3GPP TSG RAN WG1 #105-e            	   	                         R1-2104489

e-Meeting, May 19th – 27th, 2021

Source:	CATT, GOHIGH

Title:	Discussion on resource allocation for power saving

Agenda Item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

At RAN1#104b-e meeting, the following conclusion and agreements were made related to resource allocation for power saving in sidelink enhancements [1].

Conclusion: - In periodic-based partial sensing, - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2. Agreements: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc. - Note: companies are encouraged to provide more evaluations Agreement: - When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - Partial sensing configured by higher layer in the UE.

In this contribution, the resource allocation mechanisms on power saving schemes and the impact of SL DRX are discussed for sidelink enhancements.

Discussion on resource allocation for power saving

Periodic-based partial sensing based Resource Selection

In RAN1#104b-e meeting, the scheme of periodic-based partial sensing based resource selection was discussed and concluded [1]. The following agreements were achieved:

Agreements: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc. - Note: companies are encouraged to provide more evaluations

In this section, the following remaining issues are discussed:

- Remaining issues of condition(s) and timing(s)

- Determination of Y candidate slots for periodic transmission

- Determination of periodic sensing occasions for periodic transmission

    - Remaining issues of condition(s) and timing(s)

Conditions of periodic-based partial sensing are discussed. It is agreed that at least all of the followings should be met:

- Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing configured by higher layer in the UE

Besides the above, one remaining issue should be discussed, which is the condition for periodic-based partial sensing when considering pre-emption.

For periodic-based partial sensing, there is a difference between LTE-V2X and NR-V2X. Re-evaluation and pre-emption are not defined in LTE-V2X but specified in Release 16 NR-V2X. So periodic-based partial sensing mechanism design should consider re-evaluation and pre-emption for the duration that a TB arrives but resource (re)selection is not triggered.

For re-evaluation, it is not performed for resources that have been signaled in current period or previous periods. As a result, there is no need to perform partial sensing for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered for the purpose of re-evaluation.

Observation 1: Considering the purpose of re-evaluation, there is no need to perform periodic-based partial sensing for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

For pre-emption, it is performed for selected and signaled resource(s). This principle should be kept consistently with full sensing mechanism. That means, if pre-emption is enabled in the current resource pool, for the arriving packet within the non-resource-selection period, periodic-based partial sensing should be performed as illustrated in Figure 1(a). While pre-emption is disabled, there is no need to perform periodic-based partial sensing as illustrated in Figure 1(b).

(a) Periodic-based partial sensing when pre-emption is enabled

(b) Periodic-based partial sensing when pre-emption is disabled

Figure. 1 Periodic-based partial sensing considering pre-emption

Observation 2: Considering the purpose of pre-emption, whether periodic-based partial sensing should be performed or not depends on pre-emption enable or disable.

Proposal 1: For packet transmission in the duration that a TB arrives but resource (re)selection is not triggered, periodic-based partial sensing should be performed based on the following rules considering pre-emption:

- When sl-PreemptionEnable-r16 is provided, periodic-based partial sensing should be performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

- When sl-PreemptionEnable-r16 is not provided, periodic-based partial sensing should not be performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

    - Determination of Y candidate slots for periodic transmission

In RAN1 #104-e meeting, the following agreements were achieved [2]:

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Large intersections in bustling downtown areas, roads around large stadiums, in such scenarios, even if the packets sent by each P-UE are small, may cause congestion due to the large number of concurrent P-UE. Therefore, it is necessary to configure the CBR for P-UE to control channel congestion.

In R16 NR V2X, CBR measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100•2µ slots, according to higher layer parameter timeWindowSize-CBR. For R17 P-UE, the definition of CBR in R16 maybe need to be updated to adapt to the partial sensing mechanism, because partial sensing may not be able to provide enough slots for the CBR measurement window.

Higher layer will configure a two-dimensional table which is configured transmission parameters mapped by the CBR of the resource pool and the priority of transmission. Eight priorities of transmission are defined, and CBR measurements can be mapped to 16 CBR quantization intervals. When UE sends the packets, according to the priority of its own transmission and the measured CBR value, it chooses the corresponding parameters from the two-dimensional table, and achieves the purpose of congestion control.

Part of the configuration of the transmission parameters of congestion control is the control of the physical transmission parameters, including maximum transmit power, range on number of retransmissions per TB, range of PSSCH sub-channel number and range of MCS, as shown in Figure 2. The other parameter, CR_limit, is used to determine whether the current transfer resource can be sent. If the CR_limit is exceeded the threshold, the current transfer needs to be dropped.

Figure 2: Principle of Congestion Control Mechanism

In partial sensing of LTE-V2X, the minimum number of candidate subframes Ymin  is (pre-)configured per resource pool according to the parameter minnumCandidateSF-R14. The exact value of Y is selected by UE itself with the restriction of minimum number of Ymin.

In case of congested scenario, the configured minimum number of candidate slots Ymin could be adjusted based on the conditions of channel congestion or priority, etc. As illustrated in Figure 3, when the system channel congestion is below the pre-configured threshold, a smaller value of Ymin can be configured, which can reduce the partial sensing duration and power consumption of UE; when the system channel congestion is above the pre-configured threshold, a larger value of Ymin can be configured, which can improve the reliability of transmission with the cost of performing more sensing.

Figure 3: Configurable minimum candidate slots based on channel congestion

Proposal 2: With the consideration of congestion control of UE with battery constraint, the minimum candidate slots for partial sensing could be configured based on CBR value.

If a resource pool (pre-)configured with at least partial sensing, and UE performs periodic-based partial sensing, CBR in slot n can be defined as the proportion of the number of sub-channels whose SL-RSSI exceeds the (pre-)configuration threshold in the total number of sub-channels in the resource pool with a CBR measurement window. The CBR measurement window could be M partial sensing occasions or periodic sensing occasions before slot n.

The corresponding bits of the M partial sensing occasions or periodic sensing occasions are configured by bitmap as that in the high-level configuration parameter gapCandidateSensing for partial sensing or periodic sensing.

Figure 4: Configurable minimum candidate slots based on channel congestion

Proposal 3: If UE performs periodic-based partial sensing, CBR in slot n can be measured by UE in M periodic partial sensing occasions before slot n, M periodic partial sensing occasions could be a subset of the configured partial sensing occasions.

    - Determination of periodic sensing occasions for periodic transmission

In RAN1#104b-e meeting, the following alternatives of the set of Preserve values are discussed for down selecting [1].

- For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values

It was agreed that the partial sensing scheme is (pre-)configured based on per resource pool, and the determination of periodic sensing occasion is also discussed. For the design of Preserve, several alternatives were discussed as candidates. In LTE-V, the parameter Pstep for periodic partial sensing is set to one value, i.e. 100ms. It is because the potential supported V2X traffic periodicities are all multiple of 100ms, which are [100, 200, …, 1000ms], and few of them are supported in the same resource pool. However, in NR V2X, up to 16 different periodicities from [1, 99] and [0, 100, 200, …, 1000] can be supported for periodic traffic. Furthermore, aperiodic traffics also co-exist in the same resource pool. If only one value of Preserve is used for partial sensing, some of the resource occupation and reservation may not be sufficiently detected by partial sensing scheme, which leads to resource collision and performance degradation.

For Alt.1, partial sensing can be able to cover all the supported periodic resource reservation information when Preserve corresponds to all the periodicity values. When the configured periodicity values in a resource pool are few, e.g. 4 different periodicities, Alt.1 can have a good periodic partial sensing expectation. However, there would be too frequent sensing occasions when the number of periodicity values is large, e.g. 16 periodicities. P-UE performing partial sensing has to perform sensing/receptions with very short or none interval, which has obvious loss of power saving gain. Therefore, Alt.1 can have better performance on both partial sensing and power saving only when the number of configured periodicities in a resource pool is few.

In Alt.2, Preserve is a set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList, and the principle on how to determine the set should be considered. For example, the configured periodicity values are integer multiple, a greatest common divisor or a least common multiple can be used as Preserve for partial sensing. When the periodicities are multiple but the least common multiple is quite small, e.g. sl-ResourceReservePeriodList of (2, 4, 8, 100, 200, 400), let Preserve equal 2 is too restricted. For this kind of case, the configured set can be divided into several sub-groups, e.g. (2, 4, 8) and (100, 200, 400), 2 and 100 can be selected as the two values of Preserve for partial sensing. For Alt.2, how to determine the set can be done by (pre-)configuration and/or UE. UE determination on the set is preferred, because it allows fully flexibility to UE. Alt.1 is one of the cases of Alt.2 when the set of Preserve is the full set.

With the analysis above, Alt.2 is preferred by UE determination on the set of configured periodicity set. When there are multiple relations among the periodicities, UE can select a/few common divisor(s) as Preserve. When there is no or only some of the periodicities have such multiple relations among the periodicities, it is up to UE to determine whether to keep those non-multiple values in the set.

Proposal 4: Preserve corresponds to a set of values from the configured set sl-ResourceReservePeriodList, and it is up to UE to determine the set.

Whether to support multiple sets of Preserve values is under discussion. One set of Preserve values can be determined based on one metrics, such as greatest common divisor or least common multiple. Even when multiple metrics are taken into account to determine Preserve values, one set still can be used. It is not necessary to support multiple sets, because all the reservation periodicity values are supported in this resource pool, and UE can select proper values to construct the set.

Proposal 5: One set of Preserve values is supported based on the determined metric(s).

It was discussed that whether the number of Preserve values in the set should be restricted based on some conditions. Based on the current mode 2 mechanism, the supported periodicities in a resource pool is up to 16 different values. The set of values can be determined by UE implementation to save power in partial sensing procedure. There is not strong motivation to have extra conditions to restrict the number of values in the set of Preserve since the total number is no more than 16.

Proposal 6: It is not necessary to restrict the set of values.

In RAN1#104b-e meeting, the following alternatives of the k value are discussed for down selecting [1].

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

For the convenience of discussion, the options as in RAN1#104-e are also listed as follows [2].

- k equals tois selected according to (down select to one) - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction - Option 3: All possible sensing occasions after n –T0 - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured. - Option 5: k is (pre-)configured, including multiple values - Option 6: (pre-)configuration of a bitmap, same as in LTE-V - Option 7: FFS others.

The main difference is whether to support more than one recent sensing occasions based on the given reservation periodicity. In the current specification, SCI can only indicate resource reservation for the current TB and the next TB.   SCIs in two or more than two recent occasions cannot indicate resource reservation for current resource selection window. Therefore, even if two or more than two most recent occasions are performed sensing, no performance gain can be obtained.

Figure 5: Simulation results for one most recent sensing occasion and two most recent sensing occasions

As illustrated in Figure 5, there are two reservation periodicity values of 100ms for V2V traffic and 1000ms for P2V traffic. For power saving UE, bitmap of 1000000001 means supporting one most recent sensing occasions for 100ms periodicity and 1000ms periodicity while 1100000001 means supporting two most recent sensing occasions for 100ms periodicity and one occasion for 1000ms periodicity. Based on the simulation results, no performance gain can be observed when two sensing occasions are supported.

Observation 3: No performance gain can be observed when two sensing occasions are supported.

Proposal 7: Only the most recent sensing occasion for a given reservation periodicity.

An additional issue was discussed in RAN1 #104b-e meeting, considering how to define the reference time to determine the periodic-based partial sensing occasions.

- FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

Figure 6. potential collision caused by shorter periodicity reservation within (n, ty0)

As illustrated in Figure 6, if time distance between n and ty0 is larger, transmissions performed within (n, ty0) and reserved with the shorter periodicity value would potentially cause collision. In order to achieve collision avoidance as much as possible, partial sensing should be performed within (n, ty0) subject to processing time restriction. However, the resource selection triggering time is specified as time n in full sensing RA mechanism, it should also be followed in periodic-based partial sensing RA mechanism, considering not introducing significant modifications. Resource (re-)selection can be triggered in slot n and transmission resource(s) can be selected. An additional resource exclusion check can be performed before ty0 using all the periodic-based partial sensing results. The detected unavailable resource(s) after checking should be excluded.

Proposal 8:

- Sensing occasions should be determined based on the reference time of the first slot of the set of Y candidate slots subject to processing time restriction.

- Resource (re-)selection triggering time should follow the current mechanism as slot n.

- An additional selected resource(s) checking using all the sensing results should be performed before the first slot of the set of Y candidate slots subject to processing time restriction.

  - The detected unavailable pre-selected resource(s) after checking should be excluded.

Contiguous partial sensing based Resource Selection

The following agreements were achieved in RAN1#104-e [2], which are related to the aspects of contiguous partial sensing based resource selection:

Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+ TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

In this section, the following issues are discussed:

- Available sensing results

- Determination of contiguous partial sensing window

- Determination of resource selection window

- Resource allocation scheme dynamic traffic

- Contiguous partial sensing and SL DRX

The main use case for contiguous partial sensing is for aperiodic traffic type. In LTE V2X, partial sensing mechanism is designed considering only for periodic traffic. However, in NR V2X, aperiodic resource reservation is introduced to support aperiodic traffic. Consequently, the aperiodic resource allocation from neighbor UEs just before the selection window may not be detected in advance by the sensing UE according to the LTE partial sensing mechanism. The agreement in RAN1#104-e is meant to solve this issue by introducing a short period of sensing window.

As can be seen this use case is highly related to the traffic type the resource pool is intended for. If the targeted traffic type of the resource pool is only for periodic traffic, then perhaps there is no need for configuration of this contiguous short term sensing. Therefore, the resource pool should have the corresponding indication. The supported configuration should include periodic partial sensing, contiguous partial sensing and random selection.

Proposal 9: Periodic partial sensing, contiguous partial sensing and random selection are configured per resource pool.

    - Available sensing results

Contiguous partial sensing should be performed when an aperiodic packet arrives. It is discussed that all available sensing results can be used for resource selection. Besides contiguous partial sensing triggered for an arrived packet, the following sensing results should be supported as a part of available sensing results (if applicable):

- Periodic-based partial sensing results

- Sensing results from DRX_on

- Contiguous partial sensing results for another packet transmission

Observation 4: Besides contiguous partial sensing triggered for an arrived packet, the following sensing results should be supported as available sensing results (if applicable):

- Periodic-based partial sensing results

- Sensing results from DRX_on

- Contiguous partial sensing results for another packet transmission

    - Determination of contiguous partial sensing window

In RAN1#104-e meeting, it is concluded that for the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB]. However, in order to determine TA and TB, the minimum contiguous sensing duration should be defined. It can be (pre-)configured considering at least priority and latency requirements. Considering resource reservation indication range in SCI, contiguous partial sensing before resource selection should not exceed 31 logical slots.

Proposal 10: The minimum contiguous sensing duration which is no more than 31 logical slots should be defined and (pre-)configured at least considering priority and latency requirements.

Considering aforementioned available sensing results, when an aperiodic packet arrives, there would be three cases as illustrated in Figure 7.

| (a) |  |
| --- | --- |
| (b) | (c) |

Figure 7: Different cases for available sensing results when an aperiodic packet arrives

For case (a), when an aperiodic packet arrives, UE has no other available sensing results. UE should be triggered to perform sensing in the next slot. When sensing duration is meet the requirement based (pre-)configured contiguous sensing duration (marked by L, in slot), UE can perform resource selection in case (a). In this situation, TA = 1 and n + TB = n + L.

For case (b), when packet arrives, UE have available sensing results more than L. UE can perform resource selection at time n. In this situation, TB =0 and resource selection time is n + TB = n. However, it is no need to determine where TA is.

For case (c), when packet arrives, UE have available sensing results but sensed duration is less than L. UE should keep performing sensing and select resource when sensing duration is equal to L. In this situation, n+ TB = n + L – (|TA |+1). TA depends on the actual sensing starting time, and |TA |+1 means that the UE has performed a contiguous sensing until slot n.

Based on the above analysis, it can be observed that TA depends on the sensing start time but cannot affect resource selection operation. And TB should be defined to determine resource selection triggering time for contiguous partial sensing scheme. If there is no sensing results or no sufficient available sensing results, resource selection triggering time is n+ TB which can be denoted as n + L – M, where M is  (|TA|+1) and L > M. While if there are sufficient available sensing results when packet arrives, resource selection triggering time is n. In this situation, L is equal to or less than M.

Additionally, one thing should be clarified. Resource selection triggering time should be defined for contiguous partial sensing based RA, but not the candidate resource set reporting time. Considering in Rel-16 there is no specification about UE reporting time for resource selection, it is suggested to keep the same rule in Rel-17 both partial sensing and contiguous sensing. The timing of reporting a subset of resources to higher layer is left to UE implementation.

Observation 5: TA depends on the actual start time of available sensing results and cannot affect resource selection operation.

Observation 6: TB is the factor to determine resource selection time.

Proposal 11: The timing of reporting a subset of resources to higher layer is left to UE implementation.

Proposal 12: Define n + TB as the resource selection time for contiguous partial sensing based resource selection.

- If there are sufficient available sensing results when packet arrives, resource selection time is n where TB is equal to 0.

- If there is no sufficient available sensing results when packet arrives, resource selection time is n + L– M, where L is the (pre-)configured contiguous partial sensing (minimum) duration and M = |TA| + 1.

  - TA depends on the actual sensing starting time, and |TA |+1 means that the UE has performed a contiguous sensing until slot n.

  - If there is no available sensing results, contiguous partial starts at the next logical slot, i.e. TA = 1.

Proposal 13: Introduce a higher layer parameter to indicate the minimum contiguous partial sensing duration before resource selection.

For periodic traffic, contiguous sensing could also be triggered. There are multiple scenarios where it is beneficial to trigger contiguous sensing for periodic traffic, for example, as has been discussed in section 2.1, when there are insufficient sensing results from periodic partial sensing, contiguous sensing could be triggered.

Proposal 14: Contiguous sensing could also be triggered for periodic traffic.

  - FFS triggering conditions if needed.

    - Determination of resource selection window

T1 and T2 are defined to determine resource window. For contiguous partial sensing scheme, when resource selection start time is determined as n + TB, resource selection window is [n + TB + T1, n + TB + T2]. T1 is related to resource selection start time and PSCCH/PSSCH transmission preparation time and do not need to be modified. Because contiguous partial sensing is one kind of short term sensing, a new restriction should be introduced for T2. As illustrated in Figure 8, because resource reservation in SCI should be indicated within 32 logical slots, if T2 is more than 32 logical slots, no sensing results are useful for indicating reserved resources. Therefore, only candidate resources in the earlier 32 logical slots would be excluded but the later ones cannot be excluded. Then transmission resource would be selected within the non-sensed candidate resource with higher probability, i.e. these resources are nearly randomly selected.

Figure 8: Introducing a new restriction for T2

Another issue for contiguous partial sensing is re-evaluation and pre-emption. Triggering condition of pre-emption depends on the service priority of power saving UE. However, allocation of service priority is not in RAN scope. So triggering condition of pre-emption for power saving UEs is same as that of full sensing UEs.  For re-evaluation, it is introduced for aperiodic traffic to ensure the reliability and it should also be supported.

On the other side, for power saving UE, in order to achieve power saving gain, there should be some enhancements for re-evaluation and pre-emption. In current specification, the end of checking window is re-evaluation/pre-emption checking time + T2, which should not exceed remaining PDB. For power saving UE, if there is no restriction for checking window, UE may perform re-evaluation/pre-emption until of the PDB, it may lead to extra power consumption. And if the checking window is same as the initial resource selection window of n + TB, both power saving gain and reliability gain can be obtained.

Observation 7: In order to avoid selecting no sensed resources, T2 should not exceed 32 logical slots.

Observation 8: In order to achieve power saving gain, when performing re-evaluation/pre-emption after contiguous partial sensing based resource selection, the end of checking window should be fixed to n + TB + T2.

Furthermore, n + TB is the resource selection time but not the ending time of contiguous partial sensing. Considering performing re-evaluation/pre-emption,  ending time of contiguous partial sensing should be the same as ending time of re-evaluation checking or pre-emption checking, which can be following rules of re-evaluation and pre-emption.

Observation 9: The ending time of contiguous partial sensing should be the same as ending time of re-evaluation/pre-emption checking (i.e. m-T3) which follows the rules of re-evaluation/pre-emption in R16.

    - Resource allocation schemes for aperiodic traffic

As discussed in the past meetings, for aperiodic traffic, there are four candidate resource allocation schemes as follows:

- Full sensing based resource selection

- Contiguous partial sensing based resource selection with re-evaluation

  - T2 is 60 logical slots and re-evaluation checking window is not restricted

  - T2 is 32 logical slots and re-evaluation checking window does not exceed n + TB  + T2

- Random resource selection with re-evaluation and/or pre-emption

- Random resource selection without any sensing

Simulation is performed to evaluate PRR performance and power consumption. For contiguous partial sensing based resource selection, both solutions without enhancement and with enhancement for resource selection window and re-evaluation checking window are evaluated. Simulation parameters are illustrated in Annex A. Simulation results for PRR performance are illustrated in Figure 9 and for power consumption are illustrated in Figure 10.

Figure 9 PRR performances of different resource allocation schemes for aperiodic traffic

Figure 10: Power consumption CDF of different resource allocation schemes for aperiodic traffic

Table 1: Power consumption of different resource allocation schemes for aperiodic traffic

| Resource allocation schemes | Power consumption @ 99% CDF |
| --- | --- |
| Full sensing | 584700 |
| Contiguous partial sensing with T2=60 slots | 102400 |
| Contiguous partial sensing with T2=32 slots and re-evaluation not exceeding n + TB + T2 | 72570 |
| Random resource selection with re-evaluation and/or pre-emption | 60830 |
| Random resource selection without any sensing | 37550 |

Based on the simulation results, it can be observed as follows:

- Full sensing based RA scheme has the best PRR performance

- RA scheme of random resource selection without sensing has the worst PRR performance

  - Comparing random selection without sensing:

    - No obvious PRR performance gain can be observed for random selection with re-evaluation but power consumption is increased.

- Contiguous partial sensing based RA can obtain PRR performance gain

  - Comparing contiguous partial sensing based RA without enhancement:

    - PRR performance gain can be observed for enhanced scheme with T2 = 32 logical slots and performing re-evaluation not exceed n + TB + T2.

    - PRR performance of enhanced scheme is similar with that of full sensing based RA.

    - Power saving gain of enhanced scheme can be observed.

- Significant power saving gain can be observed of all the four power saving RA schemes.

Based on simulation results and analysis, there is no need to support random selection with re-evaluation. Comparing with random selection without any sensing, there is no gain but extra power consumption. Enhanced contiguous partial sensing RA can be supported because it can obtain the similar PRR performance with full sensing based RA and achieve almost 30% power saving gain with non-enhanced contiguous partial sensing based RA.

Proposal 15: Support the following two resource allocation schemes for power saving:

- Random resource selection without any sensing.

- Contiguous partial sensing based RA scheme with T2 not exceeding 32 logical slots and re-evaluation/pre-emption checking window not later than n + TB + T2.

Proposal 16: Random selection with re-evaluation is not supported.

    - Contiguous partial sensing and SL DRX

Another potential use case for contiguous partial sensing is when SL DRX is configured and the UE’s periodic partial sensing is affected by the DRX configuration. DRX active time is a semi-static period that is determined based on (pre-)configuration and inter-UE negotiation, which means it cannot dynamically change and may not always perfectly match with the sensing/partial sensing window. If Tx UE is restricted and can perform sensing only during DRX-on duration, the performance of the partial sensing operation may be degraded. As will be discussed later, it is proposed that sensing operation is allowed in Tx UE’s SL DRX inactive duration without the restriction of its own SL DRX settings, at least there should be such configuration.

In fact, the interaction between DRX and sensing configuration should be carefully considered for the trade-off between power saving and sensing performance. The case that sensing is not restricted by the DRX configuration can be viewed as a configuration most favorable for sensing performance, while the case that sensing is disabled during DRX inactive duration can be viewed as a configuration most favorable for DRX power saving. For the latter scenario, mitigation approach could be beneficial. Short term contiguous sensing could be one of the candidate solutions. The UE could use the extra sensing results from the short term contiguous sensing after the DRX inactive duration to reduce the collision probability of resource selection. For more flexible trade-off, reduced sensing configurations could be provisioned for DRX inactive duration and enabled with SL DRX.

Proposal 17: Contiguous sensing in the DRX ON duration is beneficial to reduce the collision probability of resource selection caused by muted sensing activity during DRX inactive duration.

Proposal 18: Reduced sensing configuration(s) could be provisioned for DRX inactive duration and enabled with SL DRX.

Random resource selection

The following agreement was made in RAN1#104-e [2] related to random resource selection:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection

The remaining issue is how to define the triggering/operating conditions for random selection scheme.

There are several aspects should be considered as follows:

- Conditions for random resource selection

For a UE, whether to use random selection for resource selection can firstly consider the hardware capability of the UE, when the hardware capability of the UE does not support PSCCH reception operation, the UE should use random resource selection. For the UE with the capability of PSCCH reception or resource sensing, the power saving requirement can be considered as a condition for random resource selection. When the UE has a strict requirement on power saving, or the UE’s power is very low, or the UE enters into power saving mode for other reasons, random resource selection can be allowed to choose the sidelink resources. As for the UE without strict requirement on power saving, the channel congestion can be another factor, for example, when the value of CBR is low, the UE can be allowed to do random resource selection to reduce the latency.  Finally, whether or not the UE can use random selection also depends on the resource pool configuration.

Another potential solution is to use (pre-)configured minimum contiguous sensing duration which is discussed in section 2.2.2. If UE determines the minimum contiguous sensing duration is 0 based on CBR, it means random resource selection scheme can be allowed. This condition can be independently and co-exist with aforementioned conditions.

Proposal 19: The UE capability, requirement on power saving, resource pool configuration, congestion condition (as indicated by CBR etc.) and (pre-)configured minimum contiguous partial sensing duration can be the criteria  for random resource selection.

It is important to consider the criteria for random resource selection with other considerations such as coexistence of different power saving resource selection schemes. As a result, the decision for conditions of random resource selection should be further investigated.

Proposal 20: Conditions of random selection scheme should be further investigated after schemes of periodic-based partial sensing and contiguous partial sensing are determined, considering the following aspects:

- Coexistence of different resource selection schemes.

- Congestion level

- (Pre-)configured minimum contiguous partial sensing duration

  - When determined value of pre-configured minimum contiguous partial sensing duration is 0, UE can be allowed to select random resource selection scheme.

- Periodically collision due to periodic transmission with random resource selection

The agreement above from RAN1#104-e support random resource selection applying to periodic transmission, one possible scenario shows in the Figure 11 is that a UE may collide with other UE periodically due to the random resource selection without sensing. If the UE do not have the capability of PSCCH or PSFCH reception, it can do nothing about the periodically collision. For the UE with reception or sensing capability, for instance, the type B UE which is able to receive PSFCH, could recognize whether the periodically collision happened according to the feedback information, and reduce the collision occasions by reselecting the resources.

Figure 11: Periodically collision due to periodic transmission with random resource selection

Proposal 21: The UE with reception capability of PSFCH can reselect the resource according to the HARQ feedback information to reduce periodically collision occasions.

Coexistence of different RA mechanisms in same resource pool

The following agreements were achieved in RAN1#103-e [3], which is related to configuration of Tx resource pool for sidelink mode 2:

Agreements: - In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

In LTE-V2X, resource selection mechanism of P-UE is resource pool specific configured by IE SL-P2X-ResourceSelectionConfig-r14. This IE can configure at least one of the resource selection mechanisms from partial sensing and random selection. From specification perspective, any combinations of configuration of full sensing, partial sensing and random selection for a resource pool have been supported in Rel-14 LTE-V2X.

In NR Sidelink enhancement, coexistence of full sensing, periodic-based partial sensing, contiguous partial sensing and random resource selection should be considered.

In our company's contribution R1-2100351[4] in RAN1 #104-e meeting, coexistence of full sensing, periodic-based partial sensing and random resource selection were discussed and evaluated. It can be observed in R1-2100351[4] that there is no significant performance loss for coexistence of periodic-based partial sensing and full sensing and coexistence of random resource selection and full sensing. However, because the details of periodic-based partial sensing and the schemes of contiguous partial sensing are not concluded, coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied.

Proposal 22: Coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied after schemes of periodic-based partial sensing and contiguous partial sensing are determined.

Discussion on the impact of SL DRX

Discontinuous reception (DRX) is a common power saving mechanism in Uu interface. When DRX mechanism is enabled, UE monitors PDCCH during the DRX active-time and switches to sleep mode during the DRX inactive-time. Considering the power saving requirements of some sidelink UEs, such as pedestrians, e-bikes, e-motorcycles and other devices with battery constraint, the WID [5] for R17 sidelink enhancement decided to introduce SL DRX.

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

Although DRX is mainly discussed by RAN2, RAN1 still needs to consider the potential impact of the introduction of SL DRX on sensing and resource selection, which was captured in the revised WID[6].

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

According to the revised WID, the following issues regarding the impact of SL DRX are discussed in this section:

- The impact of SL DRX on partial sensing

- The impact of SL DRX on resource selection

The impact of SL DRX on partial sensing

In RAN2 #112-e meeting, the following working assumption on sidelink DRX has been made. An LS on SL DRX design was sent to RAN1 [7], and RAN1 was asked to provide feedback on the consideration of the working assumption.

Working assumption: - SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

From the perspective of RAN1, if the motivation of this working assumption is to restrict Tx UE to performing sensing only during DRX active-time, it may degrade the performance of the partial sensing operation in mode-2. With such working assumption, Tx UE should either give up the sensing occasions which fall into the DRX inactive-time or configure the DRX cycle to be exactly aligned with the minimum configured resource reservation periods for partial sensing. However, the former may cause a loss of resource selection performance due to insufficient sensing results, and the latter may increase the DRX active-time, leading to increased power consumption. The details can be found in our contribution [8]. We propose that sensing operation is allowed in Tx UE’s SL DRX inactive time without the restriction of its own SL DRX configuration.

Proposal 23: Reply LS to RAN2.

- From RAN1’s perspective, sensing operation is allowed in SL DRX inactive time without the restriction of its own SL DRX configuration. And RAN1 respectfully asks RAN2 to take the information into account.

In R17 NR Sidelink design, the requirements of applications such as V2X and D2D, including public safety and commercial use cases, need to be considered. SL DRX is a way to save the power consumption in these applications, and is especially beneficial for V2P and D2D/P2P scenarios where the receivers are UEs with battery constraint. Since the transmitters need to perform sensing and resource selection according to the DRX configuration, it is important to know the relevant DRX parameters in advance. In RAN2 #113-e meeting, the following agreements were achieved regarding SL DRX configurations [9]:

Agreements on high-level principles for SL DRX

1: 	For SL unicast (after SL unicast link is established), SL DRX configuration can be configured per a pair of source/destination. FFS whether SL DRX operates per direction or for both directions.

2:	For SL groupcast/broadcast, SL DRX configuration can be configured in common. FFS on granularity of SL DRX configuration.

Agreements on SL DRX configurations

1: 	For broadcast/groupcast, for out-of-coverage case, TX-UE/RX-UE obtain DRX configuration from pre-configuration.

2:	For broadcast/groupcast, for in-coverage case, RRC_IDLE/INACTIVE TX-UE/RX-UE obtain DRX configuration from SIB. It is up to network implementation how to coordinate active time between different cells.

3:	For broadcast/groupcast, for in-coverage case, for RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB. FFS on whether dedicated-RRC is also used.

4:	For unicast, for OOC scenario, the UE who sends out the DRX configuration decides on the DRX configuration. FFS on whether pre-configuration and/or the assistance information from the peer UE is also taken into account when determining the DRX configuration.

5: 	For unicast, for OOC scenario, adopt per-direction DRX configuration is as baseline. FFS on whether it is TX-centric or Rx-centric, i.e. TX UE or RX UE decides it.

From RAN2’s agreements, it can be observed that UE-specific DRX configurations can be indicated by the Tx UE or Rx UE in the unicast scenario, while in the broadcast or groupcast scenario, a UE-common or intra-group UE-common DRX configuration can be pre-configured or configured by the network.

Observation 10: In the V2P/D2D/P2P unicast scenario, UE-specific DRX configurations can be indicated by the Tx UE or Rx UE. In the V2P/D2D/P2P broadcast or groupcast scenario, a UE-common or intra-group UE-common DRX configuration can be pre-configured or configured by the network.

In RAN1 #104-e meeting, the following agreements were reached on the periodic-based partial sensing [2]:

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

According to the above agreements, there are still remaining issues regarding the relationship between SL DRX and partial sensing. For P2P communication with power saving requirements on both transmitting and receiving UE, Tx UE can perform partial sensing-based resource selection while Rx UE can enable SL-DRX, in order to reduce power consumption. In this case, it is necessary to consider the relationship between the candidate resources selected by Tx UE and the DRX active time of Rx UE. If Tx UE does not consider the DRX configuration of Rx UE when determining Y candidate slots and performing resource selection, the transmission resources selected by the Tx UE may correspond to the DRX inactive time of the Rx UE. Due to the fact that the Rx UE is in a sleep state and cannot perform data reception during the DRX inactive time, packet loss inevitably occurs, which leads to the decrease of the reliability of P2P communication.

Therefore, when Tx UE performs partial sensing-based resource selection, DRX configuration of Rx UE needs to be considered by Tx UE to ensure that the selected resources for the initial transmission and retransmission(s) can be within the DRX active time of Rx UE, so that the Rx UE can receive the corresponding packets.

Proposal 24: For P2P communication, the transmission resources selected by Tx UE need to be within the DRX active time of Rx UE.

In RAN2 #113b-e meeting, the following agreements were achieved regarding the details of SL DRX timer [10]:

Agreements on details of timer

19:	Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

20:	The value(s) of the SL HARQ RTT Timer, when explicitly configured and not determined via SCI (if agreed to do so), is determined by UE or NW implementation.

21:	For unicast, sidelink retransmission timer can be supported for at least some cases of HARQ disabled transmissions. FFS whether HARQ RTT is supported or not.

22:	For transmissions with HARQ feedback, the RX UE starts the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission.

23:	If the RX UE does not transmit PSFCH for a HARQ enabled transmission (e.g. due to UL/SL prioritization) the RX UE still starts the HARQ RTT timer in the symbol/slot following the end of PSFCH resource.

24:	For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the TX UE) the RX UE uses a configured retransmission timer.

25:	Retransmission timer can be started upon expiry of the HARQ RTT timer.

26:	The value(s) of the SL retransmission timer can be determined by UE or NW implementation.

27:	The SL active time of the RX UE includes the time in which any of its applicable sl-drx-OnDuration(s), sl-DRXInactivityTimer(s), or sl-drx-RetransmissionTimer(s) are running.

From RAN2’s latest agreements, it can be observed that the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource for a HARQ enabled transmission, regardless of whether the transmission is accompanied by HARQ feedback. And when the SL HARQ RTT timer is expired, the Rx UE can start a configured retransmission timer which is determined by UE or NW implementation.

From RAN1’s perspective, similar to the retransmission timer, a configured SL HARQ RTT timer is still required for cases where there is some uncertainty in the timing of a retransmission. Since the Rx UE is in DRX inactive time during the SL HARQ RTT timer running duration, SCI decoding cannot be performed during this period. If SL HARQ RTT timer is entirely derived from the SCI indication, some potential problems may occur for the pre-emption enabled scenarios. An example is shown in Figure 12. The Tx UE’s SCI indicates a later resource, but the Tx UE may reselect a resource before the indicated resource due to pre-emption. As the reselected resource is in the DRX inactive time (i.e. SL HARQ RTT timer running duration) of the Rx UE, this packet will be missed. Thus, for cases where the retransmission timing is uncertain, such as pre-emption enabled scenarios, configured SL HARQ RTT timer should be considered as a baseline.

Figure 12: Packet loss due to the SCI indication derived SL HARQ RTT timer

Observation 11: For a HARQ enabled transmission, the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource, regardless of whether the transmission is accompanied by HARQ feedback.

Observation 12: The Rx UE can start a configured retransmission timer when the SL HARQ RTT timer is expired.

Proposal 25: For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the Tx UE), the Rx UE uses a configured SL HARQ RTT timer.

Based on the above observations, it can be concluded that once the transmission resource and the resource pool/PSFCH configuration are determined, the running duration of the corresponding retransmission timer can be determined. And the Tx UE can take the retransmission timer running duration into consideration for determining the allowable transmission time in order to ensure that Tx UE selected resources can be within DRX active time.

Figure 13: Determining the retransmission timer running duration based on the previous transmission resource

As shown in Figure 13, Tx UE sets the Y candidate resources to partially overlap with the DRX on-duration of Rx UE, ensuring that there are candidate resources within DRX active time. Tx UE can select the initial transmission resource to be within the DRX on-duration when resource selection procedure is triggered. Then, according to the resource pool/ PSFCH configuration and the DRX configuration of the Rx UE, the first running duration of drx-RetransmissionTimer can be determined. The Tx UE can take this running duration into consideration when selecting the retransmission resource(s). After selecting the first retransmission resource, the second running duration of drx-RetransmissionTimer that should be considered when selecting the remaining retransmission resource is also determined. Since this operation is related to RAN2’s specific, RAN1 should send an LS to RAN2 to inform RAN1’s consideration of sensing and resource selection.

Proposal 26: The Tx UE can take the Rx UE’s drx-RetransmissionTimer running duration into consideration for determining the allowable transmission time, in order to ensure that the transmission resources selected by the Tx UE can be within Rx UE’s DRX active time.

The impact of SL DRX on resource selection

Different from the NR Uu interface, Sidelink UE performs autonomous resource selection according to the channel sensing results. If the resources selected by the Tx UE fail to align with the DRX on-duration of the Rx UE, as shown in Figure 14 packet loss may occur. Thus, it is necessary to consider the impact of DRX parameters on the UE's resource selection when introducing the DRX mechanism into NR Sidelink enhancement.

Figure 14: Misalignment between resource selection window and DRX on duration

Observation 13: Misalignment between resource selection window and DRX on-duration may result in packet loss.

Proposal 27: The DRX parameters of the Rx UE need to be considered by Tx UE when the Tx UE performs resource selection.

In the case that the Tx UE has already got the DRX configuration of Rx UE(s) by means of such as network configuration or pre-configuration, the Tx UE can performs resource selection based on the DRX parameters of the Rx UE to avoid the misalignment between resource selection window and DRX on-duration. The following two schemes of selection window configuration may have benefits for the packet transmission. The first scheme is to have the resource selection window fully aligned with DRX on-duration of the Rx UE, that is, all the transmissions of a TB must be performed during the DRX on-duration. With the same DRX configuration, this scheme can maximize the power saving effect of the Rx UE, but it can also cause channel congestion to a certain extent. To decrease the channel congestion, another scheme is to ensure that at least the initial transmission can be made during DRX on-duration. In this case, for the reception of additional retransmission, Rx UE(s) can start sl-drx-InactivityTimer or sl-drx-RetransmissionTimer for the unicast and groupcast scenarios, or wake up at specific slots as indicated by the initial SCI for pre-emption disabled scenarios. But the above operations may also cause additional power consumption.

In the following, the system-level evaluation results of baseline and two schemes of resource selection and DRX are provided:

Baseline: Tx UE performs sensing and resource selection without any DRX-related restrictions, while Rx UE is DRX disabled.

Scheme 1: All the transmissions of a TB, including initial transmission and retransmissions, must be made during DRX on-duration of the Rx UE.

Scheme 2: Only the initial transmission of a TB must be made during DRX on-duration of the Rx UE.

- The examples of the above baseline and two schemes are illustrated in the Figure 15.

- Baseline: DRX disable

- Scheme 1: All the transmissions are made during DRX on-duration

- Scheme 2: Only the initial transmission is made during DRX on-duration

Figure 15: The baseline and two schemes of resource selection and DRX

The following comparative analysis of the above baseline and two schemes is provided with the system-level simulation results in the city broadcast scenarios, where 50% vehicle UEs transmit V2P periodic packets and the other 50% transmit V2V periodic packets as background interference.  The calculation of PRR and power consumption are according to TR 37.885 [11]and the agreements of RAN1 [12]. The detailed system-level simulation assumptions are summarized in Annex B.

Figure 16: The PRR of power saving UE reception for V2P broadcast services in 60km/h urban scenario

Figure 17: The power consumption of power saving UE reception for V2P broadcast services in 60km/h urban scenario

As shown in Figure 16, better PRR performance can be achieved when Rx UE is DRX disabled and Tx UE performs resource selection without DRX-related restrictions. For scheme 1 and scheme 2, DRX cycle is configured as 100ms, while the DRX on-duration in each DRX cycle is configured as 20ms or 15ms and starts at slot 0, i.e. DRX slot offset is 0. It is obviously that restricting the resource selection window to be fully aligned with DRX on-duration of the Rx UE will significantly decrease PRR due to channel congestion. Comparing to scheme 1, scheme 2 only restricts the initial transmission of a TB must be made during DRX on-duration, thus alleviating the channel congestion and improving the PRR performance by nearly 5%.

It can be noted that PRR performance of scheme 2 is also better than that of the baseline when the distance between Tx UE and Rx UE is longer than 150m. This phenomenon can be explained as that due to the restriction of scheme 2 that the initial transmission needs to be made during the DRX on-duration; the interference during the DRX off-duration is relaxed compared with that of the baseline where the interference is evenly distributed. For UEs that are near to the Tx UE, resource sensing is effective. The Tx UE can avoid resource collision based on the sensing result. For UEs that are relative far away from the Tx UE, the resource selection mechanism is close to random selection since the RSRP threshold has been raised to a relatively high value. In this case, the performance is directly related to the interference in the resource pool. Therefore, when there is a relatively long distance between the Tx UE and the Rx UE, the performance of scheme 2 will be better than the baseline.

Figure 17 shows the CDF diagram of the power consumption of the abovementioned baseline and two schemes of resource selection and DRX. Compared with the baseline, scheme 1 and scheme 2 which use the SL DRX mechanism achieved a power saving gain of up to 80%. For scheme 1, the power consumption of all PUEs is the same because all PUEs have the same wake-up time, i.e. during DRX on-duration. For scheme 2, since the PUE has additional wake-up time in addition to the DRX on-duration, the power consumption of all PUEs is greater than that of scheme 1 under the same DRX configuration. When the DRX on-duration of scheme 2 is configured shorter than that of scheme 1, such as 15ms, the average power consumption of PUE can be reduced to 3.9, which is significantly lower than that of scheme 1.

As can be seen from the above simulation results, scheme 2 can improve the power saving performance of PUEs by shortening the DRX on-duration, while reducing the loss of PRR performance. Thus, when the Rx UE is DRX enabled, ensuring that at least the initial transmission of a TB can be made during DRX on-duration of the Rx UE can achieve a trade-off between PRR and power consumption.

Observation 14: Comparing with scheme that the resource selection window for all the transmissions are restricted to be fully aligned with DRX on-duration of the Rx UE, the scheme that only initial transmission is transmitted during the DRX on-duration of Rx UE can archive better PRR performance.

Proposal 28: When the Rx UE is DRX enabled, Tx UE should ensure that at least the initial transmission can be made during DRX on-duration of the Rx UE to achieve a trade-off between Packet Reception Ratio (PRR) and power consumption.

Conclusion

In this contribution, issues and enhancements of resource allocation for power saving and SL DRX are discussed. Observations and proposals are given as follows:

Observation 1: Considering the purpose of re-evaluation, there is no need to perform periodic-based partial sensing  for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

Observation 2: Considering the purpose of pre-emption, whether periodic-based partial sensing should be performed or not depends on pre-emption enable or disable.

Proposal 1: For packet transmission in the duration that a TB arrives but resource (re)selection is not triggered, periodic-based partial sensing should be performed based on the following rules considering pre-emption:

- When sl-PreemptionEnable-r16 is provided, periodic-based partial sensing should be performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

- When sl-PreemptionEnable-r16 is not provided, periodic-based partial sensing should not be performed for packet transmission in the duration that a TB arrives but resource (re)selection is not triggered.

Proposal 2: With the consideration of congestion control of UE with battery constraint, the minimum candidate slots for partial sensing could be configured based on CBR value.

Proposal 3: If UE performs periodic-based partial sensing, CBR in slot n can be measured by UE in M periodic partial sensing occasions before slot n, M periodic partial sensing occasions could be a subset of the configured partial sensing occasions.

Proposal 4: Preserve corresponds to a set of values from the configured set sl-ResourceReservePeriodList, and it is up to UE to determine the set.

Proposal 5: One set of Preserve values is supported based on the determined metric(s).

Proposal 6: It is not necessary to restrict the set of values.

Observation 3: No performance gain can be observed when two sensing occasions are supported.

Proposal 7: Only the most recent sensing occasion for a given reservation periodicity.

Proposal 8:

- Sensing occasions should be determined based on the reference time of the first slot of the set of Y candidate slots subject to processing time restriction.

- Resource triggering time should follow the current mechanism as slot n.

- An additional selected resource(s) checking using all the sensing results should be performed before the first slot of the set of Y candidate slots subject to processing time restriction.

  - The detected unavailable pre-selected resource(s) after checking should be excluded.

Proposal 9: Periodic partial sensing, contiguous partial sensing and random selection are configured per resource pool.

Observation 4: Besides contiguous partial sensing triggered for an arrived packet, the following sensing results should be supported as available sensing results (if applicable):

- Periodic-based partial sensing results

- Sensing results from DRX_on

- Contiguous partial sensing results for another packet transmission

Proposal 10: The minimum contiguous sensing duration which is no more than 31 logical slots should be defined and (pre-)configured at least considering priority and latency requirements.

Observation 5: TA depends on the actual start time of available sensing results and cannot affect resource selection operation.

Observation 6: TB is the factor to determine resource selection time.

Proposal 11: The timing of reporting a subset of resources to higher layer is left to UE implementation.

Proposal 12: Define n + TB as the resource selection time for contiguous partial sensing based resource selection.

- If there are sufficient available sensing results when packet arrives, resource selection time is n where TB is equal to 0.

- If there is no sufficient available sensing results when packet arrives, resource selection time is n + L– M, where L is the (pre-)configured contiguous partial sensing (minimum) duration and M = |TA| + 1.

  - TA depends on the actual sensing starting time, and |TA |+1 means that the UE has performed a contiguous sensing until slot n.

  - If there is no available sensing results, contiguous partial starts at the next logical slot, i.e. TA = 1.

Proposal 13: Introduce a higher layer parameter to indicate the minimum contiguous partial sensing duration before resource selection.

Proposal 14: Contiguous sensing could also be triggered for periodic traffic.

  - FFS triggering conditions if needed.

Observation 7: In order to avoid selecting no sensed resources, T2 should not exceed 32 logical slots.

Observation 8: In order to achieve power saving gain, when performing re-evaluation of pre-emption after contiguous partial sensing based resource selection, the end of checking window should be fixed to n + TB + T2.

Observation 9: The ending time of contiguous partial sensing should be the same as the ending time of re-evaluation/pre-emption checking (i.e. m-T3) which follows the rules of re-evaluation/pre-emption in R16.

Proposal 15: Support the following two resource allocation schemes for power saving:

- Random resource selection without any sensing.

- Contiguous partial sensing based RA scheme with T2 not exceeding 32 logical slots and re-evaluation/pre-emption checking window not later than n + TB + T2.

Proposal 16: Random selection with re-evaluation is not supported.

Proposal 17: Contiguous sensing in the DRX ON duration is beneficial to reduce the collision probability of resource selection caused by muted sensing activity during DRX inactive duration.

Proposal 18: Reduced sensing configuration(s) could be provisioned for DRX inactive duration and enabled with SL DRX.

Proposal 19: The UE capability, requirement on power saving, resource pool configuration, congestion condition ( as indicated by CBR etc. ) and (pre-)configured minimum contiguous partial sensing duration can be the criteria  for random resource selection.

Proposal 20: Conditions of random selection scheme should be further investigated after schemes of periodic-based partial sensing and contiguous partial sensing are determined, considering the following aspects:

- Coexistence of different resource selection schemes.

- Congestion level

- (Pre-)configured minimum contiguous partial sensing duration

  - When determined value of pre-configured minimum contiguous partial sensing duration is 0, UE can be allowed to select random resource selection scheme.

Proposal 21: The UE with reception capability of PSFCH can reselect the resource according to the HARQ feedback information to reduce periodically collision occasions.

Proposal 22: Coexistence of periodic-based partial sensing, contiguous partial sensing and random resource selection should be further studied after schemes of periodic-based partial sensing and contiguous partial sensing are determined.

Proposal 23: Reply LS to RAN2.

- From RAN1’s perspective, sensing operation is allowed in SL DRX inactive time without the restriction of its own SL DRX configuration. And RAN1 respectfully asks RAN2 to take the information into account.

Observation 10: In the V2P/D2D/P2P unicast scenario, UE-specific DRX configurations can be indicated by the Tx UE or Rx UE. In the V2P/D2D/P2P broadcast or groupcast scenario, a UE-common or intra-group UE-common DRX configuration can be pre-configured or configured by the network.

Proposal 24: For P2P communication, the candidate resources selected by Tx UE need to be within the DRX active time of Rx UE.

Observation 11: For a HARQ enabled transmission, the Rx UE will start the SL HARQ RTT timer in the symbol/slot following the end of PSFCH transmission/resource, regardless of whether the transmission is accompanied by HARQ feedback.

Observation 12: The Rx UE can start a configured retransmission timer when the SL HARQ RTT timer is expired.

Proposal 25: For cases where there is some uncertainty in the timing of a retransmission for a HARQ process (e.g. due to no retransmission resource indicated in the SCI, or possible reselection by the Tx UE), the Rx UE uses a configured SL HARQ RTT timer.

Proposal 26: The Tx UE can take the Rx UE’s drx-RetransmissionTimer running duration into consideration for determining the allowable transmission time, in order to ensure that the transmission resources selected by the Tx UE can be within Rx UE’s DRX active time.

Observation 13: Misalignment between resource selection window and DRX on-duration may result in packet loss.

Proposal 27: The DRX parameters of the Rx UE need to be considered by Tx UE when the Tx UE performs resource selection.

Observation 14: Comparing with scheme that the resource selection window for all the transmissions are restricted to be fully aligned with DRX on-duration of the Rx UE, the scheme that only initial transmission is transmitted during the DRX on-duration of Rx UE can archive better PRR performance.

Proposal 28: When the Rx UE is DRX enabled, Tx UE should ensure that at least the initial transmission can be made during DRX on-duration of the Rx UE to achieve a trade-off between Packet Reception Ratio (PRR) and power consumption.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #104b-e Chairman's Notes, April. 2021.

  - 3GPP TSG RAN WG1 Meeting RAN1#104-e Chairman's Notes, Feb. 2021.

  - 3GPP TSG RAN WG1 Meeting RAN1 #103-e Chairman's Notes, Novemeber. 2020.

  - R1-2100351, “Discussion on resource allocation for power saving”, CATT, GOHIGH, RAN1 #104-e meeting.

  - RP-193257, “New WID on NR sidelink enhancement”, LG Electronics, RAN #86-E meeting.

  - RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, RAN #90-E meeting.

  - R1-2100021, “LS to RAN1 on SL DRX design”, RAN2.

  - R1-2104465, “Discussion on LS from RAN2 on SL DRX design”,  CATT, GOHIGH, RAN1 #105-e meeting.

  - 3GPP TSG RAN WG2 Meeting RAN2 #113-e Chairman's Notes, Febrary. 2021.

  - 3GPP TSG RAN WG2 Meeting RAN2 #113b-e Chairman's Notes, April. 2021.

  - 3GPP TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR; (Release 15)”, V15.3.0.

  - 3GPP TSG RAN WG1 Meeting RAN1 #102-e Chairman's Notes, Aug. 2020.

  - 3GPP TR 36.885, “Study on LTE-based V2X Services; (Release 14)”, V14.0.0.

  - 3GPP TSG RAN WG1 Meeting RAN1#104b-e Chairman's Notes, Apr. 2021.

- Annex A: System Level Evaluation Assumptions for SL power saving

| Parameter | Value |
| --- | --- |
| Deployment scenario | City scenario with Vehicles and Pedestrians - Vehicle UEs - Vehicle type distribution: 100% vehicle type 2. - Vehicle speed is 60 km/h in all the lanes. - Pedestrians are generated in sidewalk as defined in TR36.885[13] |
| Channel model | Sidelink: Urban-LOS & Urban NLOS[11] |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth:40 MHz |
| Subcarrier spacing | 30 kHz |
| Traffic model | - P2V - Periodical traffic Option 1: Traffic model for P-UE's transmission specified in TS 36.885 - The message size is fixed at 300 bytes and transmission frequency is 1 Hz - ‘100ms’ latency requirement - Aperiodic traffic Option 4: Aperiodic Model 1 specified in TR37.885 with following changes: - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms - Packet size: Uniformly random in the range between 200 bytes and 800 bytes with the quantization step of 200 bytes - Latency requirement: 250 ms or 100 ms - V2V - 50% Vehicle UEs transmit periodic packets. Model 2 (medium traffic intensity)[11] - Inter-packet arrival time: 100 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms - 50% Vehicle UEs transmit aperiodic packets. Model 1 (medium traffic intensity)[11] - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Packet size: Uniformly random in the range between 400 bytes and 2000 bytes with the quantization step of 400 bytes - Latency requirement: 50 ms |
| SCI/Data frequency resource allocation and Tx parameters | - PSCCH: 10 PRB, 3 OS, QPSK - PSSCH: - 20 PRB for 300 Bytes P2V periodic packets, 16QAM - 10 PRB per 200Bytes P2V aperiodic packets, 16QAM - 30 PRB for 800 Bytes and 1200 Bytes V2V periodic packets, 16QAM - 10 PRB per 400 Bytes for V2V aperiodic packets, 16QAM |
| TX number per TB | - 3 |
| Channel coding | PSCCH: Polar code PSSCH: LDPC |
| Antenna configuration | (Tx, Rx) = (2, 4) |
| Power saving mechanism | - Power saving UE(Pedestrian UE) - Periodic traffic - Periodically partial sensing with performing re-evaluation - Aperiodic traffic - Full sensing with performing re-evaluation - Contiguous partial sensing(31 logical slots before resource selection) with performing re-evaluation - T2 is 60 logical slots and re-evaluation checking window is not restricted - T2 is 32 logical slots and re-evaluation checking window does not exceed n + TB + T2 - Random selection with performing re-evaluation - Random selection without sensing and Re-evaluation - Vehicle UE - Full sensing with performing Re-evaluation |
| Performance metric | - PRR - Power consumption |

- Annex B: System Level Evaluation Assumptions for SL DRX

| Parameter | Value |  |
| --- | --- | --- |
| Deployment scenario | City: Option A scenario[8] - Vehicle speed = 60 km/h |  |
| Channel model | Sidelink: Urban-LOS & Urban NLOS[11] |  |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth:20 MHz |  |
| Subcarrier spacing | 30 kHz |  |
| Traffic model | V2V | - 50% Vehicle UEs transmit packets - Periodic: Model 2 (medium traffic intensity)[11] - Inter-packet arrival time: 100 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms |
|  | V2P | - 50% Vehicle UEs transmit packets - Option 7: Periodic Model 2 specified in TR 37.885 with following change:[11] - Inter-packet arrival time: 500ms - Latency requirement: 100 ms |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB, 3 OS - PSSCH: 30 PRB for packet size of 800 and 1200 Bytes |  |
| Data Packet Tx parameters | Periodic variable packet size evaluations: - 800 Byte packet: 16-QAM, 1 TTI (CR = 0.444) - 1200 Byte packet: 16-QAM, 1 TTI (CR = 0.667) |  |
| Channel coding | PSCCH: Polar code PSSCH: LDPC |  |
| Antenna configuration | (Tx, Rx) = (2, 4) |  |
| Transmission number per TB | - 3 |  |
| DRX configuration | - Power saving UE (Pedestrian UE) - DRX cycle: 100ms - DRX slot offset: 0 - DRX on-duration: - Scheme 1: 20ms - Scheme 2: 15ms or 20ms - All pedestrians use the same DRX configuration - Vehicle UE - DRX disabled - V2P UEs were informed of PUE’s DRX configuration in advance |  |
| Performance metric | - PRR - Power consumption |  |