3GPP TSG RAN WG1 #105-e		R1- 2104496

e-Meeting, May 19th – 27th, 2021

Source:                CATT

Title:	Discussion on multi-cell PDSCH scheduling via a single DCI

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

At RAN1#104 e-meeting, the discussion on multi-cell PDSCH scheduling via a single DCI focused on the simulation results in different scenarios. The following agreements were achieved:

Agreement - The proposal in section 2.6 of R1-2102138 for PDCCH blocking probability is taken as RAN1 observation - The proposal in section 2.6 of R1-2102138 for PDSCH throughput is taken as RAN1 observation with the following revision for the note: - Note: Combinations 1 and 2 were agreed for evaluation. Some companies provided evaluation results for Combinations 3 and 4. Agreement The observations for multi-cell PDSCH scheduling via a single DCI to be summarized in the status report along with explantion on different combinations that were considered for submission to RAN.

In this contribution, we provide our views on multi-cell PDSCH scheduling via a single DCI.

Discussion

Benefits of multi-cell PDSCH scheduling via a single DCI

DSS provides a useful migration from LTE to NR by allowing LTE and NR to share the same carrier. One of the PDCCH enhancements proposed for R17-DSS is multi-cell PDSCH scheduling via a single DCI. Currently, one DCI can only schedule a single serving cell. Multi-cell scheduling requires multiple PDCCHs which increase the overhead of PDCCH from system perspective. As the number of NR devices increases in a network, the resource consumption of PDCCH would be a huge burden for the scheduling cell, no matter the scheduling cell is a sharing cell or a normal cell. Multi-cell PDSCH scheduling via a single DCI provide a useful tool to make DSS more feasible and powerful. According to the simulation results provided during RAN1#104 e-meeting, at least the following advantages can be achieved by supporting multi-cell PDSCH scheduling via a single DCI:

- Reduction on CCE consumption: The ratio of CCE saving is about 20%~40% for a DSS DCI with different payload size in range of 72~108 bits [1]-[5].

- Reduction on PDCCH blocking probability: As the total number of PDCCHs is reduced, the blockage issue can be alleviated. The gain of PDCCH blocking rate reduction is increased as the growing ratio of DSS UE in a serving cell [1][2]. Detailed results on PDCCH capacity for a DSS DCI with different payload size can be found in our companion contribution [6]. The system performance can be improved accordingly.

- Improvement on PDSCH throughput: More PDCCHs can be transmitted in the same CORESET and more users can be paired together in terms of MU-MIMO [7]. It is possible to share some of the DCI fields between two carriers without throughput loss [4][5] and even up to 8%~10% throughput gain can be achieved [2].

- Reduction on UE power consumption: UE power consumption can be reduced since UE only needs to monitor the DCI in a single search space on the scheduling cell instead of two scheduling cells. Up to 6.67%~15% power consumption can be saved according to the simulation [2].

It can be seen that the benefits from multi-cell PDSCH scheduling via single DCI have been simulated and analyzed from many aspects, as shown above, at least including CCE consumption, PDCCH blocking possibility, PDSCH throughput and power consumption. Hence, multi-cell PDSCH scheduling via single DCI should be supported and specified. Given the channel condition and scheduling requirements on different serving cells are quite different, two separate TBs should be transmitted on two serving cells.

Proposal 1: Multi-cell PDSCH scheduling via a single DCI should be supported considering it can bring significant benefits in terms of PDCCH capacity, PDSCH throughput and UE power saving.

Proposal 2:  Two TBs should be scheduled separately on different serving cells for multi-cell PDSCH scheduling via a single DCI.

If multi-cell PDSCH scheduling via a single DCI is supported, the enhancement for the scheduling DCI is needed. As the current DCI format can only support single serving cell scheduling, at least the following aspects are needed to be further studied:

DCI content

The benefits of scheduling multi-cells via a single DCI come from the PDCCH overhead reduction and less CRC attachment. PDCCH overhead reduction can be achieved via sharing some bit fields for the scheduled cells. In the extreme case, each bit field carried by the DCI can be shared between two scheduled PDSCHs, which is the upper bound of bonus. However, the scheduling flexibility may be a problem as the channel condition is typically different and the same DL assignment cannot be comfortable for both serving cells. In other words, there is always a trade-off between the benefits and the loss of flexibility.

If the same information of a bit field would be indicated to the two scheduled PDSCH, the bit field can be shared to save the DCI overhead. For example, identifier for DCI formats can always be shared because both TBs are downlink transmissions. Further, DCI bit fields such as TPC command for scheduled PUCCH, PUCCH resource indicator, PDSCH-to-HARQ feedback timing indicator can be shared if the HARQ-ACK feedback for the two scheduled PDSCH would be transmitted in the same codebook.

On the other hand, if the information corresponding to the two scheduled PDSCHs is different, the bit field should be indicated separately. The concrete number of bits for each bit field can be further discussed. The simplest way is to double the bit field. For example, modulation and coding scheme, new data indicator and redundancy version for each transport block can be doubled to achieve the maximum flexibility, which also increases the payload of the DCI. On the contrary, half of the bit size can be indicated for each scheduled PDSCH to maintain the total payload size. For example, the lower two bits of TDRA field are used to indicate the time domain resource for the first PDSCH and the higher two bits of TDRA field are used to indicate the time domain resource for the second PDSCH. In this time, only four rows in a TDRA table can be chosen for a PDSCH and the scheduling flexibility in time domain is greatly jeopardized. Hence, a trade-off between overhead and flexibility can be considered for some bit field, such as TDRA field with 6bits.

Observation 1: The size of the bit field used for separate indication can be further discussed to achieve trade-off between overhead and flexibility.

The full flexibility should not be pursued in DSS scenario as the ultimate aim is to make sure the network can work well. In Table 1, we provide some preliminary DSS-DCI design with different payload sizes provided in the simulation assumption. It should be noted that the following table only shows the possible shared bit fields and separate bit fields. The details on how to further compress the bit width and how to realize the functionality with the compact bit fields can be further studied. The block with highlight background means the corresponding bit field can be shared. D1 has fully shared bit fields and hence can obtain the most benefits. D5 has the least shared bit fields and therefore achieve the best flexibility.

Proposal 3: The design of the compact DSS-DCI should be further studied.

Table 1: exemplary DSS DCI design assuming different payload size

| DCI format 1_1 | DSS DCI format design(bit) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | D1: 60 | D2: 72 | D3: 84 | D4: 96 | D5:108 |
| Identifier for DCI formats | 1 |  |  |  |  |
| Carrier indicator | 3 bits |  |  |  |  |
| Bandwidth part indicator | 0 |  |  |  |  |
| Frequency domain resource allocation | 13 bits |  |  |  |  |
| TDRA | 4 |  |  |  |  |
| VRB-to-PRB mapping | 0 |  |  |  |  |
| PRB bundling size indicator | 0 |  |  |  |  |
| Rate matching indicator | 0 |  |  |  |  |
| ZP CSI-RS trigger | 1 |  |  |  |  |
| MCS for TB1 | 5 |  |  |  |  |
| NDI for TB1 | 1 |  |  |  |  |
| RV for TB1 | 2 |  |  |  |  |
| MCS for TB2 | 5 |  |  |  |  |
| NDI for TB2 | 1 |  |  |  |  |
| RV for TB2 | 2 |  |  |  |  |
| HARQ process number | 4 |  |  |  |  |
| Downlink assignment index | 2 |  |  |  |  |
| TPC command for scheduled PUCCH | 2 |  |  |  |  |
| PUCCH resource indicator | 3 |  |  |  |  |
| PDSCH-to-HARQ_feedback timing indicator | 3 |  |  |  |  |
| Antenna port(s) | 4 |  |  |  |  |
| Transmission configuration indication | 3 |  |  |  |  |
| SRS request | 0 |  |  |  |  |
| CBG transmission information | 0 |  |  |  |  |
| CBG flushing out information | 0 |  |  |  |  |
| DMRS sequence initialization | 1 |  |  |  |  |

DCI budget

According to the WID, the total PDCCH blind decoding budget should not be changed. For the purpose of flexibility, UE should support to monitor both legacy DCI and DSS-DCI in a serving cell simultaneously. In order to avoid changing DCI budget due to DSS-DCI, the following alternatives can be further considered once the DSS-DCI has a different payload compared to the current DCI formats:

- Alt.1: Zero-padding for legacy DCI. The payload size of legacy DCI with zero-padding would be equal to the DSS-DCI. However, it brings in the performance loss of legacy DCI due to the redundant zero bits.

- Alt.2: Restricted configuration for DSS-DCI. gNB should guarantee the payload size of DSS-DCI equals to that of a reference legacy DCI. Non-fallback DCI with large payload size is preferred to be the reference legacy DCI.

- Alt.3: Restricted DCI format configuration. gNB should guarantee the DCI budget is not challenged by DCI format configuration, e.g. DSS-DCI and DCI format 0_2/1_2 should not be configured simultaneously.

There is a tradeoff between the scheduling flexibility and transmission performance for each alternative. However, it is more flexible and friendly for DSS operation if gNB can make a decision on scheduling data transmission based on legacy DCI format or DSS-DCI format without any restriction. Hence alt.1 and alt.2 are slightly preferred. For alt.1 and alt.2, UE may need to differentiate whether a DCI is a DSS-DCI or not as a same payload size for both DSS-DCI and legacy DCI is pursued. Consequently, the solution to differentiate the DCI formats would be necessary.

Proposal 4: How to maintain the DCI budget due to the introduction of DSS-DCI needs further discussion.

Target cells scheduled by DSS DCI

If a DCI is used for DSS, the target cell needs to be identified. In our view, the DSS DCI could schedule different combinations of target cells, including itself or not should be supported.

Figure 1: Illustration on different target cell combinations scheduled by a single DCI

Proposal 5: It can be supported that all of the target cells scheduled by a DSS DCI are different from the scheduling cell.

The HARQ-ACK feedback procedure may also need to be further studied accordingly, e.g. the SCS and scheduling/feedback timing may be different for the different scheduled cells. We also provide some tentative insights below from our side:

- For type1 HARQ-ACK codebook, current mechanism can be directly reused if two separate PDSCHs are scheduled on different cells respectively.

- Design of C-DAI and T-DAI in one DCI for counting multiple PDSCHs scheduled by one DCI should be considered.

- HARQ-ACK timing needs to be further considered as the scheduling timing and feedback timing may be both different on the two scheduled cells.

Proposal 6:  The HARQ feedback procedure need to be further studied.

Conclusion

This contribution discusses the relevant issues associated with multi-cell PDSCH scheduling. We have the following observation and proposals:

Observation 1: The size of the bit field used for separate indication can be further discussed to achieve trade-off between overhead and flexibility.

Proposal 1: Multi-cell PDSCH scheduling via a single DCI should be supported considering it can bring significant benefits in terms of PDCCH capacity, PDSCH throughput and UE power saving.

Proposal 2:  Two TBs should be scheduled separately on different serving cells for multi-cell PDSCH scheduling via a single DCI.

Proposal 3: The design of the compact DSS-DCI should be further studied.

Proposal 4: How to maintain the DCI budget due to the introduction of DSS-DCI needs further discussion.

Proposal 5: It can be supported that all of the target cells scheduled by a DSS DCI are different from the scheduling cell.

Proposal 6:  The HARQ feedback procedure need to be further studied.

References

  - R1-2100359, “Discussion on multi-cell PDSCH scheduling via a single DCI”, CATT, RAN1#104e.

  - R1-2100194, “Discussion on multi-carrier scheduling using single PDCCH”, HW, RAN1#104e.

  - R1-2100678, “On 2-cell scheduling via single DCI”, Intel, RAN1#104e.

  - R1-2100474, “Discussion on joint scheduling”, vivo, RAN1#104e.

  - R1-2100611, “On Multi-cell PDSCH Scheduling via Single DCI”, MediaTek, RAN1#104e.

  - R1-2104498, “Simulation results for multi-cell PDSCH scheduling via a single DCI”, CATT, RAN1#104e.

  - RP-210660, “On DSS WID objective of single DCI for scheduling PDSCH on 2 cells”, HW, RAN#91.