3GPP TSG RAN WG1 #105-e		                                                      R1-2104515

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	Intra-UE multiplexing and prioritization

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

At the RAN1#104b-e meeting, intra-UE multiplexing and prioritization were discussed and the following agreements were reached [1].

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

In this contribution, we give our further considerations for intra-UE multiplexing and prioritization based on previous agreements.

Multiplexing of PUCCH and PUCCH/PUSCH with different priorities

Time unit for multiplexing

In Rel-16, time unit of HARQ-ACK transmission may be different for different priorities and multiplexing is performed only among channels with same priority. However, for multiplexing of different priorities in Rel-17, it is not clear how to determine the multiplexing time unit. It was agreed that PUCCH resource in the second PUCCH-Config is used for multiplexing at least in case the total number of LP and HP HARQ-ACK bits is more than 2. We prefer to apply the same rule for other cases, that is to multiplex UCIs with different priorities on a PUCCH resource with high priority, hence the time unit of high priority PUCCH should be used as the time unit for multiplexing. It was agreed to support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met. Then, for a low priority PUCCH which goes across multiple time units of high priority PUCCH, it is not clear that the low priority PUCCH belongs to which time unit for multiplexing.

In order to reuse the pseudo code for multiplexing in Rel-16 as much as possible, the following option can be considered. The low priority PUCCH joins the multiplexing procedure in each of the overlapping time units for multiplexing from the first overlapping slot/sub-slot, unless the low priority PUCCH was determined to be dropped or multiplexed with other channels. As an example shown in Figure 1, multiplexing is performed in the first HP sub-slot followed by the second HP sub-slot. In the first HP sub-slot, HP PUCCH-1, HP PUCCH-2 and LP PUCCH-1 are included in Q set, the multiplexing pseudo code of Rel-16 is reused. It is determined that UCI of HP PUCCH-2 is multiplexed on HP PUCCH-1 so that UE has TDMed HP PUCCH-1 and LP PUCCH-1 after the multiplexing procedure of the first HP sub-slot. Then in the second HP sub-slot, LP PUCCH and HP PUCCH-3 are included in Q set, UE determine that UCIs of LP PUCCH-1 is multiplexed on HP PUCCH-3, only HP PUCCH-3 can be transmitted in the second HP sub-slot.

Figure 1: LP PUCCH-1 join the multiplexing procedure of both HP sub-slots

As another example shown in Figure 2, in the first HP sub-slot, HP PUCCH-1, HP PUCCH-2 and LP PUCCH-1 are included in Q set. It is determined that UCIs of HP PUCCH-2 and LP PUCCH-1 are multiplexed on HP PUCCH-1 so that UE has only HP PUCCH-1 after the multiplexing procedure of the first HP sub-slot. Then in the second HP sub-slot, LP PUCCH-1 is not considered in multiplexing any more, and only HP PUCCH-3 is transmitted.

Figure 2: LP PUCCH-1 join the multiplexing procedure of first HP sub-slot only

Proposal 1: The time unit of high priority PUCCH is used as the time unit for multiplexing.

Proposal 2: For a low priority PUCCH which goes across multiple time units for multiplexing, the low priority PUCCH joins the multiplexing procedure in each of the overlapping time units for multiplexing from the first overlapping time unit, unless the low priority PUCCH is determined to be dropped or multiplexed with other channels.

Detailed multiplexing schemes for UCI with different priorities

Multiplexing of 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK

For multiplexing a 1 bit HP HARQ-ACK and a 1 bit LP HARQ-ACK into a PUCCH, it was agreed to further discuss how to multiplex on a PUCCH resource with PUCCH format 0 or PUCCH format 1. In this case, 1 bit HP and 1 bit LP HARQ-ACK can be treated as two HP HARQ-ACK bits and transmitted on the PUCCH resource configured for HP HARQ-ACK.

Proposal 3: For multiplexing of HP HARQ-ACK and LP HARQ-ACK when total number of bits is 2, 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK are treated as two HP HARQ-ACK bits and transmitted on the time-frequency resource for the HP HARQ-ACK transmission.

Multiplexing of HP SR and LP HARQ-ACK

Several options were proposed for the case of HP SR with PF0 overlaps with LP HARQ-ACK with PF0.

 Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.3: No enhancement over Rel-16.

For option 1, HP SR resource is used for multiplexing, this ensures that the PUCCH resource after the multiplexing would not go across multiple time units of multiplexing, which could reduce the complexity of multiplexing.

For option 2 and 3, LP HARQ-ACK resource would be used for multiplexing, which would make the multiplexing more complicated as shown in Figure 3. If a positive HP SR is multiplexed on a LP HARQ-ACK resource, but the PUCCH resource for low priority HARQ-ACK overlaps with a HP HARQ-ACK in another sub-slot, then the three channels should be multiplexed together which makes the multiplexing more complicated.

Figure 3: HP HARQ-ACK/SR is impacted if HP SR is multiplexed on PUCCH resource for LP HARQ-ACK

Similar as the case of HP SR with PF0 overlapping with LP HARQ-ACK with PF0, for the case of HP SR with PF0 overlapping with LP HARQ-ACK with PF1 or for HP SR with PF1 overlapping with LP HARQ-ACK with PF0, we prefer to multiplex positive SR and HARQ-ACK on SR resource. For negative SR, LP HARQ-ACK can be transmitted on PUCCH resource for LP HARQ-ACK. Then, a unified solution can be applied for the cases of overlapping between HP SR and LP HARQ-ACK with PF0/1.

Proposal 4: For multiplexing of HP SR and LP HARQ-ACK with PF0/1,

- positive SR and HARQ-ACK are multiplexed on the SR resource;

- for negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

For the case of HP SR overlapping with LP HARQ-ACK with PF2/3/4, HARQ-ACK cannot be multiplex on HP SR resource without compression. In order to reduce the complexity, we prefer to drop LP HARQ-ACK for the positive SR, and transmit LP HARQ-ACK on PUCCH resource for LP HARQ-ACK for negative SR.

Proposal 5: For multiplexing of HP SR and LP HARQ-ACK with PF2/3/4,

- for positive SR, drop LP HARQ-ACK;

- for negative SR, transmit only HARQ-ACK on the HARQ-ACK resource.

Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR

If there is a HP SR to be transmitted together with the HP HARQ-ACK and LP HARQ-ACK, the following options can be considered:

- Option 1: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 2/3/4 for HP HARQ-ACK

- Option 2: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 0/1 for HP HARQ-ACK

- Option 3: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to an additional configured PUCCH resource for multiplexing of UCI with different priorities.

Option 1 is the most intuitive and simple scheme. Separate coding cannot bring much benefit since there is only 1 bit LP HARQ-ACK, and coding of 1 bit UCI is not supported for UCI transmission on PUCCH. Joint coding can be used in this case to reduce the specification impact.

For option 2, the multiplexing scheme for 2 bits HP HARQ-ACK and 1 bit HP SR can be reused by treating 1 bit LP HARQ-ACK as 1 bit HP HARQ-ACK. But an issue for this option is that there may be misunderstandings between gNB and UE if the DCI corresponding to LP HARQ-ACK is missed by the UE. For example, UE will transmit HP HARQ-ACK and positive SR using sequence cyclic shifts 3/9 when DCI corresponding to LP HARQ-ACK is missed by the UE, and gNB may consider UE receives the DCI corresponding to LP HARQ-ACK and the SR is negative.

For option 3, additional PUCCH resource should be configured for multiplexing UCIs with different priorities. UE can multiplex both HARQ-ACKs and 1 bit HP SR on the additional PUCCH resource if DCI corresponding to LP HARQ-ACK is detected by the UE, the drawback of this option is that additional PUCCH resource is needed.

Hence, option 1 and option 2 can be further discussed, and option 1 is slightly preferred.

Proposal 6: For multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bits HP SR, the following two options can be further considered:

- Option 1: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 2/3/4 for HP HARQ-ACK

- Option 2: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 0/1 for HP HARQ-ACK

Multiplexing of HP HARQ-ACK and LP HARQ-ACK when total number of bits is more than 2

If the total number of HP HARQ-ACK and LP HARQ-ACK bits is more than 2, it was agreed that a PUCCH resource for HP HARQ-ACK can be used for multiplexing and separate coding is used. The working assumption for separate coding is as follows:

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

We agree to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1 and let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. A remaining issue is that for separate coding on PUCCH, coding for 1 or 2 bits is not applied in Rel-15 since this scenario does not exist. For the case when HP HARQ-ACK or LP HARQ-ACK includes only 1 or 2 bits, it should be considered whether padding to 3 bits or reuse the coding scheme used for UCI on PUSCH. Considering that the coding scheme used for UCI on PUSCH is already supported in specification, and there is no difference of using coding for 1/2 bits on PUCCH or PUSCH, hence we think we can reuse the 1/2 bits coding scheme used for UCI on PUSCH for the case when HP HARQ-ACK or LP HARQ-ACK includes only 1 or 2 bits and multiplexed on PUCCH.

Proposal 7: For multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH when total number of bits is more than 2 and when HP HARQ-ACK or LP HARQ-ACK includes only 1 or 2 bits, reuse the 1/2 bits coding scheme used for UCI on PUSCH.

Proposal 8: For separate coding of HP HARQ-ACK and LP HARQ-ACK when multiplexing on PUCCH:

- Let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1;

- Let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Impact of missing DCIs corresponding to LP HARQ-ACK

If DCI(s) corresponding to LP HARQ-ACK codebook are missed at UE side, the transmission of the HP HARQ-ACK codebook would be impacted. The following options can be considered to resolve the issue:

- Option 1: Define a reference number of bits for LP HARQ-ACK codebook

- Option 2: Indicate whether LP HARQ-ACK is multiplexed with HP HARQ-ACK or indicate T-DAI for LP HARQ-ACK by DCI corresponding to HP HARQ-ACK

For option 1, a reference number of bits for LP HARQ-ACK codebook is determined, which can be different from the actual number of bits of LP HARQ-ACK codebook. For example, the reference number of bits for LP HARQ-ACK codebook can be predetermined or configured by RRC, or implicitly determined by the number of bits for HP HARQ-ACK codebook. UE always transmits the LP HARQ-ACK based on the reference number of bits. Additional bit(s) are appended to the LP HARQ-ACK when the original LP HARQ-ACK codebook size is smaller than the reference number of bits, and LP HARQ-ACK bits are partially dropped when the original LP HARQ-ACK codebook size is larger than the reference number of bits.

For option 2, additional DCI field can be added in DCI for HP HARQ-ACK. But it cannot be applied for semi-static HP PUCCH, this could be resolved by gNB implementation, for example, gNB can avoid scheduling a dynamic LP PUCCH overlaps with a semi-static HP PUCCH, then gNB and UE can always have the same understanding on the multiplexing result.

Proposal 9: The following two options can be considered to avoid the impact on HP HARQ-ACK(s) due to missing DCIs corresponding to LP HARQ-ACK codebook.

- Option 1: Define a reference number of bits for LP HARQ-ACK codebook

- Option 2: Indicate information for determine the number of LP HARQ-ACK bits by DCI corresponding to HP HARQ-ACK

Indication for enabling or disable multiplexing

It was agreed that a mechanism for gNB to enable/disable the multiplexing is supported. The type of mechanism to enable/disable the multiplexing should be considered.

Regarding semi-static RRC configuration, it should be supported as a baseline, not only for multiplexing between a high-priority HARQ-ACK and a low-priority HARQ-ACK, but also for all multiplexing between channels with different priorities.

Then if multiplexing between channels with different priorities is enabled, whether multiplexing is always applied can be considered. Considering that the details of multiplexing between channels with different priorities is still being discussed, there may be different conditions or mechanisms to enable/disable the multiplexing for different cases, whether dynamic indication the multiplexing between a high-priority HARQ-ACK and a low-priority HARQ-ACK can be further discussed after a detailed multiplexing rule is defined.

Proposal 10: Semi-static RRC configuration to enable/disable the multiplexing between channels with different priorities is supported.

Multiplexing of UCI and PUSCH with different priorities

For UCI multiplexing on PUSCH, it was agreed to support 0< beta-offset <1 for multiplexing LP HARQ-ACK in a HP PUSCH. To support a mechanism for gNB to enable/disable the multiplexing, beta-offset=0 can be considered. For example, when URLLC PUSCH collides with PUCCH carrying eMBB UCI and the PUSCH resource is limited, the gNB may indicate that the eMBB UCI is dropped. Compared with semi-static configuration, dynamically indicating whether UCI is transmitted on PUSCH is preferable to provide more scheduling flexibility at gNB side. For type-2 configured grant PUSCH, such dynamic indication can be considered in the activation DCI. For type-1 configured grant PUSCH, only semi-static configuration or predefined rule can be used.

Proposal 11: A value of zero for beta-offset in a DCI can be used to dynamically indicate that LP UCI is not multiplexed on the HP PUSCH scheduled by the DCI.

Furthermore, considering that the coding rate offset between UCI and data may be different for different priority combination of UCI and PUSCH, the additional configuration of beta-offset values should be introduced for UCI with different priorities multiplexing on a PUSCH. For example, four sets of beta offset values can be configured to the UE to indicate beta offset values for different cases.

In Rel-15, UL DAI in a UL DCI is used to indicate the number of HARQ-ACK bits multiplexed on a PUSCH scheduled by the DCI. To support multiplexing between PUCCH and PUSCH with different priorities, an additional UL DAI bit field can be considered to be added in the UL DCI. Then, one UL DAI bit field is used for indicating information for multiplexing between PUCCH and PUSCH with same priority and the other UL DAI bit field is used for indicating information for multiplexing PUCCH and PUSCH with different priority.

Proposal 12: For a UE supporting multiplexing between different priorities, consider enhancements to UCI multiplexed on PUSCH based on independent beta offsets for different priority combinations of UCI and PUSCH.

Proposal 13: An additional UL DAI bit field can be considered to be added in the UL DCI for multiplexing PUCCH and PUSCH with different priorities.

For overlapping between PUCCH and multiple PUSCHs with different priorities, how to choose the PUSCH for multiplexing needs to be considered. To reduce the impact to HP PUCCH or HP PUSCH, it is preferred that UE first select a PUSCH which has the same priority with the PUCCH. If there is no PUSCH with same priority overlaps with the PUCCH, then UCI can be multiplexed to a PUSCH with different priority when supported.

Proposal 14: For overlapping between PUCCH and multiple PUSCHs with different priorities, it is proposed that UCI of PUCCH is multiplexed on a PUSCH with different priority only when there is no PUSCH with same priority overlaps with the PUCCH.

Simultaneous PUCCH/PUSCH transmission

It was agreed that simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells for inter-band CA is supported. The feature can be RRC configured within the same PUCCH group. We do not see the benefit of dynamic indication of simultaneous PUCCH/PUSCH transmission, hence it should not be supported.

Proposal 15: Dynamic indication of simultaneous PUCCH/PUSCH transmission is not supported.

For simultaneous PUCCH/PUSCH transmission of same PHY priority, it may not require much extra work to support it. And it may be beneficial for some special cases where UCI may be dropped according to current rules. For example, simultaneous SR and PUSCH can be supported, or when PUSCH resource is limited, simultaneous PUCCH and PUSCH would reduce the impact on the data performance. Hence, we support simultaneous PUCCH/PUSCH transmission of same PHY priority over different cells for inter-band CA.

Proposal 16: Simultaneous PUCCH/PUSCH transmission of same PHY priority over different cells for inter-band CA can be supported.

For simultaneous PUCCH/PUSCH transmission of intra-band CA, it was supported in LTE. In addition, we do not see much difference between simultaneous PUSCH transmissions over different cells and simultaneous PUCCH/PUSCH transmissions over different cells. Hence, we support simultaneous PUCCH/PUSCH transmission for intra-band CA.

Proposal 17: Simultaneous PUCCH/PUSCH transmission for intra-band CA can be supported.

Conclusion

In this contribution, we discuss possible enhancements for intra-UE multiplexing and prioritization and give the following observations and proposals.

Proposal 1: The time unit of high priority PUCCH is used as the time unit for multiplexing.

Proposal 2: For a low priority PUCCH which goes across multiple time units for multiplexing, the low priority PUCCH joins the multiplexing procedure in each of the overlapping time units for multiplexing from the first overlapping time unit, unless the low priority PUCCH is determined to be dropped or multiplexed with other channels.

Proposal 3: For multiplexing of HP HARQ-ACK and LP HARQ-ACK when total number of bits is 2, 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK are treated as two HP HARQ-ACK bits and transmitted on the time-frequency resource for the HP HARQ-ACK transmission.

Proposal 4: For multiplexing of HP SR and LP HARQ-ACK with PF0/1,

- positive SR and HARQ-ACK are multiplexed on the SR resource;

- for negative SR, the UE transmits only HARQ-ACK on the HARQ-ACK resource.

Proposal 5: For multiplexing of HP SR and LP HARQ-ACK with PF2/3/4,

- for positive SR, drop LP HARQ-ACK;

- for negative SR, transmit only HARQ-ACK on the HARQ-ACK resource.

Proposal 6: For multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bits HP SR, the following two options can be further considered:

- Option 1: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 2/3/4 for HP HARQ-ACK

- Option 2: Multiplexing of 1 bit HP HARQ-ACK, 1 bit LP HARQ-ACK and 1 bit HP SR to a PUCCH resource with PF 0/1 for HP HARQ-ACK

Proposal 7: For multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH when total number of bits is more than 2 and when HP HARQ-ACK or LP HARQ-ACK includes only 1 or 2 bits, reuse the 1/2 bits coding scheme used for UCI on PUSCH.

Proposal 8: For separate coding of HP HARQ-ACK and LP HARQ-ACK when multiplexing on PUCCH:

- Let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1;

- Let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Proposal 9: The following two options can be considered to avoid the impact on HP HARQ-ACK(s) due to missing DCIs corresponding to LP HARQ-ACK codebook.

- Option 1: Define a reference number of bits for LP HARQ-ACK codebook

- Option 2: Indicate information for determine the number of LP HARQ-ACK bits by DCI corresponding to HP HARQ-ACK

Proposal 10: Semi-static RRC configuration to enable/disable the multiplexing between channels with different priorities is supported.

Proposal 11: A value of zero for beta-offset in a DCI can be used to dynamically indicate that LP UCI is not multiplexed on the HP PUSCH scheduled by the DCI.

Proposal 12: For a UE supporting multiplexing between different priorities, consider enhancements to UCI multiplexed on PUSCH based on independent beta offsets for different priority combinations of UCI and PUSCH.

Proposal 13: An additional UL DAI bit field can be considered to be added in the UL DCI for multiplexing PUCCH and PUSCH with different priorities.

Proposal 14: For overlapping between PUCCH and multiple PUSCHs with different priorities, it is proposed that UCI of PUCCH is multiplexed on a PUSCH with different priority only when there is no PUSCH with same priority overlaps with the PUCCH.

Proposal 15: Dynamic indication of simultaneous PUCCH/PUSCH transmission is not supported.

Proposal 16: Simultaneous PUCCH/PUSCH transmission of same PHY priority over different cells for inter-band CA can be supported.

Proposal 17: Simultaneous PUCCH/PUSCH transmission for intra-band CA can be supported.

Reference

- Chairman's Notes RAN1#104b-e, 3GPP TSG RAN WG1 Meeting #104b-e, e-Meeting, April 12th – 20th, 2021.