3GPP TSG RAN WG1 #105-e                                              R1-2104516

e-Meeting, May 19th – May 27th, 2021

Agenda Item:	8.4.1

Source:	CATT

Title:	Timing relationship enhancement for NTN

Document for:	Discussion and Decision

Introduction

Based on NTN Chairman's Notes 104bis-e [1], there are some agreements and working assumption on timing relationship of NTN, and some FFS points are also mentioned as follows:

Agreement:

For updating K_offset after initial access, at least one of the following options is supported:

- Option 1: RRC reconfiguration

- Option 2: MAC CE

FFS: Other options

Agreement:

- For determination of cell-specific K_offset in system information, down-select one option from below:

  - Option 1: Signal one offset value for K_offset

    - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

  - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

    - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Agreement:

Confirm the following working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

Agreement:

When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement.

Agreement:

UE can be provided by network with a K_mac value.

- When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

In this contribution, several issues related to timing relationship and enhancements are discussed. Potential problems and solutions are presented as well.

Discussion

The start of RAR window

Moderator recommendation on Issue #9: Companies are encouraged to propose a refined formulation on how to offset the start of RAR window using the below formulation as a starting point: - The start of ra-ResponseWindow and msgB-ResponseWindow are compensated by UE-gNB RTT. - If downlink and uplink frame timing are not aligned at gNB, feeder link RTT is signaled to UE. - FFS signaling details - Note: If downlink and uplink frame timing are aligned at gNB, there is no need to signal feeder link RTT. Instead, UE can determine the start of RAR window based on downlink timing.

Based on the discussion in the last meeting, when downlink and uplink frame timing are aligned at gNB as shown in Figure 1a, the UE_specific TA equals to the UE_RTT. Hence, indication of the start of RAR window is not needed. While indication of the start of RAR window is needed when downlink and uplink frame timing are not aligned at gNB as shown in Figure 1b. When downlink and uplink frame timing are aligned at reference point (RP), assume that RP is located in the satellite, UE is only aware of the TA between UE and RP, but has no knowledge to the RTT between UE and gNB.

Figure 1a: The start of RAR window considering downlink and uplink frame timing aligned at gNB

Figure 1b: The start of RAR window considering downlink and uplink frame timing not aligned at gNB

For UE, UE_specific TA can be obtained based on GNSS and ephemeris by UE itself. So UE_RTT = UE_specific TA + feeder link RTT. Indicating the feeder link RTT to help UE to derive the RAR reception timing is necessary.

Proposal 1: Indicating the feeder link RTT to help UE to derive the RAR reception timing is supported.

MAC CE action timing

RAN1#103-e: Agreement: Denote by K_mac a scheduling offset other than K_offset: - If downlink and uplink frame timing are aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - If downlink and uplink frame timing are not aligned at gNB: - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed. - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed. - Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac.

As agreed in RAN1#103-e, if downlink and uplink frame timing are unaligned at gNB, K_mac is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH. As shown in Figure 2, 	the UL slot n (at UE UL) indexed by m + k1 + K_offset is the slot where UE transmits HARQ-ACK corresponding to the received PDSCH carrying the MAC CE command, and the MAC CE command is activated in the DL slot p (at UE DL) indexed by n + 3Nslotsubframe,µ + K_mac, where μ is the sub-carrier spacing configured for uplink. Herein K_mac is the offset equaled to RP-gNB RTT, where K_mac = feeder link RTT - the common TA. Consider that the reference point (RP) is set on the satellite, where K_mac = feeder link RTT. Considering that the K_mac value is related to the feeder link RTT, it is necessary to indicate the feeder link RTT.

Figure 2: The MAC CE action timing considering downlink and uplink frame timing unaligned at gNB

The common TA is defined as the common component of propagation delay shared by all UEs within the cell coverage and corresponds to the Round Trip Delay (RTD) between the RP and the satellite. According to Chairman's Notes RAN1#104bis-e 8.4 [1], the common TA can be zero for the case when the RP is set on gNB. Hence, UE can derive K_mac based on the value of the common TA and the feeder link RTT.

Proposal 2: K_mac can be derived by the common TA and feeder link RTT.

Updating K_offset after initial access

In last meeting, it has been agreed that updating K_offset after initial access can use RRC/MAC CE/Other options. But on how to update this parameter, it should have careful consideration. We think two requirements should be followed: 1) gNB can’t frequently update the K_offset; 2) UE assistant information should be minimized.

The main purpose of updating K_offset after initial access is to reduce the scheduling delay. Regardless of whether the K_offset is updated through beam-specific or UE-specific, frequently updating is not desired. In case of signaling design, RRC configuration for K-offset can be supported.

Regarding the TA reporting, the overhead should be controllable. Though periodical TA reporting is beneficial for timely TA tracking, however, in most of cases, for example, one larger K_offset can be used for a long period, which can cover small TA variation. In this case, it’s suitable to set one threshold to trigger the aperiodic TA reporting. For example, when TA’s change exceeds one threshold, UE will report one new TA to gNB. Then gNB can adjust the K_offset accordingly.

In addition, in order to minimize reporting overhead, it’s desirable that UE only reports TA associated time range. The time range can be pre-configured, which can be mapped to one TA range and expressed by a few bits. For example, ‘00’ denotes [-10ms, -5ms], ‘01’ denotes [-5ms, 0], ‘10’ denotes [0ms, 5ms], ‘11’ denotes [5ms, 10ms].

For the signaling design of TA reporting, RRC signaling is preferred, Moreover, coarse TA range reporting can be used for signaling overhead, rather than accurate Tc level reporting.

Proposal 3: RRC signaling to indicate K_offset can be supported.

Proposal 4: One threshold is used for TA report triggering.

Proposal 5: Coarse TA range reporting with larger granularity can be supported, rather than accurate TA reporting.

On K1/K2 range extension

As agreed in RAN1#104-e meeting, for unpaired spectrum, extend the value range of K1 from (0..15) to (0..31). For FDD, it is no need to extend K1 range extension to paired spectrum. However, the DCI change should be avoided. The description of the list of timing for given PDSCH to the DL ACK in TS 38.331 [2] is “SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)”. Hence, the modification can be “SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)” to meet the requirements, where dl-DataToUL-ACK-r17 would be relevant for the increased K1 value range.

Proposal 6: Extend K1/K2 range without changing the DCI, and dynamically configure the list of K1/K2 values corresponding to the DCI size.

Conclusion

In this contribution we analyzed timing relationship issues in NTN.

We proposed:

Proposal 1: Indicating the feeder link RTT to help UE to derive the RAR reception timing is supported.

Proposal 2: K_mac can be calculated by the common TA and feeder link RTT.

Proposal 3: RRC signaling to indicate K_offset can be supported.

Proposal 4: One threshold is used for TA report triggering.

Proposal 5: Coarse TA range reporting with larger granularity can be supported, rather than accurate TA reporting.

Proposal 6: Extend K1/K2 range without changing the DCI, and dynamically configure the list of K1/K2 values corresponding to the DCI size.

References

[1] Chairman's Notes RAN1#104bis-e 8.4 eom

[2] TS 38.331.