3GPP TSG RAN WG1 Meeting #105-e	R1-2104554

e-Meeting, May 19th – 27th, 2021

Agenda item:		8.6.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Other UE Complexity Reduction Aspects

Document for:		Discussion and Decision

# Introduction

In Rel-17 WI for reduced capability devices [1], UE complexity reduction features are to be specified. Two aspects are considered in this agenda item –

- Maximum number of DL MIMO layers:

  - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

  - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

- Relaxed maximum modulation order:

  - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE.

  - No other relaxations of maximum modulation order are specified for a RedCap UE.

In RAN1#104-e, the following agreements have been made on these aspects –

Agreements:

- For relaxed maximum number of DL MIMO layers:

  - FFS: need for modification of DCI fields/formats

  - FFS: need for modification of CSI measurement/reporting

Agreements:

- The MCS tables currently defined are re-used for RedCap UEs

  - FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries)

  - FFS mandatory/optional of the MCS tables

  - Note: there is no new MCS table to be introduced for RedCap UEs

Agreements:

- The CQI tables currently defined are re-used for RedCap UEs.

  - FFS mandatory/optional of the CQI tables

  - There is no new CQI table to be introduced for RedCap UEs

In this contribution, we address issues related these two aspects of reduced complexity.

# Maximum number of DL MIMO layers

The WI states that for a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. In addiiton, for a RedCap UE with 1 Rx branch, 1 DL MIMO is supported. Thus, the number of supported DL MIMO layer is directly dependent on the number of Rx branches. During initial access and in idle mode, all downlink transmission uses a single layer, so there is no issue. In connected mode, UE capability is known and the gNB can handle each UE type appropriately. In addition, there exists capability parameter maxNumberMIMO-LayersPDSCH that can be used by the UE to report the maximum number of DL MIMO layers supported by the UE. Therefore, there is no RAN1 specification impact for RedCap UE supporting 1 or 2 downlink MIMO layer.

In RAN1#104-e, two potential optimization were noted for further study – DCI fields/formats, and CSI measurement/reporting. With respect to DCI field optimization, it has been proposed that the the size of the antenna port fields in DCI 1_1 can be reduced by at least 1 bit for UE with 1 Rx branch. This, however, is a small optimization and unlikely to improve PDCCH detection performance meaningfully. In addition, there could be a mixture of RedCap UEs with 1 or 2 Rx branches in the network and it would simplify implementation if the same DCI size is used. Therefore, it is proposed that there is no need to optimize DCI size for RedCap UE.

Proposal 1: There is no need to modify DCI fields/formats for RedCap UE supporting 1 DL MIMO layer.

The other potential optimization is from CSI measurement and reporting aspect. For instance, it has been suggested that CSI determination can be based on rank 1 only or limited to small number of CSI ports for small BWP. This is unnecessarily restriction and, in our view, unlikely to result in significant power saving. In addition, this configuration can be enabled by the gNB via implementation should it wishes to do so. Therefore, it is proposed that there is no need to optimize CSI measurement for RedCap UE.

Proposal 2: There is no need to modify CSI measurement/reporting for RedCap UE with relaxed maximum number of DL MIMO layers.

# Maximum modulation order

The issues of MCS and CQI tables have been discussed under modulation relaxation although we feel they are more related to reduced number of Rx branches. This is because the discussion was focused around whether low-SE table should be mandatory and whether it should become the default table.

MCS Tables

In RAN1#104e, it was agreed that the MCS tables currently defined are re-used for RedCap UEs. However, it was FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries). Furthermore, it was also FFS which table(s) would be mandatory. The tables are shown in Figure 2. From the coverage results shown in [2] where most results have been submitted using MSC Table 1 (i.e. Table 5.1.3.1-1 in 38.214), it is seen that MCS Table 1 is sufficient even for UE with 1 Rx branch. For instance, Figure 1 illustrates the CDF of DL SINR for UE with 1Rx at 2.6 GHz. It can be seen from the CDF that most UEs have SINR greater than 0 dB and ~90% of UEs have SINR greater than 10dB. Therefore, UEs will typically not require low-SE MCS even with 1 Rx branch and reduced antenna efficiency. For UE with 2 Rx branches, there is no need to consider the low SE tables since clearly most UEs will be in better channel conditions and can take advantage of higher SE entries in Table 1.

Figure 1. CDF of DL SINR.

Furthermore, besides Msg2, H-ARQ can be used so there should not be a coverage issue. For Msg2, TBS scaling is available in case additional redundancy is needed. In addition, during initial access, legacy UEs would only use MCS Table 1. Therefore, if MCS Table 3 is the default table for RedCap UE, it would be necessary to differentiate RedCap UE starting from Msg1. In addition, it may also be necessary to differentiate between RedCap UE with 1 or 2 Rx branches. This is against the WI objective that RedCap UE can be identified in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network.

Proposal 3: MCS Tables 5.1.3.1-1 and 6.1.4.1-1 are the default tables for RedCap UE.

Note that some RedCap use cases have higher reliability requirements (e.g. industrial sensor requires commercial service availability of 99.99% and video surveillance requires reliability of up to 99.9%). In addition, for some services it would be beneficial to use MCS Table 3 to improve coverage. Therefore, support for MCS Table 3 would be beneficial for some RedCap use cases or UEs. However, there is no need to make Table 3 support mandatory for all RedCap UEs. Therefore, it is proposed that MCS Table 3 can be optionally supported for RedCap UE.

Proposal 4: Low SE MCS Tables 5.1.3.1-3 and 6.1.4.1-2 can be optionally supported for RedCap UE.

| MCS Index IMCS; Modulation Order Qm; Target code Rate R x [1024]; Spectral efficiency. 0; 2; 120; 0.2344. 1; 2; 157; 0.3066. 2; 2; 193; 0.3770. 3; 2; 251; 0.4902. 4; 2; 308; 0.6016. 5; 2; 379; 0.7402. 6; 2; 449; 0.8770. 7; 2; 526; 1.0273. 8; 2; 602; 1.1758. 9; 2; 679; 1.3262. 10; 4; 340; 1.3281. 11; 4; 378; 1.4766. 12; 4; 434; 1.6953. 13; 4; 490; 1.9141. 14; 4; 553; 2.1602. 15; 4; 616; 2.4063. 16; 4; 658; 2.5703. 17; 6; 438; 2.5664. 18; 6; 466; 2.7305. 19; 6; 517; 3.0293. 20; 6; 567; 3.3223. 21; 6; 616; 3.6094. 22; 6; 666; 3.9023. 23; 6; 719; 4.2129. 24; 6; 772; 4.5234. 25; 6; 822; 4.8164. 26; 6; 873; 5.1152. 27; 6; 910; 5.3320. 28; 6; 948; 5.5547. 29; 2; reserved. 30; 4; reserved. 31; 6; reserved. - MCS index table 1 | MCS Index IMCS; Modulation Order Qm; Target code Rate R x [1024]; Spectral efficiency. 0; 2; 30; 0.0586. 1; 2; 40; 0.0781. 2; 2; 50; 0.0977. 3; 2; 64; 0.1250. 4; 2; 78; 0.1523. 5; 2; 99; 0.1934. 6; 2; 120; 0.2344. 7; 2; 157; 0.3066. 8; 2; 193; 0.3770. 9; 2; 251; 0.4902. 10; 2; 308; 0.6016. 11; 2; 379; 0.7402. 12; 2; 449; 0.8770. 13; 2; 526; 1.0273. 14; 2; 602; 1.1758. 15; 4; 340; 1.3281. 16; 4; 378; 1.4766. 17; 4; 434; 1.6953. 18; 4; 490; 1.9141. 19; 4; 553; 2.1602. 20; 4; 616; 2.4063. 21; 6; 438; 2.5664. 22; 6; 466; 2.7305. 23; 6; 517; 3.0293. 24; 6; 567; 3.3223. 25; 6; 616; 3.6094. 26; 6; 666; 3.9023. 27; 6; 719; 4.2129. 28; 6; 772; 4.5234. 29; 2; reserved. 30; 4; reserved. 31; 6; reserved. (b) MCS index table 3 |
| --- | --- |

Figure 2. MCS tables for downlink and uplink.

CQI Tables

Similar to the agreement on MCS tables in RAN1#104-e, it was agreed that the CQI tables currently defined are re-used for RedCap UEs. Furthermore, it was also FFS which table(s) would be mandatory as this would naturally follow the decision on MCS tables. In NR, there are 3 CQI tables defined with one-to-one correspondence to the configured MCS table. Following our proposals regarding MCS tables, we make analogous proposals for CQI tables below.

Proposal 5: CQI Table 5.2.2.1-2 is the default table for RedCap UE.

Proposal 6: Low SE CQI Table 5.2.2.1-4 can be optionally supported for RedCap UE.

# Conclusions

In this contribution, we consider remaining aspects of UE complexity reduction related to the number of DL MIMO layers and relaxed modulation order and make the following proposals –

Proposal 1: There is no need to modify DCI fields/formats for RedCap UE supporting 1 DL MIMO layer.

Proposal 2: There is no need to modify CSI measurement/reporting for RedCap UE with relaxed maximum number of DL MIMO layers.

Proposal 3: MCS Tables 5.1.3.1-1 and 6.1.4.1-1 are the default tables for RedCap UE.

Proposal 4: Low SE MCS Tables 5.1.3.1-3 and 6.1.4.1-2 can be optionally supported for RedCap UE.

Proposal 5: CQI Table 5.2.2.1-2 is the default table for RedCap UE.

Proposal 6: Low SE CQI Table 5.2.2.1-4 can be optionally supported for RedCap UE.

# References

- RP-210918, “Revised WID on support of reduced capability NR devices,” Nokia, Ericsson, RAN#91-e, electronic meeting.

- TR 38.875, “Study on support of reduced capability NR devices,” v1.0.0, December 2020.

- R1-2103967, “FL summary #4 on reduced number of Rx branches for RedCap,” Moderator (Apple), RAN1#104bis-e, electronic meeting.