3GPP TSG RAN WG1 #105-e	R1-2104555

e-Meeting, May 19th – May 27th, 2021

Agenda item:		8.14.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On Traffic Model for XR study

Document for:		Discussion and Decision

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]:

### 4.1 Objective of SI or Core part WI or Testing part WI The following applications are to be considered as starting points for this study: - VR1: “Viewport dependent streaming” - VR2: “Split Rendering: Viewport rendering with Time Warp in device” - AR1: “XR Distributed Computing” - AR2: “XR Conversational” - CG: Cloud Gaming Note: Use cases in quotes are from TR26.928. The following traffic parameters for the different applications are to be considered as starting point for the study: Traffic characteristics: - UL and DL File Size distribution (e.g., Pareto with given parameters) - UL and DL File arrival time distribution (e.g., Periodic every 1/60 seconds) Traffic requirements: - Round-trip-time or UL and DL one-way Packet delay budget (PDB) - UL and DL Packet error rate (PER) The objective of this study item are as follows: - Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs Note 1: eURLLC SI/WI work relevant to XR should be taken into consideration. Note 2: Traffic model for the performance evaluation shall be based on the standardization in SA WG4

In this contribution we present our views on traffic models for various XR and CG applications.

# Traffic Models

During RAN1-103-e meeting, five different applications of interest were agreed:

- VR1: Viewport-dependent streaming.

- VR2: Split Rendering: Viewport rendering with Time Warp in device.

- AR1: XR Distributed Computing.

- AR2: XR Conversational.

- CG: Cloud Gaming.

Furthermore, during RAN1 104-e and 104b-e meetings, it was agreed to adopt a parametrized statistical traffic model for the evaluation of the above-mentioned applications and almost all important parameters were defined.

Below, we provide a summary of all the baseline and optional parameters for the baseline traffic model as well as the combinations of the streams agreed. Most of the agreed parameters are based on the input from SA4 [2].

## Summary of the parameters for the baseline traffic model

In this section, we summarize the parameters agreed for the UL and DL traffic models for VR, AR, and CG. Particularly, in Table 1, all DL traffic model parameters are introduced. Following the agreements, the DL video stream for all applications of interest is a single stream with all main parameters being finalized and agreed.

Table 1 – Parameters for the DL video stream for VR, AR, and CG.

| DL video stream | VR | AR | CG |
| --- | --- | --- | --- |
| Packet | A packet is assumed to represent multiple IP packets corresponding to a single video frame for modelling/evaluation purposes |  |  |
| Packet size | Truncated Gaussian distribution with parameters: - Mean = fps / Av. Data rate/8 [bytes] - STD: 10.5% of mean - Max size: 150% of mean - Min size: 50% of mean Other values that can be used for evaluation: [STD, Max, Min] = [4, 112, 88] % of mean for single eye buffer [STD, Max, Min] = [3, 109, 91] % of mean for dual eye buffer |  |  |
| Arrival time of packet k | k/X*1000 [ms] + J [ms], where X is the fps value and J is a random variable jitter |  |  |
| Jitter | Truncated Gaussian distribution with parameters: - mean: 0 ms - STD: 2 ms - min and max values: [-4, 4] ms – baseline - min and max values: [-5, 5] ms – optional - Other values can be optionally evaluated |  |  |
| Frame per second (fps) | 60 fps – baseline 120 fps – optional Other values, e.g., 30, 90 fps can be also optionally evaluated |  |  |
| Av. Data rate | 30, 45 Mbps @60fps - baseline 60 Mbps @60fps – optional Other values (in combination with fps) can be also optionally evaluated. |  | 8, 30 Mbps @60fps - baseline 45 Mbps @60fps – optional Other values (in combination with fps) can be also optionally evaluated. |
| Packet delay budget (PDB) | 10 ms Other values, e.g., 5ms, 20 ms can be optionally evaluated |  | 15 ms Other values, e.g., 10ms, 30ms can be optionally evaluated |

Further, in Table 2, we provide the parameters for the UL pose/control stream agreed for the VR, AR, and CG.

Table 2 – Parameters for the UL pose/control stream for VR, AR, and CG.

| UL pose/control stream | VR | CG | AR |
| --- | --- | --- | --- |
| Packet size | Fixed: 100 bytes |  |  |
| Arrival time of packet k | Periodic: 4 ms (no jitter) Other values can be optionally evaluated |  |  |
| Packet delay budget (PDB) | 10 ms |  |  |

We continue with a summary of the agreed parameters for the UL video stream for AR applications provided in Table 3.

Table 3 – Parameters for the UL video stream for AR.

| UL video stream | AR |
| --- | --- |
| Packet size | Truncated Gaussian distribution with the parameter values same as for the DL video stream (see Table 1) |
| Arrival time of packet k | k/X*1000 [ms] k/X*1000 [ms] + J [ms] – optional jitter where X is the fps value and J is a random variable jitter |
| Jitter (optional) | same model as for the DL video stream (see Table 1) |
| Frame per second (fps) | 60 fps – baseline |
| Av. Data rate | 10 Mbps @60fps – baseline 20 Mbps @60fps – optional |
| Packet delay budget (PDB) | [ffs: 60] ms – baseline [ffs: 10/15] ms – optional |

Observation 1: Table 1, 2, and 3 are the summary of the main parameters agreed for the DL and UL baseline traffic models and employed for the evaluation purposes.

## Combinations of streams in UL and DL for AR, VR, and CG

In this section, we present the combinations of different streams in UL and DL for VR, AR, CG agreed as mandatory and optional for the evaluation purposes.

In Table 4, we present the agreed baseline streams in DL and UL employed for the evaluation of the VR, AR, and CG.

Table 4 – Mandatory streams in DL and UL for VR, AR, and CG.

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Downlink | Single video stream (with parameters in Table 1) |  |  |
| Uplink | Single pose/control stream (with parameters in Table 2) |  | 1) Two streams (for power and capacity KPIs): - Stream 1: video (with parameters in Table 3) - Stream 2: pose/control (with parameters in Table 2) OR (only for capacity KPIs) 2) Single video stream (with parameters in Table 3) |

In addition to the mandatory streams as per Table 4, we further present the optional streams that can be used in evaluation of VR, AR, and CG given in Table 5.

Table 5 – Optional streams in DL and UL for VR, AR, and CG.

|  | CG | VR | AR |
| --- | --- | --- | --- |
| Downlink | Two-stream model (Optional): - Option 1: I-frame + P-frame - Option 1A: slice-based traffic model - Option 1B: Group-Of-Picture (GOP) based traffic model - Option 2: video + audio/data - Option 3: FOV + omnidirectional stream NOTE: Companies to report detailed parameters NOTE: Companies strive to unify their parameters |  |  |
| Uplink |  |  | Option 1 (Optional): Three streams as defined below - Stream 1: pose/control - Stream 2: A stream aggregating streams of scene and video - Stream 3: A stream aggregating streams of audio and data Option 2 (Optional): Three streams as defined below - Stream 1: pose/control - Stream 2: I-stream for video - Stream 3: P-stream for video |

Observation 2: Table 4 is the summary of the mandatory streams to be used for evaluation of VR, AR, and CG. Table 5 shows the optional streams that can be also modelled when evaluating VR, AR, and CG applications.

# Conclusion

In this contribution, we summarize the traffic models to be used for evaluating the performance of XR and CG applications on NR. We make the following observations for the traffic models:

Observation 1: Table 1, 2, and 3 are the summary of the main parameters agreed for the DL and UL baseline traffic models and employed for the evaluation purposes.

Observation 2: Table 4 is the summary of the mandatory streams to be used for evaluation of VR, AR, and CG. Table 5 shows the optional streams that can be also modelled when evaluating VR, AR, and CG applications.

# References

[1] RP-210460 Revised SID on XR Evaluations for NR, RAN#91

[2] S4aV200640 Summary of XR Traffic Models for RAN1 and Open Issues