3GPP TSG RAN WG1 Meeting #105-e                                                                    R1-2104596

e-Meeting, May 10th – 27th, 2021

Source:              ZTE, CMCC, China Telecom, China Unicom, SoftBank, NTT DOCOMO, Sanechips, vivo

Title:                   TEI-17 proposal on NR codeword mapping

Agenda Item:     8.16

Document for:   Discussion and Decision

Introduction

Currently, NR release 15 and 16 specification has been completed, and many useful features have been actually deployed.

One difference between NR and LTE is layer-to-codeword mapping, where NR only supports 1 codeword (CW) but LTE can support 2 codewords in the case when the number of transmission layers is not larger than 4.

Taking 2 layers as an example as shown in Figure 1-1, two layers can map to 2 CWs respectively in LTE, and independent MCS can be indicated to the two layers to match the potentially different channel conditions. In NR, in order to simplify implementation complexity, only 1 MCS can be indicated for the two layers no matter channel conditions are closed or different. However, some issues are identified in the real test of NR deployment. The NR performance is impacted by NR CW mapping in some cases when the SINR difference for the two layers are large.

Figure 1-1 Comparison of CW mapping between LTE and NR

In this contribution, we mainly analyze the issues of NR codeword mapping based on the measurement results in real tests and link-level and system-level simulation results. We propose to address the issues in Rel-17 TEI by adopting LTE liked solution in this contribution.

It is noted that this contribution is an update of our previous document R1-2102671.

Issues in the real test

So far, NR UL codebook based PUSCH transmission scheme is well deployed. Considering the UE cost and complexity, many of UEs in the market are configured with two transmit antennas for UL transmission. Because of higher hardware requirement, most of UEs are just deployed with non-coherent antennas as shown in Figure 2-1 as UE may not be able to adjust the phase shift among its antennas.

In such case, for 1 layer transmission, TPMI =  or  is usually used based on Rel-15 specification. Rel-16 full power TPMI in mode 1 or mode 2 can also be used for UEs without full power capability for some antennas.

Figure 2-1 UL two non-coherent antennas

For 2 layers based transmission, TPMI =  is used to get Tx diversity gain and high peak data rate. However, in the real test of some NR deployments for UL transmission with 2 non-coherent UE antennas, the peak date rate is restricted because of much different channel conditions of two antennas which correspond to two layers of rank 2 transmission. As shown in the description of real tests in Guangzhou city and Fuzhou city of China, receiving power and demodulation constellation mapping are quite different for two layers which is configured with the same MCS for NR UEs.

To make sure the TB is decoded correctly, high MCS values may not be selected because of the layer with lowest receiving power/SINR. That is, if the receiving power/SINR gap between two layers are much, higher MCS value cannot be indicated.

Real test in Guangzhou city of China

In section 2.1, we provide some real test results outside an office building in Guangzhou. We have done testing in many different UE positions and show some exemplary results below corresponding to different UE positions. Center frequency is around 2.6GHz, subcarrier spacing is 30 KHz. In the following test results, rank 2 PUSCH transmission is scheduled.

      - Result for test position 1:

In this real test, the PUSCH is scheduled with about 150 PRBs, the modulation order is 256QAM.  As shown in Figure 2-2-1, Y-axis is receiving power with 0.25 dB unit, X-axis is frequency range with 1 PRB granularity. The blue curve refers to the receiving power for the first UE antenna, and the green curve refers to the receiving power for the second UE antenna. The red curve refers to noise power. In Figure 2-2-2 and 2-2-3, the red dots refer to ideal constellation, and blue dots refer to the real demodulation constellation symbol.

Observation 1: In the real test position 1, the receiving power gap between the two layers is about 10 dB. The constellation demapping of the first layer is much better than that of the second layer in the case of rank 2 transmission.

Analysis: Because the receiving power of the first layer is much better than the second one, the constellation mapping of demodulation for the first layer is better than that of the second layer. Also, EVM of the first layer is lower than that of the second layer. Although the data of first layer may be demodulated correctly, the whole TB may still not be demodulated and decoded correctly due to the bad performance of the second layer.

Figure 2-2-1 Receiving power comparison of two ports Ps0 and Ps1

Figure 2-2-2 Constellation mapping of demodulation symbols for the first layer, EVM=3.144, SINR=30.05

Figure 2-2-3 Constellation mapping of demodulation symbols for the second layer, EVM=5.79, SINR=24.7

      - Result for test position 2:

In this test, PUSCH is scheduled with more than 200 PRBs, the modulation order is 64QAM. As shown in Figure 2-3-1, Y-axis is receiving power with 0.25 dB granularity, X-axis is frequency range with 1 PRB unit. The green curve refers to the receiving power for the first UE antenna, and the blue curve refers to the receiving power for the second UE antenna. The red curve refers to noise power.  In Figure 2-3-2 (a) and (b) for the first layer and the second layer respectively, the red dots refer to ideal constellation, and blue dots refer to the real demodulation constellation mapping.

From the test results, it can be observed that the constellation demapping of the second layer is much better than that of the first layer.

Observation 2: In the real test position 2, the receiving power gap between the two layers is about 10 dB. The constellation demapping of the second layer is much better than that of the second layer in the case of rank 2 transmission.

Figure 2-3-1 Receiving power comparison of two ports

- EVM=10.53, SINR=15.48 for first layer       (b) EVM=7.99, SINR=21.95 for second layer

Figure 2-3-2 Constellation mapping of demodulation symbols for the two layers

Real test in Fuzhou city of China

In section 2.2, we provide some real measured results tested inside an office building in Fuzhou city of Fujian province of China. Indoor gNB antennas are omnidirectional and installed in room ceiling as shown in Figure 2-4.  Center frequency is around 2.6GHz, subcarrier spacing is 30 KHz.  DMRS is placed in OFDM symbol 2 and 11.

Figure 2-4 testing done inside a building with indoor gNB

      - Result for test position 3:

In this real test, the tested UE is fixed in a position of the corridor of the 8th floor of the building. In Figure 2-5-1, the blue curve is the measured receiving power of antenna 0, and the orange curve is the measured receiving power of antenna 1. The receiving SINR gap is large in some PRBs.

From Figure 2-5-2 and 2-5-3, it can be observed that the performance of the first layer is better than that of the second layer, especially in PRB 200~250.

Figure 2-5-1 Receiving power comparison of two ports

- first layer                                                 (b) second layer

Figure 2-5-2 Constellation mapping of demodulation symbols

- first layer                                                 (b) second layer

Figure 2-5-3 EVM comparison

Based on the test results, we have the following observation:

Observation 3:  The SINR gap between two UL MIMO layers is often large in our test results e.g. larger than 10dB.  It is larger than what we observed in simulations possibly due to some practical differences e.g. inaccurate modeling of antenna placements in simulations, different blockage for different antennas, etc.

Scenario with similar issues for UL

Based on the exemplary test results elaborated in section 2, it can be observed that the receiving SINR gap between two layers can be very large in the case of two layer PUSCH transmission. If the same MCS/modulation order/TB is used for both layers as described in the current NR specification, higher MCS values may not be selected because of lower SINR layer. Otherwise, the TB may not be decoded correctly.

Besides, the issue mentioned above (large receiving SINR gap for two UL MIMO layers, named as scenario 0), the similar issues exist for UL in TDD scenarios with DL heavy traffic due to the limitation of the current NR codeword mapping.

That’s because, more contiguous DL slots are usually configured in each of slot format period in many real deployments where DL traffic is much heavier than UL. The slot format structure as shown in Figure 3-1 is one of typical configurations for power network scenarios with heavy DL traffic, and with frequently small packet size UL traffic. In the scenario, small size packets may frequently come to UE for UL PUSCH transmission, e.g. 4000 packets per seconds, and each packet has 300 Bytes. Averagely, two new packets come to the UE buffer in every slot.

| D | D | D | D | D | D | D | S | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

slot 0             slot1            slot2            slot3           slot 4        slot 5          slot 6           slot 7           slot8         slot 9

Figure 3-1 Slot format with 5ms period, SCS = 30KHz

In such scenario, all new packets come to the UE buffer should be transmitted in the two UL slots. However, once there is PUSCH re-transmission in slot 8 and/or slot 9, the new packets cannot be transmitted together with the re-transmitted packets in the same slot since only one TB/CW is supported at a given time. Thus, the new packets must be deferred to the next UL slot or even to the UL slot in the next period, i.e. slot 18 or 19.

To solve this issue, it can also be considered to support two TBs in one slot scheduling as LTE supported for UL.

Observation 4: The current NR codeword mapping has limitations in some scenarios including the scenarios with large receiving SINR gap for transmission layers, and TDD scenarios with heavy DL traffic.

Simulation

Based on the above observations and analyses, we provide some simulation results for maximum UL 2 layers transmission (scenario 0 with UL two antenna ports) to evaluate performance when there is SNR gap between two layers. Most simulation assumptions are aligned with the real test described in section 2. The detailed simulation assumptions can be found in Appendix.

SLS

In the system level simulation for maximum UL two layers with two UL antenna ports, only non-coherent precoder is used. Table 4-1 and 4-2 show the performance comparisons with 9dB and 6dB average receiving power gaps between two layers, respectively.

From the simulation results, it can be observed that two MCS/TB can bring obvious gain than single TB/MCS especially for the case of the large power gap. That’s because more flexible MCS indication for two layers can better match the channel condition of respective layers.

Table 4-1 Throughput (Mbps) for UEs with 9 dB average receiving power gap between two layers

| RU | Solution | Average UE-Throughput | 5% UE-Throughput | 50% UE-Throughput | 95% UE-Throughput. |
| --- | --- | --- | --- | --- | --- |
| 60% | One MCS/TB | 15.91 | 3.61 | 11.83 | 42.21 |
|  | Two MCS/TB | 20.65 | 4.45 | 16.68 | 54.82 |
|  | Gain | 29.8% | 23.1% | 41.0% | 29.9% |
| 40% | One MCS/TB | 26.93 | 6.60 | 18.65 | 74.56 |
|  | Two MCS/TB | 31.08 | 7.86 | 23.25 | 75.38 |
|  | Gain | 15.4% | 19.0% | 24.6% | 1.1% |
| 20% | One MCS/TB | 37.52 | 10.62 | 31.37 | 76.30 |
|  | Two MCS/TB | 41.15 | 14.17 | 36.00 | 76.30 |
|  | Gain | 9.7% | 33.5% | 14.8% | 0.0% |

Table 4-2 Throughput (Mbps) for UEs with 6 dB average receiving power gap between two layers

| RU | Solution | Average UE-Throughput | 5% UE-Throughput | 50% UE-Throughput | 95% UE-Throughput. |
| --- | --- | --- | --- | --- | --- |
| 60% | One MCS/TB | 22.09 | 5.06 | 15.96 | 62.57 |
|  | Two MCS/TB | 23.28 | 5.46 | 17.41 | 62.90 |
|  | Gain | 5.4% | 7.8% | 9.1% | 0.5% |
| 40% | One MCS/TB | 31.38 | 7.43 | 23.73 | 76.83 |
|  | Two MCS/TB | 32.81 | 9.14 | 25.40 | 76.56 |
|  | Gain | 4.5% | 23.0% | 7.0% | -0.3% |
| 20% | One MCS/TB | 41.93 | 12.96 | 36.77 | 76.30 |
|  | Two MCS/TB | 42.59 | 14.06 | 37.67 | 76.30 |
|  | Gain | 1.6% | 8.5% | 2.4% | 0.0% |

LLS

In the LLS simulation for UL two layers, only non-coherent precoders are used. To align with the real test case in section 2, the average receiving power ratio between two UE antennas is assumed as 3 dB, 6dB and 9dB. In Figure 4-1 and 4-2, transmission rank is adaptive between rank 1 and rank 2.

Based on the results for two UL antenna ports from Figure 4-1 to 4-2 which correspond to 4 and 8 receiving antennas respectively, obvious performance gain derived from two TB/MCS can be observed especially in large SNR range.

Figure 4-1 Spectral efficiency comparisons for UL, 2T4R, rank adaption

Figure 4-2 Spectral efficiency comparisons for UL, 2T8R, rank adaption

Observation 5: Based on the simulation results from both SLS and LLS, two TB/MCS can bring obvious performance gain than single TB/MCS for the case when receiving power gap is large between two layers.

Potential solutions

Based on the real test results from section 2.1 and 2.2, the analyses and simulation results in section 3, the performance and latency may be impacted due to the limitation of the current codeword mapping mechanism.

To address the issues mentioned in above sections, we propose to support 2 codewords with 2 MCS for rank 2-4 uplink transmission, i.e. LTE-like CW mapping. Since the aforementioned issues reflect the urgent requirement of NR products, and the relevant solution does not need much spec effort, we suggest to discuss them in Rel-17 TEI agenda.

Proposal: Support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping)

Conclusion

In this contribution, we analyze the issues of NR codeword mapping based on the test results in real deployment, analyzed issues in some scenarios and provide some simulation results. To address the issues, we have some observations and propose to enhance NR codeword mapping.

Observation 1: In the real test position 1, the receiving power gap between the two layers is about 10 dB. The constellation demapping of the first layer is much better than that of the second layer in the case of rank 2 transmission.

Observation 2: In the real test position 2, the receiving power gap between the two layers is about 10 dB. The constellation demapping of the second layer is much better than that of the second layer in the case of rank 2 transmission.

Observation 3:  The SINR gap between two UL MIMO layers is often large in our test results e.g. larger than 10dB.  It is larger than what we observed in simulations possibly due to some practical differences e.g. inaccurate modeling of antenna placements in simulations, different blockage for different antennas, etc.

Observation 4: The current NR codeword mapping has limitations in some scenarios including the scenarios with large receiving SINR gap for transmission layers, and TDD scenarios with heavy DL traffic.

Observation 5: Based on the simulation results from both SLS and LLS, two TB/MCS can bring obvious performance gain than single TB/MCS for the case when receiving power gap is large between two layers.

Proposal: Support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping).

Appendix

Table 6-1 LLS assumptions

| Parameter | Value |
| --- | --- |
| Metric | UL spectrum efficiency |
| Carrier frequency | 2.6 GHz |
| SCS | 30kHz |
| Channel bandwidth | 10MHz |
| Channel model | TDL-A in TR 38.901 |
| Delay spread | 300ns |
| UE speed | 3km/h |
| Number of UE antennas | 2T |
| Number of gNB antennas | 4R, 8R |
| UE antenna configuration | Omni, cross-polarization |
| Precoding granularity | 4 RBs |
| CSI periodicity | 5 slots |
| rank | rank adaption |
| Modulation | Up to 64QAM |
| overhead | 14 OFDM symbols PUSCH |
| Re-transmission | No |

Table 6-2 SLS assumptions

| Parameter | Value |
| --- | --- |
| Metric | UL throughput |
| UL CSI | 5 slots periodicity with 6 slots delay |
| Carrier frequency, SCS and system bandwidth | 2.6 GHz, 30KHz and 20MHz |
| Number of gNB antennas | (M, N, P, Mg,Ng) = (1,1,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| Number of UE antennas | (M, N, P, Mg,Ng) = (1,1,2,1,1) |
| Traffic model | FTP 1 |
| Scenario | Indoor |
| Multiple access | OFDMA |
| Channel model | According to the TR 38.901 |
| UE antenna height & gain | Follow TR 36.873 |
| UE receiver noise figure | 9 dB |
| Modulation | Up to 64 QAM |
| Coding on PUSCH | LDPC |
| Slot | 14 OFDM symbols |
| Frame structure | FDD |
| MIMO scheme | SU-MIMO with rank 1-2 transmission |