3GPP TSG RAN WG1 #105-e																																																				R1-2104608

e-Meeting, May 10th – 27th, 2021

Source:	CMCC

Title:	Enhancements on UL time and frequency synchronization for NTN

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In RAN1#104b-e meeting, agreements were achieved on enhancements on UL time and frequency synchronization for NTN [1]:

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e. Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported Conclusion: The orbital propagator model to be used at UE side can be left to implementation.

In this contribution, we will discuss UL time and frequency synchronization related aspects for NTN.

Discussion

Issue#1: NTN UE Timing Advance formula

In RAN1#104b-e meeting, an NTN UE Timing Advance formula was agreed, while the details of NTA update/accumulation and NTA,common signaling were left FFS [1].

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The details of NTA update/accumulation will be separately discussed in Issue #6 (TA update in RRC_CONNECTED state).

The details of NTA,common signaling will be separately discussed in Issue #2 (Indication of Common TA drift rate), Issue #3 (Granularity and signalling of Common TA).

Issue#2: Indication of Common TA drift rate

In RAN1#104b-e meeting, the below proposal was discussed during the GTW session but it was not agreed [2].

Updated proposal 2: The Network may periodically broadcast: - Common TA - Common TA drift rate - Common TA draft rate with value of 0 is supported - FFS: Common TA drift variation rate

As discussed in our company’s contribution [3], regarding the following potential deployment scenarios, we think scenario 1/2-a/2-b are valid deployment scenario which need more study, while scenario 3 is not within Rel-17 scope.

- Scenario 1: RU located at gNB

- Scenario 2-a: RU located at gateway, with gateway and gNB co-located

- Scenario 2-b: RU located at gateway, with gateway and gNB located away from each other

- Scenario 3: RU located at satellite

For LEO in scenario 1/2-a/2-b, common TA and common TA drift rate are both needed to indicate the time-variant RTT in the feeder link.

Nevertheless, for GEO/HAPS/ATG, common TA may be still needed to absorb the TA margin, while common TA drift rate can be 0.

Therefore, it is suggested to support the cited FL’s proposal above at least with the first 2 bullets.

Proposal 1: The Network may periodically broadcast:

- Common TA

- Common TA drift rate

  - Common TA draft rate with value of 0 is supported

Issue#3: Granularity and signalling of Common TA

In RAN1#104b-e meeting, Moderator suggested for more inputs to RAN1#105-e on granularity and signaling of Common TA [2].

FL recommendation 3: On the signaling and the granularity of Common TA ( NTA,common ) companies are encouraged to conduct more investigations and provide inputs to RAN1#105-e.

In our view, Common TA is used for TA determinization, and its granularity can be the same as the granularity of NTA, i.e., 16∙642μ∙Tc.

Regarding the concern raised by some companies that

- The total UL timing error is the sum of many components, including DL sync inaccuracy, UE-specific TA inaccuracy and granularity, closed-loop TA granularity, common TA linear (or 2nd order) approximation and granularity.

- Thus, a finer granularity of common TA is proposed to allow more inaccuracy to be “spent” on the more challenging UE-specific TA calculation.

In our view, the granularity of Common TA affects the quantization error of open-loop TA control mechanism, and the maximum quantization error is equal to the granularity of Common TA, e.g., 16∙642μ∙Tc. Note that the TA adjust range via MAC CE TA command is -31,32*16∙642μ∙Tc, then the quantization error of up to 16∙642μ∙Tc seems acceptable. Thus, there is no need for a finer granularity of common TA than 16∙642μ∙Tc.

Proposal 2: The granularity of Common TA is set to be the same as the granularity of NTA, i.e., 16∙642μ∙Tc.

In order to reduce the signaling overhead, update only relative values of Common TA (NTA,common) can be considered.

For instance, the common TA can be approximated by a linear function as

NTA,commont=NTA,common, t0+TAcommondrift, t0∙(t-t0+δt0)

where:

t is the time for targeted UL transmission at UE’s clock.

t0 is the time for receiving common TA signaling at UE’s clock.

NTA,common, t0 is the common TA (in 16∙642μ∙Tc units) at t0.

TAcommondrift, t0 is the common TA drift rate.

δt0 is used to pre-compensate the ageing of NTA,common, t0values due to the long propagation delay in NTN.

Furthermore,

NTA,common, t0=A+X

where:

A represents the synthetical impact of time-invariant parameters, e.g., TA margin and the minimum RTT on the feeder link.

X are time-variant parameters, and they need to be updated with time.

Note:

Indication of A via SIB is mandatory.

Indication of X is optional depending on time reference point configured at the gNB or at the satellite. For DL and UL aligned at satellite, indications may be absented; otherwise, e.g., for DL and UL aligned at gNB, indications are necessary.

Based on above discussion, for DL and UL aligned at satellite,

- Parameter A (a time-invariant offset of NTA,common, t0) should be indicated to absorb TA margin.

For DL and UL not aligned at satellite (including aligned at gNB),

- Parameter A (a time-invariant offset of NTA,common, t0) should be indicated to absorb TA margin and the minimum RTT on the feeder link.

- Parameter X should be indicated and updated to capture the rapidly changed RTT on the feeder link.

Proposal 3: Update relative values of Common TA to reduce the signaling overhead.

Proposal 4: The common TA is determined as

NTA,commont=NTA,common, t0+TAcommondrift, t0∙(t-t0+δt0)

where:

t is the time for targeted UL transmission at UE’s clock.

t0 is the time for receiving common TA signaling at UE’s clock.

NTA,common, t0 is the common TA (in 16∙642μ∙Tc units) at t0.

TAcommondrift, t0 is the common TA drift rate.

δt0 is used to pre-compensate the ageing of NTA,common, t0values due to the long propagation delay in NTN.

and,

NTA,common, t0=A+X

where:

A represents the synthetical impact of time-invariant parameters, e.g., TA margin and the minimum RTT on the feeder link.

X are time-variant parameters, and they need to be updated with time.

Proposal 5: Indication of parameter A (a time-invariant offset of NTA,common, t0) should be supported.

- For DL and UL aligned at satellite, A can be used to absorb TA margin.

- For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link.

Proposal 6: Indication of parameter X at least should be supported.

- For DL and UL not aligned at satellite (including aligned at gNB), X should be indicated and updated to capture the rapidly changed RTT on the feeder link.

Proposal 7: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE.

Issue#6: TA update in RRC_CONNECTED state

In RAN1#104b-e meeting, Moderator suggested for more inputs to RAN1#105-e on TA update in RRC_CONNECTED state [2].

FL recommendation 6: Companies are encouraged to provide inputs to RAN1#105-e about the combination of open and closed loop TA control

As agreed in RAN1#104b-e, the Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay.

NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

NTA,offset is a fixed offset used to calculate the timing advance.

In our view, for an NR NTN UE in RRC_CONNECTED, there is no need to update NTA,common regularly.

Note that the system information NTA,common can either be periodically broadcasted on DL-SCL, or send in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. If regularly update of NTA,common for a UE in RRC_CONNECTED is supported, then some issues may arise.

- If UE updates NTA,common based on the broadcasted SI, and if the UE stops to monitor SI carrying NTA,common for a long interval for the purpose of power saving, then very frequent signaling of legacy MAC CE TA command may be needed to track the rapidly changed RTT in the feeder link.

- If UE updates NTA,common based on dedicated RRC signaling indicated by the network, then there are redundant closed loop TA control mechanisms, i.e., one for NTA,common re-configuration, and one for NTA update.

Based on the above discussion, for an NR NTN UE in RRC_CONNECTED, regular update of NTA,common can be disabled.

Thus, for an NR NTN UE in RRC_CONNECTED,

TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc

where NTA,common0 is the common TA obtained in initial access procedure.

Proposal 8: For an NR NTN UE in RRC_CONNECTED, regular update of NTA,common can be disabled.

Proposal 9: For an NR NTN UE in RRC_CONNECTED,

TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc

where NTA,common0 is the common TA obtained in initial access procedure.

In the case of disabled NTA,common update for an NR NTN UE in RRC_CONNECTED, both the legacy MAC CE TA command and an enhanced MAC CE TA command consists of TA,δ,TA,drift are to be supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. The enhanced MAC CE TA command is used to track the rapidly changed RTT in the feeder link. UE may alternatively receive the legacy MAC CE TA command and an enhanced MAC CE TA command, up to network mplementation.

Compared with the common TA drift rate indication which is a cell specific signaling to compensate the drift rate of the feeder link RTT, TA,drift is a UE specific TA drift rate, which can compensate the total experienced drift rate of RTT at gNB side. The bit field size of common TA drift rate and TA,drift may be similar.

Compared with the common TA indication which is an absolute value of the total feeder link RTT, TA,δ is a delta value for TA adjustment with much smaller bit field size then common TA indication.

Based on the above discussion, compared with the technique combination of regular update of NTA,common via dedicated RRC signaling, and the legacy MAC CE TA command, the technique combination of disabled NTA,common update and the enhanced MAC CE TA command may significantly reduce the signaling overhead.

Observation 1: Compared with the technique combination of regular update of NTA,common via dedicated RRC signaling, and the legacy MAC CE TA command, the technique combination of disabled NTA,common update and the enhanced MAC CE TA command may significantly reduce the signaling overhead.

Proposal 10: The legacy MAC CE TA command and the enhanced MAC CE TA command consists of TA,δ,TA,drift to be both supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate.

It is clear that the legacy MAC CE TA command can be seen as a special case of the enhanced MAC CE TA command with TA,drift=0, i.e., when set TA,drift=0, the enhanced MAC CE TA command degrades into the legacy one.

Observation 2: The legacy MAC CE TA command is a special case of the enhanced MAC CE TA command with TA,drift=0.

The generalized formula for NTA update based on 	the enhanced MAC CE TA command is shown as following:

For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as

NTAti=NTAaccumm+TA,driftm∙ti-tm'

where,

- m is the last received enhanced TA command before ti.

- NTAaccumm is the m-th accumulated TA command.

- Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', it will update the m-th accumulated TA command NTAaccumm as

NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0

Figure 1 illustrates the computational procedure of NTA update based on the enhanced MAC CE TA command.

Figure 1: Illustration of computational procedure of NTA update.

Proposal 11: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as

NTAti=NTAaccumm+TA,driftm∙ti-tm'

where,

- m is the last received enhanced TA command before ti.

- NTAaccumm is the m-th accumulated TA command.

- Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as

NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0

Issue#8: TA reporting

Note that without any assist information as reported by the UE on the service link RTD (tS), gNB can NOT determine what exactly K_offset value should be, let alone subsequent network indication for K_offset updating. Thus, RAN1 needs to further study the details of UE reporting TA related information to facilitate network updating K_offset after initial access.

The following reporting information can be further studied:

- Option 1: UE location.

- Option 2: UE self-estimated TA (NTA,UE-specific).

- Option 3: the difference value (in unit of slot) between K_offset signaled in system information and a UE specific K_offset as suggest by UE.

Note that Option 3 is beneficial for less signalling overhead.

Proposal 12: RAN1 to further study the details of UE reporting TA related information to facilitate network updating K_offset after initial access. The following reporting information can be further studied.

- Option 1: UE location.

- Option 2: UE self-estimated TA (NTA,UE-specific).

- Option 3: the difference value (in unit of slot) between K_offset signaled in system information and a UE specific K_offset as suggest by UE.

Issue#9: Broadcasting the position of a reference point

In RAN1#104b-e meeting, Moderator suggested for more discussion on broadcasting the position of a reference point [2].

FL recommendation 9: On support of broadcasting a reference point, proponents are encouraged to provide more details on the feasibility of such solution by taking into account the concern related to disclosing the network architecture.

In our view, broadcast of a reference point would be definitely beneficial for UE to handle the time and frequency synchronization compensation on the feeder link. E.g., if the position of the GW is signaled to the UE, the UE can autonomously determine the time offset of both the service link (i.e., satellite-to-UE RTT) and the feeder link (i.e., satellite-to-GW RTT), which would simplify the time compensation procedures and reduce signaling overhead for frequent update of NTA,common.

Observation 3: Broadcasting the position of a reference point of the feeder link is beneficial to simplify the time compensation procedures and reduce signaling overhead for frequent update of NTA,common.

Regarding to the concern related to disclosing the network architecture, it would be at the discretion of the network operator. Furthermore, the position of a reference point of the feeder link may be broadcasted with certain artificial bias to address the security issue.

Note that when TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as follows.

NTA=TA*16*642μ

where μ=0,1,2,3,4,5, and TA is an integer between 0 and 3846. The maximum TA values for different numerology are illustrated in Table 2.

Table 2: Maximum TA value when TA=3846.

| Subcarrier spacing | 15 kHz µ= 0 | 30 kHz µ= 1 | 60 kHz µ= 2 | 120 kHz µ= 3 | 240 kHz µ= 4 | 480 kHz µ= 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum timing advance (TA = 3846) (ms) | 0.67 | 0.335 | 0.1675 | 0.0838 | 0.0419 | 0.0209 |
| Maximum timing advance (TA = 3846) (km) | 300 | 150 | 75 | 37.5 | 18.75 | 9.38 |

It can be concluded that for TA pre-compensation, broadcasting the position of a reference point of the feeder link with certain accuracy (<= 9.38km) is feasible. Thus, security may be not a big issue if the broadcasted position of the reference point position is with artificial bias.

Observation 4: For TA pre-compensation, broadcasting the position of a reference point of the feeder link with certain accuracy is feasible.

Observation 5: Security issue can be fixed by broadcasting the position of a reference point of the feeder link with certain artificial bias.

Regarding to the concern related to scenario 2b (i.e., gNB location away from GW location), GW location (maybe with certain artificial bias) may be broadcasted in addition to the broadcast of fixed common TA, where the fixed common TA is used to compensate for the RTT of GW-to-gNB and possibly other latencies.

For example, if GW location is broadcasted to UE,

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

then in the above TA determining formula, NTA,UE-specific may cover both the service link RTT (i.e., satellite -to-UE RTT) and a part of the feeder link RTT (i.e., satellite -to-GW RTT), while a fixed common TA (NTA,common) is still broadcasted to cover the fixed RTT of GW-to-gNB and possibly other latencies considered necessary by the network (e.g., TA margin).

Based on above discussion, broadcasting the position of a reference point of the feeder link with certain artificial bias can be supported. A fixed common TA can be additional broadcasted, if needed.

, in addition to the broadcast of a fixed common TA if needed, can be supported.

Proposal 13: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. A fixed common TA can be additional broadcasted, if needed.

Issue#10 : Common Doppler shift pre/post compensation on the feeder link

In RAN1#104b-e meeting, the below proposal was discussed during the GTW session but it was not agreed [2].

Working assumption: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

Both the GW/Satellite and gNB may have the capability to compensate the Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink in a way transparent to UE.

From UE’s perspective, there is no need to distinguish who does the compensation.

Regarding the cited FL’s proposal above cover both candidates, hence it is to be supported.

Proposal 14: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

Issue#11: Indication of common frequency pre-compensation offset on DL service link

In RAN1#104b-e meeting, Moderator suggested for more discussion on Indication of common frequency pre-compensation offset on DL service link [2].

FL Recommendation 11: Companies are encouraged provide inputs to RAN1#105-e regarding the granularity and how to indicate the amount of frequency compensation when DL frequency compensation for the Doppler on service link is applied.

In our view, in the case UE known its position, if NR NTN gNB applies frequency pre-compensation in DL, indication of the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link is necessary.

Figure 2: UL frequency synchronization for UE only with positioning knowledge.

As shown in Figure 2, if UE only know its position, indication of the pre-compensated Common Frequency Offset on DL transmission, which is equal to the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link, is necessary. Otherwise, the UE cannot correctly determine the UE transmission frequency without residual UL frequency error at satellite.

  - If the pre-compensated Common Frequency Offset on DL transmission is unknown, UE may determine it UL transmission frequency (denoted as ④) as ④=①+③-②, where, ① is the frequency of the received DL signal, ③ is the difference between reference UL and DL frequency, ② is the estimated Doppler shift on the service link. The residual UL frequency error at satellite (denoted as ⑤) is about ⑤=②-⑥, where, ⑥ is the pre-compensated Common Frequency Offset on DL transmission.

  - On the contrary, if the pre-compensated Common Frequency Offset on DL transmission (⑥) is indicated, UE can determine its UL transmission frequency (④) as ④=①+③-②-②-⑥=0, to eliminate the residual UL frequency error at satellite.

Note that the above indication is common for both earth-fixed and earth-moving cells.

Proposal 15: Support indication of DL frequency compensation for the service link Doppler, i.e.,

- If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link.

Issue#14: Serving satellite ephemeris format

In RAN1#104b-e meeting, support serving-satellite ephemeris broadcast based on one or more of the Set 1/Set 2 was agreed [1].

Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported

Regarding the last FFS “Whether down-selection is needed or both sets are supported”, we are open to support both options for progress. However, if down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Set1) should be supported for implicit compatibility to support HAPS and ATG scenarios.

Proposal 16: Support both ephemeris formats based on satellite position and velocity state vectors (Set 1) and based on orbital elements (Set 2), if possible.

Proposal 17: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Set 1) should be supported for implicit compatibility to support HAPS and ATG scenarios.

In RAN1#104b-e meeting, Moderator suggested for more study on whether epoch time for ephemeris information is implicitly (derived from DL subframe) or explicitly signaled. [2].

FL recommendation: RAN1 to study whether epoch time for ephemeris information is implicitly (derived from DL subframe) or explicitly signaled.

In our view, implicitly (derived from DL subframe) indication of epoch time for ephemeris information is preferred to save signaling overhead.

Proposal 18: Support implicitly indication of epoch time for ephemeris information.

Conclusions

In this contribution, we share our views on related issues on UL time and frequency synchronization enhancements for NTN. The observations and proposals are summarised as follows:

Observation 1: Compared with the technique combination of regular update of NTA,common via dedicated RRC signaling, and the legacy MAC CE TA command, the technique combination of disabled NTA,common update and the enhanced MAC CE TA command may significantly reduce the signaling overhead.

Observation 2: The legacy MAC CE TA command is a special case of the enhanced MAC CE TA command with TA,drift=0.

Observation 3: Broadcasting the position of a reference point of the feeder link is beneficial to simplify the time compensation procedures and reduce signaling overhead for frequent update of NTA,common.

Observation 4: For TA pre-compensation, broadcasting the position of a reference point of the feeder link with certain accuracy is feasible.

Observation 5: Security issue can be fixed by broadcasting the position of a reference point of the feeder link with certain artificial bias.

Proposal 1: The Network may periodically broadcast:

- Common TA

- Common TA drift rate

  - Common TA draft rate with value of 0 is supported

Proposal 2: The granularity of Common TA is set to be the same as the granularity of NTA, i.e., 16∙642μ∙Tc.

Proposal 3: Update relative values of Common TA to reduce the signaling overhead.

Proposal 4: The common TA is determined as

NTA,commont=NTA,common, t0+TAcommondrift, t0∙(t-t0+δt0)

where:

t is the time for targeted UL transmission at UE’s clock.

t0 is the time for receiving common TA signaling at UE’s clock.

NTA,common, t0 is the common TA (in 16∙642μ∙Tc units) at t0.

TAcommondrift, t0 is the common TA drift rate.

δt0 is used to pre-compensate the ageing of NTA,common, t0values due to the long propagation delay in NTN.

and,

NTA,common, t0=A+X

where:

A represents the synthetical impact of time-invariant parameters, e.g., TA margin and the minimum RTT on the feeder link.

X are time-variant parameters, and they need to be updated with time.

Proposal 5: Indication of parameter A (a time-invariant offset of NTA,common, t0) should be supported.

- For DL and UL aligned at satellite, A can be used to absorb TA margin.

- For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link.

Proposal 6: Indication of parameter X at least should be supported.

- For DL and UL not aligned at satellite (including aligned at gNB), X should be indicated and updated to capture the rapidly changed RTT on the feeder link.

Proposal 7: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE.

Proposal 8: For an NR NTN UE in RRC_CONNECTED, regular update of NTA,common can be disabled.

Proposal 9: For an NR NTN UE in RRC_CONNECTED,

TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc

where NTA,common0 is the common TA obtained in initial access procedure.

Proposal 10: The legacy MAC CE TA command and the enhanced MAC CE TA command consists of TA,δ,TA,drift to be both supported, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate.

Proposal 11: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as

NTAti=NTAaccumm+TA,driftm∙ti-tm'

where,

- m is the last received enhanced TA command before ti.

- NTAaccumm is the m-th accumulated TA command.

- Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as

NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0

Proposal 12: RAN1 to further study the details of UE reporting TA related information to facilitate network updating K_offset after initial access. The following reporting information can be further studied.

- Option 1: UE location.

- Option 2: UE self-estimated TA (NTA,UE-specific).

- Option 3: the difference value (in unit of slot) between K_offset signaled in system information and a UE specific K_offset as suggest by UE.

Proposal 13: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. A fixed common TA can be additional broadcasted, if needed.

Proposal 14: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

Proposal 15: Support indication of DL frequency compensation for the service link Doppler, i.e.,

- If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link.

Proposal 16: Support both ephemeris formats based on satellite position and velocity state vectors (Set 1) and based on orbital elements (Set 2), if possible.

Proposal 17: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Set 1) should be supported for implicit compatibility to support HAPS and ATG scenarios.

Proposal 18: Support implicitly indication of epoch time for ephemeris information.

References

- Chairman's Notes RAN1#104b-e, April 12th – 20th, 2021.

- R1-2104076, “Feature lead summary #5 on enhancements on UL timing and frequency synchronization”, e-Meeting, April 12th – 20th, 2021.

- R1-2104607, “Discussion on timing relationship enhancements for NTN”, e-Meeting, May 10th – 27th, 2021.