3GPP TSG RAN WG1 #105-e		   R1-2104612

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.5.2

Source: 	CMCC

Title: 	Discussion on UL-AoA enhancements

Document for:	Discussion and Decision

Introduction

In RAN#90-e meeting, a new work item for NR positioning enhancements was approved [1], in which the motivation to support positioning enhancements to attain Rel-17 target positioning requirement was justified for both commercial and IIoT use cases.

The objectives of this new work item are to specify solutions to provide higher accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.), where the RAN1 centric objectives are listed as follows:

RAN1 centric objectives: - Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1] - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1] - UL AoA for network-based positioning solutions. - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions. Note: RAN1 will discuss the candidate solutions and provide updates for this objective, with status to be reviewed in RAN#91e.

In RAN1#104-b-e meeting, the following agreements were made on the accuracy improvements for UL-AoA positioning solutions:

Agreement: - The following option is supported to enhance signaling of UL-AOA measurement report in case of a linear array - Option 2: The z-axis of LCS is defined along the linear array axis. gNB reports only the ZoA relative to z-axis in the LCS, and the LCS-to-GCS translation function is used to set up the specific z-axis direction - UL-AOA signalling details for support of Option 2 are left up to RAN WG3 Agreement: - Uncertainty range for expected UL AoA/ZoA is defined as follows - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival - Expected zenith angle of arrival as (θAOA - ΔθAOA/2, θAOA + ΔθAOA/2) - θAOA - expected zenith angle of arrival, ΔθAOA – uncertainty range for expected zenith angle of arrival - Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication - FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range) Agreement: - Reporting to LMF of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp is supported.

In this contribution, we continually provide our views on the remaining issues.

Discussion

In the last meeting, the assistance information details of how to define the expected AoA/ZoA and the uncertainty range was agreed. There are still two remaining issues on the assistance signalling enhancement:

- The coordinate system of the assistance information

In the agreement, two alternatives of the coordination system were captured for further down-selection. In our view, for Alt. 2, the LMF should have the GCS-to-LCS translation information, which may not always be the case. From this point of view, Alt. 1 is the simplest way to transfer the assistance information from the LMF to the TRP, and let the TRP do the GCS-to-LCS translation locally.

Proposal 1: GCS is supported for AoA/ZoA assistance information indication.

- Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range)

In the last meeting, except for the definition of expected AoA/ZoA and its uncertainty range that has already agreed in the above agreement, some companies also proposed to support the LMF to indicate DL-PRS resource for the expected AoA/ZoA and uncertainty range, where the expected AoA/ZoA is represented by boresight information of a DL-PRS resource (beam) and uncertainty range is associated with DL-PRS beam width.

In R16, the configuration of SRS pos includes the spatial relation information of both serving and neighboring TRPs, of which the source RS can be DL PRS. In such a case, we believe that this proposed solution has already been supported in the current spec, to help the TRP to narrow down the beam sweeping range and enhance the measurement performance. Consider the case that no spatial relation information is configured, and the SRS pos is sent in an omnidirectional manner, the indication of DL-PRS resources is a way to provide the AoA/ZoA search window for the TRP; however, we think that the already supported method can provide full flexibility in indicating the AoA/ZoA search window, and support additional indication method seems no sufficient benefit.

Observation 1: It is not necessary to support additional signaling for AoA/ZoA assistance information (expected value and uncertainty range)

- Whether the AoA/ZoA search window can apply to other positioning

During the discussion in the last meeting, companies shared diverse views on whether the AoA/ZoA search window can be additionally applied to UL-TDOA/multi-RTT positioning. In our view, in the current spec, the positioning reference signal and corresponding measurement and signaling is configured per positioning method. In this sense, if a hybrid positioning method including UL-AoA and UL-TDOA/multi-RTT positioning is configured, the AoA/ZoA search window can also be used to assist the timing-based measurement; however, it seems not necessary to explicitly specify that signaling of AoA/ZoA assistance information (expected value and uncertainty range) is supported for UL-TDOA and Multi-RTT positioning methods as well.

Observation 2: It is not necessary to explicitly support the signaling of AoA/ZoA assistance information (expected value and uncertainty range) for UL-TDOA and Multi-RTT positioning methods.

Conclusions

In this contribution, we provide our views on the UL-AoA enhancements, and the following observations and proposals are provided:

Proposal 1: GCS is supported for AoA/ZoA assistance information indication.

Observation 1: It is not necessary to support additional signaling for AoA/ZoA assistance information (expected value and uncertainty range).

Observation 2: It is not necessary to explicitly support the signaling of AoA/ZoA assistance information (expected value and uncertainty range) for UL-TDOA and Multi-RTT positioning methods.

References

- RP-202900, New WID on NR positioning enhancements, CATT, Intel Corporation, Ericsson.