3GPP TSG RAN WG1 #105-e		R1-2104618

e-Meeting, May 10th – 27th, 2021

Source: 	CMCC

Title:	Discussion on collision handling of HD-FDD operation

Agenda item:	8.6.1.3

Document for:	Discussion & Decision

- Introduction

In RAN1#104b-e meeting, the following agreements were made regarding the HD-FDD operation of RedCap UEs [1].

Agreements:

For Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum.

- FFS whether the timeline is extended to include the RX/TX switching time for HD-FDD

For Case 4: dynamically scheduled DL reception vs. dynamic scheduled UL transmission, reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum

- That is, it is considered as an error case if a dynamically scheduled DL reception overlaps with a dynamically scheduled UL transmission

For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier/single cell in unpaired spectrum

- The semi-statically configured DL reception may include PDCCH (excluding ULCI), SPS PDSCH, CSI-RS or PRS.

  - FFS on PDCCH carrying ULCI, including whether or not it is supported by RedCap UEs (including potential difference between HD vs. FD RedCap UEs)

- The dynamically scheduled UL transmission may include PUSCH, PUCCH, SRS or PRACH triggered by PDCCH order

Agreements:

# For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

- FFS: whether or not there are conditions that need to be considered

Working assumption: For HD-FDD, no additional UE behavior for switching position determination is specified as compared to the existing specification.

Conclusion: Enhancement for potential UL and DL collision handling due to TA misalignment is not considered for Type-A HD-FDD operation of RedCap UEs

Working Assumption: For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

- A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

- A HD-FDD UE is not expected to receive in the downlink earlier than[NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell

- FFS NTX-RX and NRX-TX

- FFS: how it jointly works with the agreement for other collision cases

# Working assumption:

- If a dynamically scheduled UL transmission overlaps with an SSB, down-select one of the following options:

  - Option 1: Follow the handling of case 2 that dynamic UL is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamic UL

  - Option 3: Leave to UE implementation whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- If a semi-static configured UL transmission overlaps with an SSB, down-select from the following options

  - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL

  - Option 3: Leave to UE implementation whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols

- FFS: whether or not the semi-static configured UL transmission includes a valid RO

In this contribution, considerations on collision handling of HD-FDD operation and related specs influences are discussed and proposals are given.

- Discussion on collision handling of HD-FDD operation

In last meeting, several cases of potential collisions are listed to further study whether HD-FDD case can reuse the existing TDD collision handling method.

Last meeting has agreed that Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) can reuse the existing TDD collision handling method, as described in TS38.213.

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols.

-	The UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols.

Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0.

Whether to cancel the high layer configured uplink transmission is related to PUSCH preparation time Tproc,2. And according to TS38.214, section 6.4, BWP switching time if triggered and uplink switching gap duration for uplink Tx switch are considered when determining Tproc,2. RX/TX switching time should also be included in Tproc,2.

Proposal 1: For case 1, the timeline is extended to include the RX/TX switching time for HD-FDD, e.g. RX/TX switching time is included in Tproc,2 for HD-FDD case.

It has been agreed that Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) can reuse the existing TDD collision handling method, where the semi-statically configured DL reception may include PDCCH (excluding UL CI), SPS PDSCH, CSI-RS or PRS. PDCCH carrying UL CI should be supported by HD and FD RedCap UEs, since that RedCap UEs will cause severe interference to URLLC UEs without UL CI. Based on the existing TDD collision handling method for Case 2, when PDCCH monitoring occasion of UL CI collides with dynamically scheduled UL transmission, UL CI is cancelled, resulting in the failure of cancelling the uplink resources of RedCap UEs that conflict with URLLC UEs and the loss of reliability of URLLC UEs.

But we think this collision can be avoided from gNB implementation. When URLLC service is multiplexed with eMBB or RedCap service, and search space for monitoring CI indication has been configured, gNB should avoid scheduling PUSCH on the overlapped symbols with CI monitoring to keep a high priority for URLLC service. .

Proposal 2: For case 2, existing TDD collision handling method is reused, the semi-statically configured DL reception include PDCCH carrying UL CI.

Proposal 3: gNB avoids the possible collision of PDCCH carrying UL CI and dynamically scheduled UL transmission by implementation.

For Case 3, cell-specifically configured DL reception vs. cell-specifically configured UL transmission needs to be further discussed. Cell-specifically configured UL transmission includes RO, cell-specifically configured DL transmission includes SSB, paging occasions and SI occasions. For the cases that overlap with Case 5 and Case 8, e.g. SSB vs. cell-specifically configured UL transmission and RO vs. cell-specifically configured DL transmission, a unified collision handling method is preferred.

Proposal 4: For the overlapping case in Case 3, Case 5 and Case 8, including SSB vs. cell-specifically configured UL transmission and RO vs. cell-specifically configured DL transmission, a unified collision handling method is preferred.

For Case 5, configured SSB collides with dynamically scheduled or configured UL transmission. Based on existing spec, the UE does not perform UL transmission in the slot if a transmission would overlap with any symbol from the set of SIB1 configured SSB symbols. Differently, for HD-FDD case, gNB can transmit and receive simultaneously on paired spectrum during the overlapping symbols. When a RedCap UE doesn’t need to receive SSB, dynamically scheduled or configured UL transmission may not be cancelled to improve resource utilization. That is, whether to receive the SSB or transmit the UL transmission should be left to UE implementation.

For Case 8, dynamic or semi-static DL reception collides with valid RO. Similarly as Case 5, when a RedCap UE does not need to send PRACH, dynamic or semi-static DL reception may not be cancelled.

Proposal 5: For Case 5, whether to receive the SSB or transmit the UL transmission should be left to UE implementation.

Proposal 6: For Case 8, whether to transmit PRACH or receive the DL transmission should be left to UE implementation.

Preparing UL received DCI and parallel

Preparing UL received DCI and parallel

无法加载全部结果

重试

正在重试…

正在重试…

- Conclusions

In this contribution, considerations on collision handling of HD-FDD operation are discussed, and the following proposals are made.

Proposal 1: For case 1, the timeline is extended to include the RX/TX switching time for HD-FDD, e.g. RX/TX switching time is included in Tproc,2 for HD-FDD case.

Proposal 2: For case 2, existing TDD collision handling method is reused, the semi-statically configured DL reception include PDCCH carrying UL CI.

Proposal 3: gNB avoids the possible collision of PDCCH carrying UL CI and dynamically scheduled UL transmission by implementation.

Proposal 4: For the overlapping case in Case 3, Case 5 and Case 8, including SSB vs. cell-specifically configured UL transmission and RO vs. cell-specifically configured DL transmission, a unified collision handling method is preferred.

Proposal 5: For Case 5, whether to receive the SSB or transmit the UL transmission should be left to UE implementation.

Proposal 6: For Case 8, whether to transmit PRACH or receive the DL transmission should be left to UE implementation.

- References

- [1]	RAN1 Chairman’s Notes, RAN1#104b-e, e-Meeting, 12th April–20th April, 2021.