3GPP TSG RAN WG1 #105-e		R1-2104619

e-Meeting, May 10th – 27th, 2021

Source: 	CMCC

Title:	Discussion on other aspects of reduced UE complexity

Agenda item:	8.6.1.4

Document for:	Discussion & Decision

- Introduction

In RAN1#104-e meeting, the following agreements were made regarding the MCS table of RedCap UEs [1].

Agreements:

- The MCS tables currently defined are re-used for RedCap UEs

  - FFS which MCS table is the default one for RedCap (i.e., the default one for non-RedCap UEs or the one with low SE entries)

  - FFS mandatory/optional of the MCS tables

  - Note: there is no new MCS table to be introduced for RedCap UEs

In this contribution, considerations on relaxed maximum modulation order and related specs influences are discussed and proposals are given.

Discussion on relaxed maximum modulation order

In WID, 256QAM in the DL is an optional capability for an FR1 RedCap UE. The MCS tables currently defined (i.e. Table 5.1.3.1-1, Table 5.1.3.1-3 in TS 38.214) can be re-used for RedCap UEs that do not support 256QAM in DL. The default MCS table of legacy UEs is Table 5.1.3.1-1.

- Table 5.1.3.1-1: supports spectral efficiencies from 0.2344 to 5.5547 (QPSK to 64-QAM)

- Table 5.1.3.1-2: supports spectral efficiencies from 0.2344 to 7.4063 (QPSK to 256-QAM)

- Table 5.1.3.1-3: supports spectral efficiencies from 0.0586 to 4.5234 (QPSK to 64-QAM)

During initial access, the default MCS table of RedCap UEs should be the same as legacy UEs. Since early identification is an optional capability, if early identification is not performed, the default MCS table of RedCap UEs should be the same as legacy UEs, so that the same MCS index in DCI indicates the same modulation order and target code rate for RedCap UEs and legacy UEs. If the default MCS table of RedCap UEs is different from legacy UEs, early identification is necessary to achieve appropriate scheduling for RedCap UEs and legacy UEs respectively. This leads an opposite way to the existing agreement that early identification is configurable by gNB.

After initial access, the low SE MCS Table 5.1.3.1-3 can be optional for RedCap UEs.

Proposal 1: The default MCS table of RedCap UEs should be the same as the default one of legacy UEs.

Proposal 2: The low SE MCS table can be optional supported for RedCap UEs.

Preparing UL received DCI and parallel

Preparing UL received DCI and parallel

无法加载全部结果

重试

正在重试…

正在重试…

- Conclusions

In this contribution, considerations on relaxed maximum modulation order are discussed, and the following proposals are made.

Proposal 1: The default MCS table of RedCap UEs should be the same as the default one of legacy UEs.

Proposal 2: The low SE MCS table can be optional supported for RedCap UEs.

- References

- RAN1 Chairman’s Notes, RAN1#104-e, e-Meeting, 25th January–5th February, 2021.