3GPP TSG RAN WG1 #105-e			R1-2104670

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.4.4

Source: 	Qualcomm Incorporated

Title: 	BWP operation and other issues for NTN

Document for:	Discussion/Decision

# Introduction

# In RAN1#102e and #103e, the following agreements on satellite beam arrangements and polarization were made [1][2]:

Agreement:

One-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN.

- FFS: The need for potential enhancement for beam management

- FFS: The need for potential enhancement on association of SSBs, beams and BWPs

Agreement:

Potential enhancements for support of polarisation signalling in NR NTN can consider at least the following:

- Configuration of DL and UL transmit polarization including Right hand and Left hand circular polarizations (RHCP, LHCP)

- Network broadcast DL and UL transmit polarization configuration

- UE polarization capability (RHCP, LHCP, Linear)

- Dependence of polarisation signaling on deployment scenarios. For example,

  - Resource reuse mode with/without polarization for the beam management enhancement

  - Fixed polarization per cell/beam for polarization reuse and circular polarisation with intra-UE and inter-UE multiplexing (intra-UE and inter-UE) signalling

Agreement:

Indication of polarization information for DL and UL by the network is supported.

- FFS: Signaling details

Additionally, the following agreement and conclusions were made in RAN1 #104e[3]:

Agreement:

Support at least explicit indication of polarization information for DL by the network

- FFS: whether the indication is done by SIB, other RRC signaling, DCI.

- FFS: Whether separate signaling is needed for the UL and if so, whether or not a same polarization is indicated for DL and UL

Conclusion:

Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management:

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

Conclusion:

Discuss the necessity of reporting UE polarization capability considering at least following aspects,

- Deployment scenarios.

- UE implementation aspects with respect to polarization.

- Satellite implementation aspects for switching between polarization states.

- Satellite implementation aspects for realizing multiplexing of UEs having different polarization capabilities.

In this document, we discuss cell planning and SSB arrangements, BWP switching, antenna diversity for PRACH, and several other issues.

# Cell Planning and SSB Arrangement

## Cell/beam/frequency planning

A satellite may use multiple antennas to form multiple satellite beams. Among the satellite beams of a satellite, frequency reuse factors less than 1 have been shown to provide significant gain in link budget [4]. Hence, beams of a satellite may use different frequency intervals. If a cell consists of multiple satellite beams with different frequency intervals, it is natural to associate a bandwidth part (BWP) with each of the frequency intervals. Also, a BWP configuration comes with other configurations essential to the operation of the network. For example, a DL BWP configuration also contains configurations about CORESETs and search spaces.

Four options for cell/beam/frequency planning are shown in Figure 1, where Opt 1 is one-beam-per-cell and Opt 2 (beams on the same BWP grouped as a cell), Opt 3 (beams in proximity grouped as a cell) and Opt 4 (all beams grouped as a cell) are multiple-beams-per-cell. Note that frequency reuse is applied in all options. These options have their respective pros and cons.  For example, Opt 1 can be readily supported with the current specifications, but the UE will experience frequent cell handover in cases of moving cells with LEO satellites, which is expensive in terms of signaling overhead. Opt 4 can minimize cell handover but needs changes to the specifications on BWPs and beam management.

Observation 1: Different options for cell/beam/frequency planning call for flexible standard design.

Cell/beam/frequency planning Opt 1 is supported by the current NR specification. Opt 4 is the building block for both Opt 2 and Opt 3, and hence we will focus on Opt 4 in the remainder of the document.

Figure 1. Options for cell/beam/frequency planning among the beams from a same satellite

In terrestrial networks where different network transmit beams require different receive beams or spatial directions at UE for maximal antenna gain. In NTN, however, the best spatial direction of a UE transceiver is typically the same for all the satellite beams. That is, a VSAT type UE should always have antenna direction pointing towards the satellite, regardless the satellite beam it is connected to.

Observation 2: Different beams of a satellite may have different carrier frequencies but require the same transmit and receive spatial direction at a UE.

As such, it is natural to associate a satellite beam with a BWP, at least an initial BWP.

Proposal 1: Support satellite beam specific initial BWPs.

For Rel-17, it is important to support diverse satellite implementation options and hence it is necessary to support flexible design of SSB arrangements and BWP operations. To avoid backward compatibility issue in the future, it is important to ensure flexible design in the first release of NTN specification.

## SSB arrangements

The SSB arrangement for Opt 1 of cell/beam/frequency planning is trivial, because there is only one beam within each cell. We focus on the SSB arrangement for Opt 4, which is the basis for both Opt 2 and Opt 3. The SSB transmissions in a cell may be arranged such that they occupy a same frequency interval (Alt 1), or they are within the BWPs of their respective satellite beams (Alt 2), as shown in Figure 2.

Alt 1: All SSB transmissions in a cell occupy a same frequency interval (which may be fully contained in a BWP or straddle two BWPs)

- Pros: there is a smaller number of potential SSB frequencies for the UE to search for, and they can be placed in a sparse synch raster, which reduces initial access time

- Cons: A satellite beam needs to be tuned from the BWP that it uses for data communication to the same SSB frequency periodically. This may increase the hardware complexity of the satellite.

Alt 2: The SSB transmission in a satellite beam of a cell is within the BWP associated with the satellite beam

- Pros: A satellite beam does not need to tune away from its BWP to transmit an SSB, and this avoids the increase in hardware complexity.

- Cons: The number of SSB frequencies will be larger than in Alt 1, and the SSB frequencies need to be placed on a dense synch raster, leading to an increase in the initial access time.

Figure 2. SSB arrangements in time and frequency: Alt 1: SSBs are in a same frequency interval, and Alt 2: SSBs are within respective BWPs.

Proposal 3: Support the following SSB arrangements

- Alt 1: SSBs of all satellite beams in a same cell are transmitted within a same frequency interval and do not overlap in time

- Alt 2: SSBs of a cell are transmitted in different frequency intervals, i.e., within their respective BWPs.

## Enhancements on signalling

We fist consider SSB arrangement Alt1 (SSBs are in a same frequency interval or BWP). In Design 1, as shown in Figure 3, the SSBs are identical (except for the SFN and SSB index), and the configurations of CORESET#0 (and search space zero) in the MIB are the same.  The CORESET#0 leads to the same SIB1 transmission, which provides the initial DL BWP and the initial UL BWP for each of the satellite beams. In Design 2, as shown in Figure 3, the SSBs differ in not only SFN and SSB index but also CORESET#0 configuration. These different CORESET#0 configurations lead the UEs to the CORESET#0’s associated with the respective satellite beams. A PDCCH determined by a CORESET#0 (and search space zero) then schedules a SIB1 transmission, which signals to the UEs the configuration for the initial DL BWP and the initial UL BWP for the satellite beam that transmits the SIB1. The SIB1 transmission may also convey the configuration for the initial DL BWPs and initial UL BWPs, and the CORESET#0’s for other satellite beams. This information is important for mobility handling later on, and it is efficient to broadcast it as system information.

Design 1 allows for soft combining in decoding the MIB in the SSBs for improved decoding performance. Design 2 transmits a generally smaller amount of information in the SIB1 message to each UE which allows for improved efficiency.

Figure 3. Design options for beam-specific configurations for SSB arrangement Alt 1: Design 1, satellite beams are separate in frequency starting from initial BWPs. Design 2, satellite beams are separate in frequency starting from CORSET 0.

Design 1 has more commonality with existing NR than Design 2. Compared to existing NR design, Design 1 requires signalling of initial BWPs per SSB.  For the above reasons, only Design 1 is to be considered.

Below, we discuss how to enable beam-specific initial BWPs after initial access. We now consider SSB arrangement Alt 2 (SSBs are in different BWPs). As shown in Figure 4, the SSBs are transmitted in different BWPs and the UE can follow the procedure in the existing NR specification to decode the SSB, extract the configuration on CORESET#0 (and search space zero) for its associated satellite beam, decode the PDCCH and receive the SIB1 transmission, which carries not only the configuration for the initial DL BWP and initial UL BWP for the associated satellite beam, but also the initial DL BWPs and initial UL BWPs, and the CORESET#0’s for other satellite beams.

Figure 4. Design for beam-specific configurations for SSB arrangement Alt 2 (SSBs are in different BWPs): signaling the configurations for initial DL and UL BWPs of all satellite beams and the configuration for CORESET#0’s of other satellite beams.

The signalling design must support both of the SSB arrangements. In order for efficient beam switching with reduced signalling overhead, all relevant information including initial DL and UL BWP for the satellite beams are to be made available to all UEs, regardless of the SSB arrangement (Alt 1 or Alt2).

For both SSB arrangement Atl. 1 and Alt. 2, UE procedure can be summarized by the following steps:

- UE synchronizes to a first SSB and reads the information of the associated CORSET 0.

- From CORSET 0, UE decodes SIB1 that contains information about initial BWPs of the first SSB and also others.

- UE switches to the initial BWPs of the first SSB.

Proposal 4: Support signalling of the following configurations in SIB1

- initial BWPs of other satellite beams,

- CORSET#0 of other satellite beams if different from that of the serving beam.

# BWP Switching

A satellite may form many beams, which are mapped to multiple BWPs. Depending on the frequency reuse factor and the beam footprint layout, the number of BWPs in which a UE operates over time may be greater than what is allowed in the current specification. For example, if a UE traverses along the straight line in Figure 5, seven BWPs would need to be defined for that UE, greater than 4, the maximum number of dedicated (DL) BWPs that can be configured for a UE. One way to resolve this issue is to increase the range of BWP ID.

Figure 5. A UE moves along a straight line and traverses 7 BWPs, where each color represents a BWP.

A BWP configuration contains many parameters including frequency location and bandwidth, subcarrier spacing, CP duration, CORESETs (for DL), search spaces (for DL), cell-specific PUCCH resources (for UL) and PUSCH (for UL). The signaling overhead of configuring BWPs in NTN may be significantly higher than that in a terrestrial network if more BWPs to be configured in NTN. However, the beams from the same satellite may provide similar services, and the desired BWP configurations may be similar. For example, the BWPs may be identical except the frequency location. To save on the signaling overhead of BWP configurations, the network can signal the full configuration of only one BWP (referred to as a reference BWP), and then signal only the difference of other BWPs relative to the reference BWP.

Proposal 5: Consider increasing the number of bits for the BWP ID (bwp-Id)

- The network can signal the difference between a BWP and a reference BWP to reduce the signalling overhead of BWP configuration.

Alternatively, the network can signal a frequency shift and a UE applies the frequency shift to a reference BWP, without changing the other parameters of the BWP. The reference BWP may be configured via RRC signaling. Also network can configure a basic unit of frequency shift, denoted as Δf. The frequency shift (how many units of frequency shift) may be signaled with DCI using a new field or reusing some existing field. For example, a value x can be signaled in the DCI to indicate a frequency shift value x×Δf to be applied an existing BWP.

For BWP frequency shift, the network needs to ensure that configured resources such as resources for SPS and PUCCH and reference signals such as CSI-RS still work without conflicts in the new BWP after frequency shift. This may or may not impose some constraints on network resource allocation and scheduling depending on network load and the type of services of a UE. Whenever the BWP configurations can apply in different frequency intervals, BWP switching through frequency shift can be an efficient tool to support scheduled/configured satellite beam switch.

Proposal 6: Support BWP frequency shift through DCI and/or configuration.

Proposal 7: Support RRC configuration to indicate BWP switching through DCI by either BWP ID or BWP frequency hopping.

To help the network decide on the best BWP or satellite beam for a UE to switch to, it is important that the UE measures the quality of its current serving satellite beam and neighboring satellite beams and reports the measurements back to the network. Frequent satellite beam switching leads to frequent reconfiguration of reference signals for radio link measurement. Because the measurement of a neighboring satellite beam may need to be performed in a BWP that is different from the active BWP, extra time is needed for frequency retuning and preparation for the frequency compensation and it is beneficial to minimize the measurement overhead.

Proposal 7: Consider enhancements on radio link monitoring to support efficient reference resource configuration and measurement outside the active BWP.

# Synch Raster & SIB Arrangements

The synch raster defines the allowed center frequencies for the SSBs, which a UE tries to detect during initial access. An important performance metric for initial access is the delay. For NTN, the synch raster does not have to be the same as the one in the terrestrial case. For example, the step size can be greater than that in the terrestrial network.

Proposal 9: Consider synch raster design to reduce initial access time.

For an NTN cell, some system information may change faster than other system information. For example, ephemeris information needs to be updated periodically. For reduced system overhead and UE power consumption, system information elements should be grouped in SIBs based on required updating rate..

Proposal 10: Consider grouping of system information elements in SIBs based on the required updating rate.

# Reduced MCS for power limited PUSCH transmission

The large propagation delay in NTN makes it difficult to do fast link adaptation. This could cause long bursts of errors if the channel quality suddenly degrades. For the UL transmission, the closed-loop power control command may be outdated due to large delay and a sudden channel degradation caused by shadowing can be mitigated by UL transmit power control based on the DL path loss estimation. However, UL power control on the PUSCH may be infeasible if the UE has already reached the maximum transmit power. One potential solution is to allow the network to configure a number of reduced (or fallback) MCS’s and let a UE choose a reduced MCS that compensates for the sudden degradation on the link quality.

Proposal 11: Support autonomous reduction of MCS for PUSCH at least for cases when UE is power limited

- Study the exact triggering condition and indication of the reduced MCS

# Signalling of polarization

Polarization is an effective tool for increasing system capacity. One coverage area can be served by two different polarizations and two polarizations can have different beam lay out as shown in Figure 6.

Figure 6. Polarization and beam layout.

To have broad support of different deployment options with regard to polarization, it is best to support at least polarization per BWP.

Proposal 12: Consider at least signalling of polarization per BWP.

# Antenna Diversity for PRACH

In RAN meeting #90e, a work item on NR coverage enhancements was approved [5]. Enhancements for PUSCH, PUCCH, and Masg3 are all included including increased repetitions, DMRS bundling across slots, and others. NR NTN coverage is expected to benefit from the work item. It is therefore important to examine PRACH coverage particularly for smart phones in S band.

Smart phones in FR1 typically have more than one linearly polarized transmit antennas. However, the exact polarization direction of antennas is unknown due to the uncertainty of handset orientation. In addition, phase shifts among antennas are typically unknown due to the separation of their associated RF chains. As a result, transmit diversity is important for PRACH transmission. In Figure 7, we compare the performance of PRACH format 2 with and without transmit antenna diversity. For both cases, coherent reception of the first half, i.e., the first and the second repetitions, and of the second half of the preamble was performed. The outputs of the first and the second half are then combined non-coherently.  Due to the long duration of PRACH format 2, it was found that with 200 Hz frequency error coherent reception among four repetitions will lead to worse performance that is not shown in the figure. For transmit diversity, the antenna switch point is set at CP/2 before the end of repetition 2 as shown in Figure 8. As such, the PRACH format 2 can accommodate a maximal differential delay CP/2 or delays in the range [-CP/4, CP/4]. Since the CP duration of PRACH format is 152.6 ms, CP/2 is more than enough to accommodate UEs with GNSS capability.

Figure 7. Antenna switching performance for PRACH format 2: the probability of miss detection under the constraint that the probability of false alarm is kept within the range [0.63%, 0.73%].

Figure 8. Antenna switching point for PRACH format 2.

Switching Point

CP/2

As can be seen from Figure 6, Antenna diversity provides about 1 dB gain at 1% miss detection rate. Since the bandwidth of PRACH format 2 is about 1 MHz, a X dB SNR for PRACH format 2 corresponds to  a X+7.6 dB SNR for 1 PRB PUSCH transmission of with 15 kHz subcarrier spacing. Hence it is important to allow transmit antenna diversity for PRACH format 2. To do so, we need to define the antenna switching point.

Proposal 13: For PRACH format 2 in NTN, antenna switching point is CP/2 before the end of the second repletion or 22232x64 Tc of the second repetition.

# Conclusions

In this contribution, we have discussed BWP operation and associated signalling enhancements to support different deployment options and efficient beam switching. In addition, some other issues and enhancements including PAPR reduction techniques are discussed. Concerning Rel-17 work, the following proposals have been developed, and observations have been made:

Observation 1: Different options for cell/beam/frequency planning call for flexible standard design.

Observation 2: Different beams of a satellite may have different carrier frequencies but the same corresponding UE transmit and receive spatial direction.

Proposal 1: Support satellite beam specific initial BWPs.

Proposal 2: 3GPP RAN1 to have an agenda item dedicated to SSB arrangements and BWP operation.

Proposal 3: Support the following SSB arrangements

- Alt 1: SSBs of all satellite beams in a same cell are transmitted within a same frequency interval and do not overlap in time

- Alt 2: SSBs of a cell are transmitted in different frequency interval, i.e., within their respective BWPs.

Proposal 4: Support signalling of the following configurations in SIB1

- initial BWPs of other satellite beams,

- CORSET#0 of other satellite beams if different from that of the serving beam.

Proposal 5: Consider increasing the number of bits for the BWP ID (bwp-Id)

- The network can signal the difference between a BWP and a reference BWP to reduce the signalling overhead of BWP configuration.

Proposal 6: Support BWP frequency shift through DCI and/or configuration.

Proposal 7: Support RRC configuration to indicate BWP switching through DCI by either BWP ID or BWP frequency hopping.

Proposal 8: Consider enhancements on radio link monitoring to support efficient reference resource configuration and measurement outside the active BWP.

Proposal 9: Consider synch raster design to reduce initial access time.

Proposal 10: Consider grouping of system information elements in SIBs based on the required updating rate.

Proposal 11: Support autonomous reduction of MCS for PUSCH at least for cases when UE is power limited

- Study the exact triggering condition and indication of the reduced MCS

Proposal 12: Consider at least signalling of polarization per BWP.

Proposal 13: For PRACH format 2 in NTN, antenna switching point is CP/2 before the end of the second repletion or 22232x64 Tc of the second repetition.

# References

- Chairman’s note, 3GPP RAN1#102e.

- Chairman’s note, 3GPP RAN1#103e.

- Chairman’s note, 3GPP RAN1#104e.

- 3GPP TR38.821, “Solutions for NR to support non-terrestrial networks (NTN)”.

- RP202928, “New WID on NR coverage enhancements,” China Telecom, 3GPP TSG RAN#90e, Dec. 2020.