3GPP TSG RAN WG1 #105-e                                                 R1-2104717

e-Meeting, May 10th – 27th, 2021

Source:               ZTE

Title:                    Remaining issues on 14-HARQ processes in DL for eMTC

Agenda item:      8.9.2

Document for:    Discussion and Decision

Introduction

In RAN1 #104bis e-meeting, the following agreements were achieved for 14-HARQ processes in DL for LTE-MTC [1]:

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be down-selected in RAN1#105-e from:

Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - FFS: The expression consisting of different subframe types.

  - FFS: Signaling Details.

Alt-2: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - FFS: The percentage of presence of non-BL/CE DL subframes and non-BL/CE UL subframes to be handled.

  - FFS: HARQ-ACK delay values and length of the HARQ-ACK delay set.

  - FFS: Signaling Details.

The following aspects will be considered towards the down-selection of one of the two alternatives (i.e., Alt-1 or Alt-2) for the HARQ-ACK delay solution:

- Total number of bits required in DCI

- Scenarios that can be handled, including:

(a) different numbers of scheduled HARQ processes per burst (including dynamically switching between more than 10 HARQ processes and 10 or less HARQ processes)

(b) different % of invalid subframes for both 10 and 40 SF long bitmaps

- Robustness against loss of DCIs

- Flexibility

- RRC signaling overhead

In this contribution, we discuss the remaining HARQ-ACK delay and DCI signaling issues on 14-HARQ processes in DL for LTE-MTC.

HARQ-ACK delay

As agreed in last RAN1 meeting, total number of required DCI bits, Scenarios to be handled, robustness against loss of DCIs, flexibility and RRC signaling overhead will be considered towards the comparison between Alt-1 and Alt-2.

The intention of introducing 14 HARQ processes is to achieve a target peak data rate. Ideally, the peak data rate of 14 HARQ processes is 705 kbps. However, the peak data rate of 14 HARQ processes (571 kbps) in the case of 20% presence of non-BL/CE subframes would be similar to the ideal peak data of 10 HARQ processes (588 kbps). When the percentage of presence of non-BL/CE subframes is larger than 20%, the peak data rate of 14 HARQ processes is much lower than the target peak data. For scenarios to be handled, there is no need to enable 14 HARQ processes for any percentage of presence of non-BL/CE subframes.

Proposal 1: For scenarios to be handled, there is no need to enable 14 HARQ processes for any percentage of presence of non-BL/CE subframes.

HARQ-ACK feedback delay may be different for different percentage of presence of DL/UL non-BL/CE subframes. Similar to legacy 10 HARQ processes, the “HARQ-ACK delay” can be expressed in terms of “absolute subframes” (Alt-2).

If percentage of invalid subframes is 0, when PDSCH scheduling delay is 2, the legacy HARQ-ACK delay value, i.e., 4~11, can be reused. When PDSCH scheduling delay is 7, the HARQ-ACK delay value is 12, 13. If Percentage of DL invalid subframes is 20% and percentage of UL invalid subframes is 0, as shown in Figure 1, the corresponding HARQ-ACK delay value range is 4~15 for PDSCH scheduling delay of 2 while the corresponding HARQ-ACK delay value range is 12~17 for PDSCH scheduling delay of 7.

Figure 1

If percentage of DL/UL invalid subframes is 20%, as shown in Figure 2, the maximum HARQ-ACK delay value would be 17 for PDSCH scheduling delay of 2 while the maximum HARQ-ACK delay value would be 19 for PDSCH scheduling delay of 7.

Figure 2

If only scenarios with percentage of DL/UL invalid subframes no larger than 20% is to be handled, the HARQ-ACK delay value range is 4~17 for PDSCH scheduling delay of 2 while the HARQ-ACK delay value range is 12~19 for PDSCH scheduling delay of 7. For Alt-2, there may be no need to introduce additional DCI bits for 14 HARQ processes if percentage of invalid subframes is not larger than 20%. Even though 14 HARQ processes would be used for the scenarios with percentage of invalid subframes larger than 20%, up to one additional bit (3~4 bits in total) is required to indicate HARQ-ACK delay. By adding up to one additional bit, Alt-2 is flexible to indicate the corresponding HARQ-ACK delay value. For Alt-2, if the maximum percentage of invalid subframes to be handled is determined, the potential HARQ-ACK delay values would be determined. There is no need to semi-statically configure the HARQ-ACK delay value set by RRC signaling.

Alternatively, the HARQ-ACK delay can be signaled through an expression consisting of different subframe types (Alt-1). For example, the HARQ-ACK delay is indicated by d1 BL/CE DL subframes + 1 subframe + d2 BL/CE UL subframes. To keep the same flexibility as Alt-2, Alt-1 needs up to 4 bits to indicate d1 and 2 bits to indicate d2. Therefore, the number of required DCI bits for Alt-1 would be larger than that for Alt-2.

Based on above analysis, Alt-2 is preferred for HARQ-ACK delay solution.

Proposal 2: For 14 HARQ processes, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

- The HARQ-ACK delay value range is 4~17 for PDSCH scheduling delay of 2.

- The HARQ-ACK delay value range is 12~19 for PDSCH scheduling delay of 7.

Details of signaling

For 14 HARQ processes, the PDSCH scheduling delay and HARQ-ACK feedback delay should be indicated by DCI. From the perspective of signaling overhead, the joint indication is preferred. From the perspective of peak data rate, feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions. Therefore, the repetition number field can be repurposed when 14-HARQ processes is enabled. The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

Observation 1: For 14-HARQ processes, the ‘repetition number’ field in DCI can be repurposed since feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions.

Proposal 3: The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

Conclusions

In this contribution, we have discussed the additional PDSCH scheduling delay and related HARQ-ACK delay to support DL 14-HARQ processes for H-FDD LTE-MTC. We make the following observations and proposals:

Observation 1: For 14-HARQ processes, the ‘repetition number’ field in DCI can be repurposed since feature of 14-HARQ processes is not enabled in the case of PDSCH repetitions.

Proposal 1: For scenarios to be handled, there is no need to enable 14 HARQ processes for any percentage of presence of non-BL/CE subframes.

Proposal 2: For 14 HARQ processes, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

- The HARQ-ACK delay value range is 4~17 for PDSCH scheduling delay of 2.

- The HARQ-ACK delay value range is 12~19 for PDSCH scheduling delay of 7.

Proposal 3: The PDSCH scheduling delay and HARQ-ACK feedback delay can be jointly indicated by repetition number field and HARQ-ACK delay field.

- References

- 3GPP, RAN1 Chairman’s Notes, RAN1 #104bis-e