3GPP TSG RAN WG1 #105-e		R1-2104737

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Discussion on Rel-17 Tx Switching enhancement

Agenda Item:	5.1

Document for:	Discussion and Decision

Introduction

In Rel-16, there was a work item for the support of UL Tx switching between two different NR carriers, i.e., the support of UL Tx switching between case 1 and case 2 [1]. By the great efforts of RAN1 and RAN4, Rel-16 specification supports the UL Tx switching between two different carriers for three scenarios: EN-DC, NR CA and SUL. On the basis of Rel-16 functionality, a Rel-17 package for further enhancement of UL Tx switching was approved in RAN#90e [2].  In RAN1#104bis-e meeting, some pertinent agreements were made as below [3]:

Agreements: For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA option 1 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Agreements: For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA option 2 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 1; 1T+1T; 1P+0P, 1P+1P, 0P+1P. Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Conclusion: - For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, - If the state of Tx chains is 1Tx on Band A and 1Tx on Band B, 1Tx is available simultaneously on both uplink carriers on band B for a UE. - If the state of Tx chains is 0Tx on Band A and 2Tx on Band B, 2Tx are available simultaneously on both uplink carriers on band B for a UE. Agreement: Send LS to RAN4 asking following question: - Question: For UL Tx switching in a band pair of a band combination, whether or not the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching. Agreements: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Agreements: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Case 3; 2T+0T; 2P+(0P+0P), 1P+(0P+0P). Agreements: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Agreements: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for UL CA Option 2 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P), 1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Case 3; 2T+0T; 2P+(0P+0P), 1P+(0P+0P).

In this contribution, we continue to discuss RAN1 impacts of Rel-17 Tx switching enhancement and provide ours views on the corresponding designs.

Discussion

- In Rel-16, there are two cases considered for UL Tx switching, which are case 1 and case 2, defined in below table [1]

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

According to the R17 WID [2], the Rel-17 enhancements on top of Rel-16 UL Tx switching focus on the following two aspects from the perspective of RAN1 impacts:

- UL MIMO are supported in all UL carriers

- More than 2 UL carriers are supported for UL Tx switching.

- For the two-carrier scenarios, the following cases are considered for Rel-17 enhancement

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

For the three-carrier scenarios where 1 carrier is in band A and 2 contiguous aggregated carriers are in band B, the following cases are considered in Rel-17 enhancement:

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

In order to ensure the consistency of Rel-16 and Rel-17 designs, we should reuse Rel-16 design principle and schemes as much as possible. Rel-16 UL Tx switching supports two different operation modes that are 'switchedUL' and 'dualUL', respectively. It is nature to support these two operation modes for Rel-17 UL Tx switching as well. If so, UE has the flexibility to choose different implementations for the support of Rel-17 UL Tx switching.

Proposal 1: For Rel-17 enhanced UL Tx switching of UL CA, support UE capability signaling for UE to report the support of 'switchedUL' or 'dualUL’.

2Tx-2Tx switching between 2 carriers

SUL and UL CA Option 1

- For the scenarios of SUL and UL CA Option 1, there are no simultaneous UL transmissions in the two UL carriers. Thus, the UL Tx switching is triggered by a transmission on a carrier after a transmission on the other carrier. Compared to Rel-16 feature, the new thing of Rel-17 counterpart is the support of UL MIMO in all carriers. However, the support of UL MIMO in carrier 1 does not affect the UL Tx switching procedure. Thus, Rel-16 mechanism of UL Tx switching can be reused in principle.

- For SUL scenario, the current version of TS 38.214 specifies the existence of switching period based on the transmissions on different carriers, regardless the number of transmission ports. Thus, we have the following observation and proposal:

Observation 1: For 2Tx-2Tx switching between 2 carriers of the SUL scenario, Rel-16 procedures can be reused.

Proposal 2: For 2Tx-2Tx switching between 2 carriers of SUL, the Rel-16 mechanism of uplink switching specified in Section 6.1.6.3 of TS 38.214 can be reused.

However, for the CA scenario, the current version of TS 38.214 specifies the existence of switching period based on the transmission ports on different carriers. There are two cases (namely case 2 and case 3) for the status of Tx chains for different carriers. Accordingly, there will various different combinations of two neighboring UL transmissions. Whether a switching period is needed or not should be investigated for each combination. By going through all the possible combinations, the following table summarizes all the transmission combinations where a switching period is needed:

| No | Preceding uplink transmission | Next uplink transmission |  |
| --- | --- | --- | --- |
| 1 | 1-port Carrier 1 | 1-port on Carrier 2 | Supported by Rel-16 mechanism |
| 2 | 1-port on Carrier 1 | 2-port on Carrier 2 | Supported by Rel-16 mechanism |
| 3 | 2-port on Carrier 1 | 1-port on Carrier 2 | Supported by Rel-16 mechanism |
| 4 | 2-port on Carrier 1 | 2-port on Carrier 2 | Not covered in Rel-16 |

Based on the table, we can see that Rel-16 mechanism doesn’t cover the scenario of the 2-port transmission on both carriers (namely, No.4 combination).

Observation 2: For 2Tx-2Tx switching between 2 carriers of UL CA Option 1, Rel-16 procedures doesn’t cover the case where the two neighboring 2-port transmissions are on different carriers.

Since the solution will be the same as UL CA Option 2, the detailed proposal will be provided in the next section for UL CA Option 2.

UL CA Option 2

For the scenario of UL CA Option 2, there can be simultaneous UL transmissions in the two UL carriers. There are three cases (namely, case 1, case 2 and case 3) for the status of Tx chains for different carriers. Accordingly, there will various different combinations of two neighboring UL transmissions. Whether a switching period is needed or not should be investigated for each combination. By going through all the possible combinations, the following table summarizes all the transmission combinations where a switching period is needed:

| No | Preceding uplink transmission | Next uplink transmission |  |
| --- | --- | --- | --- |
| 1 | 1-port on Carrier 1 and UE is under the operation state in which 2-port transmission can be supported | 1-port on Carrier 2 | Supported by Rel-16 mechanism |
| 2 | 1-port on Carrier 1 | 2-port on Carrier 2 | Supported by Rel-16 mechanism |
| 3 | 2-port on Carrier 1 | 1-port on Carrier 2 | Supported by Rel-16 mechanism |
| 4 | 1-port Carrier 1 and UE is under the operation state in which 2-port transmission cannot be supported | 1-port on Carrier 1 | Supported by Rel-16 mechanism |
| 5 | 2-port on Carrier 1 | 2-port on Carrier 2 | Not covered in Rel-16 |

Based on table, we can see that the combination of No.1-4 has been supported by Rel-16 mechanism. However, the combination of No.5 has not been covered by Rel-16.

Observation 3: For 2Tx-2Tx switching between 2 carriers of UL CA Option 2, Rel-16 procedures doesn’t cover the case where the two neighboring 2-port transmissions are on different carriers.

In order to address the issue shown by Observation 2 and 3, we suggest to introduce new rule on top of Rel-16 mechanism as below:

Proposal 3: For 2Tx-2Tx switching between 2 carriers of UL CA Option 1 or UL CA Option 2, the mechanism of uplink switching specified in Section 6.1.6.2 of TS 38.214 is reused with an additional rule as below

    - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

UL Tx switching between two bands with 3 carriers

- In order to facilitate the discussion, we have the following assumptions

- Carrier 1: in Band A

- Carrier 2 and Carrier 3: in Band B

- For SUL cases, Carrier 1 and Carrier 2 are the corresponding SUL and non-SUL carrier, respectively.

SUL and UL CA Option 1

The Rel-16 mechanism only describes the UL Tx switching from one carrier to another carrier, but cannot fully support the 3-carier scenarios. For example, Rel-16 spec does not differentiate these two combinations: {1 carrier in band A + 1 carrier in Band B} and {1 carrier in band B and another carrier in band B}. On the other hand, it is straightforward and easy to extent Rel-16 procedure to support the 3-carier scenarios. Following the similar principle of Rel-16 mechanisms, we can summarize all the transmission combinations that requires a UL Tx switching period

| No | Preceding uplink transmission | Next uplink transmission |  |
| --- | --- | --- | --- |
| 1 | 1-port on any carrier in one Band | 1-port on any carrier in another Band | Extend Rel-16 mechanism |
| 2 | 1-port on any carrier in one Band | 2-port on any carrier in another Band | Extend Rel-16 mechanism |
| 3 | 2-port on any carrier in one Band | 1-port on any carrier in another Band | Extend Rel-16 mechanism |
| 4 | 2-port on any carrier in one Band | 2-port on any carrier in another Band | New rule of Rel-17 |

In the table, “Extend Rel-16 mechanism” means the description can be reused by modifying the two carriers to two bands. Based on the summary, we have the following proposal:

Proposal 4: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if SUL is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - If the UE is to transmit any uplink channel or signal on a different uplink in one band from the preceding transmission occasion in another band based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2,.

Proposal 5: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if UL CA Option 1 is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of   NTx1-Tx2 on any of the two carriers.

    - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier in one band, and any transmission on another uplink carrier in another band.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

UL CA Option 2

Based on the similar analysis as SUL and UL CA Option 1, we can summarize all the transmission combinations that requires a UL Tx switching period for UL CA Option 2 as below:

| No | Preceding uplink transmission | Next uplink transmission |  |
| --- | --- | --- | --- |
| 1 | 1-port on any carrier in one band and UE is under the operation state in which 2-port transmission can be supported in this band | 1-port on any carrier in another Band | Extend Rel-16 mechanism |
| 2 | 1-port on any carrier in one band | 2-port on any carrier in another Band | Extend Rel-16 mechanism |
| 3 | 2-port on any carrier in one band | 1-port on any carrier in another Band | Extend Rel-16 mechanism |
| 4 | 1-port on any carrier in one band and UE is under the operation state in which 2-port transmission cannot be supported in this band | 1-port on any carrier in the same band | Extend Rel-16 mechanism |
| 5 | 2-port on any carrier in one band | 2-port on any carrier in another Band | New rule of Rel-17 |

Based on the summary, we have the following proposal:

Proposal 6: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if UL CA Option 1 is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any the same uplink carrier in the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any another uplink carrier in another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier in one band, and any transmission on another uplink carrier in another band.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

- We summarize our proposals in the following tables:

- Table: SUL scenarios

|  | - 2-Carrier | - 3-Carrier |
| --- | --- | --- |
| - Case 1 -> Case 2 | - / | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 2 -> Case 1 | - / | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 1 -> Case 3 | - / | - / |
| - Case 3 -> Case 1 | - / | - / |
| - Case 2 -> Case 3 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 3 -> Case 2 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |

- Table: CA scenarios

|  | - 2-Carrier | - 3-Carrier |
| --- | --- | --- |
| - Case 1 -> Case 2 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 2 -> Case 1 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 1 -> Case 3 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 3 -> Case 1 | - Reuse Rel-16 | - Rel-17 - (extend corresponding Rel-16 procedure) |
| - Case 2 -> Case 3 | - Rel-17 | - Rel-17 |
| - Case 3 -> Case 2 | - Rel-17 | - Rel-17 |

Conclusion

In this contribution, we discuss the Rel-17 scenarios for the enhancement of UL Tx switching. We analyzed whether Rel-16 procedure can be reused or Rel-17 new design is needed case by case and made the following observations and proposals:

Observation 1: For 2Tx-2Tx switching between 2 carriers of the SUL scenario, Rel-16 procedures can be reused.

Observation 2: For 2Tx-2Tx switching between 2 carriers of UL CA Option 1, Rel-16 procedures doesn’t cover the case where the two neighboring 2-port transmissions are on different carriers.

Observation 3: For 2Tx-2Tx switching between 2 carriers of UL CA Option 2, Rel-16 procedures doesn’t cover the case where the two neighboring 2-port transmissions are on different carriers.

Proposal 1: For Rel-17 enhanced UL Tx switching of UL CA, support UE capability signaling for UE to report the support of 'switchedUL' or 'dualUL’.

Proposal 2: For 2Tx-2Tx switching between 2 carriers of SUL, the Rel-16 mechanism of uplink switching specified in Section 6.1.6.3 of TS 38.214 can be reused.

Proposal 3: For 2Tx-2Tx switching between 2 carriers of UL CA Option 1 or UL CA Option 2, the mechanism of uplink switching specified in Section 6.1.6.2 of TS 38.214 is reused with an additional rule as below

    - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

Proposal 4: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if SUL is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - If the UE is to transmit any uplink channel or signal on a different uplink in one band from the preceding transmission occasion in another band based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2,.

Proposal 5: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if UL CA Option 1 is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of   NTx1-Tx2 on any of the two carriers.

    - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier in one band, and any transmission on another uplink carrier in another band.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

Proposal 6: For UL Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, if UL CA Option 1 is configured, the switching period is required when the two UL transmissions are switched between band A and band B, i.e.,

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 1-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any the same uplink carrier in the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - When the UE is to transmit a 1-port transmission on one uplink carrier in one band and if the preceding uplink transmission was a 1-port transmission on any another uplink carrier in another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

    - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier in one band, and any transmission on another uplink carrier in another band.

    - When the UE is to transmit a 2-port transmission on one uplink carrier in one band and if the preceding uplink transmission is a 2-port transmission on any another uplink carrier in another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

References

- RP-193266, RF requirements for NR frequency range 1 (FR1), Ericsson

- RP-202799, RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon

- Chairman’s notes of RAN1#104bis-e