3GPP TSG RAN WG1 #105-e		R1-2104739

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Agenda Item:	8.5.1

Document for:	Discussion and Decision

- Introduction

The Rel-17 WID of NR positioning enhancement was approved in RAN#90e [1], focusing on RAN1-centric objectives. Then, it was updated to include more objectives in RAN1#91e [2].

Mitigating of Rx/Tx timing delays is one of the key objectives. RAN1#104e had an extensive discussion on it and got some agreements as below [3]

Agreement: The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. Agreement: - Study specification impact for enabling a reference device with known location to support the following functionalities: - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF; - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF; - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations; - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information - FFS: The device with the known location being a UE and/or a gNB - FFS: Precision to which location of reference device is known - Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

There were also dozens of alternatives proposed for the gNB/UE Tx/Rx timing error mitigation. In RAN1#104bis-e meeting, some down-selection on the alternatives [4].

In this contribution, we will continue to discuss the enhancement for positioning to resolve the issues of UE Rx/Tx and gNB Rx/Tx timing delays from the perspective of positioning methods, measurement, signaling and procedures.

- Discussion on timing error group (TEG)

In RAN1#104bis-e meeting, there was a discussion on the association of TEG and MIMO SRS port, but no consensus was made.  From the perspective of specification, MIMO SRS can be used for some positioning, e.g., UL-RTOA. However, from UE side, there are only four usages for MIMO SRS resource(s) and positioning is not one of these four usages according to the NR specifications. In this sense, whether a MIMO SRS is used for positioning or not can be only known by network and is totally transparent to UE.

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]]

}

Thus, we have the following observation:

Observation 1: From the UE perspective, whether a MIMO is used for positioning or not is totally transparent

From the perspective of implementation, UE may transmit MIMO SRS by using different antenna virtualization or precoding according to the usage configured by RRC signaling. As MIMO SRS is widely used to support various NR features, we need to carefully check the potential impact on UE implementation of MIMO SRS for various use cases and scenarios.

Usage: non-codebook based UL MIMO. Let’s take a 2-Tx UE as an example.

- If each port is associated with one Tx, the timing difference of two SRS ports is related to the timing difference between different RF chains/antennas.

- If the two ports are implemented by vector [1,  1] and [1, -1] respectively, the timing difference of two ports is likely to be the same.  (e.g., used for non-codebook based UL MIMO)

Obviously, the two different mapping of SRS will have different assumption of timing difference and UE doesn’t know whether it will be used for positioning or not.  Moreover, the mapping of SRS port and physical antenna/PA may be different in different transmission occasions as UE will adjust the precoder according to the current DL channel status.

Usage: codebook based UL MIMO. Let consider a 4-Tx UE that supports UL Full power transmission. If a gNB configures 2-port SRS for UL full power transmission (Codebook based UL MIMO), then UE may have different mapping of MIMO SRS

- Alt.1: each port is mapped to one Tx

- Alt.2: each port is mapped to two Tx via CDD or precoding.

TEG information is likely to be different for Alt.1 and Alt.2. Moreover, UE implementation of Alt.1 or Alt.2 is transparent to gNB/LMF.

Usage: antenna switching. Let’s assume antenna switching of 2T4R.We cannot assume the timing difference between the two 2 Tx chains/antennas for the two transmissions (each corresponding to one 2-port SRS resource) are the same since antennas are in different positions and different switchers are used. That is to say, there may exist such case:  timing difference between the 2 ports of one SRS resource is within a margin, whereas timing difference between the 2 ports of another SRS resource is larger than this margin.

Based on the discussion, we can have the following observation:

Observation 2: At least for some usage(s) of MIMO SRS resource, the TEG information may be varying for different transmission occasions.

On the other hand, for a MIMO SRS resource configured with one usage in Rel-15/Rel-16, UE knows the intention of this SRS resource and may do some dedicated mapping for this SRS. If additional information on the MIMO SRS used for positioning is received, UE has to make some decision, for example

- Option1: Prioritize the configured usage

- Option2: Prioritize positioning

- Option3: Try to make some optimization to achieve a good trade-off between the configured usage and positioning

Different options will have different impact on the UE implementation and the performance of MIMO and positioning, which is not friendly from UE perspective. Thus, the benefits to asscociate TEG with MIMO SRS port are not well justified.

Proposal 1: Rel-17 doesn’t support the association of TEG with MIMO SRS port(s).

- TEG based method for DL Positioning

In RAN1#104-bis, an agreement for the enhancement of DL-TDOA positioning by down-selecting some basic schemes out of 17 alternatives:

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

According to the agreement, the basic enhancement scheme for DL positioning is determined to mitigate the UE Rx timing errors and/or TRP Tx timing errors. One of the remaining issues is that how to signal the information of TEG. One way is to introduce TEG indices. In this way, the measurement result will be reported along with the index of the associated TEG and the transmit signals (namely PRS) are also reported or signaled with the associated TEG index. However, this way may suffer two disadvantages:

- TEG is a concept for discussion and it is tightly related to hardware and UE implementation. From the perspective of RAN1 design, a general principle is to avoid the usage of some terminology tightly coupled with UE implementation.

- Association of TEG index with each measurement result or DL PRS resource will introduce larger signaling overhead

In order to avoid the above-mentioned disadvantages, one possible approach is to group the measurement results or DL PRS resources. That is to say, if UE reports more than one group of positioning measurement results, then the results from different groups are implicitly associated with different UE Rx TEGs. Likewise, the DL PRS resources from different groups are belonging different TRP Tx TEGs in implicit way. Thus, we have the following observation and proposal:

Observation 3: The information of TEG can be implicitly indicated by grouping the measurements and DL PRS resources.

Proposal 2: Rel-17 supports UE to report the grouping of DL TDOA measurement results via LPP for the implicit indication of UE Rx TEG information.

Proposal 3: Rel-17 supports LMF to signal the grouping of DL PRS resources to UE via LPP for the implicit indication of TRP Tx TEG information.

Proposal 4: Rel-17 supports gNB to signal the grouping of DL PRS resources to LMF via NRPPa for the implicit indication of TRP Tx TEG information.

- TEG based method UL Positioning method

In RAN1#104bis-e, an agreement for the enhancement of UL-TDOA positioning by down-selecting some schemes out of 10 alternatives:

Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the Signaling, procedures, and UE capability

One of the remaining issues is about the MIMO SRS. As we discussed in Section 2, whether a MIMO resource is used for positioning or not is transparent to UE. The analysis of potential impacts on positioning and MIMO are also applicable for this issue. Thus, we propose not to support the association of MIMO SRS resources and TEG information.

Proposal 5: Rel-17 doesn’t support the association of TEGs with MIMO SRS resources.

Another open issue is how to report the association of TEG and Positioning SRS resources. There are two alternatives:

- Alt.1: The association information is sent directly from UE to LMF

- Alt.2: The association information is first provided to gNB and then forwarded to LMF

The SRS resource for positioning is configured by gNB and signaled to UE via RRC. That means the SRS configuration for each are “locally” configured with in the serving cell of the gNB. As a result, there may be some ambiguity from LMF side on the associated TRPs of the SRS for Alt.1. For example, two gNBs (gNB A, gNB B) with long distance has UE A and UE B, respectively. If each gNB configures two SRS resources for its own UEs for positioning and these SRS configuration for UE A and UE B are the same, then same LMF will cannot differentiate these SRS resource for positioning, e.g., whether the reported association information for UE A is applicable for gNB A or gNB B.  In order to address this issue, some new design will be needed and the design principle of Rel-16 signaling mechanism will be broken. Thus, Alt.1 is not a good approach for the signaling of association information.

Observation 4: Alt.1 that the association information is sent directly from UE to LMF may lead to ambiguity issue at LMF.

According to the above discussion, we suggest to select Alt.2 to move forward.

Proposal 6: For the association information of TEGs and SRS resources for positioning, Rel-17 supports UE to report it to gNB and gNB to forward it to LMF via NRPPa.

- TEG based method Multi-RTT Positioning Method

In RAN1#104bis-e, an agreement for mitigating UE Rx/Tx timing errors in DL+UL positioning by down-selecting some basic schemes out of 9 alternatives:

Agreement: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS; - Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to the one of the 2 following options: - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS. - For both alterntives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the signalling, procedures, and UE capability

For DL-TDOA and UL-TDOA positioning method, the Rx TEG and Tx TEG are used. As a general framework for all positioning method is preferred. Moreover, the signaling of the association information can reuse that of DL-TDOA and UL-TDOA, rather than some new design based on RxTx TEG. Thus, we suggest to support Alt.1 for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning.

Proposal 7: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- UE provides the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS

In RAN1#104bis-e, an agreement for mitigating UE Rx/Tx timing errors in DL+UL positioning by down-selecting some basic schemes out of 8 alternatives:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures

Based on the same reason for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, we suggest to support Alt.1 for mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning as well. Thus, we have the following proposal:

Proposal 8: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF

- Multiple results for a report

In RAN1#104e meeting, it was also agreed to report one or more measurement instances in a single measurement report to LMF.

From UE perspective, the current LPP signaling has supported multiple measurement results in a report. For example, a UE can report up to 4 results for NR Multi-RTT measurements and NR DL-TDOA measurements. The corresponding signaling is copied as below

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-UE-RxTxTimeDiff-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-Multi-RTT-AdditionalMeasurements-r16 NR-Multi-RTT-AdditionalMeasurements-r16 OPTIONAL, ... } NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 CHOICE { k0-r16 INTEGER (0..1970049), k1-r16 INTEGER (0..985025), k2-r16 INTEGER (0..492513), k3-r16 INTEGER (0..246257), k4-r16 INTEGER (0..123129), k5-r16 INTEGER (0..61565), ... }, nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingQuality-r16 NR-TimingQuality-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126) OPTIONAL, nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16 OPTIONAL, ... } NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF NR-DL-TDOA-AdditionalMeasurementElement-r16

Therefore, the current signaling can support this feature with an exception that more than 4 results may be needed.

Proposal 9: The current LPP signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF, with potential extension to support a larger number than 4.

From network perspective, the current NRPPa signaling has supported multiple measurement results in a report. For example, a UE can report up to 16384 results for NR Multi-RTT measurements and NR UL RTOA measurements. The corresponding signaling is copied as below

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Measured Result Item; 1 .. <maxnoPosMeas>. >CHOICE Measured Results Value; M. >>UL Angle of Arrival; M; 9.2.38. >>UL SRS-RSRP; M; INTEGER (0..126). >>UL RTOA; M; 9.2.39. >>gNB Rx-Tx Time Difference; M; 9.2.40. >Time Stamp; M; 9.2.42. >Measurement Quality; O; 9.2.43. >Measurement Beam Information; O; 9.2.57. Range bound; Explanation. maxnoPosMeas; Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384..

Therefore, the current NRPPa signaling can support  reporting of multiple measurement instances. For the same type of measurement instances, the current mechanism seems sufficient. However, for the different types of measurement instances, there should be some association between them. For example, the TROA and AoA results are measured on the same SRS resource(s).

Observation 5: The current NRPPa signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF

Proposal 10: No enhancement is needed for the current NRPPa signaling to support the feature that TRP reports one or more measurement instances with the same quantity in a single measurement report to LMF.

Proposal 11:  Enhancement on the association of measurement instances should be introduced to support the feature that TRP reports one or more measurement instances with different quantities in a single measurement report to LMF.

- Reference device based solution

There was another study on the mitigation of Tx/Rx timing delays based on the reference device with known location. To be specific, two different approaches can be considered for this reference device based solution:

Approach 1:

1.1	determine the real location of the UE (non-3GPP)

1.2	transit the real location information to UE (non-3GPP)

1.3	UE does measurements (3GPP)

1.4	NW requests the reporting (3GPP)

1.5	 UE sends measurement results and its real location information to LMF (3GPP)

1.6	LMF do the calculation (based on NW implementation, transparent to the spec)

Approach 2:

2.1	determine the real location of the UE (non-3GPP)

2.2	transit the real location information to LMF (non-3GPP)

2.3	 UE does measurements (3GPP)

2.4	NW requests the reporting (3GPP)

2.5	 UE sends reporting to LMF (3GPP)

2.6	LMF do the calculation (based on NW implementation, transparent to the spec)

The differences between the two approaches are as below

- The real location information is transmitted to UE and LMF in Component 1.2 and 2.2, respectively. They are both based on non-3GPP technology.

- Component 1.5 needs additional standardization efforts as UE needs to report the real positioning info (i.e., known location) to LMF whereas Step 2.5 can reuse the current spec.

Based on the above discussion, we have the following observation:

Observation 6: Approach 1 and Approach 2 can achieve the same functionality, but Approach 2 has no or less impact.

Proposal 12: Implementation based approach (Approach 2) is supported for the positioning based on reference device(s) with known location.

- Conclusion

In this contribution, we presented our views on the impact of Tx/Rx timing delays on the DL, UL and DL/UL positioning and we also discussed the potential enhancements to address the issues. Based on the discussions, the following observations and proposal were made:

Observation 1: From the UE perspective, whether a MIMO is used for positioning or not is totally transparent

Observation 2: At least for some usage(s) of MIMO SRS resource, the TEG information may be varying for different transmission occasions.

Observation 3: The information of TEG can be implicitly indicated by grouping the measurements and DL PRS resources.

Observation 4: Alt.1 that the association information is sent directly from UE to LMF may lead to ambiguity issue at LMF.

Observation 5: The current NRPPa signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF

Observation 6: Approach 1 and Approach 2 can achieve the same functionality, but Approach 2 has no or less impact.

Proposal 1: Rel-17 doesn’t support the association of TEG with MIMO SRS port(s).

Proposal 2: Rel-17 supports UE to report the grouping of DL TDOA measurement results via LPP for the implicit indication of UE Rx TEG information.

Proposal 3: Rel-17 supports LMF to signal the grouping of DL PRS resources to UE via LPP for the implicit indication of TRP Tx TEG information.

Proposal 4: Rel-17 supports gNB to signal the grouping of DL PRS resources to LMF via NRPPa for the implicit indication of TRP Tx TEG information.

Proposal 5: Rel-17 doesn’t support the association of TEGs with MIMO SRS resources.

Proposal 6: For the association information of TEGs and SRS resources for positioning, Rel-17 supports UE to report it to gNB and gNB to forward it to LMF via NRPPa.

Proposal 7: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- UE provides the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS

Proposal 8: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, Rel-17 NR support Alt.1, i.e.,

- gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF

- Proposal 9: The current LPP signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF, with potential extension to support a larger number than 4.

Proposal 9: The current LPP signaling can support the feature that UE reports one or more measurement instances in a single measurement report to LMF, with potential extension to support a larger number than 4.

Proposal 10: No enhancement is needed for the current NRPPa signaling to support the feature that TRP reports one or more measurement instances with the same quantity in a single measurement report to LMF.

Proposal 11:  Enhancement on the association of measurement instances should be introduced to support the feature that TRP reports one or more measurement instances with different quantities in a single measurement report to LMF.

Proposal 12: Implementation based approach (Approach 2) is supported for the positioning based on reference device(s) with known location.

- Reference

RP-202900 New WID on NR Positioning Enhancements

RP-210903, Revised WID on NR Positioning Enhancements

Chairman’s note of RAN1#104e

Chairman’s note of RAN1#104bis-e