3GPP TSG RAN WG1 #105-e		R1-2104769

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Discussion on other aspects

Agenda Item:	8.2.7

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our simulation assumptions, CM/PAPR and required SNR results for PUCCH based on the agreed simulation assumptions.

- PUCCH evaluation

  - Evaluation assumptions

In this section, we present the CM/PAPR comparison and the required SNR fulfilling the detection criterion:

- For PF0/1 (payload of 1 or 2 bits) the detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and the criterion is defined as the SNR for which P(ACK to Error) ≤ 1% AND P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only.

- For PF4 (payload greater than 2 bits): the detection criterion is the UCI block error probability BLER ≤ 1% (as in TS38.104 Section 8.3.6)

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier Spacing [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) Note: If other values used, companies to report values |
| PUCCH Frequency Hopping | On the two extreme sides of the bandwidth |
| PUCCH Frequency Domain Resource Mapping | N_RB contiguous RBs per hop (with all REs allocated per PRB) N_RB = 1, 12, 32 for 120kHz N_RB = 1, 4, 8 for 480kHz N_RB = 1, 2, 4 for 960kHz Full-PRB: all RE in the PRB are allocated for PUCCH Sub-PRB: - Case 1: 6 consecutive RE in PRB are allocated for PUCCH - Case 2: 1 RE per PRB is allocated for PUCCH |
| Waveform | CP-OFDM for PF0 DFT-s-OFDM for PF4 |
| CP Type | Normal CP |
| Channel Model | TDL-A model as defined in of TR38.901 Section 7.7.2: - Delay spread (DS) = {5ns, 10ns, 20ns} |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| PA Model | None |
| gNB TRP PN Model | Zero phase noise |
| UE PN Model | Zero phase noise |
| Pre-loaded Tx EVM | 0% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |

Table 1: LLS assumption for CM/PAPR and required SNR

  - CM/PAPR and required SNR

The PUCCH format 0 and 4 simulation results are summarized in the table 2 and table 3, where frequency hopping is applied and PUCCH resource are allocated on two extreme sides of the band, e.g. for 1RB, the first hop is the first PRB and the second hop is the last PRB of the simulation band, and so on for N_RB>1.

For PUCCH format 4, OCC = [1,1] with spreading factor 2.

The analysis of the simulation results are given in our companion document [2].

| PF 0 | SCS (kHz) | N_RB (RB) | mapping | sequence | CM | Required SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DS=5 ns | DS = 10 ns | DS = 20 ns |
| 1bit | 120 | 1 | Full-PRB | 12-ZC | 0.78 | 0.22 | 0.04 | -0.13 |
|  |  | 12 | Full-PRB | Cycling | 1.476 | -10.86 | -10.42 | -9.49 |
|  |  | 12 | Full-PRB | Long ZC | 2.048 | -10.33 | -10.40 | -9.62 |
|  |  | 12 | Sub-case 1 | Long ZC | 2.693 | -7.63 | -7.45 | -6.65 |
|  |  | 12 | Sub-case 2 | 12-ZC | 0.7756 | 0.44 | 0.36 | 1.21 |
|  |  | 32 | Full-PRB | Cycling | 3.60 | -14.38 | -13.91 | -12.20 |
|  |  | 32 | Full-PRB | Long ZC | 2.38 | -14.33 | -13.87 | -11.63 |
|  |  | 32 | Sub-case 1 | Long ZC | 6.93 | -11.31 | -10.87 | -10.09 |
|  | 480 | 1 | Full-PRB | 12-ZC | 0.78 | -0.28 | 0.35 | 0.62 |
|  |  | 4 | Full-PRB | Cycling | 3.3 | -5.46 | -4.65 | -3.81 |
|  |  | 4 | Full-PRB | Long ZC | 2.22 | -5.59 | -4.65 | -3.53 |
|  |  | 8 | Full-PRB | Cycling | 2.36 | -8.03 | -6.91 | -4.40 |
|  |  | 8 | Full-PRB | Long ZC | 1.59 | -8.10 | -6.84 | -4.26 |
|  | 960 | 1 | Full-PRB | 12-ZC | 0.78 | 0.78 | 0.60 | 0.85 |
|  |  | 2 | Full-PRB | Cycling | 2.678 | -1.97 | -2.02 | -0.77 |
|  |  | 2 | Full-PRB | Long ZC | 0.78 | -1.99 | -2.46 | -1.24 |
|  |  | 4 | Full-PRB | Cycling | 3.30 | -4.49 | -4.53 | -2.02 |
|  |  | 4 | Full-PRB | Long ZC | 2.16 | -4.46 | -4.76 | -1.91 |
| 2 bit | 120 | 1 | Full-PRB | 12-ZC | 0.78 | 0.60 | 0.62 | 0.18 |
|  |  | 12 | Full-PRB | Cycling | 1.476 | -10.13 | -9.86 | -9.09 |
|  |  | 12 | Full-PRB | Long ZC | 2.048 | -9.91 | -10.17 | -9.17 |
|  |  | 12 | Sub-case 1 | Long ZC | 2.692 | -6.97 | -6.75 | -5.96 |
|  |  | 12 | Sub-case 2 | 12-ZC | 0.775 | 0.84 | 0.97 | 1.56 |
|  |  | 32 | Full-PRB | Cycling | 3.60 | -13.98 | -13.44 | -11.02 |
|  |  | 32 | Full-PRB | Long ZC | 2.38 | -14.08 | -13.39 | -11.28 |
|  |  | 32 | Sub-case 1 | Long ZC | 6.93 | -10.96 | -10.54 | -8.18 |
|  | 480 | 1 | Full-PRB | 12-ZC | 0.78 | 0.31 | 0.70 | 1.18 |
|  |  | 4 | Full-PRB | Cycling | 3.3 | -5.47 | -4.14 | -3.54 |
|  |  | 4 | Full-PRB | Long ZC | 2.22 | -5.44 | -4.08 | -3.15 |
|  |  | 8 | Full-PRB | Cycling | 2.36 | -7.74 | -6.41 | -3.94 |
|  |  | 8 | Full-PRB | Long ZC | 1.59 | -8.04 | -6.31 | -3.90 |
|  | 960 | 1 | Full-PRB | 12-ZC | 0.78 | 1.35 | 0.71 | 1.24 |
|  |  | 2 | Full-PRB | Cycling | 2.6785 | -1.24 | -1.61 | -0.88 |
|  |  | 2 | Full-PRB | Long ZC | 0.78 | -1.24 | -1.34 | -0.97 |
|  |  | 4 | Full-PRB | Cycling | 3.30 | -4.15 | -4.10 | -1.82 |
|  |  | 4 | Full-PRB | Long ZC | 2.16 | -3.92 | -3.82 | -1.75 |

Table 2: simulation results of PUCCH format 0 for CM/PAPR and required SNR

| PF 4 | SCS (kHz) | N_RB (RB) | Coding rate | mapping | CM | Required SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DS=5 ns | DS = 10 ns | DS = 20 ns |
| 4-bit | 120 | 1 | 0.0278 | Full-PRB | 3.09 | -5.1 | -5.53 | -5.75 |
|  |  | 12 | 2.32e-3 | Full-PRB | 1.964 | -14.8 | -15.2 | -15.07 |
|  |  | 12 | 0.0278 | Sub-case 2 | 3.087 | -4.117 | -4.037 | -3.895 |
|  |  | 32 | 8.68e-4 | Full-PRB | 1.87 | -18.28 | -18.33 | -17.83 |
|  | 480 | 1 | 0.0278 | Full-PRB | 3.09 | -5.66 | -5.59 | -5.16 |
|  |  | 4 | 0.0069 | Full-PRB | 2.157 | -11.75 | -10.88 | -10.71 |
|  |  | 8 | 0.0035 | Full-PRB | 1.87 | -13.72 | -13.19 | -12.23 |
|  | 960 | 1 | 0.0278 | Full-PRB | 3.09 | -4.3 | -5.25 | -5.16 |
|  |  | 2 | 0.0139 | Full-PRB | 2.415 | -8.068 | -7.909 | -8.06 |
|  |  | 4 | 0.0069 | Full-PRB | 2.16 | -10.63 | -11.02 | -10.39 |
| 11-bit | 120 | 1 | 0.0764 | Full-PRB | 2.58 | -2.74 | -2.66 | -3.23 |
|  |  | 12 | 0.0063 | Full-PRB | 2.231 | -11.21 | -11.628 | -12.68 |
|  |  | 12 | 0.0764 | Sub-case 2 | 2.58 | -1.122 | -1.39 | -1.063 |
|  |  | 32 | 0.0024 | Full-PRB | 2.26 | -15.58 | -15.93 | -15.70 |
|  | 480 | 1 | 0.0764 | Full-PRB | 2.58 | -2.98 | -2.67 | -2.30 |
|  |  | 4 | 0.0191 | Full-PRB | 2.216 | -8.83 | -8.17 | -8.15 |
|  |  | 8 | 0.0095 | Full-PRB | 2.26 | -11.23 | -10.60 | -10.26 |
|  | 960 | 1 | 0.0764 | Full-PRB | 2.58 | -2.22 | -2.55 | -2.70 |
|  |  | 2 | 0.0382 | Full-PRB | 2.35 | -5.54 | -5.435 | -5.71 |
|  |  | 4 | 0.0191 | Full-PRB | 2.22 | -8.19 | -8.86 | -8.24 |
| 22-bit | 120 | 1 | 0.1528 | Full-PRB | 2.58 | -0.34 | -0.6 | -0.77 |
|  |  | 12 | 0.0127 | Full-PRB | 1.82 | -11.135 | -11.5 | -11.946 |
|  |  | 12 | 0.1528 | Sub-case 2 | 2.58 | 0.433 | 0.566 | 1.34 |
|  |  | 32 | 0.0048 | Full-PRB | 1.63 | -15.03 | -15.66 | -15.43 |
|  | 480 | 1 | 0.1528 | Full-PRB | 2.58 | -0.85 | -0.68 | -0.2 |
|  |  | 4 | 0.0382 | Full-PRB | 2.148 | -7.05 | -6.91 | -6.972 |
|  |  | 8 | 0.0191 | Full-PRB | 1.93 | -10.42 | -10.08 | -9.60 |
|  | 960 | 1 | 0.1528 | Full-PRB | 2.58 | 0.33 | -0.36 | -0.44 |
|  |  | 2 | 0.0764 | Full-PRB | 2.363 | -3.65 | -3.732 | -4.16 |
|  |  | 4 | 0.0382 | Full-PRB | 2.15 | -6.74 | -7.56 | -7.26 |

Table 3: simulation results of PUCCH format 4 for CM/PAPR and required SNR

In the table 4, we provide the PUCCH format 0 simulation results with and without interference for full-PRB and sub-case 2 mapping respectively, where two users are multiplexed by using different CS indexes, i.e., UE1 uses CS index 0, and UE2 uses CS index 3. The CM and required SNR for UE1 are summarized in the following table.

| PF 0 | SCS 120 (kHz) | Scenario | N_RB (RB) | mapping | sequence | CM | Required SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | DS=5 ns | DS = 10 ns | DS = 20 ns |
|  |  | Without interference | 12 | Full-PRB | 144-ZC | 2.048 | -10.33 | -10.40 | -9.62 |
|  |  |  |  | Sub-case 2 | 12-ZC | 0.7756 | 0.44 | 0.36 | 1.21 |
|  |  | With interference |  | Full-PRB | 144-ZC | 2.048 | -10.15 | -10.16 | -9.49 |
|  |  |  |  | Sub-case 2 | 12-ZC | 0.7756 | 0.49 | 0.29 | 0.91 |

Table 4: simulation results of user multiplexing for CM/PAPR and required SNR

- Conclusion

This contribution mainly provides the evaluation assumptions, CM/PAPR and required SNR results for PUCCH enhancement.

- Reference

| [1] | R1-2102127, “FL Summary 4 for Enhancements for PUCCH formats 0/1/4”, Moderator (Ericsson), RAN1#104-e |
| --- | --- |
| [2] | R1-2104766, “Discussion on enhancements for PUCCH format 0/1/4”, OPPO, RAN1#105-e |