3GPP TSG-RAN WG1 Meeting #105-e	R1-2104811

e-Meeting, May 19th – 27th, 2021

Agenda Item:	8.4.2

Source:	Ericsson

Title:	On UL time and frequency synchronization enhancements for NTN

Document for:	Discussion

# Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the current work item description (WID) can be found in [1]. One objective is to specify uplink (UL) time and frequency synchronization enhancements for NTN.

TR 38.821 [3] summarizes the observations made during the Release 16 study on solutions for NR to support NTN. With regards to downlink (DL) time and frequency synchronization it was concluded that robust performance can be supported without taking any special measures on the network side. The potential impact is limited to increased UE complexity.

For the UL time and frequency synchronization it was agreed that existing preamble formats can be reused during random access in case the UE can perform pre-compensation of timing and frequency offset. The suitability of the pre-compensation option is dependent on the UE’s ability to support GNSS in the different RRC states, on the availability of accurate ephemeris data, and on the UE’s ability to make use of the GNSS and ephemeris information for performing time and frequency pre-compensation to compensate for Doppler effects.

The agreements made in RAN103-e, RAN1#104-e and RAN1#104bis-e can be found in the Appendix.

In the next sections, we express our view on this critical functionality. For convenience the term access offset will be used to denote both the time and frequency offset used by the UE when transmitting in the UL to compensate for propagation delay and Doppler shift, respectively.

# UE transmission timing calculations for NTN

## Overview of propagation delays in NTN

To determine the UE transmission timing, it is necessary to calculate the delay of the service link and the feeder link at a given point in time. These delays can be calculated based on the distances between the UE and satellite, and GW and satellite, respectively. However, to accurately calculate the delay that a given signal has experienced (or will experience), it is the distance between the position of the transmitting node at the time of transmission to the position of the receiving node at the time of reception that determines the delay. Since the time of transmission is different from the time of reception, this does not correspond to the distance between the nodes at any given point in time. In other words, accurately determining the link delays requires a dynamic view of the signal propagation, as opposed to a static snapshot.

Figure 1 illustrates the feeder and service link delays when transmission delays are taken into account. A DL signal is sent from GW at time t0, relayed by the satellite at time t1, and used for DL synchronization by the UE at time t2. An UL signal is sent from the UE at time t3, relayed by the satellite at time t4, and received by the GW/gNB at time t5. The timing relationships can be expressed as follows:

- t1=t0+df,DL=t0+1cpGWt0-psatt1

- t2=t1+ds,DL=t1+1cpsatt1-pUEt2

- t3=t2+dUE (UE-internal delay from DL sync to UL TX)

- t4=t3+ds,UL=t3+1cpUEt3-psatt4

- t5=t4+df,UL=t4+1cpsatt4-pGWt5

where psatt, pGWt and pUEt are satellite, GW and UE position vectors, respectively, in ECI (earth-centered inertial) coordinates, df,DL, df,UL, ds,DL and ds,UL  denote the delays of the DL feeder link, UL feeder link, DL service link, and UL service link, respectively,  and c is the speed of light. Note that in ECI, even though the GW moves at the speed of up to 466 m/s due to Earth rotation, it is still relatively slow compared to satellite speed of up to ~7700 m/s.

The UE needs to derive df,DL, df,UL, ds,DL and ds,UL to calculate its transmission timing. In the following we will look closer at these delays, how they can be calculated and whether it is necessary to take the dynamics into account. Note that the detailed methods for calculation should be up to implementation. What is relevant from a specification perspective is how the delays are defined and how accurate they need to be calculated.

Figure 1: Signal timing relationships for NTN.

## DL service link delay

First consider the DL service link delay, ds,DL of the signal used by the UE to synchronize at time t2. This is determined by the following expression:

ds,DL=1cpsatt2-ds,DL-pUEt2

Here, the reception time at the UE, t2, is known while the transmission time of the satellite, t2-ds,DL, is unknown. Therefore, the satellite position needs to be propagated backwards in time to calculate the service link delay. This can be done with various numerical methods. A very accurate solution can be found using e.g. the Newton-Raphson method. Below we investigate two simplified solutions from accuracy point of view.

- A crude approximation is to ignore the satellite movement and use the satellite-UE distance at sync time t2, i.e.:

ds,DL≈1cpsatt2-pUEt2

This method will be denoted “sync time” in the figure legend below.

- A simple method to take into account the approximate movement of the satellite is to first approximate the delay based on the distance at sync time t2 and then calculate the satellite position at time t2-d1 and re-calculate ds,DL:

d1=1cpsatt2-pUEt2

ds,DL=1cpsatt2-d1-pUEt2

This method will be denoted “two-step” in the figure legend below.

To assess the performance of the methods, simulations were run. A LEO satellite with circular orbit is modelled. Orbit altitudes ranging from 600 km to 1800 km are tested. For each altitude, the satellite passes from horizon to horizon (≥10° elevation angle) above a UE and a gateway on the Earth surface. Every 10 ms the UE calculates the DL service link delay based on UE position and satellite ephemeris. The two methods described above are tested and the performance is shown in Figure 2. The reference ds,DL is calculated using the Newton-Raphson method. The blue curve corresponds to the “sync time” method. The red curve corresponds to the “two-step” method.

It can be seen that using the “sync time” method (i.e., ignoring the satellite movement during the time-of-flight from satellite to UE) results in an error of up to 0.25 µs, depending on satellite altitude. This corresponds to approximately 5.3% of the CP length for a PUSCH or PUCCH with 15 kHz SCS and 43% of the CP with 120 kHz SCS.

Also, it can be seen that the method taking into account satellite movement gives good accuracy.

This is summarized in the following observation:

When calculating the DL service link delay of a signal, ignoring the satellite movement during the signal propagation from satellite to UE will lead to a calculation error of up to 0.25 µs, which corresponds to 5% of the PUCCH/PUSCH CP length for 15 kHz SCS and 43% of the CP length for 120 kHz SCS.

Using a low-complex approximative “two-step” method is sufficient to get a good approximation of the impact of satellite movement on DL service link delay.

RAN1 to discuss how to take into account satellite movement during DL signal propagation from satellite to UE when defining the UE-specific TA.

Figure 2: Max DL service link delay estimation error for different approximative calculation methods.

## UL service link delay

Similar calculations can be made when determining the other contributions to the TA. From the UE perspective, the UL service link delay must also be determined before the UE can transmit. The UE will then target a particular UL slot at the gNB receiver (based on UL grant, etc.), i.e., the desired reception time at the gNB (t5 in Figure 1) is known. Based on this and on signaled common TA from the network, the UE can calculate the desired reception time at the satellite (t4 in Figure 1), and further the required transmission time from the UE (t3 in Figure 1). Therefore, the UL service link delay will be the solution to the following equation:

ds,UL=1cpsatt4-pUEt4-ds,UL

In this equation it is only the UE position that depends on the delay, and since the UE speed is relatively low (even when considering the impact of Earth rotation), it can probably be ignored in practice.

RAN1 to discuss how to take into account satellite movement during UL signal propagation from UE to satellite when defining the UE-specific TA.

## Feeder link delay

For the feeder link delay, similar considerations need to be made by the network.

For the DL, the UE will first determine the service link delay based on the sync time (t2 in Figure 1) as described in section 2.2, after which it can derive the time the DL signal was relayed at the satellite (t1 in Figure 1). Thus, the network needs to provide the UE with an expression for the DL feeder link delay that is a function of the time the DL signal passed the satellite, t1.

For the UL, the desired reception time at the GW/gNB (t5 in Figure 1) is known to the UE. To determine the UL feeder link delay for the signal received by the GW/gNB at time t5, the UE needs to be provided with an expression for the the UL feeder link delay that is a function of the time t5.

It has been proposed to characterize the feeder link delay using a common TA base value and a common TA drift rate, i.e., as a linear approximation. Looking at the equations for calculating the DL and UL feeder link delays,

df,DL=1cpsatt1-pGWt1-df,DL

df,UL=1cpsatt5-df,UL-pGWt5

it can be noted that the expressions are not the same (even if disregarding the different variable names) since in the first case, the time of transmission is before the time of reception, and vice versa in the second case. This implies that, to be accurate, it would be necessary to broadcast separate expressions for common TA for DL and UL. Since this is undesirable, it is necessary to use an approximative expression for the (one-way) feeder link delay, disregarding the node movement from transmission to reception time:

df(t)=1cpsatt-pGWt

To enable compensation for the feeder link delay, the network would then approximate this function with a piecewise linear function, characterized by base values and drift rate values that are broadcast at regular intervals.

Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift

In order to minimize the error of disregarding node movement from time of transmission to time of reception, it is important that the UE uses the time at which the signal passes the satellite when calculating Dcommont since the speed of the other nodes is much lower. For DL, this corresponds to

df,DL=Dcommont1=Dcommont2-ds,DL

where t2 is sync time (see Figure 1) and ds,DL is the DL service link delay as described in section 2.2. For the UL, the corresponding expression is

df,UL=Dcommont4=Dcommont5-df,UL

Similar to ds,DL, the UL feeder link delay is defined by the solution to an equation. However, since Dcommon(t) is a line, the solution is simple.

In section 3.2, we describe how the above analysis should be taken into account for TA design.

RAN1 to discuss how to take into account satellite movement during signal propagation between gateway and satellite when defining the common TA.

# Time and frequency compensation

## Reference point for time and frequency

The reference point is the point in the network at which UL time slots are received at their nominal time and carriers are transmitted and received at their nominal DL and UL frequencies, respectively.  The reference point for time and frequency has been the BS antenna connector, as defined in section 4.3.1 of TS 38.104 [6], up to and including release-16. For release-17 NTN WI, there are at least two options, the BS antenna connector of the gNB (as in release-16) or the antenna connector of the satellite.

If the satellite is time and frequency reference point, then UL signals will be time and frequency aligned and received at nominal UL frequency at satellite RX. Then all time slots will be misaligned by twice the feeder link delay and the frequency will be affected by feeder link Doppler shift. This is shown in Figure 3.

Figure 3: Impact of having the time reference for UL timing synchronization at the satellite on DL/UL framing alignment.

All time slots will be misaligned by twice the feeder link delay and the frequency will be affected by the feeder link Doppler shift, if the satellite is used as reference for time and frequency requirements.

Using satellite as reference for time and frequency requirements severely affects compatibility with existing Rel-16 gNB.

If the gNB is the reference point, then UL signals will be time and frequency aligned at gNB. This is illustrated in Figure 4.

Figure 4: Impact of having the time reference for UL timing synchronization at the gNB on DL/UL framing alignment.

In our view, the reference points should be under control of the network. At least the option of having gNB as the reference point should be supported. The UE should be responsible for autonomously determining and pre-compensating the access offset required for time and frequency alignment at the satellite, while the network may configure additional time and frequency offsets that are signaled to the UE. The signaled offsets are determined by the gNB and may be used to move the reference point e.g. to the gNB.

It should be noted that even if the location of the reference point can be flexible from a physical layer point of view, as discussed under agenda item 8.4.1 (see e.g. [5]), having the reference point in the satellite will require continuous adjustment of the DL and UL time at the gNB that is complex to maintain and will also require significant additional specification effort in other RAN groups to specify how the timing adjustment must be performed by the gNB, the accuracy levels expected for this operation and testing procedures for product validation. Also, a varying DL/UL slot misalignment at the gNB, will have significant impact to the scheduler.

The reference point for time and frequency in an NTN should be under control of the network and should at least support the option of having gNB as the reference point.

The support of a reference point for time that results in a time-varying DL/UL slot misalignment at the gNB will have significant impact to gNB implementation and result in heavy specification work in other RAN groups.

## Time synchronization

### Rel-16 time synchronization

The current (Rel-16) TA is defined as follows [2]:

TTA=(NTA+NTA,offset)×Tc

It consists of two parts:

- NTA is the timing advance which is dynamically controlled by the network. For initial access (PRACH transmission), it is zero. After initial access, it is updated through an absolute timing advance command in RAR and subsequently through timing advance commands in MAC CE.

- NTA,offset is a semi-static offset which is dependent on frequency band and LTE/NR coexistence and can be explicitly signaled by the NW.

### NTN time synchronization

At RAN1#104bis-e, the following agreement was made:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

The TA for NTN UE consists of four components. These are further discussed below.

#### Closed-loop TA, NTA

Regarding the closed loop control of TA, it is proposed to reuse the legacy procedures. Since the open control loops will handle most of the TA drift, the legacy procedures should be able to manage the residual offset, if any.

The closed-loop component of the TA, NTA, should be defined as in Rel-16.

#### Fixed TA offset, NTA,offset

NTA,offset is proposed to be defined as in Rel-16 (see section 3.2.1).

The fixed TA offset, NTA,offset, should be defined as in Rel-16.

#### UE-specific TA, NTA,UE-specific

The purpose of the UE-specific TA is to compensate for the RTT of the service link. The UE-specific TA is autonomously determined by the UE based on GNSS-acquired UE position and serving satellite position from broadcast ephemeris.

As discussed in Section 2, to accurately calculate the service link delay, it is necessary to take into account that the UL and DL signals are offset in time and therefore subject to different service link delays. It is also necessary to take into account when calculating the service link delay that the satellite moves during the transmission across the service link.

NTA, UE-specific=ds,DL+ds,UL/Tc

where ds,DL and ds,UL are the DL and UL service link delays derived by the UE, respectively.

The UE calculates the UE-specific TA (in Tc units) as follows:
NTA, UE-specific=ds,DL+ds,UL/Tc
where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

NOTE: Whether the granularity of the UE-specific TA should be Tc or a multiple of Tc is FFS.

#### Common TA, NTA,common

The purpose of the common TA is to compensate for the RTT of the feeder link and possibly other latencies in the satellite-gNB path. The common TA varies with time. As discussed in Section 2, it is beneficial to consider UL and DL delays separately since the UL and DL signals are offset in time and therefore subject to different delays. The common delay can be approximated by a linear function as follows:

Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift

where:

t is the time the signal passes the satellite

Tcommon,base is a reference time of the broadcast common delay

Dcommon, base is the common one-way delay at time Tcommon,base

Dcommon, drift is the common one-way delay drift rate

The parameters Dcommon, base, Dcommon, drift are broadcast regularly by the network to allow the UE to calculate the common TA, whereas the reference (epoch) time Tcommon,base could be broadcast or implicit from the DL subframe in which the parameters are signaled.

The network broadcasts parameters describing the (one-way) common delay by a linear function as follows:
Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift
where:
t is the time the signal passes the satellite
Tcommon,base is a reference time of the broadcast common delay that may be implicitly (derived from DL subframe) or explicitly signaled
Dcommon, base is the common one-way delay at time Tcommon,base 
Dcommon, drift is the common one-way delay drift rate

The common TA is then calculated as the sum of the DL and UL common delays, and expressed in Tc units as follows:

NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc

where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to be transmitted by the UE will be relayed by the satellite.

The UE determines the common TA (in Tc units) as follows:
NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc
where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite.

NOTE: Whether the granularity of the common TA should be Tc or a multiple of Tc is FFS.

To demonstrate the need for drift rate information, consider a scenario with VLEO at 200 km, which can be considered as a worst case. In this scenario, the feeder link RTT (i.e., DL+UL feeder link delay) changes at a rate of up to 50 µs/s as shown in Figure 5.

Figure 5: Feeder link RTT drift rate versus time of passing VLEO 200 satellite.

This means that a common TA that is correct at one slot will be off by 10% of the CP length 10 slots later, as shown in Table 1. Very frequent signaling of the common delay parameters would be needed if drift rate is not included that allows the UE to autonomously update NTA, common with time.

Table 1: Feeder link RTT drift for different SCS

| SCS [kHz] | Slot length [ms] | CP length PUCCH/PUSCH [µs] | Feeder link RTT drift per slot [% of CP] | Slots before drift exceeds 10% of CP |
| --- | --- | --- | --- | --- |
| 15 | 1 | 4.69 | 1.1% | 10 |
| 30 | 0.5 | 2.34 | 1.1% | 10 |
| 60 | 0.25 | 1.17 | 1.1% | 10 |
| 120 | 0.125 | 0.59 | 1.1% | 10 |

The benefit of providing a drift rate is further illustrated in Figure 6. The blue curve shows the feeder link RTT during one pass of a VLEO 200 satellite. The red curve shows a linear approximation based on a base value + drift rate value that are updated every 1.6 seconds (1600 slots with SCS=15 kHz). The approximation error is less than 10% of the CP length. The significant reduction in signaling justifies the use of drift rate for the common TA.

Drift rate information significantly reduces the signaling load for common delay.

The characterization of the common delay should include drift rate information.

Figure 6: Feeder Link RTT vs time during one pass of a VLEO 200 satellite.

##### Granularity of common TA

The granularity of NTA,common will impact the UE transmit timing error and thereby the timing error of the signal received by the gNB. The accuracy requirements for UE transmit timing are not yet defined but the design target should be that the total timing errror can be handled by the CP also in the presence of channel time dispersion. There are several sources of error that add up to the total UE transmit timing error. Some sources of error such as DL synchronization inaccuracy, UE positioning inaccuracy and satellite ephemeris inaccuracy may be costly to reduce. To allow for more feasible requirements on these error souces, the quantization error of common TA should preferably consume only a small part of the total error budget. Therefore, the NTA,common applied by the UE should have a granularity of e.g. 64/2μ, which corresponds to less than 1% of the normal CP length for PUCCH/PUSCH.

The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length.

In section 3.2.2.4 it is proposed to broadcast the parameters Dcommon, base and Dcommon, drift reflecting the one-way common delay and one-way common delay drift rate, respectively, from which the UE calculates NTA,common. Dcommon, base should be broadcast with granularity (64/2μ)Tc to match the proposed granularity of NTA,common. The maximum propagation delay between gateway and satellite is 271 ms for a GEO satellite [3]. This means that the total range of Dcommon, base of 0-271 ms can be signaled with granularity (64/2μ)Tc using 23-26 bits (depending on μ).

It can be noted that the feeder link delay cannot be smaller than what is implied by the satellite height. Therefore the number of bits may be reduced by making the encoding of common delay dependent on the broadcast serving satellite ephemeris. E.g., for a GEO satellite, the satellite height of 35786 km can be derived from the satellite ephemeris, which implies that the lower bound on the common delay is 238 ms. The range of 238-271 ms can be signaled with with granularity (64/2μ)Tc using 20-23 bits (depending on μ).

The common delay can be signaled with granularity (64/2µ)Tc using 23-26 bits. The number of bits can be reduced by taking into account that delay values lower than what is implied by the satellite height need not be signaled.

The common delay drift rate Dcommon, drift can be in the range ±25 µs/s. The timing error due to quantization error of Dcommon, drift depends on the prediction horizon and the granularity of Dcommon, drift. This is FFS.

#### TA margin

To account for the estimation uncertainty of the UE-specific TA, one could subtract a TA margin, whose value could be included in the broadcast common TA. The network can derive this value by taking into account the maximum TA uncertainty (this could include any factors that the network may deem necessary). With this, the TA command in Msg2 does not need to support bipolar range.

For NTN the TA command in Msg2 would be used to compensate for errors in the UE-autonomous determination of the UE-specific TA. The accuracy of the UE-specific TA is expected to be significantly better than the maximum range of the legacy TA command. Therefore, there is no need to extend the range of the legacy TA command in Msg2.

If the common TA includes a margin for maximum estimation error of the UE-specific TA, and the accuracy requirements of the UE-specific TA are appropriately set, the current unipolar TA command in Msg2 is sufficient, i.e., bipolar TA command or extended TA range is not needed in Msg2.

## Frequency synchronization

Doppler shift compensation will be needed in UL. Optionally, Doppler shift pre-compensation may also be used in DL. Doppler shift compensation can be in the form of pre-compensation by the transmitter, post-compensation by the receiver, or both.

- The gNB may optionally apply frequency pre-compensation to its DL signals. The DL frequency pre-compensation may compensate for Doppler shift on the feeder link and the service link. The gNB cannot fully compensate for the service link Doppler shift for all UEs in the cell, but it may eliminate the Doppler shift for some reference point, e.g., the cell or beam center. The UE will then only experience a residual Doppler shift, i.e., the difference in Doppler shift at its position compared to the reference point. The purpose of DL pre-compensation is to reduce the frequency range in which the UE has to search for SSB when synchronizing to a cell/beam. This may reduce synchronization time and battery consumption.

- The UE must pre-compensate its UL signals. This pre-compensation may compensate for Doppler shifts on both the feeder link and the service link. At a minimum, the UE needs to pre-compensate for the Doppler shift on the service link since this Doppler shift is UE-specific and would otherwise destroy the UL orthogonality. In addition, the UE should be able to pre-compensate for the Doppler shift of the feeder link, but the gNB may also post-compensate for this Doppler shift at the RX side. The UE should be responsible for determining the Doppler shift of the service link while the network should be responsible for determining additional Doppler shift for which the UE should pre-compensate, i.e., the feeder link or parts thereof. The network should be able to provide the UE with information about the amount of additional UL frequency pre-compensation.

To facilitate this, it is proposed to provide two new broadcast parameters related to frequency pre-compensation:

- The amount of DL pre-compensation applied. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. A UE that uses the gNB DL frequency as frequency reference needs this information to determine its nominal UL TX frequency. The amount of DL pre-compensation applied should be implementation specific.

- The amount of additional UL pre-compensation to apply. This parameter tells the UE how much pre-compensation to apply in the UL in addition to the amount the UE has determined is needed to compensate for the Doppler shift on the service link. The gNB can set this parameter equal to the amount of Doppler shift on the feeder link to eliminate the need for post-compensation at the gNB receiver, but it may also set it to a different value, or omit it, in case it prefers to perform (partial) post-compensation. The value of this parameter should be implementation specific.

In summary, the following are proposed:

If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link.

The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link.

As an alternative to the feeder link Doppler shift compensation described above with broadcast of a common Doppler shift to be applied by the UE, it should be considered to let the gateway pre/post-compensate the common Doppler shift over the feeder link in a way transparent to the UE and gNB. This has been proposed e.g. in [4]. This is a convenient solution from the gNB and UE perspective but since it is currently unclear how to view the gateway from a 3GPP architecture point of view, the possibility to put requirements on the gateway to perform Doppler shift compensation needs further study.

RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE.

## Alternative UE-centric solution

In the previous sections, solutions for time and frequency compensation assume that the gNB/gateway is responsible for determining the feeder link compensation and instructing UE to apply additional access offsets accordingly. Alternatively, if the position of a reference point (which may or may not be the gateway) and the UL and DL carrier frequencies of the feeder link were signaled to the UE, the UE could autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. This would simplify the time and frequency compensation procedures.

As mentioned, the reference point may or may not be the gateway. It is up to network to configure. In scenarios where it is acceptable to broadcast gateway position, the network can configure the reference point to be the position of the gateway. In scenarios where it is not acceptable to broadcast gateway position, the network can configure the reference point to be the “approximate” position of the gateway. If the reference point is not the gateway, it is up to gNB how to handle the (small) variations in timing and frequency offset.

If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signaled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link.

# GNSS requirements

RAN1 needs to discuss how to deal with the case of no GNSS coverage. The relevant case of interest is when the NR-NTN satellite is accessible while GNSS is out of coverage. For instance, it is reasonable to assume that neither of the two systems will provide indoor coverage. However, there may be corner cases where e.g. a mountain or a high-rise building blocks line-of-sight to a majority of the GNSS satellites but not all NTN satellites. How to deal with this case needs further discussion.

RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage.

# Satellite ephemeris

## Epoch time

The issue of implicit or explicit epoch time for serving satellite ephemeris was discussed at RAN1#104bis-e without conclusion.

If explicit epoch time is used, the epoch time is broadcast together with the satellite ephemeris. In this case there is no ambiguity regarding the epoch time for the satellite ephemeris.

If implicit epoch time is used, the epoch time of the broadcast satellite ephemeris is defined by the DL subframe containing the ephemeris information. In this case, it is necessary to define how the epoch time is derived from the subframe. This has not been discussed in RAN1 and needs to be clarified before the issue of implicit or explicit epoch time is concluded.

How and where the broadcast ephemeris data is generated is an implementation choice. E.g., it may be generated in a node external to the gNB (e.g., an NTN Control Center, NCC) or generated (propagated) by the gNB based on ephemeris data from an NCC. The latency between the generation of the ephemeris data and the UE can be tens or hundreds of milliseconds (depending on the satellite orbit), during which time the satellite moves a considerable distance. Therefore, it is necessary to define a reference point that defines the epoch time when the subframe containing the ephemeris data is at that reference point. It is then the responsibility of the network to generate ephemeris data that matches the implicit epoch time at the reference point. Possible reference points include the gNB transmitter, the satellite transmitter and the UE receiver.

- Reference point at gNB: The epoch time is the time when the subframe is transmitted from the gNB. This allows the network (e.g., the NCC or the gNB) to predict the satellite ephemeris with a fixed prediction time offset corresponding to the delay between the NCC and the gNB. It also allows the UE to determine the epoch time based on broadcast common TA information and satellite ephemeris, assuming that the common TA reflects the full delay from satellite to gNB.

- Reference point at satellite transmitter: The epoch time is the time when the subframe is transmitted from the satellite. Due to the time-varying delay of the feeder link, this option requires the network (e.g., the NCC or the gNB) to predict the satellite ephemeris with a time-varying prediction time offset corresponding to the delay between the NCC and the satellite). It allows the UE to determine the epoch time based on the broadcast satellite ephemeris.

- Reference point at UE receiver: The epoch time is the time when the subframe is received by the UE. This is impractical since the UE-specific delay is unknown and differs between UE. The differential delay in a cell can be in the order of 3 ms for LEO [3], during which time the satellite moves up to approximately 25 m. Between cells of the same satellite the differential delay is in the order of 4-6 ms for a LEO satellite, which implies that cell-specific ephemeris data would be needed for different cells of the same satellite.

If the reference point for epoch time for satellite ephemeris is at the gNB transmitter, the network (e.g., the NCC or the gNB) can generate satellite ephemeris with a fixed prediction time offset. The UE can determine the epoch time based on broadcast common TA information and satellite ephemeris.

If the reference point for epoch time for satellite ephemeris is at the satellite transmitter, the network (e.g., the NCC or the gNB) must generate satellite ephemeris with a time-varying prediction time offset, adapted to the feeder link delay. The UE can determine the epoch time based on satellite ephemeris.

If the reference point for epoch time for satellite ephemeris is at the UE receiver, the network (e.g., the NCC or the gNB) cannot accurately predict the satellite ephemeris at epoch time due to the differential delay in an NTN cell. Also, cell-specific ephemeris data for the same satellite might be needed due to differential delay between cells of the same satellite.

Given that the generation of ephemeris data is an implementation choice that may involve equipment from multiple vendors, the specifications should allow flexibility in the design. A natural solution is that the reference point for epoch time is the reference point for time as defined by the common TA.

RAN1 to discuss how to define the reference point for epoch time if implicit epoch time for satellite ephemeris is used.

Another aspect of implicit epoch time is that it may require the gNB to know the absolute transmission time of the subframes in which the ephemeris is broadcast. E.g., the gNB may be provided with satellite ephemeris data with explicit epoch time from an external source and needs to propagate it to the implicit epoch time of the subframe, or the gNB may request satellite ephemeris data from an external source for a given epoch time.

The accuracy requirements for UE transmission timing are not yet determined but there are many sources of error that contribute to the total error. The impact of epoch time inaccuracy should be kept as low as possible. Assuming e.g. that the impact of the epoch time inaccuracy should not exceed 5% of the shortest CP length (PUCCH/PUSCH for 120 kHz SCS), this corresponds to a time offset of 29 ns. This corresponds to a satellite position error of c*29∙10-92=4.4 m (where c is the speed of light). A LEO 600 satellite travels this distance in approximately 0.6 ms. This implies that the gNB needs to know the absolute time of its transmissions with an accuracy of 0.6 ms.

If implicit epoch time is used, the gNB may need to know the absolute transmission time of the subframe containing the ephemeris data with an accuracy of 0.6 ms, assuming that the impact of the epoch time inaccuracy on the UE transmission time error should not exceed 5% of the shortest CP length.

RAN1 to discuss the impact of implicit epoch time on gNB.

## Satellite position prediction accuracy

To investigate how accurately the position and velocity of a satellite can be predicted based on orbital elements, orbital data from Eutelsat is analyzed. The data consists of both orbital elements (a, e, ω, Ω, i, M0) and state vectors (x, y, z, vx, vy, vz) for every second of two hours of a LEO satellite orbit at approximately 535 km altitude. The orbital elements at a given time t are used to predict the satellite position and velocity at time t+Δt using the algorithm described in [7]. The predicted position and velocity are compared to the actual satellite position and velocity from Eutelsat state vector data and the error magnitudes are calculated. This is repeated for all available times t in the data files and for Δt ranging from 1 s to 120 s. The maximum (over t) prediction error magnitude for satellite position and satellite velocity are shown in Figure 7 and Figure 8, respectively. The results indicate that prediction from orbital elements is possible with acceptable accuracy in the order of tens of seconds ahead in time (depending on the required accuracy). Note that the presented prediction errors are the total error magnitude, which will only partially be in the UE-satellite direction and therefore only partially impact the access offset determination.

Figure 7: Max position prediction error versus prediction time.

Figure 8: Max velocity prediction error versus prediction time.

Orbit prediction from orbital elements is possible with acceptable accuracy in the order of tens of seconds ahead in time (depending on the required accuracy).

Satellite ephemeris with sufficient accuracy to support timing and frequency offset pre-compensation shall be made available to the NR NTN UE.

RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation with accuracy specified by RAN4.

# Conclusions

In the previous sections, we discuss UL time and frequency synchronization enhancements for NTN. We made the following observations:

Observation 1	When calculating the DL service link delay of a signal, ignoring the satellite movement during the signal propagation from satellite to UE will lead to a calculation error of up to 0.25 µs, which corresponds to 5% of the PUCCH/PUSCH CP length for 15 kHz SCS and 43% of the CP length for 120 kHz SCS.

Observation 2	Using a low-complex approximative “two-step” method is sufficient to get a good approximation of the impact of satellite movement on DL service link delay.

Observation 3	All time slots will be misaligned by twice the feeder link delay and the frequency will be affected by the feeder link Doppler shift, if the satellite is used as reference for time and frequency requirements.

Observation 4	Using satellite as reference for time and frequency requirements severely affects compatibility with existing Rel-16 gNB.

Observation 5	The support of a reference point for time that results in a time-varying DL/UL slot misalignment at the gNB will have significant impact to gNB implementation and result in heavy specification work in other RAN groups.

Observation 6	Drift rate information significantly reduces the signaling load for common delay.

Observation 7	The common delay can be signaled with granularity (64/2µ)Tc using 23-26 bits. The number of bits can be reduced by taking into account that delay values lower than what is implied by the satellite height need not be signaled.

Observation 8	If the common TA includes a margin for maximum estimation error of the UE-specific TA, and the accuracy requirements of the UE-specific TA are appropriately set, the current unipolar TA command in Msg2 is sufficient, i.e., bipolar TA command or extended TA range is not needed in Msg2.

Observation 9	If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signaled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

Observation 10	If the reference point for epoch time for satellite ephemeris is at the gNB transmitter, the network (e.g., the NCC or the gNB) can generate satellite ephemeris with a fixed prediction time offset. The UE can determine the epoch time based on broadcast common TA information and satellite ephemeris.

Observation 11	If the reference point for epoch time for satellite ephemeris is at the satellite transmitter, the network (e.g., the NCC or the gNB) must generate satellite ephemeris with a time-varying prediction time offset, adapted to the feeder link delay. The UE can determine the epoch time based on satellite ephemeris.

Observation 12	If the reference point for epoch time for satellite ephemeris is at the UE receiver, the network (e.g., the NCC or the gNB) cannot accurately predict the satellite ephemeris at epoch time due to the differential delay in an NTN cell. Also, cell-specific ephemeris data for the same satellite might be needed due to differential delay between cells of the same satellite.

Observation 13	If implicit epoch time is used, the gNB may need to know the absolute transmission time of the subframe containing the ephemeris data with an accuracy of 0.6 ms, assuming that the impact of the epoch time inaccuracy on the UE transmission time error should not exceed 5% of the shortest CP length.

Observation 14	Orbit prediction from orbital elements is possible with acceptable accuracy in the order of tens of seconds ahead in time (depending on the required accuracy).

Observation 15	Satellite ephemeris with sufficient accuracy to support timing and frequency offset pre-compensation shall be made available to the NR NTN UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to discuss how to take into account satellite movement during DL signal propagation from satellite to UE when defining the UE-specific TA.

Proposal 2	RAN1 to discuss how to take into account satellite movement during UL signal propagation from UE to satellite when defining the UE-specific TA.

Proposal 3	RAN1 to discuss how to take into account satellite movement during signal propagation between gateway and satellite when defining the common TA.

Proposal 4	The reference point for time and frequency in an NTN should be under control of the network and should at least support the option of having gNB as the reference point.

Proposal 5	The closed-loop component of the TA, NTA, should be defined as in Rel-16.

Proposal 6	The fixed TA offset, NTA,offset, should be defined as in Rel-16.

Proposal 7	The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA.

Proposal 8	The network broadcasts parameters describing the (one-way) common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay that may be implicitly (derived from DL subframe) or explicitly signaled Dcommon, base is the common one-way delay at time Tcommon,base  Dcommon, drift is the common one-way delay drift rate

Proposal 9	The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to which the UE applies the TA will be relayed by the satellite.

Proposal 10	The characterization of the common delay should include drift rate information.

Proposal 11	The granularity of the NTA,common applied by the UE should be 64/2µ to limit its impact to the total timing error budget to less than 1% of the CP length.

Proposal 12	If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link.

Proposal 13	The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link.

Proposal 14	RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE.

Proposal 15	Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link.

Proposal 16	RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage.

Proposal 17	RAN1 to discuss how to define the reference point for epoch time if implicit epoch time for satellite ephemeris is used.

Proposal 18	RAN1 to discuss the impact of implicit epoch time on gNB.

Proposal 19	RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation with accuracy specified by RAN4.

# References

RP-210908, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #91e, March 2021.

TS 38.211, Physical channels and modulation.

TR 38.821, Solutions for NR to support non-terrestrial networks.

R1-2100520, “Considerations on UL timing and frequency synchronization in NTN”, Thales, Rakuten mobile, January 2021

R1-2101296, “Further views on DL-UL timing relationship for NTN operation”, Nokia, January 2021.

TS 38.104, Base Station (BS) radio transmission and reception, V16.5.0.

Eng, M., and René Schwarz, "Keplerian Orbit Elements → Cartesian State Vectors," available at https://downloads.rene-schwarz.com/download/M001-Keplerian_Orbit_Elements_to_Cartesian_State_Vectors.pdf.

Eng, M., and René Schwarz, "Cartesian State Vectors → Keplerian Orbit Elements," available at https://downloads.rene-schwarz.com/download/M002-Cartesian_State_Vectors_to_Keplerian_Orbit_Elements.pdf.

Agreements from previous RAN1 meetings

At RAN1#103-e, the following agreements were made:

Agreement:

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

Agreement:

An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.

Agreement:

- In NTN, the network may broadcast

- A common timing offset value

  - FFS details of the common timing offset

- FFS: A common timing drift rate

- Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows:

Where:

is derived from the User specific TA self-estimation

is derived at least from the common timing offset value if broadcasted by the network. The granularity of  and whether  is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed.

- depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2.

- is specified in TS 38.211 section 4.1.

- Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

Agreement:

An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

At RAN1#104-e, the following agreements were made:

Agreement:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

- FFS: Operation of closed loop and open loop TA control

Agreement:

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

- FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

Agreement:

RAN1 should send an LS to RAN4 with the following questions:

Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

Agreement:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

At RAN1#104bis-e, the following agreements and conclusions were made:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

Agreement:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

Conclusion:

The orbital propagator model to be used at UE side can be left to implementation.

In this description it is assumed that the network wants to fully compensate for the feeder link delay but it could be left to network implementation to decide what parameter values to broadcast.