3GPP TSG RAN WG1 #105-e	R1-2104837

e-Meeting, 10th - 27th May, 2021

Title:	Discussion on the frequency range definition for above 52.6GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.7

Document for:	Discussion and Decision

Introduction

According to the outcome of the study item on supporting NR above 52.6 GHz [1], in 3GPP RAN #91 plenary e-meeting in March 2021, an update WID [2] was approved to extend NR operation up to 71 GHz considering both licensed and unlicensed operation.

It is noted in the approved WID that RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6~71 GHz or the existing FR2 shall be extended to cover frequency range from 52.6~71 GHz. However, although RAN plenary discussed this issue in RAN #91 plenary e-meeting, it did not make a final decision. The conclusion is to postpone the final decision to the next plenary meeting and request to the WGs' analysis and input, as copied below. In the last RAN1 e-meeting in April, no progress was made although the issue was discussed. In this contribution, we present some initial understandings from RAN1 perspective.

Conclusion: RAN1, RAN2 and RAN4 are asked to provide its analysis or recommendation to RAN#92e (June 21) on how to introduce the 52.6-71GHz frequency range

Discussion on frequency range definition

The frequency range of the existing FR2 defined in TS 38.101-2 is from 24.25 GHz to 52.6 GHz. For defining frequency range of 52.6-71 GHz, there are four alternatives as show in Figure 1.

- Alt 1: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR3 for 52.6-71 GHz;

- Alt 2: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR2x for 52.6-71 GHz;

- Alt 3: Maintain the existing FR2 for 24.25~52.6 GHz, introduce FR2x for 52.6-71 GHz and FR2-e for 24.25~71 GHz;

- Alt 4: Extend the exiting FR2 to cover 24.25~71 GHz, and introduce FR2a for 24.25~52.6GHz and FR2b for 52.6-71GHz. The extended FR2 comprises FR2a and FR2b.

Figure 1: Three alternatives for defining the 52.6~71GHz frequency range

In Table 1, we analyze the impacts of the above four different frequency definitions for 52.6-71 GHz on the issues in RAN1 specifications.

Table 1: The impacts on RAN1 specifications by different frequency range definitions

| FR related issues in RAN1 specs | Explanation for the issues | Alt 1/2 spec impacts | Alt 3 spec impacts | Alt 4 spec impacts |
| --- | --- | --- | --- | --- |
| Transition time NRx-Tx and NTx-Rx | For the existing FR2, - NRx-Tx=13792. - NTx-Rx=13792. For 52.6~71GHz, - FFS and decided by RAN4 | Add the values for FR3/FR2x | Add the values for FR2x or replace FR2 with FR2-e | If same values are supported for FR2a and FR2b, there is no spec impacts. Otherwise, specify FR2a and FR2b respectively. |
| offsetToPointA | For the existing FR2, it is expressed in units of resource blocks assuming 60kHz SCS. For 52.6~71GHz, no need to change 60 kHz reference SCS. | Add the description for FR3/FR2x | Add the description for FR2x or replace FR2 with FR2-e | No spec impacts. |
| Sub-carrier level offset | For the existing FR2, SSB type B is defined, and For 52.6~71GHz, - SSB type C may be need to be defined - FFS for and | Define and for new SSB type. (e.g. SSB type C). Clarify the relationship between the range of and the existence of CORESET0 for FR3/FR2x | Define and for new SSB type. (e.g. SSB type C). Clarify the relationship between the range of and the existence of CORESET0 for FR2x | Define and for new SSB type. (e.g. SSB type C). Whether there are spec impacts depends on whether FR2a and FR2b can share the same design |
| SSB pattern in a half frame | For 52.6~71GHz, - 120kHz is supported; - FFS for 240/480/960 kHz SCS | (New) SSB pattern cases with (new) SCSs need to be defined for FR3/FR2x | (New) SSB pattern cases with (new) SCSs need to be defined for FR2x | (New) SSB pattern cases with (new) SCSs need to be defined for FR2b. Whether Case D/E can be reused for extended FR2 depends on whether FR2a and FR2b can share the same design. |
| PRACH | For 52.6~71GHz, sequence lengths, SCSs and random access configurations may need to be enhanced | Specify sequence lengths, SCSs and random access configurations supported by FR3/FR2x | Specify sequence lengths, SCSs and random access configurations supported by FR2x | Specify sequence lengths, SCSs and random access configurations supported by FR2a (instead of FR2) and FR2b respectively |
| UL power control | A lot of power related parameters are defined for the exiting FR2. It seems that there is no need to enhance UL PC for above 52.6GHz. | Add the description and update the parameters for FR3/FR2x | Replace FR2 with FR2-e | No spec impacts. |
| Reference SCS configuration | For the existing FR2, a reference SCS configuration μSFI, or μSFI,DL, or μSFI,UL, or μSFI,SUL is either 2 or 3. For 52.6~71GHz, new reference SCSs will be introduced. | Specify new reference SCS configuration for FR3/FR2x | Specify new reference SCS configuration for FR2x | Specify reference SCS configurations for FR2a (instead of FR2) and FR2b respectively. |
| BWP | It seems that the existing FR2 mentioned in BWP in RAN1 specs does not need to be enhanced for above 52.6GHz | Add the description for FR3/FR2x | Replace FR2 with FR2-e | No spec impacts |
| - UE procedure for monitoring Type0-PDCCH CSS sets | For 52.6~71GHz, parameters table of Type0-PDCCH CSS set for SS/PBCH block and CORESET multiplexing pattern may need to be defined. | Specify parameters table of Type0-PDCCH CSS set for FR3/FR2x | Specify parameters table of Type0-PDCCH CSS set for FR2x, or replace FR2 with FR2-e | Specify parameters table of Type0-PDCCH CSS set for FR2a (instead of FR2) and FR2b respectively. |
| PHY UE capabilities | A lot of UE capabilities support different functionalities between FR1 and current FR2. A lot of UE capabilities were intended for only current FR2. For 52.6~71GHz, at least UE capabilities related to supported carrier aggregation, supported BW, timeline threshold, etc. should be newly specified. | May need to add “FR1-FR2-FR3/FR2x DIFF”, “FR2-FR3/FR2x only”, “FR3/FR2x only”, etc. Specify UE capabilities for FR3/FR2x if FR3/FR2x and FR2 have different functionalities or values | May need to add “FR1-FR2-FR2x DIFF”, “FR2-e only”, “FR2x only”, etc. Specify UE capabilities for FR2x if FR2x and FR2 have different functionalities or values | May need to add “FR1-FR2a DIFF”, “FR2a only”, “FR2b only”, etc. Specify UE capabilities for FR2a and FR2b if they have different functionalities or values |
| Other new features that may be introduced by licensed & unlicensed operation in above 52.6 GHz | For 52.6~71GHz, Some new features e.g. multi-slot PDCCH monitoring, QCL assumption and timing for multi-PDSCH/ PUSCH scheduling, may be introduced in Fel-17 NR | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts |
| New features that may be introduced by unlicensed operation in above 52.6 GHz | For unlicensed operation in above 52.6GHz, some new features e.g. discovery burst, discovery burst transmission window, etc., may be introduced in Rel-17 NR | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts | No need to explicitly describe the frequency range in RAN1 specs. No spec impacts |

According to the analysis in Table 1, we can find the following observations.

- If the features of 52.6~71 GHz are the same as the existing FR2, extending the existing FR2 to cover the new frequency range from 52.6~71 GHz has the least impact on RAN1 specs.

- However, due to not all features designed for 52.6-71 GHz are the same as the existing FR2, e.g. PRACH and reference SCSs, thus a new notation, e.g. FR3/FR2x/FR2b, is needed for defining the frequency range from 52.6~71 GHz to distinguish the existing FR2 and 52.6~71 GHz.

- Alt 1 and Alt 2 have the same impacts on RAN1 specs, which are simple and clear mainly brought by adding new words/sentences/sub-clauses for FR3/FR2x. But Alt 1/2 will cause some text redundancy in the RAN1 specs.

- For Alt 3, if FR2 and FR2x have same designs, the current FR2 in RAN1 specs could be replaced by term FR2-e. Otherwise, new values / descriptions for FR2x should be specified.

- For Alt 4, whether there are RAN1 spec impacts depends on whether FR2a and FR2b can share the same designs. If same designs are supported for FR2a and FR2b, there is no RAN1 spec impacts. Otherwise, specify FR2a (instead of FR2) and FR2b respectively. However, it should be noted that even though there is little RAN1 spec impacts if most designs are the same for FR2a and FR2b, it requires modification of all current specs to replace current FR2 with FR2a if FR2b and FR2a have different features.

- In fact, no matter which alternative is adopted, the impact on RAN1 specs is relatively small (as summarized in Table 2). Therefore, we think RAN plenary should mainly refer to RAN4's analysis/conclusion to make the final decision.

Table 2: Comparison between four alternatives

| Schemes | RAN1 spec impacts |
| --- | --- |
| Alt 1: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR3 for 52.6-71 GHz | Alt 1 and Alt 2 have the same impacts on RAN1 specs. FR3/FR2x for defining 52.6-71GHz need to be specifically described in all parts involving the frequency range in RAN1 specs, e.g. transition time of RX/TX, Point A, k_ssb, SSB pattern, PRACH, UL PC, reference SCS configuration, BWP, etc., as analyzed in Table 1. |
| Alt 2: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR2x for 52.6-71 GHz |  |
| Alt 3: Maintain the existing FR2 for 24.25~52.6 GHz, introduce FR2x for 52.6-71 GHz and FR2-e for 24.25~71 GHz | If FR2 and FR2x have same designs, the current FR2 in RAN1 specs could be replaced by term FR2-e. Otherwise, new values / descriptions for FR2x should be specified in RAN1 specs. |
| Alt 4: Extend the exiting FR2 to cover 24.25~71 GHz, and introduce FR2a for 24.25~52.6GHz and FR2b for 52.6-71GHz. | Whether there are spec impacts depends on whether FR2a and FR2b can share the same designs. If same designs are supported for FR2a and FR2b, there is no RAN1 spec impacts. Otherwise, specify FR2a and FR2b respectively. Some examples are given below. - At least in Point A, UL PC, and BWP in RAN1 specs, the description of FR2 can be reused for 52.6-71GHz. - For PRACH and reference SCS configuration, some features of them should be specified for FR2a (instead of FR2) and FR2b respectively. - For transition time of RX/TX, k_ssb, and SSB pattern, whether the description of FR2 can be reused depends on Rel-17 final decision (i.e. if FR2a and FR2b can share the same design). |

Observation 1: There are four options for defining the frequency range of 52.6-71 GHz. Among the options below, the impact on RAN1 specs is relatively small no matter which option is adopted.

- Alt 1: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR3 for 52.6-71 GHz;

- Alt 2: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR2x for 52.6-71 GHz;

- Alt 3: Maintain the existing FR2 for 24.25~52.6 GHz, introduce FR2x for 52.6-71 GHz and FR2-e for 24.25~71 GHz;

- Alt 4: Extend the exiting FR2 to cover 24.25~71 GHz, and introduce FR2a for 24.25~52.6GHz and FR2b for 52.6-71GHz. The extended FR2 comprises FR2a and FR2b.

Proposal 1: RAN1 can adapt to other groups decision on FRx notation for 52-71 GHz. No critical dependency in RAN1.

Conclusion

In this contribution, we discuss frequency range definition for 52.6GHz~71GHz band and have the following observation and proposal.

Observation 1: There are four options for defining the frequency range of 52.6-71 GHz. Among the options below, the impact on RAN1 specs is relatively small no matter which option is adopted.

- Alt 1: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR3 for 52.6-71 GHz;

- Alt 2: Maintain the existing FR2 for 24.25~52.6 GHz, and introduce FR2x for 52.6-71 GHz;

- Alt 3: Maintain the existing FR2 for 24.25~52.6 GHz, introduce FR2x for 52.6-71 GHz and FR2-e for 24.25~71 GHz;

- Alt 4: Extend the exiting FR2 to cover 24.25~71 GHz, and introduce FR2a for 24.25~52.6GHz and FR2b for 52.6-71GHz. The extended FR2 comprises FR2a and FR2b.

Proposal 1: RAN1 can adapt to other groups decision on FRx notation for 52-71 GHz. No critical dependency in RAN1.

Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71GHz. Intel, Sitges, Spain, RAN#86, December 9 – 12, 2019

- RP-210862, Revised WID: Extending current NR operation to 71 GHz, CMCC, 3GPP TSG RAN Meeting #91-e, 16 - 26 March 2021

- Draft_MeetingReport_RAN_91e_210330_eom.