3GPP TSG RAN WG1 #105-e		                                    		   R1-2104861

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.8.1.3

Source:	InterDigital, Inc.

Title:	Joint channel estimation for PUSCH

Document for:	Discussion and Decision

# Introduction

In RAN1#104be, the agreements regarding details of joint channel estimation such as time windowing parameters were made [1]. In addition, LS reply to RAN4 was agreed in R1-2104119. In this contribution, we several design aspects related to DMRS bundling for PUSCH to enable joint channel estimation.

# DMRS bundling for PUSCH

## Background

Joint channel estimation at gNB can be enabled by combining channel estimates from DM-RS in multiple PUSCH(s). DM-RS bundling is a technique to allow channel estimation using multiple DM-RS symbols across different slots or repetitions. To obtain improved performance from joint channel estimation, phase and power continuity should be maintained by the UE between PUSCHs that contain DMRS to be bundled. In time-varying fading channels, using DM-RS symbols across slots or repetitions for channel estimation will be beneficial, thanks to averaged channel estimates.

Observation 1: Maintenance of phase and power continuity across bundled PUSCH(s) is critical to enable DMRS bundling

## Time window for DM-RS bundling

The following agreements were made during RAN1-104b-e regarding the parameters for a time window.

Agreements: - For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window - FFS the units the time domain windown (e.g. repetitions, slots, and/or symbols) - FFS : association between the potential use case(s) and units of the time window - FFS: single or multiple time domain windows - FFS: relation with UE capability - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms - FFS whether or not to further consider impacting of timing advance Agreements: - For the time domain window for joint channel estimation, down select on the following two options: - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions: - PUSCH repetition type A - PUSCH repetition type B, if agreed - TBoMS, if agreed - Different TB, if agreed - Option 2: The unit of the time domain window is the same for the following PUSCH transmission: - PUSCH repetition type A - PUSCH repetition type B, if agreed - TBoMS, if agreed - Different TB, if agreed Agreement: - For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases: - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A. - FFS: additional specification enhancements on top of that defined to support repetition Type A - Only for single layer transmissions - Subject to UE capability - FFS: Over back-to-back PUSCH transmissions with different TBs

### Units for time window

In RAN1-104b-e, a concept of a time window, during which the UE is expected to maintain phase and power continuity across PUSCH(s), was agreed. In addition, whether to keep the unit of the time window same or different across different schemes was discussed in the meeting. The time window can be applicable to different type of transmissions, e.g., repetition type A/B, TBoMS, different TB. Thus, the definition of time unit is important for the time window to be compatible with different types of UL transmission. So far, in Rel. 17, repetition type A and B are agreed to be supported for joint channel estimation.

In our view, the units for time window should be different for different types of repetition schemes since granularity of the time window is different between type A and type B repetition. For example, while type A enables slot-by-slot repetition, type B may be divided into nominal and actual repetitions, which may go over the slot boundary. Thus, using slots as units for the DMRS bundling time window is not desirable for repetition type B for compatibility reasons. For type B repetition, symbol-level unit for the time window seems sufficient. Thus, at least for type A and type B repetition, the following proposal is made with regards to unit of the time window.

Proposal 1: Units of the time window for repetition type A and type B are slot and symbol, respectively

### Necessity of single/multiple time windows

The above proposal is also compatible with the use cases mentioned in R1-2104119 such as back-to-back PUSCH transmission within a slot/across slots. For non-back-to-back transmission within a slot/across slots,  the above proposal is applicable as well where the time window covers gap or downlink symbols. Thus, if non-back-to-back transmission is supported for joint channel estimation, the UE is provided with one window (covering both gap and downlink symbols) and UE is expected to maintain phase/power continuity across gap or downlink symbols.

Proposal 2: For back-to-back transmission within a slot or across slots, one time window, the UE is provided with one time window, expressed in the unit dependent on the repetition type.

Proposal 3: For non-back-to-back transmission of PUSCH, the UE is provided with one time window, expressed in the unit dependent on the repetition type, which covers gap or downlink symbols.

### Indication of time windowing

To enable joint channel estimation, there should be an expectation from the gNB that the UE maintained power and phase continuity between PUSCH(s). Thus, the gNB needs to inform the UE either implicitly or explicitly which PUSCH(s) that contain DMRS needs to be bundled so that the gNB can perform joint channel estimation. DMRS bundling may be determined based on channel estimates obtained by the gNB as parameters such as UE mobility may affect the performance of joint channel estimation.

Based on the observed measurements, the gNB may determine that the channel environment is suitable for DMRS bundling. In addition, channel averaging is effective if it is performed in a long term or short term (i.e., for a set of repetitions). In addition, the UE needs to know how long the UE needs to maintain power/phase continuity. Thus, signalling details should include duration the UE is expected to maintain power/phase continuity.

Depending on grant types, different signalling mechanisms may be needed. For example, DCI which triggers the UE to perform DMRS bundling may be needed if grant is dynamic. Timing of receipt of DCI (e.g., before each PUSCH transmission), needs to be discussed and specified. DCI may be used to signal to the UE to switch DMRS bundling on or off. Disadvantage of such dynamic indication of DMRS bundling is that it may cost overhead and loss of DCI may cause performance loss and inefficiency for DMRS bundling operation.

Observation 2:  Dynamic indication of bundling (e.g., indicating to the UE to perform power/phase continuity maintenance for the next PUSCH) may cost overhead and loss of DCI may cause performance loss and inefficiency for DMRS bundling operation

As the semi-static configuration offers more robust signalling compared to dynamic signalling, the following proposal is made.

Proposal 4: Support a semi-static mechanism to indicate to the UE that DMRS from the indicated group of PUSCHs can be used by gNB for joint channel estimation

Once the UE receives the indication, the UE needs to prepare the PUSCH transmission so that the gNB can conduct joint channel estimation. The UE should maintain power and phase coherency between DMRS so that channel estimation performance can be improved by averaging the bundled DMRS at the gNB.

## Grant type dependent signalling : bundling group index

One approach to increase efficiency to organize DMRS bundling via book keeping index; a bundling group index. The UE determines to bundle DMRS in PUSCH in configured/dynamic grants if the grant is associated with the same bundling group index. The index can be indicated explicitly or implicitly. For example, the UE can receive configuration RRC or signaling in MAC-CE that associates the index with a configured grant.  The UE may bundle DMRS(s) on configured grants that are associated with the same bundling group index.

Alternatively, the UE may receive semi-static configuration for indexing which can be applied to both configured and dynamic grants. For example, a periodic time window can be defined where the UE is expected to bundle DMRS in the PUSCH repetitions that are within the window. Alternatively, a set of frequency can be defined where the UE is expected bundle DMRS in PUSCH that is scheduled in the resource. The aforementioned indication mechanisms can be used to support DMRS bundling for PUSCH with different types of grants, i.e., dynamic and configured grants.

Observation 3: Considering different grant types for PUSCH(s), an indication mechanism that is adjustable for grant type offers flexibility and robustness

Proposal 5: Support a grant-type dependent index which indicates which PUSCH(s) to bundle

## Time domain allocation of DMRS in the DMRS bundle

Whether DMRS in special slot should be bundled needs to be discussed as they also provide useful channel estimates for the gNB. In configuration such as DDDSU where the special slot contains downlink symbols, DMRS placed the special slot can assist channel estimation in the following uplink slot. An example of bundled DMRS in special and uplink slot is shown in Figure 1.

The following simulation demonstrates performance benefits of insertion of DM-RS symbols in a special slot. In Figure 2, BLER performances of various DM-RS placement in uplink and special slot are shown. The simulation parameters assumed in evaluation are shown in Table 1. In the evaluation, uniformly distributed CFO with 0.1ppm as upper bound is included. DMRS allocation considered in the evaluation is shown in Figure 1. From the figure, it is clear that additional DM-RS symbols placed in a special slot bring performance benefits. Comparing the default DM-RS allocation of  and DMRS placement in the special slot, nearly 0.3dB BLER gain can be obtained at BLER=10-2 with the enhancement.

Figure 1 DMRS insertion in a special slot

Based on the evaluation result, we make the following proposal.

Proposal 6: Support DMRS bundling which includes DMRS sybmol(s) in a special slot

## Joint channel estimation for TBoMS

Working assumption: - For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case: - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots - It’s subject to UE capability

Figure 2 DMRS bundling with DMRS in a special slot

Joint channel estimation is possible for TBoMS transmission if the TB is mapped over contiguous slots. When the TB is scheduled over the multiple contiguous slots and multi-slot transmission is enabled, the UE can be configured to maintain power/phase continuity between slots to enable joint channel estimation at the gNB which will bring further benefits in addition to inherent transmission side benefits in TBoMS.

In this contribution, link level evaluation is performed to demonstrate performance improvement of DM-RS bundling. In Figure 3, BLER evaluation results for DM-RS bundled CHEST (i.e., joint channel estimation) and slot-by-slot CHEST are shown. Simulation parameters are summarized in Table 1 in Appendix. Frame formats of DDDUU and DUUUU are assumed in the evaluation.

From the figure it is clear that performance gain can be obtained by performing DM-RS bundling for TBoMS. In the figure, BLER evaluation results in the presence of uniformly distributed CFO with 0.1 ppm as upper bound are also shown. The performance gain for performing DM-RS bundling is approximately 0.3-0.4dB, in the presence of CFO. Slight performance degradation in TBoMS in the presence of CFO can be observed by comparing BLER curves in the figure.

Observation 4: Joint channel estimation for TBoMS will provide additional performance gain from the receiver side

Proposal 7: Confirm the working assumption from RAN1#104 and support joint channel estimation for the TBoMS transmission for back-to-back PUSCH transmission

Figure 3 slot-by-slot CHEST vs. DMRS-bundling CHEST for TBoMS for DDDUU, with or without CFO, 120km/hr in the presence of CFO

## DM-RS bundling in the presence of “available slots”

In dynamic TDD, a time symbol can be downlink, uplink or flexible according to a slot format. In RAN1#104e, treatment of “available slots” was discussed for Type A repetition. The slot format can be changed dynamically using SFI (Slot Format Indicator). Thus, dynamically changing slot formats may cause interruption in repetitions scheduled via configured granst and DM-RS bundling associated with the repetition. A simple approach is to disable the DM-RS bundling if repetitions are scheduled over flexible slots/symbols and the flexible slots/symbols are converted to downlink symbols due to SFI.

Proposal 8: Disable DM-RS bundling if flexible symbols or slots included in the time window turn to downlink symbols/slots due to SFI.

## Necessity of PTRS for DM-RS bundling

In the presence of CFO, the DMRS alone may not be enough for phase correction. As indicated in the simulation results in this contribution, the presence of CFO causes performance degradation. In addition, PT-RS may be needed to correct disruption in phase continuity after switching from DL to UL slot or during transmissions of repetitions that may cross the slot boundary. Thus, the UE may need to send PTRS along within the DM-RS bundle so that the gNB can perform phase correction.

Observation 5: In the presence of CFO, PT-RS insertion may assist the gNB for the phase continuity maintenance within an acceptable range so the DM-RS coherence is maintained.

PTRS is specified in Rel. 15 to correct phase offset at the receiver. Similarly, DM-RS bundling can be implemented along with PTRS to assist the gNB phase continuity maintenance. PT-RS placed with DM-RS in the same DM-RS bundle provides additional assistance for the gNB to track time varying phase change using PT-RS as phase reference in the first place, and subsequentially for phase correction between repetitions

The PT-RS sequence has been introduced in Rel-15 for phase tracking mainly in FR2 due to typical effects in this frequency range of the phase noise. The PT-RS allocation is sparser in frequency domain and denser in time domain. They are effective and usually used at a slot-based level. Also, they are configured along with DM-RS. Using PT-RS even in both FR1 and FR2 for cross slot phase continuity evaluation and possibly correction can bring a certain advantage for the non-contiguous transmissions and the UE implementation phase drifting. An example CP-OFDM based allocation of the DM-RS and PT-RS looks like in figure below:

Figure 4 Example PT-RS and DM-RS allocation for CP-ODFM

If PT-RS is inserted over multiple repetitions and place the first PT-RS before the DM-RS symbol during repetitions, the phase shift over repetitions can be evaluated and eventually corrected by gNB.

Observation 6: The PT-RS may help mitigate the phase continuity during non-contiguous repetitions and UE based phase drifting.

At the same time, probably it will be desirable that for the repetitions slots the DM-RS symbol can be delayed allowing for timely phase evaluation. An example is given below in Figure 5 only for the first PT-RS and DM-RS symbol position:

Figure 5 Example of PT-RS and delayed DM-RS symbol during a repetition slot

With the above observations, the following proposal is made.

Proposal 9: When DM-RS bundling is enabled, PTRS should be enabled as well, at least for FR2.

# Conclusion.

In this contribution, the following observations and proposals are made.

Observation 1: Maintenance of phase and power continuity across bundled PUSCH(s) is critical to enable DMRS bundling

Observation 2:  Dynamic indication of bundling (e.g., indicating to the UE to perform power/phase continuity maintenance for the next PUSCH) may cost overhead and loss of DCI may cause performance loss and inefficiency for DMRS bundling operation

Observation 3: Considering different grant types for PUSCH(s), an indication mechanism that is adjustable for grant type offers flexibility and robustness

Observation 4: Joint channel estimation for TBoMS will provide additional performance gain from the receiver side

Observation 5: In the presence of CFO, PT-RS insertion may assist the gNB for the phase continuity maintenance within an acceptable range so the DM-RS coherence is maintained.

Observation 6: The PT-RS may help mitigate the phase continuity during non-contiguous repetitions and UE based phase drifting.

Proposal 1: Units of the time window for repetition type A and type B are slot and symbol, respectively

Proposal 2: For back-to-back transmission wihtin a slot or across slots, one time window, the UE is provided with one time window, expressed in the unit dependent on the repetition type.

Proposal 3: For non-back-to-back transmission of PUSCH, the UE is provided with one time window, expressed in the unit dependent on the repetition type, which covers gap or downlink symbols.

Proposal 4: Support a semi-static mechanism to indicate to the UE that DMRS from the indicated group of PUSCHs can be used by gNB for joint channel estimation

Proposal 5: Support a grant-type dependent index which indicates which PUSCH(s) to bundle

Proposal 6: Support DMRS bundling which includes DMRS sybmol(s) in a special slot

Proposal 7: Confirm the working assumption from RAN1#104 and support joint channel estimation for the TBoMS transmission for back-to-back PUSCH transmission

Proposal 8: Disable DM-RS bundling if flexible symbols or slots included in the time window turn to downlink symbols/slots due to SFI.

Proposal 9: When DM-RS bundling is enabled, PTRS should be enabled as well, at least for FR2.

Reference

[1] RAN1 Chairman’s notes, RAN1#104be, Apr. 2021.

[2] 3GPP, TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” V.16.5.0, Nov. 2020.

Appendix

Table 1 VoIP PUSCH parameters, TDD

| - Parameters | - Values |
| --- | --- |
| - Frequency Region | - FR1 |
| - Duplexing mode | - TDD |
| - Carrier Frequency | - 4GHz |
| - Subcarrier Spacing (kHz) | - 30 |
| - Tx Bandwidth Configuration (MHz) | - 100 |
| - Waveform | - DFTsOFDM |
| - PHY channel | - PUSCH |
| Allocation (# of PRB) | - 4 PRBs per slot, 2 PRBs per slot for TBoMS (TB spread over 2 slots) for DDDUU and 1 PRB per slot for TBoMS for DUUUU |
| DM-RS Type | - Configuration Type 1 (according to Table 6.4.1.1.3-1 in TS 38.211) |
| DM-RS (# of OFDM symbols) | - 2DM-RS symbols per slot, Type B for uplink (14) |
| Precoder | - None |
| MCS index, table, VoIP packet size | - #4, Table 6.1.4.1-1, TS 38.214, VoIP packet size=320 bits |
| HARQ sequence | - None |
| Repetition/HARQ scheme | - None |
| Frequency Hopping | Disabled |
| Antenna Configuration | 1x4 uplink channels (i.e., 1 TX at UE and 4 RX at BS) |
| PA impairment | Ideal |
| Channel model (TDL type, DS) | TDL-C, DS=300ns |
| TX pattern per frame (for TDD) | DDDSU |
| Channel estimation | Per-slot channel estimation |
| UE mobility | 3km/hr, 120km/hr |
| CFO | Uniformly distributed, 0.1 ppm as upper bound |