3GPP TSG RAN WG1 #105-e                                                                                   R1-2104866

E-meeting, May 10th – 27th, 2021

Agenda Item:	8.12.2

Source:	Lenovo, Motorola Mobility

Title:	On reliability improvement for RRC-CONNECTED UEs

Document for:	Discussion & Decision

# Introduction

The contribution is focused on reliability improvement issues for RRC-CONNECTED UEs in Rel-17 NR MBS.

In RAN1#104bis-e meeting, regarding NACK-only based HARQ-ACK feedback and HARQ-ACK codebook determination, the corresponding agreements are made as below:

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

Hence, in this contribution, more detailed views are present from our side.

# Discussion

## HARQ-ACK feedback for PTM scheme 1

HARQ-ACK feedback is widely adopted in wireless communication systems to improve transmission efficiency and reliability. As highlighted in the WID, the HARQ-ACK feedback from UEs corresponding to downlink multicast transmission is essential for the multicast services in order to satisfy the QoS requirement, e.g., reliability. Furthermore, the highlighted objective also stated that the level of reliability should be based on the requirements of the application/service which is provided by MBS. That is to say, the HARQ-ACK feedback for MBS PDSCH can be enabled or disabled according to the reliability requirements.

Generally, for a PDSCH carrying multicast service via PTM scheme 1, there are two options for HARQ-ACK feedback transmission from UE to gNB:

- Option 1: NACK-only based HARQ-ACK feedback

  - If the PDSCH is successfully received, then a UE does not transmit HARQ-ACK feedback to gNB;

  - If the PDSCH is not successfully received, then a UE transmits NACK to gNB.

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- Option 2: ACK/NACK based HARQ-ACK feedback

  - If the PDSCH is successfully received, then a UE transmits ACK feedback to gNB;

  - If the PDSCH is not successfully received, then a UE transmits NACK to gNB.

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

It is obvious that Option 1 can minimize the HARQ-ACK feedback overhead because only a sequence indicating NACK is enough as the HARQ-ACK feedback overhead. At gNB side, only a single common PUCCH resource for transmitting a NACK sequence needs to be configured to the group of UEs so as to enable this NACK-only based HARQ-ACK feedback. Furthermore, when gNB detects the sequence on the PUCCH resource for a given MBS PDSCH, the gNB has to retransmit the MBS PDSCH in subsequent transmission in a group-manner since the gNB can’t differentiate which UE transmits the NACK feedback on the common PUCCH resource shared by the group of UEs. Hence, as long as only one UE does not correctly received the MBS PDSCH, the gNB needs to retransmit the MBS PDSCH to all the UEs in same group. For those UEs which have correctly decoded the MBS PDSCH in the initial transmission and not transmitted “NACK” on the configured PUCCH resource, they may receive the retransmission of the MBS PDSCH in subsequent transmissions and neglect the retransmission.

Regarding the PUCCH format for this NACK-only based HARQ-ACK feedback, since only one-bit information of ACK or NACK is required, PUCCH format 0 is the most appropriate format for such NACK-only based feedback. Typically, one sequence is sufficient for indicating NACK and the signaling overhead is quite low because only one RB in frequency domain and one or two symbols in time domain are required. Moreover, this sequence can be shared by the group of UEs which greatly saves the PUCCH resource overhead. In addition, as for the reliability concern of NACK-to-ACK error for such sequence-based PUCCH format 0 especially when the cell coverage is large, there might be NACK-to-ACK error. Actually, it is much dependent on the receiver sensitivity of gNB. Considering Rel-16 NR V2X has adopted NACK-only based feedback for sidelink transmission from an Rx UE to a Tx UE. The receiver sensitivity is not an issue for UE even though for gNB. Furthermore, since a single PUCCH resource can be configured for the group of UEs to transmit such NACK-only based feedback, the received energy on the configured PUCCH resource is high enough when more than one UE sends the feedback on the PUCCH resource. On the other hand, PUCCH format 1 with 4 to 14 symbols for repetition in time domain and one or more DMRS symbols for coherent demodulation can further improve the transmission reliability. For the MBS transmission with large coverage, PUCCH format 1 can be configured; otherwise, PUCCH format 0 can be configured. In this way, both PUCCH format 0 and PUCCH format 1 can be used for the NACK-only based HARQ-ACK feedback. Although PUCCH format 1 requires more time-frequency resource, a single PUCCH format 1 resource can be shared by the group of UEs. So the overhead is not an issue.

Proposal 1: For NACK-only based HARQ-ACK feedback, both PUCCH format 0 and PUCCH format 1 can be configured for UEs receiving multicast.

Proposal 2: For NACK-only based HARQ-ACK feedback, a common PUCCH resource for transmitting the NACK is configured to the group of UEs receiving multicast.

On the contrast, Option 2 has minor standardization effort and gNB can differentiate each UE’s ACK or NACK at the cost of UE-specific PUCCH resource reservation. Option 2 can be supported so that PTP based retransmission can be adopted. However, when a large number of UEs in the group are receiving the multicast transmission, it will consume too much PUCCH resources if a specific PUCCH resource is configured for each UE for HARQ-ACK feedback corresponding to the multicast transmission. In that sense, two or more UEs in the group can be configured with a same PUCCH resource which is different from the PUCCH resource configured for remaining UEs. In this way, it is up to network to configure orthogonal or same PUCCH resources for the UEs in same group according to the PUCCH resources provided for the group of UEs. However, for support of PTP-based retransmission in case of PTM scheme 1 in initial transmission, UE-specific PUCCH resource should be configured, i.e., each UE is provided with a specific PUCCH resource for the UE to transmit ACK/NACK to gNB.

Proposal 3: For ACK/NACK based HARQ-ACK feedback, it is up to network to configure a PUCCH resource shared by two or more UEs or a specific PUCCH resource for each UE in the group of UEs receiving multicast.

Proposal 4: For ACK/NACK based HARQ-ACK feedback, each UE in the group of UEs receiving multicast is configured with a specific PUCCH resource for support of PTP-based retransmission.

Generally speaking, either Option 1 or Option 2 has its own pros and cons. In order to fully exploit the benefit of each option, it is better to allow the gNB to switch between Option 1 and Option 2 in case gNB decides to enable HARQ-ACK feedback for a given MBS transmission. E.g., gNB can flexibly adjust the feedback options based on the number of interested UEs, when the number of UEs is relatively small, Option 2 is better than Option 1; otherwise, Option 1 is better than Option 2; when the UE-specific PUCCH resource overhead is acceptable to gNB, then it is better for the gNB to adopt Option 2; otherwise, it is better to adopt Option 1.

Additionally, in order to fully exploit the benefit of Option 1 or Option 2, it is necessary to further allow the gNB to dynamically switch between Option 1 and Option 2 in case gNB decides to enable HARQ-ACK feedback for a given MBS transmission. As mentioned above, gNB can flexibly adjust the feedback options based on the number of interested UEs in connected mode or the PUCCH resource capacity on the serving cell so as to reach a tradeoff between transmission reliability and resource overhead. Furthermore, if gNB intends to use PTP based retransmission, the gNB should indicate UE to use Option 2.

Proposal 5: Option 1 (NACK-only based HARQ-ACK feedback) and Option 2 (ACK/NACK-based feedback) can be dynamically switched based on gNB’s scheduling policy and PUCCH resource capacity.

## HARQ-ACK feedback enabling/disabling

As mentioned in the agreement, there is one FFS issue whether to enable or disable the HARQ-ACK feedback for MBS PDSCH. Considering the reason why HARQ-ACK feedback is introduced for MBS is for the purpose of satisfying MBS service reliability requirements, from the standard point of view, it does make sense to introduce a mechanism which allows gNB to enable or disable the HARQ-ACK feedback based on gNB’s scheduling policy and service QoS. In case of high reliability requirements, gNB can enable the HARQ-ACK feedback; otherwise, gNB can disable the HARQ-ACK feedback. Hence, dynamic signaling for enabling/disabling the HARQ-ACK feedback is needed.

Regarding enabling or disabling HARQ-ACK feedback, in Rel-16 NR-U, HARQ-ACK feedback for PDSCH can be disabled by indicating a non-numerical value by the field of PDSCH-to-HARQ_timing in the DL DCI. This non-numerical value is preconfigured in the K1 set. When gNB intends to enable the HARQ-ACK feedback for PDSCH, it shall indicate a numerical value by the field of PDSCH-to-HARQ_timing; when gNB intends to disable the HARQ-ACK feedback for PDSCH, it shall indicate the non-numerical value by the field of PDSCH-to-HARQ_timing. At the UE side, whether to transmit HARQ-ACK feedback depends on the received HARQ-ACK timing value. If it is an applicable value, then UE shall transmit HARQ-ACK feedback in the indicated slot; Otherwise, UE shall disable the HARQ-ACK feedback. In this way, there is no any L1 signaling overhead introduced and gNB can dynamically enable/disable the HARQ-ACK feedback. Based on the learning of Rel-16 NR-U, this kind of mechanism can be reused for MBS.

Based on above discussion, we have below proposals:

Proposal 6: For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

## PUCCH resource configuration

For RRC_CONNECTED UEs receiving multicast, when NACK-only based HARQ-ACK feedback is adopted for PTM transmission scheme 1, if a UE has successfully decoded the scheduled group-common PDSCH, the UE shall not transmit feedback to gNB; otherwise, the UE shall transmit NACK to gNB. This kind of feedback mechanism can’t be multiplexed with ACK/NACK based feedback in same HARQ-ACK codebook. From per UE perspective, PUCCH resource configuration for NACK-only based feedback should be separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

For RRC_CONNECTED UEs receiving multicast, when ACK/NACK based HARQ-ACK feedback is adopted for PTM scheme 1, if a UE has successfully decoded the scheduled group-common PDSCH, the UE shall transmit ACK to gNB; otherwise, the UE shall transmit NACK to gNB. This feedback mechanism is same for both multicast and unicast so from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

Proposal 7: For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

## Type-2 HARQ-ACK codebook determination

For PTM transmission scheme 1, the value of the DAI in the group-common DCI should be counted in group-manner. In detail, the DAI is denoted as the accumulative number of PDCCH monitoring occasions in which PDSCH reception(s) or SPS PDSCH release associated with DCI format for multicast is present on the serving cell. Since the existing DAI is updated per UE, the DAI in the group-common DCI for multicast should be counted separately from the DAI in the UE-specific DCI for unicast. In this way, the HARQ-ACK feedback for PDSCHs scheduled by UE-specific DCI should be generated in different HARQ-ACK codebook to the HARQ-ACK feedback for PDSCHs scheduled by the group-common DCI.

Nevertheless, for ACK/NACK based feedback, when HARQ-ACK feedback for PDSCHs scheduled by the group-common DCI is to be transmitted in same slot with HARQ-ACK feedback for PDSCHs scheduled by UE-specific DCI, both HARQ-ACK feedback should be transmitted in same HARQ-ACK codebook. Since DAI is counted separately for multicast and unicast, HARQ-ACK feedback for group-common PDSCH and HARQ-ACK feedback for UE-specific PDSCH should be contained in two different sub-codebooks as defined in Rel-15 CBG-based HARQ-ACK feedback. Consequently, HARQ-ACK feedback for PDSCHs scheduled by UE-specific DCI is multiplexed in one sub-codebook and HARQ-ACK feedback for PDSCHs scheduled by the group-common DCI is multiplexed in another sub-codebook. Then the two sub-codebooks are further concatenated as the final HARQ-ACK codebook. A predefined rule is needed when concatenating the two sub-codebooks. Since the group-common DCI is transmitted usually with higher reliability than the UE-specific DCI, placing the sub-codebook for PDSCHs scheduled by the group-common DCIs at the beginning of the HARQ-ACK codebook can avoid the error case of HARQ-ACK bit shifting due to the last DCI missed.

Regarding the FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast, since DAIs for multicast are counted separately from DAIs for unicast when the HARQ-ACK feedback corresponding to the multicast PDSCHs and unicast PDSCHs is to be multiplexed in same HARQ-ACK codebook, there are two Type-2 HARQ-ACK sub-codebooks for multicast and unicast, respectively. Support of more than one Type-2 HARQ-ACK codebook for multicast implies separate DAI counting for each Type-2 HARQ-ACK codebook for each MBS and more than one HARQ-ACK sub-codebook for multicast. We don’t see the need to specify more than one Type-2 HARQ-ACK codebook for multicast. For HARQ-ACK feedback for multicast PDSCHs in one PUCCH, DAI should be contiguously counted for those multicast PDSCHs and the HARQ-ACK information bits should be contained in one Type-2 HARQ-ACK codebook.

Proposal 8: For multicast PDSCHs, don’t support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

## Type-1 HARQ-ACK codebook determination

For Type-1 HARQ-ACK codebook determination, if separate K1 set and TDRA table are configured for unicast and multicast, one open issue is how to construct the Type-1 HARQ-ACK codebook when HARQ-ACK feedback for unicast and multicast is to be multiplexed in same Type-1 HARQ-ACK codebook. One straightforward way is to generate two separate Type-1 HARQ-ACK codebooks, one for unicast and another for multicast, then concatenate the two codebooks for transmission. In more detail, for the UE receiving both unicast PDSCHs and multicast PDSCHs, downlink association set for Type-1 HARQ-ACK codebook for unicast is determined according to the configured K1 set for unicast, and downlink association set for Type-1 HARQ-ACK codebook for multicast is determined according to the configured K1 set for multicast. In each slot of the downlink association set for unicast, the candidate PDSCH reception occasions are determined according to the configured TDRA table for unicast. In each slot of the downlink association set for multicast, the candidate PDSCH reception occasions are determined according to the configured TDRA table for multicast. Type-1 HARQ-ACK codebook for unicast is generated only according to the configured K1 set and TDRA table for unicast, and Type-1 HARQ-ACK codebook for multicast is generated only according to the configured K1 set and TDRA table for multicast. Then the both Type-1 HARQ-ACK codebooks are concatenated for transmitting in same PUCCH.

The detailed steps can be listed below as example:

- Step 1-1: Determine the slots in the downlink association set for unicast according to the configured K1 set for unicast.

- Step 1-2: Determine the candidate unicast PDSCH reception occasions within the slots of the downlink association set according to the configured TDRA table for unicast.

- Step 1-3: Generate a first codebook for unicast for each candidate unicast PDSCH reception occasion within each slot of the downlink association set.

- Step 2-1: Determine the slots in the downlink association set for multicast according to the configured K1 set for multicast.

- Step 2-2: Determine the candidate multicast PDSCH reception occasions within the slots of the downlink association set according to the configured TDRA table for multicast.

- Step 2-3: Generate a second codebook for multicast for each candidate multicast PDSCH reception occasion within each slot of the downlink association set.

- Step 3: Concatenate the first codebook and the second codebook.

Proposal 9: For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, two separate Type-1 HARQ-ACK codebooks are generated for unicast and multicast, respectively, according to the respective K1 set and TDRA table.

Proposal 10: For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, Type-1 HARQ-ACK codebook for unicast is placed firstly then followed by Type-1 HARQ-ACK codebook for multicast.

## HARQ-ACK feedback SPS for multicast

As agreed in previous RAN1 meeting, for RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE and the total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. Therefore, maximum 8 SPS configurations can be configured for multicast and/or unicast. One straightforward way is to let gNB determine the number of SPS configurations for multicast or unicast so that gNB has full scheduling flexibility.

Regarding the activation/deactivation of group-common SPS PDSCH configuration, RAN1 has agreed that group-common PDCCH is supported for activation and deactivation of the group-common SPS configuration for MBS. One FFS issue is how to address the DCI missing problems when the DCI is transmitted for activation/deactivation of the group-common SPS configuration. When one UE missed the activation DCI, it implies the UE shall miss the following SPS PDSCH transmissions. However, gNB can’t know which UE has successfully received the activation DCI and which UE has missed the activation DCI. Such DTX issue needs to be addressed.

Since Rel-8, for DL SPS release, UE shall generate one bit of ACK and transmit it in the PUCCH for confirming the reception of the DL SPS release DCI. This mechanism can be reused here for confirming the reception of activation DCI and deactivation DCI. That is to say, upon reception of an activation/deactivation DCI for multicast SPS PDSCH, the UE shall transmit ACK for confirming the reception to gNB. As long as all the UEs in the group confirm the reception of activation/deactivation DCI, the gNB shall activate or deactivate the group-common SPS configuration.

Since ACK-only feedback is required, one sequence is enough for transmitting the ACK to gNB. The key point is each UE in the group should be configured with a specific PUCCH resource so that the gNB can differentiate whether a UE has received the activation/deactivation DCI or not. In case the UE has not transmitted ACK on the UE-specific PUCCH resource, gNB shall retransmit the activation/deactivation DCI. For those UEs which have reported ACK on the respective PUCCH resource may receive the retransmitted activation/deactivation DCI, it can be neglected by the UEs.

With the UE-specific PUCCH resource for the group-common SPS configuration, each UE can not only transmit ACK for confirming the reception of activation/deactivation DCI but also transmit ACK for the transmission of SPS PDSCH. After the group-common SPS configuration is activated, the UE shall receive the SPS PDSCH and transmit the corresponding ACK on the UE-specific PUCCH resource if the SPS PDSCH is correctly received or transmit nothing on the UE-specific PUCCH resource if the SPS PDSCH is incorrectly received. Based on this ACK-only based HARQ-ACK feedback, the gNB can know whether a certain UE has correctly received the SPS PDSCH and can retransmit the SPS PDSCH in PTP manner to the UE. Since the activation/deactivation DCI and the associated SPS PDSCH don’t occur in same slot, the UE-specific PUCCH resource can be used not only for confirming the reception of the activation/deactivation DCI but also for confirming the reception of the associated SPS PDSCH.

Based on above discussion, we have below proposals:

Proposal 11: For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

Proposal 12: For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

# Conclusion

In this contribution, we focus on the reliability improvement for MBS transmission and have below proposals:

Proposal 1: For NACK-only based HARQ-ACK feedback, both PUCCH format 0 and PUCCH format 1 can be configured for UEs receiving multicast.

Proposal 2: For NACK-only based HARQ-ACK feedback, a common PUCCH resource for transmitting the NACK is configured to the group of UEs receiving multicast.

Proposal 3: For ACK/NACK based HARQ-ACK feedback, it is up to network to configure a PUCCH resource shared by two or more UEs or a specific PUCCH resource for each UE in the group of UEs receiving multicast.

Proposal 4: For ACK/NACK based HARQ-ACK feedback, each UE in the group of UEs receiving multicast is configured with a specific PUCCH resource for support of PTP-based retransmission.

Proposal 5: Option 1 (NACK-only based HARQ-ACK feedback) and Option 2 (ACK/NACK-based feedback) can be dynamically switched based on gNB’s scheduling policy and PUCCH resource capacity.

Proposal 6: For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

Proposal 7: For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

Proposal 8: For multicast PDSCHs, don’t support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Proposal 9: For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, two separate Type-1 HARQ-ACK codebooks are generated for unicast and multicast, respectively, according to the respective K1 set and TDRA table.

Proposal 10: For HARQ-ACK feedback for unicast and multicast to be multiplexed in the same PUCCH, Type-1 HARQ-ACK codebook for unicast is placed firstly then followed by Type-1 HARQ-ACK codebook for multicast.

Proposal 11: For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

Proposal 12: For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

# References

Chairman’s notes on RAN1#104bis-e meeting

RP-201038, “New Work Item on NR support of Multicast and Broadcast Services”, Huawei

R1-2104865, “On group scheduling for RRC-CONNECTED UEs”, Lenovo, Motorola Mobility

R1-2104080, “Summary#9 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS”, Moderator (CMCC)

R1-2104031, “FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs”, Moderator (Huawei)