3GPP TSG RAN WG1 #105-e                                                               R1-2104893

e-Meeting, 10th – 27th May, 2021

Source: 	Intel Corporation

Title:	On CSI enhancements for MTRP and FDD

Agenda item:	8.1.4

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN Meeting #86 new work item (WI) on further enhancements on MIMO (feMIMO) for NR was agreed with WI description in [1]. One of the objectives of the WI is related to CSI enhancements for multi-TRP (MTRP) non-coherent joint transmission (NCJT) and codebook enhancement for FDD reciprocity. The corresponding objective is captured below.

- Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2 - Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

At the RAN1#103-e meeting [2] it was agreed to support enhancements for port selection codebook utilizing DL/UL reciprocity of angle and/or delay. Also, it was agreed that Rel-17 CSI measurement and reporting for DL multi-TRP and/or multi-panel transmission shall be enhanced to support and enable more dynamic channel/interference hypotheses for NCJT. In this contribution CSI enhancements to support MTRP and FDD reciprocity are discussed.

# Discussion

## 2.1. CSI enhancements for MTRP

CMR and IMR configuration

At the last two RAN1 meetings [3, 4] good progress was achieved on channel measurement resource (CMR) and interference measurement resource (IMR) configuration for NCJT. However, there are multiple open issues related to configuration of CMR and IMR. Such issues are listed in the following agreement from the RAN1#104-bis-e [4].

Agreement For CSI measurement associated with a CSI-ReportConfig for NC-JT, study following aspects: - whether to support dynamic updating, e.g. by MAC-CE, for CMR pairs for NCJT measurement hypotheses, and/or CMRs for Single-TRP measurement hypotheses, and/or TCI states in CMRs, and/or the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report - whether additional high layer signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses - For CMRs configured in the CSI-RS resource set, whether support high layer signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis

For the update of CMR pairs for NCJT and CMRs for STRP (Single-TRP) using MAC-CE, as we discussed in our contribution for the previous RAN1 meeting [5] dynamic update of CMRs can be supported by using Rel. 15 aperiodic CSI triggering design with configuration of multiple aperiodic CSI reports. However, considering that at the last RAN1 meeting it was agreed to support configuration with up to 8 CMRs, dynamic update of CMR by using aperiodic CSI triggering is challenging due to large number of required CSI reports. Considering the above, we propose to support MAC-CE based update of CMRs configured for NC-JT and STRP.

Regarding the additional higher layer configuration to select M CMRs for STRP from the set of Ks CMRs, in our view such additional configuration is required at least for FR2, where CMR for NC-JT and STRP are different. In such case subset of CMRs is used for NCJT only and other subset of CMRs is used for STRP only.

In the same time, in our view such features as the dynamic configuration of the number of CMR pairs for NCJT (N) and the number of CMRs for STRP (M) are redundant and not needed as well as the dynamic configuration of number of CSI for STR measurement hypothesis (X).

Proposal 1:

- Support MAC-CE based update of CMRs for NCJT and STRP

- Support configuration of M < Ks CMRs for STRP

The following agreements were made at the last RAN1 meeting w.r.t. additional constraints for CMR and IMR configuration.

Agreement Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses, study following Alternatives and down-select one Alternative in RAN1#105-e: - Alt 1: It is feasible for FR1 but not for FR2. - Alt 2: It is feasible for both FR1 and FR2 but subject to further UE capability for FR2. Agreement Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis, study following Alternatives and down-select one Alternative in RAN1 105e: - Alt 2: It is feasible for FR1 but it is not for FR2. For FR2, the UE is expected to have different NZP CSI-RS resources configured for all CMRs of Single-TRP and NCJT measurement hypotheses respectively. - Alt 3: It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot. For future meetings: Companies to study whether a CSI-IM can be referred by both NCJT and Single-TRP measurement hypotheses. Consider following Alternatives and FR1/FR2 differentiation: - Alt 1: CSI-IM can be shared by both NCJT and Single-TRP measurement hypotheses. - Alt 2: A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis

For the NCJT CSI with CMR pair (m, a) assuming a UE with two panels, channel measurements should be done with both panels on CMR m and CMR a in order to properly consider interference between layers transmitted on CMR m and CMR a. Furthermore, Rx spatial filters on both panels for reception of CMR m should be set exactly the same as for reception of CMR a. Thus, for a UE with two panels, Rx spatial filters for reception of CMRs a, b, m should be the same if CMR pair (m, a) and CMR pair (m, b) are configured for NCJT. Thus, such configuration is feasible at least with the same QCL Type D assumption for CMRs a, b.

Observation 1:

- Configuration of CMR pair (m, a) and CMR pair (m, b) is feasible at least with the same QCL Type D assumption for CMRs a and b

If a UE with two panels is configured with CMR pair (m, a) and CMR for STRP (m), the Rx spatial filters for reception of CMR m can be set according to QCL Type D assumption for CMR m for the 1st panel and CMR a for the 2nd panel. Thus, configuration of CMR pair (m, a) and CMR for STRP (m) is feasible. However, such configuration may have negative impact on the performance since Rx spatial filter may be not optimised for actual reception of CMR for STRP (m) or CMR pair (m, a).

Observation 2:

- Configuration of CMR pair (m, a) and CMR for STRP (m) is feasible, however it may have negative impact on the performance for FR2 since Rx spatial filters on UE panels may be not optimized for actual reception of CMR for STRP (m) or CMR pair (m, a)

At the last RAN1 meeting [4] it was agreed that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource. Following this agreement, it means that if UE is configured with the same CSI-IM resource for STRP and NCJT, a UE will apply QCL-Type D assumption of NCJT measurement hypothesis for reception of the CSI-IM. Thus, it may lead to degradation of performance due to inaccurate interference measurement for the STRP measurement hypothesis. Thus, for FR2 separate configuration of IMR for STRP and NCJT measurement hypothesis is preferred.

Observation 3:

- Configuration of the same IMR for STRP and NCJT measurement hypothesis may lead to performance degradation due to inaccurate interference measurements

At the last RAN1 meeting [4] the following agreement was made on configuration of NZP CSI-RS based interference measurements.

Agreement Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM, study following Alternatives and down-select one Alternative in RAN1#105e: - Alt 1: Yes, it is supported, subject to limitations, e.g. N=1 CMR pair and Ks=2 CMR resources - Alt 2: No, it is not supported

In the Rel. 15 CSI measurements design NZP CSI-RS based interference measurements is supported only for the case with single CSI-RS resource in the resource set. The main use case for that feature is to support measurements of intra-stream interference from transmission layers of other UEs in MU-MIMO. Since MU-MIMO is not considered together with NCJT we propose not to support NZP CSI-RS based interference measurements in Rel. 17.

Proposal 2:

- NZP CSI-RS based interference measurements is not supported for MTRP CSI in Rel. 17

CSI for multi-DCI based NCJT

At the RAN1#104-e meeting the following agreement was made on CSI for multi-DCI based NCJT which is related to working assumption from RAN1#103-e meeting.

Agreement - Strive to agree at most one of the following options, if needed - Option 1: Confirm the Working Assumption from RAN1 103e. - Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT - The time of decision is RAN1#105e (May 2021) Working Assumption For CSI measurement for multi-DCI based NCJT, down select one of following two options: - Option 1 (Explicit): CMRs corresponding to different TRPs can be associated with different reporting settings respectively, with the same configurations between two settings except for PUCCH/PUSCH resources and CMR/IMR resources setting(s) - Option 2 (Implicit): a single CSI reporting setting associated with each TRP where a NZP CSI-RS is configured for interference measurement from another TRP - FFS: how interference from CMR in the linked reporting settings in option 1 or from the NZP CSI-RS configured as IMR in option 2 is considered in CQI calculation Following restrictions apply to both options: - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Only ‘periodic’ and ‘semiPersistentOnPUCCH’ cases are supported; - The number of ports of two CMRs associated to two reporting settings for NCJT CSI measurement are the same; - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

In the above agreement option 1 corresponds to the category 2 NCJT CSI, where CSI report is transmitted to each TRP separately; option 2 corresponds to the category 1 NCJT CSI, where one CSI report is transmitted with CSI for both TRPs. Transmitting CSI report per TRP is beneficial for non-ideal backhaul since large backhaul delay can be avoided. For option 2 large backhaul delay for CSI exchange between TRP can be avoided by triggering/configuring two separate CSI reports with the same content. However, in this case additional CSI overhead is expected. Thus, option 1 is optimized for the case of non-ideal backhaul. Given that both options can be used for non-ideal backhaul and there is no significant benefit of one scheme over the over for ideal backhaul, in our view standardisation efforts to support CSI for multi-DCI based NCJT should be minimised in RAN1.

Proposal 3:

- Strive to minimize standardization efforts to support CSI for multi-DCI based NCJT

CSI reporting for NCJT

At the RAN1#104-e meeting [3] it was agreed to support CSI report with multiple measurement hypothesis including the case where measurement hypothesis for NCJT and STRP are reported by the UE. For this case CSI reporting overhead can be reduced by sharing of RI and PMI for NCJT CSI and STRP CSI. However, sharing of RI and PMI values may lead to degradation of system performance due to different optimal RI value for NCJT and STRP especially for scenarios were number of Tx antennas at the gNB is equal or higher comparing to the number of Rx antennas at the UE. For such scenario RI for STRP may be higher comparing to the number of layers transmitted from the same TRP in NCJT. Thus, if sharing of RI and PMI for NCJT CSI and STRP CSI is supported, enabling/disabling of this feature shall be considered (e.g. via RRC).

Proposal 4:

- Enabling/disabling of sharing of RI/PMI for NCJT CSI and STRP CSI via RRC shall be considered if sharing of RI/PMI for NCJT CSI and STRP CSI is supported

Omission of PMI for NCJT can be also considered to improve the efficiency of CSI reporting since CSI for NCJT measurement hypothesis (NCJT CSI) is not always needed. For example, in some cases NCJT provides lower transport block size (TBS) comparing to STRP transmission with the same CMR, there is no benefit for system if PMI for NCJT is known for this case since NCJT will not be used for transmission. Also, if channel conditions result in out of range CQI for NCJT CSI (i.e. NCJT cannot be received) there is no value to report PMI for NCJT. Based on SLS evaluations (please see the evaluation parameters in the Appendix) the probability of such event is around 25% for low resource utilization, around 35% for medium resource utilization assuming 2 Tx antennas at the gNB and 4 Rx antennas at the UE.

Observation 4:

- Based on system level evaluations, probability of NCJT CSI omission is 25% for low resource utilization and 35% for medium resource utilization with 2 Tx antennas at the gNB and 4 Rx antennas at the UE

Thus, we propose to support NCJT CSI omission in part 2 (e.g. PMI, LI and CQI for the second codeword) with indication of omission in part 1. For the indication of NCJT CSI omission any bitfield in CSI part 1 can be used, for example CRI, RI or CQI for 1st codeword.

Proposal 5:

- Support omission of CSI for NCJT measurement hypothesis in CSI part 2

  - Omission of NCJT measurement hypothesis is indicated in CSI part 1 by using CRI/RI or CQI fields

## CSI enhancements for FDD

CSI-RS port selection and selection

At the last RAN1 meeting the following agreements were made on the selection and reporting of CSI-RS ports.

Agreement For rank=1, polarization-common based free-selection should be supported for W1W1. - FFS: Whether there is a need to restrict the number of CSI-RS ports for which this is supported Agreement At least for rank 1, combinatorial coefficient is used for port selection for W1. - FFS when Wf is turned off

System level evaluations were carried for polarization-common and polarization-specific CSI-RS port selection at the UE side with different number of ports with the following parameters (P, K1, K0) = {(8,4,4), (16,8,8), (24,12,12), (32,16,16)}, where P is the number of configured CSI-RS ports, K1 is the number of CSI-RS ports selected at the UE side and reported to the gNB, K0 is the number of reported linear combining coefficients. The detailed evaluation assumptions are captured in the Appendix. Simulation results are presented in the below figures for relative gains for average UE throughput and cell-edge UE throughput.

Figure 1. Average throughput for polarization-common and polarization-specific CSI-RS port selection at the UE

Figure 2. Cell-edge throughput for polarization-common and polarization-specific CSI-RS port selection at the UE

As it can be seen from the above evaluation results, the performance and overhead are very close for polarization-common and polarization-specific CSI-RS port selection at the UE for P = {8, 16, 24, 32}. Thus, we propose to support polarization-common CSI-RS port selection for all the supported number of ports for rank 1-4.

Observation 5:

- Performance and overhead are very close for polarization-common and polarization-specific CSI-RS port selection at the UE for P = {8, 16, 24, 32}

Proposal 6:

- Support polarization-common CSI-RS port selection for all the supported number of ports for rank 1-4

Regarding the reporting of CSI-RS port selection, combinatorial coefficient reporting provides theoretically lowest number of bits for that problem. Combinatorial coefficient reporting is already supported in Rel. 15 and Rel. 16 Type II codebooks for SD and FD vectors selection. Thus, we propose to support combinatorial coefficient reporting for CSI-RS port selection for rank 1-4 and including the case with one vector in Wf matrix M = 1.

Proposal 7:

- Support combinatorial coefficient reporting for CSI-RS port selection for rank 1-4 including the case with one vector in Wf matrix M = 1

Support of multiple FD vectors in Wf (M > 1)

At the RAN1#104-e meeting [3] it was agreed that codebook design with more than one vector in Wf matrix M > 1 is supported. Furthermore, at the last RAN1 meeting [4] it was agreed that M > 1 is supported as optional feature. However, the value of M and some other details of codebook design for the case of M > 1 are still not defined. The main intention to support M > 1 is to provide additional robustness to non-ideal delay reciprocity for FDD DL/UL channels which is applicable for any number of CSI-RS ports. Larger M value leads to larger overhead due to larger dimension of W2 matrix. In our view reasonable overhead and robustness tradeoff can be achieved for M = 2. Thus, we propose to support M = 2 for all the supported number of ports as for M = 1.

Proposal 8:

- Support M = 2 for all the supported number of ports as for M = 1

The following agreements were made at the last RAN1 meeting on the selection and reporting of FD vectors.

Agreement At least for rank 1, the FD bases used for Wf quantitation are limited within a single window/set with size N configured to the UE, study and down-select one Alternative in RAN1 105e: - Alt 1: FD bases in the window must be consecutive from an orthogonal DFT matrix - Alt 2: FD bases in the set can be consecutive/non-consecutive, and are selected freely by gNB from an orthogonal DFT matrix - FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank. Agreement At least for rank 1, for relationship between N and Mv, study and down-select one Alternative from following in RAN1 105e - Alt 1: N= Mv always - Alt 2: N >= Mv and FSS candidate value(s) of N, e.g. 2, 4 - FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/ not supporting additional alternatives for higher rank.

For M = 1 there is no need for reporting of FD vector. Similar to oversampling and cyclic shift of FD vectors, UE can select arbitrary FD vector for M = 1 (e.g. with the highest average power across all the CSI-RS ports), apply it to CSI-RS port measurement, report the coefficients in W2 matrix; the performance will be the same for cases where gNB is aware of the selected FD vector or not.

Observation 6:

- Reporting of FD vector index is not needed for M = 1

For M > 1, in our view configuration of non-consecutive window for FD vectors selection is not needed since gNB may not be aware of the particular location of FD components in DL channel for proper configuration of the non-consecutive window. In order to provide additional flexibility and additional robustness against delay reciprocity in FDD channels size of the window N can be larger comparing to the number of selected FD vectors N > M. In order to maintain low UE complexity, the selection of FD vectors can be layer-common. In this case SVD operation at the UE is done M times instead of N times.

Proposal 9:

- Support consecutive window for FD vector selection with the window size N = 4 for M = 2

  - Selection and reporting of M FD vectors among N FD vectors is layer-common

Coefficient quantisation

The following agreement was made at the last RAN1 meeting for coefficient quantisation in W2.

Agreement For the quantization of W2 coefficient, study following Alternatives with Alt 1 as the baseline: - Alt1: Reusing Rel-16 quantization mechanism for Rank 1 at least, which can be summarized as following: - An indicator for the strongest coefficient - Two polarization-specific reference amplitudes: - for the polarization associated with the strongest coefficient, the reference amplitude is not reported - for the other polarization, reference amplitude is quantized to 4 bits - For coefficients other than the strongest coefficient - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits - phase is quantized to 16PSK - Alt1-1: the ref amplitude = 0 reserved in R16 can be replaced with a new value, e.g. (1/2)^(1/8), (1/2)^(3/8) - Alt2-0: Individual amplitude (e.g. 3 or 4 bits with Rel15/16 amplitude codebooks) and phase (e.g. 16PSK) quantization - FFS: amplitude codebook is uniform in db or linear scale - FFS: support a strongest coefficient indicator, and individual quantization for other non-zero coefficients. - Alt2-1: ref amp (e.g. 4 bits), Individual amplitude (e.g. 3 bits) and phase (e.g. 16PSK) quantization for each non-zero coefficient - FFS: amplitude codebook is uniform in db or linear scale - FFS: reference amplitude is polarization specific or polarization common, and corresponding codebook - Note: Other quantization schemes or enhancement on top of Alt 1 or Alt 2 are not precluded.

Different quantisation schemes were well studied during the Rel. 16 eMIMO WI. The design of the Rel. 16 codebook is very similar to the Rel. 17 codebook except the selection of SD and FD vectors. Thus, we propose to support the Rel. 16 coefficient quantisation for Rel. 17 (i.e. Alt1).

Proposal 10:

- Reuse Rel-16 coefficient quantization design for Rel. 17 (Alt1)

One more feature which can be reused for Rel. 17 codebook from Rel. 16 codebook is partial CSI omission. Since the PMI has similar structure the same partial CSI omission design can be reused.

Proposal 11:

- Reuse Rel-16 partial CSI omission design for Rel. 17

CSI-RS enhancements

At the last RAN1 meeting the following agreement was made on CSI-RS enhancements.

Agreement For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, down-select ONE option for CSI-RS configurations associated with Rel-17 PS codebook, from Option 0 (No further enhancement), Option 1 (i.e. lower CSI-RS density) and Option 3 (i.e. configuring multiple CSI-RS resources) - If there is no consensus in RAN1 105e, Option 0 is by default

System level simulations were carried out with different CSI-RS density values. The evaluation results are presented in the below figures. Detailed evaluation assumptions are presented in the Appendix.

The following CSI configurations were considered for evaluations.

- Rel. 16: Rel. 16 Enhanced Type II port selection codebook with L = 4

- Rel. 17: Rel. 17 PMI codebook with K1 = {8, 16, 24, 32}, number of FD DFT vectors at the UE side M = 1, number of CSI-RS ports is P = K1, CSI-RS density is equal to D

Figure 3. Average UE throughput for different CSI-RS density

Figure 4. Cell-edge UE throughput for different CSI-RS density

From the above evaluation results it can be observed that Rel. 17 codebook with CSI-RS density D = 0.25 has ~5% performance gain in cell-edge UE throughput comparing to Rel. 17 codebook with CSI-RS density D = 0.5. Thus, reduction of CSI-RS density is beneficial can be considered by RAN1.

Observation 7:

- Rel. 17 codebook with CSI-RS density D = 0.25 has around 5% performance gain in cell-edge UE throughput comparing to Rel. 17 codebook with CSI-RS density D = 0.5

Proposal 12:

- Support 0.25 CSI-RS density in Rel. 17

Codebook parameters

The following agreements were made at the last RAN1 meeting on the codebook parameters.

Agreement At least for rank 1, regarding the value(s) of K1 for port selection matrix W1 in NP*K1, study and down-select from the following candidate values of K1 and the maximal value of P in RAN1 105e - K1 in {2,4,8,12,16,24,32} with K1 <= P - The maximal value of P as Pmax, e.g. 32 - FFS: possible parameter combinations/dependence for K1 with other PS CB parameters, e.g. whether different candidate values of K1 should be configured for different ranks (if rank>1 is supported). - FFS: Whether any value of K1 up to P can be supported for some codebook parameters - Note: for Polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations. Note: for polarization-specific based free-selection, it means select K1 ports out of P ports Note: P is the number of CSI-RS ports for port selection (whose value depends on the outcome of the CSI-RS related study) Agreement A bitmap for indication non-zero coefficients should be supported for W2 with a compression coefficient beta<=1 whereas - FFS values of beta < =1, e.g. 1/8, 1/4, 1/2, 3/4, 1 - FFS: whether/how such a bitmap can be absent for specific codebook configuration parameters - FFS: whether a bitmap is polarization-common or polarization-specific whereas polarization-specific bitmap is the baseline - FFS: possible parameter combinations/dependence for beta with other PS CB parameters Agreement At least for rank 1, regarding the value(s) of R for Rel-17 PS codebook enhancement, study and down-select one or more than one Alternative (or a subset of corresponding values) in RAN1 105e: - Alt 0: R < 1 (e.g. 1/4, 1/2) - Alt 1: R=1 - Alt 2: R=1 and 2 - Alt 3: R=1,2, 4, and 8 - Alt 4: R= {1,2,…, D*NPRBSB} whereas D is the density of CSI-RS in frequency domain - FFS: applicable conditions: e.g. Wf turned ON/OFF and/or associated value of Mv - FFS: Whether this applies when Wf is turned OFF Note that “at least for rank 1” does not imply for the support of rank 1 only in Rel-17 or restrictions of supporting/not supporting additional alternatives for higher rank.

System level evaluations were carried out for the following parameter combinations (P, K1, K0).

- K1 = P, Beta = 1: (2,2,2), (4,4,4), (8,8,8), (16,16,16), (32,32,32)

- K1 = P, Beta = 1/2: (16,16,8), (32,32,16)

- K1 = P/2, Beta = 1: (16,8,8), (32,16,16)

- K1 = P, Beta = 1/4: (32,32,8)

- K1 = P/2, Beta = 1/2: (32,16,8)

- K1 = P/4, Beta = 1: (32,8,8)

Figure 5. Average UE throughput for different codebook combinations

Figure 6. Cell-edge UE throughput for different codebook combinations

From the above evaluation results it can be seen that considered parameter combinations have similar performance/overhead.

Observation 8:

- Parameter combinations (K1 = P, Beta = 1), (K1 = P, Beta = 1/2), (K1 = P/2, Beta = 1), (K1 = P, Beta = 1/4), (K1 = P/2, Beta = 1/2), (K1 = P/4, Beta = 1) have similar performance/overhead

In Rel. 16 it was agreed to support PMI subband size lower comparing to the subband size for CQI. This is controlled by configuration parameter R = {1, 2}, i.e. number of PMI subbands N3 = NSB ∙ R, where NSB is number of subbands for CQI. For the codebook introduced for system with FDD reciprocity subband size can be further reduced since the complexity of PMI search is similar for different values of R for the new codebook. In fact, the PMI subband size can be reduced to the minimum possible value such that PMI subband contains one sample of CSI-RS, i.e. PMI subband size is equal to 1/D PRB, where D is CSI-RS density.

Proposal 13:

- Support R = D*NPRBSB whereas D is the density of CSI-RS in frequency domain

# Conclusion

In this contribution CSI enhancements to support MTRP NC-JT and FDD reciprocity were discussed. The following proposals and observations were presented.

Proposal 1:

- Support MAC-CE based update of CMRs for NCJT and STRP

- Support configuration of M < Ks CMRs for STRP

Observation 1:

- Configuration of CMR pair (m, a) and CMR pair (m, b) is feasible at least with the same QCL Type D assumption for CMRs a and b

Observation 2:

- Configuration of CMR pair (m, a) and CMR for STRP (m) is feasible, however it may have negative impact on the performance for FR2 since Rx spatial filters on UE panels may be not optimized for actual reception of CMR for STRP (m) or CMR pair (m, a)

Observation 3:

- Configuration of the same IMR for STRP and NCJT measurement hypothesis may lead to performance degradation due to inaccurate interference measurements

Proposal 2:

- NZP CSI-RS based interference measurements is not supported for MTRP CSI in Rel. 17

Proposal 3:

- Strive to minimize standardization efforts to support CSI for multi-DCI based NCJT

Proposal 4:

- Enabling/disabling of sharing of RI/PMI for NCJT CSI and STRP CSI via RRC shall be considered if sharing of RI/PMI for NCJT CSI and STRP CSI is supported

Observation 4:

- Based on system level evaluations, probability of NCJT CSI omission is 25% for low resource utilization and 35% for medium resource utilization with 2 Tx antennas at the gNB and 4 Rx antennas at the UE

Proposal 5:

- Support omission of CSI for NCJT measurement hypothesis in CSI part 2

  - Omission of NCJT measurement hypothesis is indicated in CSI part 1 by using CRI/RI or CQI fields

Observation 5:

- Performance and overhead are very close for polarization-common and polarization-specific CSI-RS port selection at the UE for P = {8, 16, 24, 32}

Proposal 6:

- Support polarization-common CSI-RS port selection for all the supported number of ports for rank 1-4

Proposal 7:

- Support combinatorial coefficient reporting for CSI-RS port selection for rank 1-4 including the case with one vector in Wf matrix M = 1

Proposal 8:

- Support M = 2 for all the supported number of ports as for M = 1

Observation 6:

- Reporting of FD vector index is not needed for M = 1

Proposal 9:

- Support consecutive window for FD vector selection with the window size N = 4 for M = 2

  - Selection and reporting of M FD vectors among N FD vectors is layer-common

Proposal 10:

- Reuse Rel-16 coefficient quantization design for Rel. 17 (Alt1)

One more feature which can be reused for Rel. 17 codebook from Rel. 16 codebook is partial CSI omission. Since the PMI has similar structure the same partial CSI omission design can be reused.

Proposal 11:

- Reuse Rel-16 partial CSI omission design for Rel. 17

Observation 7:

- Rel. 17 codebook with CSI-RS density D = 0.25 has around 5% performance gain in cell-edge UE throughput comparing to Rel. 17 codebook with CSI-RS density D = 0.5

Proposal 12:

- Support 0.25 CSI-RS density in Rel. 17

Observation 8:

- Parameter combinations (K1 = P, Beta = 1), (K1 = P, Beta = 1/2), (K1 = P/2, Beta = 1), (K1 = P, Beta = 1/4), (K1 = P/2, Beta = 1/2), (K1 = P/4, Beta = 1) have similar performance/overhead

Proposal 13:

- Support R = D*NPRBSB whereas D is the density of CSI-RS in frequency domain

# References

- RP-193133, New WID: Further enhancements on MIMO for NR, Samsung

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-bis-e, e-Meeting

- R1-2103020 On CSI enhancements for MTRP and FDD, Intel Corporation

# Appendix

Table 1. Evaluation assumptions for FDD CSI

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) |
| Layout | Hexagonal grid with 2 tiers (19 sites) |
| ISD | 200 m |
| Carrier frequency | 2 GHz DL/UL for evaluations with full reciprocity 2 GHz DL, 1.8 GHz UL for evaluations with FDD reciprocity |
| Simulation bandwidth | 20 MHz with 15 kHz subcarrier spacing, 104 PRB |
| Tx power | 44 dBm |
| UE distribution | Uniform |
| UE antenna configuration | 2 Rx X-pol slant 0/90 degrees |
| BS antenna configuration | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8) λ |
| Traffic model | FTP 1 with 0.5 Mbytes packet size, high traffic load (~70% resource utilization) |
| TRP association | RSRP based, Handover margin = 0 dB |
| Transmission mode | MU-MIMO with rank adaptation |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| MU-MIMO precoding | MMSE, 8 BS layers max |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| UE noise figure | 9 dB |
| HARQ | 4 HARQ transmissions max |
| CSI | 10 ms periodicity, 4 ms delay, subband CQI with 8 PRB CQI subband size. Dynamic CSI-RS overhead is considered in the number of RE for PDSCH transmission. |

Table 2. Evaluation assumptions for MTRP CSI

| Parameter | Value |
| --- | --- |
| Scenario | Indoor hotspot |
| Layout | Indoor hotspot grid with 4 sites |
| ISD | 20 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 10 MHz with 15 kHz subcarrier spacing, 52 PRB |
| Tx power | 33 dBm |
| UE distribution | Uniform |
| UE antenna configuration | 4 Rx X-pol slant 0/90 degrees, dH = 0.5 λ |
| BS antenna configuration | 2 Tx X-pol slant -45/45 degrees |
| Traffic model | FTP 1 with 0.5 Mbytes packet size |
| TRP association | RSRP based Handover margin = 0 dB |
| Transmission mode | SU-MIMO with rank adaptation, STRP and MTRP |
| Scheduling | Proportional Fair, wideband |
| OLLA | 10% BLER target |
| MU-MIMO precoding | N/A |
| Elevation beamforming | N/A |
| UE receiver | MMSE-IRC |
| UE noise figure | 9 dB |
| HARQ | No HARQ retransmission |
| CSI | 5 ms periodicity, 4 ms delay, wideband CQI and PMI |