3GPP TSG RAN WG1 Meeting #105-e	R1-2104895

e-Meeting, May 19th – 27th, 2021

Source:	Intel Corporation

Title:	Discussion on PUCCH Format 0/1/4 Enhancements for NR up to 71 GHz

Agenda item:	8.2.3

Document for:	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #86, a new SID on studying the support of NR from 52.6 GHz to 71 GHz (RP-193228) [1] was approved. After completion of this study item (SI), in the 3GPP TSG RAN Meeting #90, a working item (WI) has been approved with the aim to extending NR up to 71 GHz [2]. As part of the objectives of the WI, the following aspects were included:

- Physical layer aspects including [RAN1]: - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation. - Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this context, during the prior meetings [3-4], the following agreements were made:

RAN#1 104e [3]: Agreement: Tables 1, 2, and 3 in Section 2.3 of R1-2102127 are agreed as a common set of assumptions for link level simulations and link budget calculations for evaluating enhancements to PUCCH formats 0/1/4 Note: Other parameters can be additionally considered in the evaluations Agreement: For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs - FFS: Values of N_RB for each SCS - For 480/960 kHz SCS, all REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - For 120 kHz SCS, further discuss the following two alternatives: - Alt-1: All REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping) Agreement: - The configured number of RBs for enhanced PF 0/1/4 is denoted NRB - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings - The maximum value of NRB depends on subcarrier spacing - FFS: maximum value for each SCS and each of PF0/1/4 - FFS: Allowed values of NRB within the [min/max] range - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs. - For PF4: - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers - Note: if frequency hopping is enabled, NRB is the number of RBs per hop - Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions Agreement: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement: - For DMRS of enhanced PF4, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives for sequence construction: - Alt-1: A single sequence of length equal to the total number of mapped Res of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4. - Alt-2: A single sequence of length equal to the number of mapped Res per PRB of the PUCCH resource is used, and the sequence is repeated in each PRB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact Agreement: - For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 as defined for Rel-16 PF4 - Further study the following and decide in RAN1#104-b: - Whether or not additional OCC lengths are supported - Down-select to one of the following alternatives for blockwise spreading - Alt-1: Blockwise spreading is performed across all allocated RBs - Alt-2: Blockwise spreading and DFT is performed per-RB followed by per-RB PAPR/CM reduction mechanism. - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB Specification impact RAN#1 104b-e [4]: Agreement: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions Agreement: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)] Agreement: For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16. Agreement: For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e). Agreement: For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e). Agreement: For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz. Agreement: User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

In this contribution, the following aspects related to PUCCH enhancements for extending NR up to 71 GHz are discussed:

- General Discussion on Enhancements for PUCCH format 0/1/4;

- Enhancements on PUCCH format 0/1;

- Enhancements on PUCCH format 4;

- Enhancements on PUCCH common resource sets.

# General Enhancements for PUCCH Format 0/1/4

The Rel.15 NR framework defines 5 PUCCH formats, which can be categorized into two classes based on their frequency domain span:

- Single-PRB transmission: three PUCCH formats, viz. formats 0/1/4 use single-PRB transmission.

- Multi-PRB transmission: two PUCCH formats, viz. formats 2/3 use multi-PRB transmission.

In Rel.16, in order to cope with the OCB restriction mandated by the ETSI BRAN, PUCCH format 0/1/2 and 3 have been enhanced to support a block interleaved structure. As for Rel.17, while RAN1 has already concluded that for 480/960 kHz SCS no sub-PRB interlaced mapping would be considered, no conclusion has been yet drawn for 120 KHz. In this matter, the following different resource mapping alternatives, which are illustrated in Fig. 1, are compared in terms of maximum isotropic loss (MIL) for different delay spreads.

Figure 1 – Example of resource mapping alternatives in case of a PUCCH spanning across 2 PRBs. In Alt-1, a PUCCH spans over all the available REs, and the sequence is composed by a long sequence with length equal to number of subcarriers over which the PUCCH spans across. In Alt-2, a PUCCH spans over all the available REs, and the sequence is composed by a length-12 sequence which is repeated every PRB, and cyclic shift hopping is applied across the RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured. In Alt-3, a PUCCH spans in a sub-PRB interlace manner, and the sequence is composed by a long sequence with length equal to number of subcarriers over which the PUCCH spans across. In Alt-3, a PUCCH spans in a sub-PRB interlace manner, and the sequence is composed by a length-12 sequence which is repeated every PRB, and cyclic shift hopping is applied across the RBs in similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured.

In order to compare the four alternatives, a thorough study in terms of maximum isotropic loss (MIL) has been performed using the evaluation assumptions agreed during the RAN1 104-e meeting [3]. In this matter Figure 2-5 compare Alt-1, Alt-2, Alt-3 and Alt-4 for PUCCH format 0 in terms of MIL as a function of the number of PRBs over which the sequences span. In particular Figure 2-3 provide MIL evaluation when PF0 spans over 1 OFDM symbol, while Figure 4-5 provide MIL evaluation when PF0 spans over 2 OFDM symbols. For all these figures, the assumption is that the UCI payload would be one bit, and UE’s maximum EIRP is equal to 25 dBm while the UE’s maximum conducted power is equal to 21 dBm. Further, the curves are obtained by fixing the subcarrier spacing to 120 KHz and by varying the delay spread. For each figure, starting from the left, each subfigure is obtained by accounting for the US regulatory restrictions, the ETSI BRAN constrains, and the South Korea regulatory requirements. Figure 2-5 highlight that for PF0, Alt-1 always outperforms all other alternatives, and more importantly that as the channel becomes more frequency dispersive, the sub-PRB interlace mapping performance degrades more than the case when a PUCCH is mapped over all the REs with a PRB. Figure 2-5 show that as expected the MIL increases with the number of PRBs as the required SNR needed to fulfill the detection criteria decreases, but it starts declining after a certain value since the detrimental effect of the CM impairment, the noise power and the limitation of the counted power for different regulatory requirements become more predominant as the bandwidth over which the PUCCH spans becomes larger.

- MIL by accounting for the US regulatory restrictions                         b) MIL by accounting for the European regulatory restrictions

- MIL by accounting for the South Korean regulatory restrictions

Figure 2– MIL for 1 symbol PUCCH format 0 as function of the number of PRBs over which the sequence spans for the case when the delay spread is equal to 5ns.

- MIL by accounting for the US regulatory restrictions                         b) MIL by accounting for the European regulatory restrictions

- MIL by accounting for the South Korean regulatory restrictions

Figure 3– MIL for 1 symbol PUCCH format 0 as function of the number of PRBs over which the sequence spans for the case when the delay spread is equal to 40ns.

- MIL by accounting for the US regulatory restrictions                         b) MIL by accounting for the European regulatory restrictions

- MIL by accounting for the South Korean regulatory restrictions

Figure 4– MIL for 2 symbols PUCCH format 0 as function of the number of PRBs over which the sequence spans for the case when the delay spread is equal to 5ns.

- MIL by accounting for the US regulatory restrictions                         b) MIL by accounting for the European regulatory restrictions

- MIL by accounting for the South Korean regulatory restrictions

Figure 5– MIL for 2 symbols PUCCH format 0 as function of the number of PRBs over which the sequence spans for the case when the delay spread is equal to 40ns.

Observation 1:  The intra-PRB interlaced mapping does not provide any benefits in terms of MIL.

While it has been observed from the above evaluations that a sub-PRB interlace mapping does not provide any gains compared to the case when a PUCCH is mapped across all available REs, if we further consider potential maximum transmit power reduction due to inter-modulation distortions (IMD) from interlace design, which is not considered in MIL calculation, we expect further coverage loss. On the other hand, the interlace design would only complicate the design and the implementation.

Proposal 1:  For the enhanced (multi-RB) PUCCH formats 0/1/4 for 120 kHz SCS all REs within each RB are mapped.

During RAN1#104-e meeting [3], it was concluded that due to the regulatory restrictions imposed globally, and specifically in Europe/CEPT, USA and South Korea, which highly limit the maximum conducted power, the three PUCCH formats with single-PRB frequency domain span (i.e., PUCCH formats 0/1/4) would need to be enhanced over NRB contiguous RBs. Furthermore, in prior RAN#1 meeting [4], it has been agreed that the maximum number of PRBs required to be able to transmit using the maximum allowed conducted power would be equal to 12 RBs, 3 RBs and 2 RBs for 120, 480 and 960KHz subcarrier spacing, respectively. However, the above agreement was made under the assumption that the maximum UE EIRP is 23 dBm, its maximum conducted power is 21 dBm, and its minimum TX beamforming gain is 6 dBm, and it was left for further study to revise the above values in case a lower TX beamforming gain or an higher UE EIRP or conducted power should be supported. In this matter, it is important to note that UEs may have different power classes and TX beamforming gains, and based on these factors and the range of values that these could assume the number of PRBs required by a UE may be substantially different among UEs. For this reason, while RAN1 should strive to identify the lowest TX beamforming gain and the highest UE EIRP or conducted power that can be supported, it should also discuss on whether a proper framework is needed for a UE to implicitly or explicitly indicate its own power class and beamforming gain to the gNB, so that the gNB may be able to configure more properly the number of PRBs over which a PUCCH format 0/1/4 should span.

Proposal 2: Depending on the LS reply from RAN4, RAN1 should discuss whether a proper framework is needed for the UE to implicitly or explicitly indicate its nominal beamforming gain in addition to the power class to the gNB.

# Potential Enhancements on PUCCH Format 0/1

In Rel-15, NR PUCCH formats 0/1 allow UE multiplexing in the code domain (using cyclic shift and/or OCC) and occupy 1 PRB in frequency domain. While format 0 can span 1~2 OFDM symbol(s) within a slot, format 1 has a more dynamic timespan, with a range of 4~14 symbols within a slot. Formats 0/1 are intended to carry small payload size, e.g. 1~2 UCI bit(s). In prior RAN1 meeting [3], it has been agreed to enhance these two formats in frequency domain by allowing them to span over NRB contiguous RBs. In this matter, it has been agreed to support Type-1 low PAPR sequences, but to down-select among one of the following alternatives:

- Alt-1: A single sequence of length equal to the total number of mapped REs of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured.

- Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction:

  - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured.

In order to compare the two alternatives, a thorough study in terms of cubic metric (CM) and maximum isotropic loss (MIL) has been performed using the evaluation assumptions agreed during the prior RAN1 meeting [3].

Figure 3 shows the 95th percentile of the CM of all sequences as a function of the number of PRBs over which the sequence spans. This figure highlights that for most of the number of PRBs, Alt-1 outperforms Alt-2 in terms of CM. Furthermore, it is important to note that while the 95th percentile CM of Alt-1 remains mainly within a given range of values as the number of PRBs increases, for Alt-2 the CM starts to deteriorate greatly as the cyclic shift sequence is repeated.

Figure 3 – 95th percentile CM of all the sequence as a function of the number of PRBs over which the sequence spans.

Figure 4-9 compare Alt-1 against Alt-2 for PUCCH format 0 in terms of MIL as a function of the number of PRBs over which the sequences span. In particular Figure 4-6 provide MIL evaluation when PF0 spans over 1 OFDM symbol, while Figure 7-9 provide MIL evaluation when PF0 spans over 2 OFDM symbols. For all these figures, the assumption is that the UCI payload would be one bit, and UE’s maximum EIRP is equal to 25 dBm while the UE’s maximum conducted power is equal to 21 dBm. For each figure the curves on the left are obtained by fixing the subcarrier spacing to 120 KHz and varying the delay spread, while the curves on the right are obtained by fixing the delay spread and varying the subcarrier spacing. Figure 4 and 7 provide the evaluation under the US regulatory restrictions, Figure 5 and 8 provide the evaluation by considering the ETSI BRAN constrains, and finally Figure 6 and 9 provide the evaluation by accounting for the South Korea regulatory requirements (notice that since the predominant effect in the calculation of the conducted power limit is always the factor depending on the PSD limit, when evaluating the MIL the results between the case when an equipment is further away and when this is close by an astronomical antenna provide the same results).

Figure 4– MIL for 1 symbol PUCCH format 0 as function of the number of PRBs over which the sequence spans accounting for the US regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 5– MIL for 1 symbol PUCCH format 0 as function of the number of PRBs over which the sequence spans accounting for the European regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 6– MIL for 1 symbol PUCCH format 0 as function of the number of PRBs over which the sequence spans accounting for the South Korean regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 7– MIL for 2 symbols PUCCH format 0 as function of the number of PRBs over which the sequence spans accounting for the US regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 8– MIL for 2 symbols PUCCH format 0 as function of the number of PRBs over which the sequence spans accounting for the European regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 9– MIL for 2 symbols PUCCH format 0as function of the number of PRBs over which the sequence spans accounting for the South Korean regulatory restrictions. In the left figure, the MIL is parameterized over different values of the delays spread. In the right figure, the MIL is parameterized over different values of the subcarrier spacing.

Figure 4-9 highlight that for PF0, Alt-1 outperforms Alt-2 mostly for all subcarrier spacing, delay spread and number of PRBs, except for some limited cases. In fact, Alt-1 is slightly better than Alt-2 (<0.5 dB) when the number of PRBs are 12, 15 and 20. Figure 4-9 show that as expected the MIL increases with the number of PRBs as the required SNR needed to fulfill the detection criteria decreases, but it starts declining after a certain value since the detrimental effect of the CM impairment, the noise power and the limitation of the counted power for different regulatory requirements become more predominant as the bandwidth over which the PUCCH spans becomes larger. This last effect becomes even more evident by inspecting the curves when these are parameterized over the subcarrier spacing. In fact, as the subcarrier spacing increases the MIL decays more abruptly.

Figure 10-12 compare Alt-1 against Alt-2 for PUCCH format 1 with different lengths (i.e., 4, 8 and 14 OFDM symbols) in terms of MIL as a function the number of PRBs over which the sequences span. For all these figures, the assumption is that the UCI payload would be two bits, the UE’s maximum EIRP is equal to 25 dBm while the UE’s maximum conducted power is equal to 21 dBm. For each figure the curves on the left are obtained by fixing the subcarrier spacing to 120 KHz and the delay spread to 10ns, while varying the PUCCH length. On the other hand, the curves on the right are obtained by fixing the delay spread to 10ns and the PUCCH length to 14 OFDM symbols, while varying the subcarrier spacing. Figure 10 provides the evaluation under the US regulatory restrictions, Figure 11 provides the evaluation by considering the ETSI BRAN constrains, and finally Figure 12 provides the evaluation by accounting for the South Korea regulatory requirements.

Figure 10– MIL for PUCCH format 1 as function of the number of PRBs over which the sequence spans accounting for the US regulatory restrictions.

Figure 11– MIL for PUCCH format 1 as function of the number of PRBs over which the sequence spans accounting for the European regulatory restrictions.

Figure 12– MIL for PUCCH format 1 as function of the number of PRBs over which the sequence spans accounting for the South Korean regulatory restrictions.

Figure 10-12 highlight that for PUCCH forma 1, Alt-1 outperforms Alt-2 mostly for all subcarrier spacing, delay spread and number of PRBs, except for some limited cases. In fact, as for PUCCH format 0, Alt-1 is slightly better than Alt-2 (<0.5 dB) when the number of PRBs are 12, 15 and 20. Figure 10-12 show once again that as expected the MIL increases with the number of PRBs as the required SNR needed to fulfill the detection criteria decreases, but it starts declining after a certain value since the detrimental effect of the CM impairment, the noise power and the limitation of the counted power for different regulatory requirements become more predominant as the bandwidth over which the PUCCH spans becomes larger.

With that said, while it is acknowledged that Alt-2 may increase the user multiplexing capabilities with misaligned PUCCH resources, it is important to note that this may not be important for a system that would highly benefit from spatial multiplexing on top of the multiplexing capability provided through cyclic shifting. On the hand, depending on the number of PRBs over which a PUCCH span across, Alt-2 may incur in a loss which may be up to 2 dB. Therefore given the technical merit in terms of both CM and MIL of Alt-1 over Alt-2, for PUCCH format 0 and 1 the sequence should be generated by using a Type-1 low PAPR sequence of length equal to the number of subcarriers over which the PUCCH spans across.

Proposal 3: For PUCCH format 0 and 1, the sequence is generated by using a Type-1 low PAPR sequence

of length equal to the number of subcarriers over which the PUCCH spans across.

# Potential Enhancements on PUCCH Format 4

While in prior RAN#1 meeting, it was concluded that for the enhanced PF4, a pre-DFT blockwise spreading is performed across all allocated RBs, however, it was not specified which orthogonal cover codes (OCCs) would be applied. In this matter, given the evaluations and through study performed during Rel.16, for PUCCH format 4, block-wised orthogonal cover code (OCC) similar to PUCCH format 3 as defined in Rel-16 can be applied for the enhanced PUCCH format 4.

Proposal 4: PUCCH format 4, block-wised orthogonal cover code (OCC) similar to PUCCH format 3 as defined in Rel-16 can be applied for the enhanced PUCCH format 4.

# Potential Enhancements on the PUCCH Common Resource Sets

In NR, a PUCCH resource set is provided before dedicated PUCCH resource configuration by the higher layer parameter pucch-ResourceCommon, which is indicated via a 4-bit remaining minimum system information (RMSI) index. The 4-bit RMSI indicates an entry into a 16-row table (Table 9.2.1-1 of TS 38.213, which is provided below in Table VI), where each row in the table configures a set of 16 cell-specific PUCCH resources. Each PUCCH resource is indicated by 5 parameters:

- a PUCCH format;

- a starting symbol;

- a duration;

- a PRB offset RBBWPoffset;

- a set of initial cyclic shift (CS) indices.

Some group of rows within the table are defined to have orthogonal PUCCH resources between different rows, in order to account for multiple sectorized cell sites. For instance, {index1, index2}, {index4, index5, index6}, {index8, index9, index10}, {index12, index13, index14, index 15} are defined so that orthogonal resources are assigned to different resource sets within the same group. Notice that these resource sets support only format 0 and 1: i) for PUCCH format 0, only starting symbol #12 and symbol duration 2 is supported; ii) for PUCCH format 1, starting symbol #10, #4, and #0 and symbol duration 4/10/14 are supported.

Table VI - PUCCH resource sets before dedicated PUCCH resource configuration (Table 9.2.1-1 from TS 38.213)

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

When PUCCH format 0 and 1 are enhanced so that they span across a number of PRBs larger than 1, for some numerologies (i.e., subcarrier spacing and bandwidth), and some of the NR PUCCH resources sets, the total number of PRBs used for the transmission of a PUCCH is so large that the PUCCH resource partitioning in frequency domain is no longer possible.

As an example, let’s consider the case when 960 kHz subcarrier spacing is used with a bandwidth of 400 MHz. In this case the total number of available PRBs would be approximately 32 PRBs. If a PUCCH format 1 is configured to be able to operate in Europe with the maximum transmit power, then the PUCCH should span over at least 5 PRBs. If frequency hopping is used, then 10 PRBs will be occupied by each resource set. For each of the resource set groups composed by more than 2 orthogonal resource sets the number of PRBs required would be greater than the one available: for instance, for the groups composed by {index4, index5, index6} and {index8, index9, index10} then a minimum of 36 PRBs would be needed, while for the group composed by {index12, index13, index14, index 15}, a minimum of 48 PRBs would be needed.

Observation 2: For some numerologies, initial UL bandwidth part and PUCCH resource configurations, the total number of PRBs is not sufficient for PUCCH resource partitioning in frequency domain.

Given that the aforementioned issue limits not only the resource sets that can be used, but more importantly the multiplexing capability of the system, RAN1 should discuss how to enhance the PUCCH common resource sets so that for each resource group at least the same number of orthogonal resources are supported. This enhancement could be achieved via either additional starting symbols or orthogonal cover codes.

Proposal 5: Enhance PUCCH resource sets before dedicated PUCCH resource configuration to support sufficient resource partitioning via either additional starting symbols or orthogonal cover codes.

# Conclusions

In this contribution, we discussed several aspects related to PUCCH enhancements for extending NR up to 71 GHz, and we derived the following proposals, and observations:

Observation 1:  The intra-PRB interlaced mapping does not provide any benefits in terms of MIL.

Proposal 1:  For the enhanced (multi-RB) PUCCH formats 0/1/4 for 120 kHz SCS all REs within each RB are mapped.

Proposal 2: Depending on the LS reply from RAN4, RAN1 should discuss whether a proper framework is needed for the UE to implicitly or explicitly indicate its nominal beamforming gain in addition to the power class to the gNB.

Proposal 3: For PUCCH format 0 and 1, the sequence is generated by using a Type-1 low PAPR sequence

of length equal to the number of subcarriers over which the PUCCH spans across.

Proposal 4: PUCCH format 4, block-wised orthogonal cover code (OCC) similar to PUCCH format 3 as defined in Rel-16 NR-U can be applied for the enhanced PUCCH format 4.

Observation 2: For some numerologies, initial UL bandwidth part and PUCCH resource configurations, the total number of PRBs is not sufficient for PUCCH resource partitioning in frequency domain.

Proposal 5: Enhance PUCCH resource sets before dedicated PUCCH resource configuration to support sufficient resource partitioning via either additional starting symbols or orthogonal cover codes.

# References

- RP-193228, “New SID Study on supporting NR above 52.6 GHz”, Intel Corporation, 3GPP TSG RAN Meeting #86, Dec. 2019.

- RP-202925, “Revised WID: Extending current NR operation to 71GHz,” CMCC, 3GPP TSG RAN Meeting #90, Dec. 2020.

- Chairman’s notes, RAN1 #104 e-Meeting, February 2021.

- Chairman’s notes, RAN1 #104b e-Meeting, April 2021.

- R1-2103023, “ Discussion on PUCCH Format 0/1/4 Enhancements for NR up to 71 GHz”, Intel, 3GPP TSG-RAN WG1 Meeting #104b-e, April 2021.

- RP-2102127, “FL Summary 4 for Enhancements for PUCCH formats 0/1/4”, Ericsson, 3GPP TSG-RAN WG1 Meeting #104-e, February 2021.