3GPP TSG RAN WG1 Meeting #105-e                                                                    R1-2104901

e-Meeting, May 10th – 27th, 2021

Source: 	Intel Corporation

Title: 	On the Harmonization of UL CG Between NR-U and URLLC and COT-Initiator Determination

Agenda item: 	8.3.2

Document for: 	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #88 [1], a new WID related to enhancements to industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) was approved. As part of the objectives of this working item (WI), the following aspects were included:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this context, the agreements and conclusions included within the appendix of this document, which are related to the enhancements to enable URLLC to operate in the sub-6 GHz unlicensed band, were made during the previous RAN1 meetings [2-5].  In relation to this topic, in this document the following aspects will be discussed:

- Determination of COT-initiator for UEs performing scheduled transmissions;

- Determination of COT-initiator for UEs performing configured-grant (CG);

- Additional details for the determination of the initiator of a COT;

- Further details and considerations to harmonize the uplink (UL) CG enhancements introduced in NR-U and URLLC during Rel.16.

# Discussion on the Initiator of a COT for Scheduled UL Transmissions

During RAN1 #104-e meeting [4], it was concluded that in semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. In this matter, RAN1 identified the following two alternatives:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI (If absent, determination based on the rules applied for configured UL transmissions is applied)

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

- Alt-b: Determination based on the rules applied for a configured UL transmission

While both options are valuable solutions, it is important to consider the following technical aspects while down-selecting one of them: 1) miss-detection; 2) overhead; 3) early COT termination; 4) cross-FFP scheduling rules.

Miss-detection: Alt-a is able to provide explicit indication via the scheduling DCI (either fall-back and non-fall back DCI) about whether a UE would need to acquire its own FFP or operate within a g-FFP to perform a specific transmission, and this would inherently make this solution a bit resilient against possible miss-detections. In fact, no ambiguity could be created between a UE and its serving gNB at least for the UL transmissions which are scheduled to start at the beginning of a u-FFP in case that UE receives explicit indication on how to operate. On the other hand, as shown in Fig.1, there may be cases in which Alt-a may lead to ambiguity between a UE and a gNB for the following UL transmissions within a u-FFP when this may fall within a g-FFP if the DCI scheduling the first transmission used by the UE to initiate the u-FFP is miss-detected. In this case, then the UE may not be able to perform the following transmission since the UE would not have been able to acquire its own COT. However, since the gNB may not detect the first UL transmission, it may be able to overcome this issue by overwriting the scheduling decision of the second UL transmission if enough time elapses between the first and the second transmission by indicating to the UE that the second transmission could be operated within a g-FFP.

Figure 1 – Illustration of a case when ambiguity between a gNB and a UE could occur due to miss-detection

However, it is important to note that miss-detection is a very rare event and both Alt-a and Alt-b are not exempt from ambiguities between UE and gNB that this may cause. As an example, for both Alt-a and Alt-b if the gNB actually miss-detect the first UL transmission, then the second UL transmission even if it would occur, would not be expected by the gNB and would be missed.

Overhead: When considering the overhead of Alt-a, it is worth noticing that in prior RAN1 #104-e meeting [4], it was agreed that for Rel.16 when operating in semi-static channel access mode both fallback and non-fallback DCIs would carry a 2 bits bitfield. This field jointly indicates a specific set of combinations of CP extension and LBT channel access type, which are meant for this type of channel access mode when a gNB is the only initiator device. However, moving forward to Rel.17, it is possible to extend the content of this field, without increasing overhead, to jointly indicate not only the CP extension and LBT channel access type, but also whether the UE should operate as initiating or responding device. For instance, for UL scheduling DCIs:

- The reserved entry within bitfield ChannelAccess-CPext could be used to indicate that a UE must perform the channel sensing as defined by clause 4.3 in TS 37.213, while operating as initiating device.

- The entry with index 2 could be used to indicate that a UE must perform the channel sensing as defined by clause 4.3 in TS 37.213, and in this case as in Rel.16 should be operating as responding device.

- If a UE is indicated no sensing (i.e., index 0 or 1), it would assume that it operates as an initiating device only if the UE has previously received explicit indication to operate as initiating device within a specific u-FFP, it was able to succeed LBT, and the time-domain resources for the current UL burst fall within the same u-FFP.

Early COT termination: another important aspect worth noticing is that by using the aforementioned framework, this may allow inherently (without any additional overhead) to allow a gNB to early terminate a COT by cancelling/overriding a prior decision made by the gNB on how a FFP could be used, and how a device should operate within it. This may be particularly useful in terms of power consumption at least in the following cases:

  - A gNB after initiating its g-FFP, and initial transmissions may have nothing else to transmit, may not intend to acquire the following g-FFP, and the u-FFP may start within the current g-FFP. In this case, if Alt-b is used, the UE may not be allowed to initiate its own u-FFP given that there is an already active g-FFP, which the UE has detected. Given that in this case the UE would operate as responding device, the UE may not be allowed neither to transmit within the gNB’s idle period nor to perform any transmissions after that until it initiates its own u-FFP. However, if Alt-a is used the gNB could indicate an early termination of its COT to the UE and indicate to this that it can operate as initiating device within a g-FFP which was previously acquired. In this case, the UE could better utilize the unused resources, and would be able to transmit within the idle period of the g-FFP as a initiating device, which would have not been able to be used if the UE would have operated as a responding device. This would not only keep the UE from performing presence detection and therefore saving power, but also allow the UE to perform a back-to-back transmission which would have been not possible otherwise.

Figure 2 – Illustration of a case when the gNB could indicate to the UE that intends to terminate its COT earlier, so that the UE can initiate its own COT and better utilize its unused resources.

  - A UE may have unused resources that would like to share within its own u-FFP. However, a gNB may have nothing to transmit within that u-FFP. In this case, by early terminating the u-FFP the gNB is able to schedule transmission of other UEs within those unused resources, which would have been otherwise unable to do with Alt-b due to the regulatory restrictions, and prior agreements. In this matter, Figure 3 illustrates this scenario.

Figure 3 – Illustration of a case when the gNB could indicate to UE1 to early terminate its COT, so that by acting as initiating device within the active u-FFP1 it can schedule a transmission to UE2.

Cross-FFP scheduling: In the prior meeting it was discussed on whether a gNB may be allowed to perform cross-FFP scheduling or not, and it was agreed that a gNB can schedule by both an UL and DL DCI a transmission in a later g-FFP, which is different from the one which carries the scheduling DCI. However, it was left for further study on defining the channel access requirements that are needed to both the scheduled UL or DL transmissions to occur without violating the regulatory requirements mandated by the ETSI BRAN [6]. In this matter, according to [6, Sec. 4.2.7.3.1.4 and Sec. 4.2.7.3.1.5], an initiating device is mandated to grant an authorization to one or more associated responding devices to transmit only on the current operating channel within the current FFP:

- Sec. 4.2.7.3.1.4: “(3) An Initiating Device is allowed to grant an authorization to one or more associated Responding Devices to transmit on the current Operating Channel within the current Channel Occupancy Time. A Responding Device that receives such a grant shall follow the procedure described in clause 4.2.7.3.1.5.” - Sec. 4.2.7.3.1.5: “Clause 4.2.7.3.1.4, point 3) describes the possibility whereby an Initiating Device grants an authorization to one or more associated Responding Devices to transmit on the current Operating Channel within the current Fixed Frame Period.”

This implies that transmissions from responding devices are only allowed if any authorization is granted by an initiating device within the current channel occupancy time or FFP. If an authorization is granted by an initiating device to another device within a different FFP from the one where the authorization was firstly granted, then in that FFP the authorized device should operate as an initiating device.

When translating the aforementioned requirements into the Rel.17 IIoT/URLLC design, if a gNB operates as an initiating device and schedules an UL transmission outside of its FFP, then the UE must assume that the scheduled UL transmission would need to be performed as if the UE is the initiating device irrespectively from any explicit indication provided by the gNB or any implicit assumptions that the UE may be able to make*. However, no additional channel access requirements would be needed for any DL transmissions scheduled in a different FFP, given that in this case the authorizing and authorized device would coincide with the same device.

Proposal 1: If a gNB operates as an initiating device and schedules an UL transmission outside of its FFP, then the UE must assume that the scheduled UL transmission would need to be performed as if the UE is the initiating device irrespectively from any explicit indication provided by the gNB within the scheduled DCI or any implicit assumptions that the UE may be able to make.

Proposal 2: If a gNB operates as an initiating device and schedules via a DCI a DL transmission outside of its FFP, no special considerations are needed in terms of channel access requirements or COT initiator.

With that said, given that Alt-a does not require any additional overhead to explicitly indicate the COT initiator information, and would inherently make the design more resilient against possible miss-detections and possibly allow to enable early COT indication, this solution is the more technically valid, and desirable than Alt-b.

Proposal 3: In semi-static channel access mode, a UE performing scheduled UL transmissions determines the initiator of the COT based on the content of the scheduling DCI (Alt-a). In particular the bitfield ChannelAccess-CPext jointly indicates not only the channel access type and CP extension to use, but also whether a UE should operate as initiating or responding device.

Proposal 4: In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through scheduling DCI any prior decision regarding the initiator of the COT.

* It is worth noting that consideration regarding the channel access requirements are needed regardless of whether Alt-a or Alt-b is selected.

# Discussion on the Initiator of a COT for UL CG Transmissions

For what concerns the UEs performing CG transmissions, during RAN1 #103-e meeting [3] it was agreed that when a configured UL transmission starts after a u-FFP boundary and ends before the idle period of that u-FFP, then:

- if the UE has already initiated the u-FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT;

- otherwise, if the transmission is confined within a g-FFP before the idle period of that g-FFP, and if the UE has already determined that gNB has initiated that g-FFP, then the UE assumes that the configured UL transmission corresponds to the gNB-initiated COT.

However, for the case when a CG UL transmission is aligned with a u-FFP boundary and it ends before the idle period of that u-FFP, during RAN1 #104-e meeting [4] it was decided that RAN1 would need to down select one among the following options :

- Alt-a: If the transmission is confined within a g-FFP before the idle period of that g-FFP, and the UE has already determined that gNB is initiated that g-FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT;

- Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

While both aforementioned options are valuable solutions, it is important to consider the following technical aspects while down-selecting one of them: 1) power consumption; 2) scheduling restrictions; 3) small gaps advantages; 4) inter-relationship with DL traffic.

Power consumption: it is important to note that through Alt-b it is indeed possible to achieve some power saving by assuming that the UE would always operate as initiating device despite of any ongoing/active g-FFP, which the UE should no longer need to detect. However, it is worth highlighting that this is only a corner case, compared to normal operation where presence detection would be used (i.e., if the UE is not configured to operate as initiating device, and when even if the UE is configured to operate as initiating device but the CG-resources do not align with the u-FFP), and the effective saving would be negligible.

Scheduling restrictions: While Alt-a may not pose any scheduling restrictions, through Alt-b the UE would be always operating as initiating device, and therefore the gNB may be constrained to choose certain FFP parameters and configure a certain length of the UL transmission so that the UE may to block a gNB from acquiring the following g-FFP: this could easily occur if the g-FFP and u-FFP are misaligned and the UL transmission overlaps the gNB’s idle period as illustrated by Figure 4.

Figure 3 – Example highlighting potential UE’s blocking or scheduling constraints deriving from Alt-b.

Small gaps advantages:  On the contrary of Alt-a, Alt-b precludes the gNB’s from sharing its COT with the UE, since the UE must operate always as initiating device. This would inherently increase the LBT overhead, since single shot LBT would be always required for a transmission starting at the u-FFP boundary, when in reality through DL COT sharing procedure and proper scheduling, the UE may not be even required to perform LBT if the gap among bursts would be less than 16us. In other words, through Alt-a by allowing the DL COT sharing procedure, it is possible to achieve a better spectrum utilization, and reduce LBT overhead (and correspondingly also in terms of latency).

Inter-relationship with DL traffic: During the prior meeting, it was argued that Alt-a may preclude UL transmissions not aligned with the u-FFP boundary to be performed in case of sparse DL traffic. However, it is worth highlighting that this is not correct, since in absence of DL traffic (no DL transmission detected), the UE could indeed operate as initiating device, and Alt-a and Alt-b would be equivalent, and the transmission of the UL transmission would be only conditional to succeeding the LBT procedure prior the u-FFP and performing a transmission at the start of that u-FFP.

With that said, given that the only advantage of Alt-b over Alt-a is some negligible power saving, coming at the cost of restricting the scheduler decisions, and precluding the UE from operating with no LBT, and to gNB’s from sharing its COT, Alt-a is preferred.

Proposal 5: When a configured UL transmission is aligned with a u-FFP boundary and ends before the idle period of that u-FFP, if the transmission is confined within a g-FFP before the idle period of that g-FFP, and the UE has already determined that gNB has initiated that g-FFP, then the UE assumes that the configured UL transmission corresponds to gNB-initiated COT (Alt-a). Otherwise, the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

# Additional Details for the Determination of the Initiator of a COT

When discussing the assumptions that a UE should make regarding the COT initiator, RAN1 should also make considerations in the context of wideband operation. When a system operates in wideband, the CCA procedure in each LBT bandwidth (BW) may have a different outcome. While in principle a UE may assume that it can operate differently in every LBT BW, this may have several drawbacks. In fact, this may not only complicate the design, but it may induce additional interference among devices, while on the other hand allowing the UE to switch in ad-hoc manner between responding and initiating device operation based on the outcome of the LBT increasing the likelihood of a UE to be able to transmit. Given that the targeted scenario of URLLC is within a controlled environment where maintaining a coordination among gNB and UEs and reducing power consumption may be critical, it may be preferred if there may be alignment among the assumptions made by a UE across all LBT BWs, and in particular that:

- A UE operates as an initiating device in all LBT BWs, if for at least one LBT BW i) the UE assesses that it shall operate as initiating device in that LBT BW or ii) the UE has received indication from the gNB that it shall operate as an initiating device;

- UE operates as a responding device in all LBT BWs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from the gNB that it shall operate as responding device.

Proposal 6: When operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs. In this case, a UE could assume to operate:

- as an initiating device over all RBs if for at least one LBT BW i) the UE assesses that it shall operate as initiating in that LBT BW or ii) the UE has received indication to the gNB that it shall operate as an initiating device; or

- as a responding device over all RBs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from the gNB that it shall operate as responding device.

# Harmonization of the UL CG Enhancements

Together with enabling a UE from operating as an initiating device, an additional objective of this WI lies into harmonizing the enhancements made in Rel.16 for the UL CG design between URLLC and NR-U, given that these have been introduced having in mind different requirements. During RAN1 #102-e meeting [2], it was agreed that for operation in shared spectrum the configuration of the cg-RetransmissionTimer is up to the network. During RAN1 #103-e meeting [3], it was further discussed on whether the cg-RetransmissionTimer, CG-UCI, and CG-DFI should be enabled or disabled jointly or separately, and the following options were formulated:

Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

Finally during the prior RAN1 #104b-e meeting [5], Option 2-b and Option 3 have been ruled-out, and RAN1 should now strive to down-select between Option 1 and Option 2-a. From Table I, it is possible to infer that the use of the CG-UCI when the cg-RetransmissionTimer-r16 is disabled should be left up to the network, so that this may decide on whether it is appropriate to use it or not based on the use case, application and the reliability and latency targets.

Table I – Summary of use cases enabled by Option 1 and Option 2a.

| Index | Enabling options | CG-UCI | CG-DFI | Retransmission Timer | Comments | Support |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Option 1, 2a | 0 | 0 | 0 | This corresponds to the licensed IIoT like mechanism and could be used in cases where no LBT failures are possible (e.g., single operator in factory settings with complete absence of incumbent technologies, where resources are always orthogonally scheduled). | YES |
| #2 | 2a | 1 | 0 | 0 | This may be used in cases where LBT failure is infrequent (e.g., single operator in factory settings with sporadic presence of incumbent technologies), but still likely. In this case, UCI is transmitted with the only aim to allow COT sharing for a better use of the medium under the assumption of some very infrequent LBT failures. | YES |
| #3 | Option 1, 2a | 1 | 1 | 1 | This corresponds to the NR-U like mechanism and could be used in cases where LBT failures are very frequent. For instance, in case of a deployment with two NR operators or a deployment with one NR operator and presence of incumbent technology. | YES |

With respect to selecting between option 1 and option 2a, it is important to note that the CG-UCI has been introduced in Rel.16 to enable a more “UE-centric” design, where given a certain time-domain resource assignment (TDRA), the UE has the freedom to transmit on its own. To combat latency deriving from possible LBT failures, the UE autonomously selects the HARQ-ID from a given set and indicates this information within the CG-UCI together with the RV used, which is also up to UE. The CG-UCI also carries the NDI information, which is mainly used by the gNB to discern a new transmission from a retransmission, and more importantly information that is necessary to enable the UE’s COT sharing procedure. Moving forward to Rel.17, when the cg-RetransmissionTimer is not configured, if CG-UCI is used, this may become the bottleneck for applications that require high reliability. However, on the other hand, its payload could be substantially reduced since this CG-UCI may only be required to carry information that may allow to enable the UE’s COT sharing procedure, which is a fundamental component introduced as mentioned above so that to limit latency in case of LBT failures and allow a better spectrum utilization.

Proposal 7: The “CG-UCI based procedures” is enabled or disabled through a new RRC parameter, while the “CG-DFI based procedures” is enabled or disabled based on the cg-RetransmissionTimer-r16 (Option 2-a).

6     Conclusion

In this contribution, we discussed several aspects related to the UL enhancements for URLLC operating in unlicensed spectrum, and made the following proposals:

Proposal 1: If a gNB operates as an initiating device and schedules an UL transmission outside of its FFP, then the UE must assume that the scheduled UL transmission would need to be performed as if the UE is the initiating device irrespectively from any explicit indication provided by the gNB within the scheduled DCI or any implicit assumptions that the UE may be able to make.

Proposal 2: If a gNB operates as an initiating device and schedules via a DCI a DL transmission outside of its FFP, no special considerations are needed in terms of channel access requirements or COT initiator.

Proposal 3: In semi-static channel access mode, a UE performing scheduled UL transmissions determines the initiator of the COT based on the content of the scheduling DCI (Alt-a). In particular the bitfield ChannelAccess-CPext jointly indicates not only the channel access type and CP extension to use, but also whether a UE should operate as initiating or responding device.

Proposal 4: In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through scheduling DCI any prior decision regarding the initiator of the COT.

Proposal 5: When a configured UL transmission is aligned with a u-FFP boundary and ends before the idle period of that u-FFP, if the transmission is confined within a g-FFP before the idle period of that g-FFP, and the UE has already determined that gNB has initiated that g-FFP, then the UE assumes that the configured UL transmission corresponds to gNB-initiated COT (Alt-a). Otherwise, the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 6: When operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs. In this case, a UE could assume to operate:

- as an initiating device over all RBs if for at least one LBT BW i) the UE assesses that it shall operate as initiating in that LBT BW or ii) the UE has received indication to the gNB that it shall operate as an initiating device; or

- as a responding device over all RBs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from the gNB that it shall operate as responding device.

Proposal 7: The “CG-UCI based procedures” is enabled or disabled through a new RRC parameter, while the “CG-DFI based procedures” is enabled or disabled based on the cg-RetransmissionTimer-r16 (Option 2-a).

# References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR,” Nokia, Nokia Shanghai Bell.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #102-e, August, 2020.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #103-e, November, 2020.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #104-e, February, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #104b-e, April, 2021.

- ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, (2017-05).

# Appendix – Prior Agreements and Conclusions

RAN1 102-e [2]: Agreements: - For semi-static channel access mode, - If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity. - For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported. Agreements: - For semi-static channel access mode, - When gNB operates as an initiating device - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT - When a UE operates as an initiating device - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB - When the gNB shares a COT initiated by a UE during an FFP associated with the UE - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE - FFS whether/how to support additional restrictions to the idle period Agreements: - For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode - FFS the case when the UE is IDLE/INACTIVE mode Agreements: - A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst. Agreements: - Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles. Agreements: - UE-to- gNB COT sharing in semi-static channel access mode is supported. - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP. - FFS details - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP. - FFS whether/how UE to gNB COT sharing when the gap is >16us Agreements: For semi-static channel access mode, o Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. - FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Agreements: - For semi-static channel access mode, - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling. - FFS on to be provided by SIB-1 - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters Agreements: - At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. Conclusion: Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision. RAN1 103-e [3]: Agreements: - In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms. Conclusion: - For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB. Agreements: - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported Conclusion: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. o Note: Any value for the period, shall be at least 1ms and at most 10ms. o Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. RAN1 104-e [4]: Agreement: - PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG - FFS whether/how to enhance Agreement: - In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5, 10}. - FFS on other values Agreement: - In semi-static channel access mode: - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary. - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT - FFS on X (e.g. X=0, or X= even index number) Agreement: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Agreement: In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported. RAN1 104b-e [5]: Agreement: - Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode. Agreement: - For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. Agreement: - For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell: - Do not support any additional period value Agreement: - For semi-static channel access mode, the starting point of first UE FFP for a serving cell - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement). Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation. FFS whether/how to enhance Agreement: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreement: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. Agreement: - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization