3GPP TSG RAN WG1 #105-e		    	R1-2104928

e-Meeting, May 19th – 27th, 2021

Source:	Intel Corporation

Title:	NR MBS Group Scheduling for RRC_CONNECTED UEs

Agenda item:	8.12.1

Document for:	Discussion and Decision

# Introduction

In this paper, group scheduling for NR multicast and broadcast services (MBS) for RRC_CONECTED UEs is discussed. The new work item on NR Support of Multicast and Broadcast Services [1] has the following objectives for normative work on NR MBS related to multicast scheduling:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

Based on the agreements reached up to RAN1#104bis-e meetings [2][3], in this paper we provide further discussion on group scheduling to enable NR-MBS reception for RRC_CONNECTED UEs.

# Discussion

In this section we provide details on system design considerations for group scheduling for NR MBS including transmission schemes, common frequency resource and control signaling.

## 2.1. Common Frequency Resource for MBS Reception

In RAN1#104bis-e the following working assumption was reached for the configuration of the MBS frequency resource for scheduling group common PDSCH in the downlink.

Agreement: One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs. - FFS: Whether more than one CFR is supported per dedicated unicast BWP - FFS: Whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP Agreement: The down-selection of Option 2A and Option 2B for CFR for multicast of RRC-CONNECTED UEs will be made before the end of RAN1#105-e

For NR MBS, the configuration of a CFR was agreed for RRC_CONNECTED UEs. While it’s important to provide UE with necessary parameters via SIB or RRC signaling, a default assumption can also be defined where the default the MBS CFR is identical to active unicast BWP for the case when no configuration is provided to the UE. With this default CFR assumption, it is possible in principle to support multicast in a dedicated unicast BWP. However, the FDRA should be further discussed with respect to the reference point since multiple UEs will receive the same GC-PDCCH. Another point for further discussion is how to handle the case when some UEs in the group are provided a CFR configuration while some other UEs are not. One option is to assume that the it is up to the gNB to configure CFR for the other UEs in such a way that it aligns with the unicast BWP of the UE with the default CFR assumption.

Proposal 1: A default CFR identical to active unicast BWP can be defined for UEs when no CFR configuration is provided

While the details of CFR design for IDLE mode UEs are still under discussion, RAN1 should strive to achieve a unified CFR design for CONNECTED and IDLE mode UEs. For example, if the default CFR of IDLE mode UEs is agreed to be CORESET#0, then CONNECTED mode UEs may also be configured with same CORESET#0.

Proposal 2: RAN1 should strive for unified CFR for CONNECTED and IDLE mode UEs

The MBS CFR is a tool for FDRA for UEs to receive MBS transmission, but the region is contained within the unicast BWP and therefore, there need not be any restriction on unicast reception within the CFR.

Proposal 3: The UE expects no restriction on unicast reception within the CFR since it is contained within the active DL BWP of the UE.

The use case of more than common frequency region is unclear since by defining a large enough common region the gNB can always use further resource allocation to control the actual frequency domain scheduling.

Proposal 4: One CFR per dedicated BWP is sufficient for scheduling MBS transmissions.

## 2.2. Support of PTM Schemes for NR MBS

In RAN1#104bis-e [2] and RAN1#104-e  [3] the following agreements were reached for support of different transmission schemes based on different control signaling mechanisms:

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. Agreement: The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

PTM Scheme 1 is the baseline scheme that enables group-common transmissions for MBS. PTM Scheme 2 is additionally defined wherein UE specific PDCCH can be used to transmit or retransmit the group-common MBS PDSCH. The utility of PTM Scheme 2 for initial transmission may be limited since scheduling group-common PDSCH using UE-specific PDCCH adds large control overhead. Since PTM scheme 2 uses UE-specific PDCCH, the utility of this scheme becomes apparent only when ACK/NACK based HARQ is supported similar to the case for PTP based retransmission which was agreed in RAN1#104-e. For the NACK only feedback case, the gNB cannot discern which UE within the group has the failed TB and therefore UE specific retransmission may not be ideal since it needs to retransmit the failed TB to the entire group.

Proposal 5: PTM Scheme 2 should be supported when ACK/NACK based HARQ feedback is configured or enabled for the UEs within a group.

Since ACK/NACK based HARQ is supported for RRC_CONNECTED UEs which can receive high QoS MBS transmissions using delivery mode 1 based on RAN2 design, the support of PTP and PTM Scheme 2 simultaneously may not be mandated. Since both schemes are capable of scheduling individually to only UEs which transmitted a NACK, either one of the schemes can be supported and the support can be semi-statically configured via UE-specific RRC signaling.

Proposal 6: Only one among PTP or PTM Scheme 2 can be supported for UE specific retransmission when the initial transmission was based on PTM Scheme 1. The support of PTP or PTM Scheme 2 can be configured by UE-specific RRC signaling. Different UEs in a group can potentially support different retransmission schemes but not both simultaneously.

For the UE to associate the PTM scheme 2 based re-transmission to the initial transmission using PTM Scheme 1, the HARQ process ID can be used as an identifier similar to the PTP based retransmission case. Additionally, based on semi-static configuration of PTP or PTM scheme 2, the UE can identify whether to use C-RNTI (PTP) or the group common RNTI (PTM Scheme 2) to descramble the PDSCH. It is also assumed that the UE does not expect a unicast transmission with the same HARQ process ID as the ongoing MBS transmission.

Proposal 7: The HARQ process ID is used to associate PTM Scheme 2 based retransmission with the initial transmission using PTM Scheme 1. The UE does not expect to receive a unicast transmission using the same HARQ process ID as the ongoing MBS transmission.

In case, PTP and PTM scheme 2 needs to be simultaneously supported from the perspective of a single UE, then the HARQ process ID does not suffice since the UE will need an additional signaling especially for the case of PTP to differentiate it from PTM Scheme 2 since different RNTIs are used to scramble the PDSCH. In this case, an additional bit may be required in DCI to indicate between PTP or PTM Scheme 2. However, since the utility of simultaneously supporting the two schemes is not clear, such options may not be needed in practice. Additionally, it is assumed that the MBS frequency resource is contained within the active UE BWP. Therefore, PTM scheme 2 does not need to re-interpret the FDRA field of DCI with respect to the MBS frequency region and can simply allocate the MBS frequency region using FDRA within the active DL BWP.

In order to support both delivery mode 1 and 2, connected mode UEs can be configured with different group RNTIs each corresponding to a delivery mode and the UE can differentiate the delivery modes based on the RNTI used for scrambling.

Proposal 8: Different group RNTIs corresponding to high and low QoS delivery modes are configured for RRC_CONNECTED UEs

## 2.6. NR MBS Configuration for Delivery Mode 2

To facilitate the reception of broadcast transmission in delivery mode 2, RAN2 has agreed [4] a configuration framework similar to LTE SC-PTM, where the MBS configuration is transmitted over MCCH (carried over a PDSCH) and MBS data over MTCH.

Since MCCH carries the configuration for broadcast reception which is also received by RRC_IDLE/INACTIVE UEs. Therefore, the DCI scheduling the PDSCH carrying the MCCH for MBS configuration should be a DCI format 1_0 with CRC scrambled by SC-RNTI. The configuration change notification can be carried via a DCI 1_0 with CRC scrambled with SC-N-RNTI.

Proposal 9: For delivery mode 2, MBS configuration is provided via a MCCH carried over a PDSCH scheduled by a PDCCH using DCI format 1_0 with CRC scrambled with a new SC-RNTI. Change in configuration can be notified through a PDCCH carrying another DCI 1_0 with CRC scrambled with SC-N-RNTI.

The PDCCH which schedules the MCCH carrying the MBS configuration can be monitored in a Type0-PDCCH CSS set configured by searchSpaceZero in PDCCH-CommonConfig and associated with a CORESET#0 for both RRC_CONNECTED and IDLE mode UEs. Alternately it can be monitored in a new PDCCH CSS set e.g., mcch-searchSpace which is configured by the MBS specific PDCCH-ConfigCommon.

Proposal 10: The PDCCH scheduling the MCCH is monitored in Type0 CSS set configure by searchSpaceZero or Type0A CSS set or alternately on new mcch-searchSpace which is a CSS configured by the MBS specific PDCCH-ConfigCommon.

## 2.3. PDCCH Related Enhancements for MBS

In RAN1#104bis-e the following agreements were reached for NR MBS PDCCH related enhancements:

Agreement: For group-common PDCCH of Rel-17 MBS, support at least two DCI formats. - DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI. - DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields Agreement: If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options. - Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. - l Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - l Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - l Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following alternatives (to be decided in RAN1#105): - Alt 1: support Type-3 CSS - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS - Alt 2: support a new Type-x CSS - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS. - Alt 3: support both Alt 1 and Alt 2

UEs that support MBS reception may also support an additional CFR CORESET in addition to the already supported maximum if 3 unicast CORESETs per BWP. The support of the additional CFR CORESET may be a UE capability. Additionally, if IDLE mode UEs support a CORESET larger than CORESET#0, the gNB may configure the same CORESET as a CFR CORESET for the CONNECTED mode UEs as well.

Proposal 11: MBS capable UEs may support an additional CFR CORESET for monitoring MBS PDCCH and the support of the additional CORESET can be a UE capability.

If the UE supports an additional CFR CORESET, then option 3 can be supported with the MBS CORESET being dedicated to MBS only reception. Otherwise Option 1 is supported wherein MBS and unicast can be configured in any CORESET which is contained within the configured CFR

Proposal 12: If UE can support additional CFR CORESET, Option 3 can be supported. Otherwise Option 1 should be considered as the default for MBS and unicast reception.

For PTM Scheme 1, the CORESET for the group-common PDCCH is configured within the MBS frequency region, but for PTP or PTM scheme 2 which uses UE-specific PDCCH, the restriction is not required and the CORESET can be configured within the unicast BWP and outside the MBS frequency region.

Proposal 13: For PTP or PTM scheme 2, the CORESET scheduling MBS (re)transmission can be configured outside the MBS frequency region.

For determining the BD/CCE limits for MBS, similar methodology as in the of multi-DCI multi-TRP can be re-used for CA capable UEs as mentioned in option 2 of the third agreement above. However, for UEs without CA capability, the Rel-15/16 limits can be kept unchanged. Therefore, the two options should co-exist in specification since they address different categories of UEs.

Proposal 14: For determining BD/CEE limits for NR MBS in Rel-17, Option 1 should be supported for UEs without CA capability and Option 2 should be supported for UEs with CA capability. Down-selection is not necessary.

In NR search spaces sets are configured for a UE to monitor PDCCH candidates and each search space is associated with one CORESET. The search space set configuration for NR MBS can reuse the basic framework defined in up to Rel-16 NR [5]. Since it is already agreed that group common PDCCH based scheduling will be supported, the search space set for monitoring the group-common PDCCH can be a common search space (CSS) and the Type3-PDCCH CSS set configuration can be reused. In this case, the new RNTIs can be added to the set of RNTIs that can scramble the CRC of the DCI formats that can be monitored. Alternately, a new CSS Type 4 can also be defined for multicast purposed for DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI. For the case of groupcast transmissions, the transmission is generally limited by the UE with the worst coverage i.e., the aggregation levels for the PDCCH candidates to be monitored is provisioned based on the worst UE.  Similarly, the CORESET configuration including the precoder granularity should be based on the worst UE in the group.

Proposal 15: Search space set configuration for monitoring DCI scheduling multicast PDSCH can re-use NR Type 3 CSS configuration while additionally supporting monitoring of DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI. Alternately, a new NR CSS Type can be defined for monitoring multicast DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI

For the monitoring priority of search space sets for MBS should be kept identical to that of Rel-15/16 i.e., the priority of CSS and USS, if supported, monitoring MBS PDCCH should be same as that of unicast.

Proposal 16: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS and USS (if supported)

For groupcast transmission to RRC_CONECTED UEs using delivery mode 1 (high QoS), DCI format 1_1 is used as the baseline for the second DCI format. DCI format 1_2 is optional and it’s not clear why UE supporting MBS reception should be compelled to support DCI 1_2.

Proposal 17: DCI format 1_1 should be used as the baseline for the second DCI format for group scheduling.

## 2.4. Multiplexing with Unicast Transmissions

Agreement: At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

It has been agreed that TDM and FDM between one unicast and one group common PDSCH in a slot is supported based on UE capability. In the agreement to support simultaneous reception, the cases listed include support for receiving multiple group common PDSCHs in a slot either in TDM or FDM manner. However, the use case for the support of multiple PDSCHs is unclear. If the use case is for supporting delivery mode 1 and 2 receptions for the same UE, N, L = 2 should suffice.

Observation 1: The use case for multiple simultaneous MBS PDSCH reception should be clarified further. If the intention is to support delivery modes 1 and 2, N, L =2 is sufficient. The total number of PDSCHs that can be simultaneously received may be subject to UE capability.

Since reception of fully time overlapped PDSCHs adds to UE complexity, the agreed FDM reception of MBS and unicast should be subject to UE capability signaling

Proposal 18: The reception of MBS and unicast in FDM mode should be a UE capability

## 2.5. Semi-Persistent Scheduling

Agreement: For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability - The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. - FFS: How to allocate the total SPS configurations between MBS and unicast. Agreement: For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS - FFS: The retransmission scheme(s) - FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2 Agreement: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, - At least group-common PDCCH is supported - FFS: Whether and how to address the missed activation and deactivation - FFS: Whether UE-specific PDCCH is supported for activation/deactivation Agreement: For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total. - It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively. Agreement: The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. - FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group Agreement: Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH. - G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH - FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH. FFS: Number of G-CS-RNTI

For SPS activation for NR MBS, group common PDCCH with CRC scrambled by group RNTI can be used, where the HARQ ID field is set to all 0’s and the RV field is set to 00 for the TB being scheduled. PUCCH resource for carrying HARQ-ACK feedback of SPS PDSCH for MBS may be configured by higher layers via RMSI (SIB1), OSI or RRC signalling. The SPS release can be performed through a similar group common DCI with the HARQ ID set to all 0’s, MCS and FDRA field set to all 1s and RV set to 0. For the SPS release DCI, the UEs can send HARQ feedback through UE-specific (if ACK/NACK feedback is supported) or cell specific (if NACK only feedback is supported) PUCCH resource configured via dedicated RRC signalling.

Proposal 19: For DL SPS configuration for NR MBS

- Group common PDCCH is used for SPS activation with HARQ ID field set to all 0’s and RV field set to 00 for the TB being scheduled\

- PUCCH resource for HARQ feedback may be configured via RMSI, OSI or RRC

- For SPS release, similar group common PDCCH can be used with HARQ ID set to all 0s, MCS and FDRA set all 1’s and RV set 0. For SPS release DCI, UE can be configured with PUCCH resource via RRC

- The PUCCH resources for HARQ feedback for SPS PDSCH as well as the SPS release DCI can be UE-specific for ACK/NACK based feedback or a common PUCCH resource can be configured for the case when NACK-only feedback is configured.

## 2.6. Other Issues

For the PDSCH reception, NR PDSCH Mapping Type A [3] can be used with Type 1 DM-RS as a baseline option. PDSCH mapping Type B and Type 2 DM-RS are not precluded but their need for receiving groupcast transmission can be further discussed.

Proposal 20: NR MBS uses PDSCH Mapping Type A with DM-RS Type 1 as a baseline. PDSCH Mapping Type B and use of Type 2 DM-RS are not precluded.

One of the major design updates in NR is the capability of transmitting front-loaded DM-RS symbols i.e., DM-RS transmission always occurs in the first data symbol of the PDSCH regardless of mapping types A or B. Furthermore, there are no specified transmission modes in NR and multi-layer transmission with rank adaptation is possible. NR multicast design should not be limited in terms of number of MIMO layers. Depending on whether CSI feedback is supported or not, the gNB should be able to transmit multi-layer downlink groupcast. If feedback is available, the transmission rank can be adapted to the such that the UE with the worst channel condition in the group has sufficient spectral efficiency to support the required higher layer QoS. Rank limitation for cases without feedback can also be further discussion.

Proposal 21: For NR MBS support of multi-layer MIMO transmission with rank adaptation (from UE perspective) is not precluded.

Since all UEs in the group are receiving the same transmission within a cell, they are expected to receive the groupcast PDSCH on the same DM-RS port(s). NR DM-RS was designed to support high dimensional MU-MIMO with up to 12 orthogonal DM-RS ports. Therefore, for a given slot, the gNB should be able to use the DM-RS ports not used by the groupcast transmission to multiplex other possibly unicast transmissions in multi-user mode. This would increase the overall efficiency of NR multicast by allowing resource sharing with unicast transmission in addition to TDM and FDM transmissions.

Proposal 22: For groupcast transmission, all UEs within the group share the same DM-RS port(s). Additionally, UEs receiving unicast transmission are multiplexed on remaining orthogonal DM-RS ports.

Groupcast transmission is generally always limited by the worst UE in the group. In order to ensure coverage for the worst-case user, the QoS of users with better channel quality may be adversely affected. Advanced transmission schemes can be further considered to address such issues. An example multiuser superposition transmission (MUST) where the users are divided into two groups with good and adverse channel conditions or coverage. The MSBs of the modulation mapping is reserved for the so-called bad UE sub-group. The sub-group of UEs with good channel conditions can additionally receive the LSBs as well which can lead to additional information. For example, the same video stream can be transmitted using MUST wherein the MSBs correspond to a lower resolution video meant for user with low coverage while the LSB add higher resolution content meant for users with higher coverage. From a physical layer perspective, such transmission schemes can enhance the groupcast quality and improve the group spectral efficiency and their applicability to SC-PTM type transmissions can be further studied.

Proposal 23: Advanced transmission schemes like multiuser superposition transmission (MUST) for improving group spectral efficiency are not precluded

# Conclusions

In this paper, a high-level view of NR MBS design was presented. Group scheduling and transmission schemes were discussed, and the following proposals are made for further consideration

Proposal 1: A default CFR identical to active unicast BWP can be defined for UEs when no CFR configuration is provided

Proposal 2: RAN1 should strive for unified CFR for CONNECTED and IDLE mode UEs

Proposal 3: The UE expects no restriction on unicast reception within the CFR since it is contained within the active DL BWP of the UE.

Proposal 4: One CFR per dedicated BWP is sufficient for scheduling MBS transmissions.

Proposal 5: PTM Scheme 2 should be supported when ACK/NACK based HARQ feedback is configured or enabled for the UEs within a group.

Proposal 6: Only one among PTP or PTM Scheme 2 can be supported for UE specific retransmission when the initial transmission was based on PTM Scheme 1. The support of PTP or PTM Scheme 2 can be configured by UE-specific RRC signaling. Different UEs in a group can potentially support different retransmission schemes but not both simultaneously

Proposal 7: The HARQ process ID is used to associate PTM Scheme 2 based retransmission with the initial transmission using PTM Scheme 1. The UE does not expect to receive a unicast transmission using the same HARQ process ID as the ongoing MBS transmission.

Proposal 8: Different group RNTIs corresponding to high and low QoS delivery modes are configured for RRC_CONNECTED UEs

Proposal 9: For delivery mode 2, MBS configuration is provided via a MCCH carried over a PDSCH scheduled by a PDCCH using DCI format 1_0 with CRC scrambled with a new SC-RNTI. Change in configuration can be notified through a PDCCH carrying another DCI 1_0 with CRC scrambled with SC-N-RNTI.

Proposal 10: The PDCCH scheduling the MCCH is monitored in Type0 CSS set configure by searchSpaceZero or Type0A CSS set or alternately on new mcch-searchSpace which is a CSS configured by the MBS specific PDCCH-ConfigCommon.

Proposal 11: MBS capable UEs may support an additional CFR CORESET for monitoring MBS PDCCH and the support of the additional CORESET can be a UE capability.

Proposal 12: If UE can support additional CFR CORESET, Option 3 can be supported. Otherwise Option 1 should be considered as the default for MBS and unicast reception.

Proposal 13: For PTP or PTM scheme 2, the CORESET scheduling MBS (re)transmission can be configured outside the MBS frequency region.

Proposal 14: For determining BD/CEE limits for NR MBS in Rel-17, Option 1 should be supported for UEs without CA capability and Option 2 should be supported for UEs with CA capability. Down-selection is not necessary.

Proposal 15: Search space set configuration for monitoring DCI scheduling multicast PDSCH can re-use NR Type 3 CSS configuration while additionally supporting monitoring of DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI. Alternately, a new NR CSS Type can be defined for monitoring multicast DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI

Proposal 16: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS and USS (if supported)

Proposal 17: DCI format 1_1 should be used as the baseline for the second DCI format for group scheduling.

Proposal 18: The reception of MBS and unicast in FDM mode should be a UE capability

Proposal 19: For DL SPS configuration for NR MBS

- Group common PDCCH is used for SPS activation with HARQ ID field set to all 0’s and RV field set to 00 for the TB being scheduled\

- PUCCH resource for HARQ feedback may be configured via RMSI, OSI or RRC

- For SPS release, similar group common PDCCH can be used with HARQ ID set to all 0s, MCS and FDRA set all 1’s and RV set 0. For SPS release DCI, UE can be configured with PUCCH resource via RRC

Proposal 20: NR MBS uses PDSCH Mapping Type A with DM-RS Type 1 as a baseline. PDSCH Mapping Type B and use of Type 2 DM-RS are not precluded.

Proposal 21: For NR MBS support of multi-layer MIMO transmission with rank adaptation (from UE perspective) is not precluded.

Proposal 22: For groupcast transmission, all UEs within the group share the same DM-RS port(s). Additionally, UEs receiving unicast transmission are multiplexed on remaining orthogonal DM-RS ports.

Proposal 23: Advanced transmission schemes like multiuser superposition transmission (MUST) for improving group spectral efficiency are not precluded

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- Chairman’s Notes, 3GPP RAN1 104bis-e Meeting, April 2021

- Chairman’s Notes, 3GPP RAN1 104-e Meeting, January 2021

- R1-2104165, LS on broadcast session delivery and MCCH design, 3GPP RAN1#105e, May 2021.

- 3GPP TR 36.890 v13.0.0, Study on Single Cell Point-to-Multipoint Transmission (Release 13)

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)