3GPP TSG RAN WG1 #105-e		R1-2105143

e-Meeting, May 10th – 27th, 2021

Source:	Panasonic

Title: 	Enhancements for unlicensed band URLLC/IIoT

Agenda Item:		8.3.2

Document for:	Discussion

Introduction

A work item on enhanced IIoT and URLLC was approved [1]. One of objectives of this work item is uplink enhancements for URLLC in unlicensed controlled environments such as

- Specify support for UE-initiated COT for FBE with minimum specification effort

- Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel.16 to be applicable for unlicensed spectrum

This document provides our view on potential enhancements related to unlicensed band URLLC/IIoT.

Support of UE-initiated COT for FBE

COT-initiator determination (Configured UL transmissions aligned with UE FFP boundary)

In RAN1#104e, following agreement was made.

Agreement:

- In semi-static channel access mode when a UE can operate as UE-initiated COT

  - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT.

    - Alt.a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

    - Alt.b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Alt.a seems to have unified behaviour between “when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE” and “when a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE”. Although Alt.a may increase the power consumption at UE side to check gNB transmission compared to Alt.b, consistent behaviour with already agreed procedures for configured UL starting after UE FFP boundary would be desirable and sufficient.

Proposal 1: In semi-static channel access mode, when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, if the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

COT-initiator determination (Scheduled UL transmissions)

In RAN1#104e, following agreement was made.

Agreement:

- In semi-static channel access mode when a UE can operate as initiating device

  - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT.

    - Alt.a: Determination based on the content in the scheduling DCI

      - FFS on whether the corresponding field(s) can be absent in DCI

        - If absent, determination based on the rules applied for configured UL transmission is applied.

      - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

    - Alt.b: Determination based on the rules applied for a configured UL transmission

In our view, Alt.a has a benefit that allows the gNB to signal “no LBT” to the UE, which would beneficial for low latency transmission.

Proposal 2: In semi-static channel access mode, when a UE can operate as an initiating device, whether a scheduled UL transmission is based on a UE-initiated COT or sharing a gNB-initiated COT is determined based on the content in the scheduling DCI. If the corresponding field(s) is absent, the determination is based on the rules applied for configured UL transmission.

Harmonizing UL configured-grant enhancements in NR-U and URLLC

In RAN1#103e, following options were identified. In RAN1#104bis-e, it was agreed that Option 1 and Option 2a are considered further.

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

- Option 2a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e., new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e., new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

In Rel.16 specification, cg-RetransmissionTimer is always configured for an unlicensed band operation. In RAN1#102e, it was agreed that at least cg-RetransmissionTimer is not mandated in the unlicensed band operation. Option 1 seems simpler. One of the operations would be if the higher layer parameter cg-RetransmissionTimer is provided, NR-U CG is adopted, otherwise URLLC CG is used.

One of motivations for harmonizing CG enhancement in NR-U and URLLC is to support the same CG operation as licensed band (e.g., CG-UCI/DFI less transmission). The reason why CG-UCI/DFI should be disabled is to reduce unnecessary signalling overhead and retransmissions. For the functions related to autonomous transmission on CG (e.g., the CG retransmission timer, DFI, indication of NDI and RV on CG-UCI), when miss-detection probability at gNB is low (e.g., by configuring low MCS and dense DMRS) and/or the environment where LBT failure probability is low, these functions should be disabled. For the function related to COT sharing (i.e., COT sharing information on CG-UCI), the merit is not related to miss-detection and LBT failure probability. COT sharing can be used to reduce LBT delay by using Cat.1 or Cat.2 LBT instead of Cat.4 LBT. If the whole CG-UCI field is disabled (CG-UCI is not transmitted on CG PUSCH), delay can be increased because of the lack of COT sharing information. Then, at least COT sharing information should be indicated by CG-UCI in some cases.

Proposal 3: When configured grant Type 1 or Type 2 are configured on unlicensed spectrum, it would be useful that at least functions related to autonomous retransmission on CG and COT sharing are separately enabled/disabled by the configuration.

In RAN1#104e, it was suggested to use following table for better understanding of each option.

Table 1: Feature combinations for each option

(Note: 0 means the feature is disabled/not configured, 1 means enabled/configured.

|  | CG-UCI | CG-DFI | cg-RetransmissionTime-r16 |
| --- | --- | --- | --- |
| IIoT like mechanism (Option 1) | 0 | 0 | 0 |
| Option 2b: Combination 1 | 0 | 0 | 1 |
| Option 2b: Combination 2 | 0 | 1 | 0 |
| Option 2b and Option 2a: Combination 3 | 0 | 1 | 1 |
| Option 2b, Option 2a, Option 3: Combination 4 | 1 | 0 | 0 |
| Option 2b and Option 3: Combination 5 | 1 | 0 | 1 |
| Option 2b and Option 3: Combination 6 | 1 | 1 | 0 |
| NR-U like mechanism (Option 1) | 1 | 1 | 1 |

In our view, the functions related to autonomous transmission on CG (e.g., the CG retransmission timer, DFI, indication of NDI and RV on CG-UCI) can be enabled/disabled jointly via cg-RetransmissionTime-r16. Therefore, at least Combination 1, 2, 3, 5, and 6 are not necessary. On the other hand, Combination 4 should be supported in order that COT sharing information is indicated by CG-UCI, which is independently enabled or disabled of the functions related to autonomous transmission on CG.

Proposal 4: The functions related to autonomous transmission on CG (e.g., DFI, indication of NDI and RV on CG-UCI) are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

Proposal 5: The functions related to COT sharing information in CG-UCI is independently enabled or disabled.

Conclusion

In this contribution, we provide our view on potential enhancements related to unlicensed band URLLC/IIoT. We made following proposals.

Proposal 1: In semi-static channel access mode, when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, if the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 2: In semi-static channel access mode, when a UE can operate as an initiating device, whether a scheduled UL transmission is based on a UE-initiated COT or sharing a gNB-initiated COT is determined based on the content in the scheduling DCI. If the corresponding field(s) is absent, the determination is based on the rules applied for configured UL transmission.

Proposal 3: When configured grant Type 1 or Type 2 are configured on unlicensed spectrum, it would be useful that at least functions related to autonomous retransmission on CG and COT sharing are separately enabled/disabled by the configuration.

Proposal 4: The functions related to autonomous transmission on CG (e.g., DFI, indication of NDI and RV on CG-UCI) are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

Proposal 5: The functions related to COT sharing information in CG-UCI is independently enabled or disabled.

Reference

[1] 	RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR,” Nokia, Nokia Shanghai Bell, RAN#88e.

- Appendix: Agreements in the previous meetings

RAN1#102e

Agreement:

- For semi-static channel access mode

  - If sensing is needed, it is performed immediately before the configured / scheduled transmission opportunity.

  - For operation with semi-static channel access, the Rel.16 random starting offsets for UL configured grants with full bandwidth allocation when UE initiates a COT, is not supported.

Agreement:

- For semi-static channel access mode

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT.

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT.

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB.

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that FFP in which the gNB shares the COT initiated by the UE.

  - FFS: Whether/how to support additional restrictions to the idle period

Agreement:

- For semi-static channel access mode, support using the transmission of any scheduled / configured UL channel / signal to initiate a COT by a UE in RRC CONNECTED mode.

  - FFS: The case when the UE is IDLE / INACTIVE mode

Agreement:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9 us immediately before the UL transmission burst.

Agreement:

- Conditions on the channel access procedures with respect to sensing during and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel.16 by exchanging UE and gNB roles.

Agreement:

- UE-to-gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UE transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS: Details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS: Whether/how UE to gNB COT sharing when the gap is >16 us.

Agreement:

- For semi-static channel access mode.

  - Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

  - FFS: FFP periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT

Agreement:

- For semi-static channel access mode

  - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

    - FFS on to be provided by SIB1

  - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreement:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

- Further study and decide how to harmonize the CG features for Rel.16 URLLC and Rel.16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

RAN1#103e

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Conclusion:

- If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

- If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y.

- Note 1: One of the device X and Y is a UE and the other is its serving gNB.

- Note 2: Whether or not there is additional restriction on idle period is still FFS.

Agreement:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200 ms.

Agreement:

- In semi-static channel access mode, FFP period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

  - Note: Any value for the period, shall be at least 1 ms and at most 10 ms.

  - Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreement:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell.

Agreement:

- UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16 us is supported.

Agreement:

- In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

  - UE determines the initiator of a COT based on at least one of the following alternatives.

    - Alt.1: Introduce additional bit field in the scheduling DCI

    - Alt.2: Based on ChannelAccess-CPext field in DCI

    - Alt.3: Based on a predetermined rule(s)

    - Alt.4: Based on RRC signalling

    - Alt.5: Based on MAC CE

    - FFS: Other alternatives

  - FFS on overriding possibility and/or the assumption

  - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreement:

- In semi-static channel access mode

  - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down select one of the following.

    - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

    - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

    - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

  - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE

    - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT.

    - Otherwise, if the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

  - FFS on other conditions for determining the corresponding UE or gNB initiated COT

  - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreement:

- Down select one of the following options (target RAN1#104e).

  - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

  - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e., new parameter X and cg-RetransmissionTimer-r16, respectively.

  - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e., new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

  - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

  - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel.16 where the values of the respective fields of CG-UCI are decided by UE.

  - Note: Procedures based on CG-DFI rely on automatic retransmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for retransmissions.

RAN1#104e

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5, 10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode

  - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT.

  - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability.

  - FFS offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

    - The offset value range is 0 <= offset < FFP period of UE-initiated COT.

      - FFS on X (e.g., X = 0, or X = even index number)

Agreement:

- In semi-static channel access mode when a UE can operate as initiating device

  - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT.

    - Alt.a: Determination based on the content in the scheduling DCI

      - FFS on whether the corresponding field(s) can be absent in DCI

        - If absent, determination based on the rules applied for configured UL transmission is applied.

      - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

    - Alt.b: Determination based on the rules applied for a configured UL transmission

Agreement:

- In semi-static channel access mode when a UE can operate as UE-initiated COT

  - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT.

    - Alt.a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

    - Alt.b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NT IIoT Rel.16 based CG.

  - FFS whether/how to enhance

RAN1#104bis-e

Agreement:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC CONNECTED mode.

Agreement:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e., X = even indexed number in RAN1#104e).

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT.

  - Note: The gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carriers the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carriers the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details

Agreement:

- Option 2-b and Option 3 are not considered further for the agreement in RAN1#103e regarding CG harmonization.

Agreement:

- Select one of the following options (aiming for RAN1#105e).

  - Option 1: Do not support PUSCH repetition Type B based on NR-U Rel.16 CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B based on NR-U Rel.16 CG for unlicensed operation. FFS whether/how to enhance.

Agreement:

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreement:

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details.