3GPP TSG RAN WG1 #105-e		R1-2105161

e-Meeting, May 10th – 27th, 2021

Agenda Item: 	8.3.1.2

Source: 	Sony

Title: 	Considerations on CSI feedback enhancements

Document for: 	Discussion / decision

# Introduction

In RAN1#104bis-e we made the following agreements:

Agreements:

For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3):

- Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.)

  - Companies are encouraged to provide more details in their analysis

- FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits)

- FFS: Whether quantity reported is relative to the scheduled MCS

Agreement: Focus study on the following for new reporting Case 1:

- Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection.

  - Downselect by RAN1#105 to at most a single method from the following options:

    - Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

    - CSI based on worst IMR occasion (FFS details)

    - Interference standard deviation (FFS details)

    - Worst-M CQI (FFS details)

  - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

  - Applicable for same reporting quantity as R16 for CQI.

  - FFS: Whether network configured channel and interference measurement interval can also be applied

  - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated

  - FFS: how to report the updated CQI

  - FFS: whether the CQI processing time can be is reduced compared to Rel-16 CSI processing delay

This contribution discusses some considerations on CSI enhancement for URLLC.

# Discussions

## Case 1 CSI Enhancements

For Case 1 CSI enhancements the following schemes were agreed for further considerations [1]:

- Scheme 1: Mean-CQI/SINR and stdev-CQI/SINR

- Scheme 2: CSI based on worst IMR occasion

- Scheme 3: Interference standard deviation

- Scheme 4: Worst-M CQI

- Scheme 5: Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Scheme 6: Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

Scheme 1 reports the mean and standard deviation of the CQI/SINR so that the gNB is aware of the distribution of the CQI/SINR and therefore is able to estimate the worse/best CQI/SINR that the UE may experience.  It was noted that the gNB can collect CQI/SINR reports from the UE and calculate the mean and standard deviation but it was argued that this would lead to an excessive number of CQI/SINR reports and measurements at the UE side.  Hence, sending the mean and standard deviation is a way to compress multiple CQI/SINR measurements into a single report.  We therefore see some benefit in this scheme.

Proposal 1: Support reporting mean and standard deviation of CQI/SINR.

Scheme 2 reports CSI based on the worst IMR occasion.  This can be captured in the CQI/SINR statistics provided in Scheme 1 since the standard deviation can give the gNB a picture of the worst CQI/SINR condition.  Since Scheme 1 provides more information than just the worst CSI, we do not see a need for Scheme 2.

Proposal 2: Do not support new CSI reporting based on worst IMR occasion.

Scheme 3 reports the standard deviation of the interference [2].  Without the mean value, there is little benefit in reporting only the standard deviation.  Furthermore, similar information can be obtained from Scheme 1.  Hence, we do not see the need to include this scheme.

Proposal 3: Do not support new CSI reporting on interference standard deviation.

Scheme 4 reports the worst M sub-band CQI [3] so that the gNB is aware of the worst condition across the frequency domain which can help the gNB in either avoiding PRBs in these M sub-bands or schedule conservatively.  This information can be obtained from the existing sub-band CQI reporting.  However, sub-band CQI reporting has a lower granularity and therefore, we can consider increasing the granularity of sub-band CQI reporting as proposed in Scheme 5.  Hence this scheme can be considered together with Scheme 5.

Proposal 4: Consider increasing the granularity of sub-band CQI reporting.

Scheme 6 updates CQI only in a report where the CQI is conditioned on a previous instance of a RI/PMI/(CRI) report.  This scheme proposed that the CQI is derived from CSI-IM only instead of CSI-RS and CSI-IM as in the legacy method that reports CQI and RI/PMI/(CRI).  It was noted in [1] that the results presented in [4] using this scheme have lower percentage of satisfied UEs and lower resource utilisation compared to Rel-15 methods, that is, this scheme performs worse than Rel-15 methods.  It was argued that Scheme 6 allows faster CQI processing at the UE but it isn’t clear how much faster the processing can be done and also it is observed in [3] that faster CQI processing does not improve system level performance.  Hence, we do not see a clear benefit in this scheme.

Proposal 5: Do not support new CQI only update that is conditioned on previous RI/PMI/(CRI) report instance.

## Case 2 CSI Enhancements

The only scheme in Case 2 CSI enhancement being considered is the reporting of delta CQI/MCS.  Here the UE reports the difference between the CQI/MCS of the scheduled PDSCH and the CQI/MCS required for that PDSCH in order to achieve a target BLER, e.g. 10-5.  We consider some aspects of this scheme.

Firstly, we need to decide whether the UE is reporting delta CQI, delta MCS or both.  CQI is reported by the UE whereas gNB schedules the MCS based on the reported CQI.  There is therefore already some function that maps CQI to MCS at the gNB and so we do not think there is a need for the UE to report both CQI and MCS.

Observation 1: There is no need to report both delta CQI and delta MCS since the gNB can obtain one unit from another.

In [5] & [6], it is proposed that the UE reports delta MCS since the MCS of the scheduled PDSCH is already indicated by the gNB and the UE needs only to work out the required MCS to achieve the target BLER.  If the UE were to report delta CQI, then the UE would need to convert the MCS of the scheduled PDSCH to a CQI value and then work out the CQI required to achieve the target BLER.  Since reporting the delta MCS requires less computation at the UE, we have a preference for the UE to report only delta MCS.

Observation 2: In reporting delta MCS, the scheduled MCS is already signalled to the UE for the scheduled PDSCH.  On the other hand, in reporting delta CQI, the UE may need to convert the scheduled MCS to a scheduled CQI, which requires additional computations.

Proposal 6: For Case 2 CSI enhancement, the UE reports only delta MCS.

Secondly, it should be noted that a UE may be allocated different types of traffic, e.g. eMBB and URLLC, which have different BLER targets.  Since delta MCS reports the difference between scheduled MCS and MCS required for a target BLER, the UE needs to know which BLER target it should use in calculating the delta MCS or whether the UE needs to report the delta MCS at all.  Therefore, there is a need for the gNB to indicate either the target BLER or whether a delta MCS report is required.  One way for the gNB to indicate the target BLER of a PDSCH for delta MCS calculation is to use the RNTI of the DL Grant, namely MCS-RNTI for URLLC BLER target and C-RNTI for eMBB BLER target.

Observation 3: Since delta MCS determination is based on a target BLER and a UE can have different traffic types with different BLER targets, the UE needs to know the target BLER of each PDSCH in order to determine the delta MCS.

Proposal 7: The gNB uses the RNTI of the DL Grant, i.e. MCS-RNTI and C-RNTI, to indicate different BLER targets for the calculation of delta MCS or to indicate whether the UE needs to report delta MCS.

For SPS PDSCH transmission, the gNB can configure the target BLER that the UE should use in calculating delta MCS.  The gNB should also be able to disable/enable the reporting of delta MCS for each SPS configuration.

Proposal 8: The gNB can configure the target BLER and enable/disable delta MCS reporting for each SPS configuration.

Another consideration is whether the delta MCS is reported per PDSCH or per multiple PDSCHs.  Reporting delta MCS per PDSCH would generate a lot of overhead.  Whereas reporting delta MCS  over multiple PDSCHs would reduce overhead, we would need to define how to determine the delta MCS over multiple PDSCHs, e.g. using an average or max function.

Observation 4: Reporting delta MCS per PDSCH can lead to high uplink overhead.

Observation 5: Reporting delta MCS per multiple PDSCHs requires a function, e.g. average or max, to calculate a delta MCS value from multiple PDSCHs.

Proposal 9: Consider whether delta MCS report is on a per PDSCH or per multiple PDSCHs basis.

# Conclusion

In this contribution, we discuss some aspects of enhanced CSI.  We observe the following:

Observation 1: There is no need to report both delta CQI and delta MCS since the gNB can obtain one unit from another.

Observation 2: In reporting delta MCS, the scheduled MCS is already signalled to the UE for the scheduled PDSCH.  On the other hand, in reporting delta CQI, the UE may need to convert the scheduled MCS to a scheduled CQI, which requires additional computations.

Observation 3: Since delta MCS determination is based on a target BLER and a UE can have different traffic types with different BLER targets, the UE needs to know the target BLER of each PDSCH in order to determine the delta MCS.

Observation 4: Reporting delta MCS per PDSCH can lead to high uplink overhead.

Observation 5: Reporting delta MCS per multiple PDSCHs requires a function, e.g. average or max, to calculate a delta MCS value from multiple PDSCHs.

We therefore propose the following:

Proposal 1: Support reporting mean and standard deviation of CQI/SINR.

Proposal 2: Do not support new CSI reporting based on worst IMR occasion.

Proposal 3: Do not support new CSI reporting on interference standard deviation.

Proposal 4: Consider increasing the granularity of sub-band CQI reporting.

Proposal 5: Do not support new CQI only update that is conditioned on previous RI/PMI/(CRI) report instance.

Proposal 6: For Case 2 CSI enhancement, the UE reports only delta MCS.

Proposal 7: The gNB uses the RNTI of the DL Grant, i.e. MCS-RNTI and C-RNTI, to indicate different BLER targets for the calculation of delta MCS or to indicate whether the UE needs to report delta MCS.

Proposal 8: The gNB can configure the target BLER and enable/disable delta MCS reporting for each SPS configuration.

Proposal 9: Consider whether delta MCS report is on a per PDSCH or per multiple PDSCHs basis.

# References

[1] R1-2103956, “Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT ,” Moderator (InterDigital, Inc.), RAN1#104bis-e

[2] R1-2102759, “CSI feedback enhancements for URLLC,” Futurewei, RAN1#104bis-e

[3] R1-2103434, “CSI feedback enhancements for URLLC/IIoT use cases,” Nokia, Nokia Shanghai Bell, RAN1#104bis-e

[4] R1-2102522, “CSI feedback enhancements for Rel-17 URLLC,” vivo, RAN1#104bis-e

[5] R1-2102393, “CSI feedback enhancements for URLLC,” OPPO, RAN1#104bis-e

[6] R1-2102629, “CSI feedback enhancements,” CATT, RAN1#104bis-e