3GPP TSG RAN WG1 #105-e		                           R1-2105169

e-Meeting, May 10th – May 27th, 2021

Agenda Item: 	8.5.2

Source:	Sony

Title:	Discussion on accuracy improvements for UL-AoA positioning method

Document for:	Discussion & Decision

# Introduction

In RANP#91e, a work item on NR positioning enhancements was updated [1]. The objective of the work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. One of the objectives of this work item is:

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

In RAN1#104bis-e [2], the following agreements have been made regarding to the potential accuracy enhancement for UL-AoA positioning:

Agreement 1 - The following option is supported to enhance signaling of UL-AOA measurement report in case of a linear array - Option 2: The z-axis of LCS is defined along the linear array axis. gNB reports only the ZoA relative to z-axis in the LCS, and the LCS-to-GCS translation function is used to set up the specific z-axis direction - UL-AOA signalling details for support of Option 2 are left up to RAN WG3 Agreement 2 - Uncertainty range for expected UL AoA/ZoA is defined as follows - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival - Expected zenith angle of arrival as (θAOA - ΔθAOA/2, θAOA + ΔθAOA/2) - θAOA - expected zenith angle of arrival, ΔθAOA – uncertainty range for expected zenith angle of arrival - Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication - FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range) Agreement 3 Reporting to LMF of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp is supported.

In this contribution, we discuss and provide our view on the remaining issues of the potential solution to enhance the UL-AoA positioning accuracy.

# Discussion on Assistance Information

Regarding the coordinate system for signaling UL AoA/ZoA assistance information (noted as agreement 2 above), our view is that both alternatives are basically feasible. The expected AoA/ZoA is calculated from a coarse UE location estimation. Since this location is using GCS, the expected AoA/ZoA should also be in GCS, accordingly. The remaining question is whether the transformation from GCS to LCS should be done in LMF or not. The coordinate transformation requires knowing the orientation of a TRP Rx. In practice, this information is available in both TRP and some LMF. For consistency, our suggestion is that expected angles in assistance information should keep the same format as the regular AoA/ZoA reporting. In NRPPa, both GCS and LCS for AoA/ZoA measurements are supported, which are indicated by an IE ‘LCS to GCS Translation’ with a range of 0 or 1[2]. Alternative 2 in the agreement share the same idea as in the regular AoA measurement reporting.

Proposal 1: Support both GCS and LCS for AoA/ZoA assistance information indication to maintain the same reporting format as the regular AoA/ZoA measurement reporting.

# Potential enhancements

## 3.1 AoA uncertainty reporting

Most of the TRPs are equipped MIMO antenna with multiple element rows and columns. A typical antenna sizes are 4 by 4 MIMO in FR1 and 4 by 8 in FR2. In UL-AoA positioning, each element row can be regarded as a linear array. The AoA can be measured at each element row and the measurements are independent. Likewise, the ZoA can be independently measured by each  column of elements. As a result of it, there could be multiple independent AoA and ZoA measurements at different element rows or columns. To illustrate, we provide the following figure about multiple AoA/ZoA measurements by FR1 MIMO antenna model. In FR1 MIMO antenna model as shown in Figure 1, there could be 4 independent AoA measurements and 4 ZoA measurements in total.

AoA measurement 1

AoA measurement 2

AoA measurement 3

AoA measurement 4

ZoA1

ZoA2

ZoA3

ZoA4

Figure 1: Illustration of multiple AoA/ZoD measurements given by different rows/columns.

A simple solution to process multiple AoA/ZoA measurements is to calculate the average of them. After that, the TRP can only report the average value as the final measurement to the LMF.

Our study also shows that the uncertainty/standard deviation of multiple AoA/ZoA measurements at different rows/columns can also be utilized to indicate the channel quality and the reliability of the AoA/ZoA measurement. This uncertainty reflects the statistical property of the measured AoA results. For simplicity, we name this new parameter as “AoA uncertainty” or “uAoA”.

Similar to the usage of SRS-RSRP, the LMF can use uAoA to filter out some unreliable AoA measurements. The qualified AoA measurement would be used in the calculation of the UE position estimation. In order to verify the proposed method, a simulation on UE positioning estimation in indoor factory (InF) scenario was carried out.

We have performed simulations using InF-sparse high (SH) and InF-dense high (DH) for both FR1 and FR2 frequency range. The simulation results are shown in Figure 2. The CDF of the legacy UL-AoA positioning accuracy (with RSRP reporting) is also given in the simulation as the reference. It is shown that reporting uAoA as the AoA measurement indicator improve the UE positioning estimation accuracy. The accuracy of the proposed method in terms of horizontal location error for X% (X = 50, 67, 80, 90) of the UEs is given in the Table 1. Particularly on the horizontal accuracy for 90% CDF of the UE, one can observe that the gain of the proposed method can be up to 0.8 meters in InF-DH scenario. Even in the one of most ideal cases, InF-SH FR2 scenario, the proposed method can obtain 68% gain in positioning accuracy over the legacy UL-AoA. Simulation results in InF-SH FR1,2 and InF-DH FR1 show that the industrial IoT accuracy requirement can be fulfilled (i.e less than 0.2 m for 90% of the UEs). In InF-DH FR2, the obtained accuracy for 90% of the UEs is 0.26m, in which it is almost meeting the accuracy requirement.

Figure 2: Comparison between UL-AoA positioning with AoA uncertainty reporting and the legacy UL-AoA positioning in terms of positioning accuracy in meter.

The simulation results in Figure 2 are also summarized in Table 1.

Table 1: UL-AoA positioning - horizontal location error results (m) for X% of the UEs in meters

| Test Case | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| [Case 1], [UL-AoA], [InF-SH], [FR2], [400 MHz], [Comb-4], [6dB PB], [w/ AoA uncertainty reporting] | 0.019 | 0.024 | 0.031 | 0.044 |
| [Case 2], [UL-AoA], [InF-SH], [FR2], [400 MHz], [Comb-4], [6dB PB], [Legacy UL-AoA] | 0.019 | 0.027 | 0.040 | 0.074 |
| [Case 3], [UL-AoA], [InF-DH], [FR2], [400 MHz], [Comb-4], [6dB PB], [w/ AoA uncertainty reporting] | 0.0084 | 0.012 | 0.020 | 0.17 |
| [Case 4], [UL-AoA], [InF-DH], [FR2], [400 MHz], [Comb-4], [6dB PB], [Legacy UL-AoA] | 0.014 | 0.025 | 0.088 | 1.08 |
| [Case 5], [UL-AoA], [InF-SH], [FR1], [100 MHz], [Comb-4], [6dB PB], [w/ AoA uncertainty reporting] | 0.032 | 0.045 | 0.066 | 0.10 |
| [Case 6], [UL-AoA], [InF-SH], [FR1], [100 MHz], [Comb-4], [6dB PB], [Legacy UL-AoA] | 0.037 | 0.060 | 0.093 | 0.17 |
| [Case 7], [UL-AoA], [InF-DH], [FR1], [100 MHz], [Comb-4], [6dB PB], [w/ AoA uncertainty reporting] | 0.014 | 0.024 | 0.047 | 0.26 |
| [Case 8], [UL-AoA], [InF-DH], [FR1], [100 MHz], [Comb-4], [6dB PB], [Legacy UL-AoA] | 0.013 | 0.035 | 0.31 | 0.98 |

Observation 1: The uAoA indicator can improve the UL-AoA positioning accuracy in any scenarios. The gain from the proposed method can up to 0.8 m in InF-DH case. Even in InF-SH FR2, the method can still obtain 68% gain over the legacy AoA.

Observation 2: The accuracy of the proposed method in InF-SH FR1,2 and InF-DH FR1 can fulfil the IIoT requirement (less than 0.2 m for 90% of the UE). In InF-DH FR2, the obtained accuracy for 90% of the UEs is 0.26m, in which it is almost meeting the accuracy requirement.

We propose to enhance the reporting from gNBs to the LS by including the statistical property of the measured AoA in which it can reflect the uncertainty of the AoA (uAoA). This is to be reported as part of the measurement report which enable the LS to process (filtering) the AoA measurement effectively.

Proposal 2: Support the statistical property (e.g., standard deviation of AoA) of the measured AoA in UL-AoA measurement report from gNB to LS.

In RAN1#104e, it was discussed and agreed that multiple AoA reports based on the first path can be provided by gNB. We consider this agreement is also aligned with our proposal on providing the statistical property of the measured AoA. In practice, gNB may have obtained multiple AoA results. The number of multiple AoA that can be reported may be limited. Hence, the statistical property of the measured AoA is a measure of all AoA results that the gNB has obtained. The LMF can use this information for the UE positioning estimation.

## 3.2 SRS resource ID

Another approach to enhance AoA positioning method is by providing UE TX beam related information to the LS. UE transmits UL SRS for positioning with certain TX beam direction / properties and it also transmit in SRS resource that has been configured by the gNB. The detected received SRS (e.g. SRS that has high RSRP) in a specific resource can represent UE TX beam. For example, if there are 3 gNBs located in various directions then each gNB may detect different SRS resource that produce high RSRP. We propose gNB to report the SRS resource ID that has been used for the AoA measurement. Having this SRS resource ID information enable the LS to filter out undesired report from certain gNB or basically to filter out any anomaly results.

Furthermore, the LMF can estimate the AoD in the UE coordinate because each SRS resource ID is associated with different UE Tx beams and different AoDs. Although the orientation of the UE is unknown and the AoD can’t be transferred from LCS to GCS, LMF can still calculate the AoD differences between reference TRP and other TRP as in the figure shown below:

TRP1

Reference TRP

TRP2

UE with unknown orientation

θ1

θ2

SRS 1

SRS 2

SRS 3

SRS 4

SRS 5

Figure 2: The AoD differences can be estimated if the SRS resource IDs at different TRPs are known in LMF.

If more than 3 TRPs are used, LMF can use the AoD differences to estimate the UE location without knowing the UE orientation. Moreover, this information can assist the UE location estimation by the AoA and improve the positioning accuracy.

Proposal 3: Support SRS resource ID information of the associated reported AoA  / SRS-RSRP measurement in UL-AoA measurement report from gNB to LS.

# Conclusion

In this contribution, we discuss some aspects of positioning accuracy improvements using UL-AoA method.  We provide the following observations:

Observation 1: The uAoA indicator can improve the UL-AoA positioning accuracy in any scenarios. The gain from the proposed method can up to 0.8 m in InF-DH case. Even in InF-SH FR2, the method can still obtain 68% gain over the legacy AoA.

Observation 2: The accuracy of the proposed method in InF-SH FR1,2 and InF-DH FR1 can fulfil the IIoT requirement (less than 0.2 m for 90% of the UE). In InF-DH FR2, the obtained accuracy for 90% of the UEs is 0.26m, in which it is almost meeting the accuracy requirement.

We also provide the following proposals:

Proposal 1: Support both GCS and LCS for AoA/ZoA assistance information indication to keep the same report format as the regular AoA/ZoA measurement reporting.

Proposal 2: Support the statistical property (e.g., standard deviation of AoA) of the measured AoA in UL-AoA measurement report from gNB to LS.

Proposal 3: Support SRS resource ID information of the associated reported AoA  / SRS-RSRP measurement in UL-AoA measurement report from gNB to LS.

# Reference

RP-210903, Revised WID on NR Positioning Enhancements

RAN1, Chairman’s Notes RAN1#104b-e 8.5, RAN1#104b-e, e-Meeting, April 12th – April 20th, 2021.

3GPP TS 38.455 V16.3.0 NR Positioning Protocol A (NRPPa) (Release 16)  (2021-04)

R1-2103400, Enhancement for UL AoA positioning, Huawei, HiSilicon, 3GPP TSG RAN WG1 Meeting #104bis-e E-meeting, April 12th – April 20th, 2021

R1-2103036, Enhancements of UL-AOA Positioning Solution, Intel Corporation, 3GPP TSG RAN WG1 #104bis-e   e-Meeting, April 12th – 20th, 2021