3GPP TSG RAN WG1 #105-e                                      R1-2105197

e-Meeting, May 10th – 27th, 2021

Source:	ZTE

Title:	Discussion on additional enhancements for IoT-NTN

Agenda Item:	8.15.5

Document for:   Discussion

- Introduction

In RAN1#104e, the SI for IoT-NTN [1] has been discussed with agreements on the scenarios and UE assumptions.

Agreement:

- Capture in TR 36.763, moderator’s summary of GNSS Position fix impact on UE power consumption based on Appendix A Section 5.1

- Capture in TR 36.763, individual companies battery life analysis in Appendix A

Agreement:

Study potential impact of NTN SIB carrying the satellite ephemeris on

- UE power consumption in NB-IoT and eMTC

- Accuracy of satellite location tracking

- PRACH congestion

Agreement:

Capture the following in the TR:

The required power consumption to read SIB containing satellite ephemeris information for the short sporadic connections use case is not significant.

- Note: For this conclusion, it is assumed that the UE need not read broadcast SIB for the purpose of obtaining satellite ephemeris information in CONNECTED mode.

In this contribution, identified issues including beam management, polarization indication/reporting and power consumption are discussed.

- Technical issues to support IoT over NTN

  - Beam management

In NTN deployment, the coverage of a NTN cell can be implemented by either multiple satellite beams or a single satellite beam, as illustrated in Figure 1.

Figure 1 Mapping between beams and cell in [2]

In NR-NTN discussion, both options are supported. However, from the viewpoint of reducing frequent handover, option A (a cell with multiple beams) is preferred. More specifically, with the deployment in option B, (1) RRC_CONNECTED UEs have to handover more frequently, and (2) RRC_IDLE UEs have to perform measurements and carry out cell reselection more frequently, which is also not preferred by the IoT device. For example, in legacy NB-IoT, connected mobility (e.g., handover and measurement report) is not supported, since both NB-IoT UEs and eNBs are assumed to be generally stationary on Earth, which is not held in NTN scenarios.

Since there is no concept of beam in legacy LTE including NB-IoT, following options can be considered to define beams in IoT over NTN scenarios:

- Explicit beam ID indication:

For a cell with multiple satellite beams, beam ID can be included in system information as a new information element. For example, a beam ID can be constructed using a similar way as used by current cell ID construction, i.e., . The  can be used in PBCH bit scrambling together with the . Or the  can be used as cover code applied on multiple OFDM symbols of SSS. And the  is included in MIB as a new information element, which is only applicable for IoT over NTN scenarios.

- Implicit beam ID indication:

In IoT over NTN scenarios, a key performance criterion is coverage, and the NB-IoT PBCH repetition can be inherited to combat large path loss. The NB-IoT PBCH is transmitted with a period of 640ms. Each 640ms period contains 8 sub-period of 80ms. In each 80ms sub-period, an identical 1ms PBCH is transmitted in the subframe #0 of each radio frame. Based on high LOS probability and GNSS capability on UE side, extra cover code can be applied to each PBCH repetitions to carry. Besides, the scrambling code applied on PBCH or PBCH CRC can also be used to carry, or part of  (e.g.,  and/or) in an implicit way. Furthermore, considering resource reuse scheme among beams, a given beam may use a predefined frequency resource only, e.g., anchor/non-anchor carrier for NB-IoT and narrowband for eMTC. The association between beam and resource can be studied in this case.

In addition, in case of transmission with larger repetition, the beam switching may occur during the transmission time interval, which is different from the case in NR. As a result, for IoT over NTN, corresponding enhancements should also be studied to ensure the performance.

Proposal 1: Multiple beams per cell should be supported with potential enhancement to enable the beam management for IoT over NTN.

Proposal 2: Mechanisms to enable the beam identification in IoT over NTN should be considered via both explicit and implicit ways.

Proposal 3: In IoT over NTN, mapping between beam and frequency resource should be studied.

Proposal 4: In IoT over NTN, enhanced repeated transmission to tackle the interrupt due to beam switching should be studied.

  - Polarization indication

In the NR-NTN discussion, the intention for enabling the polarization indication is to fix the potential polarization loss for link budget due to the mismatch among UE and satellite configuration, which is beneficial for the system with terminal supporting different polarization. In IoT-NTN, since the circular the polarization can also be optional supported by the terminal, the indication of polarization can be included in system information or RRC configurations.

- Option-1: The indication of polarization information can be included in the configuration of each channel. In this way, it’s assumed that for each transmission of PDSCH/PDCCH, the corresponding polarization information should be explicitly indicated. For example, the SIB based solution is preferred for the transmission/reception of common channel. For example, in NPDSCH-ConfigCommon-NB of SystemInformationBlockType2-NB, polarization can be included.

- Option-2: The RRC based solution can also be considered for polarization indication in UE specific channel configuration. For example, in NPDCCH-ConfigDedicated-NB of RadioResourceConfigDedicated-NB, polarization can be included.

To better support terminals with different polarization capability, a satellite with advanced polarization capability can optimize its scheduling if the UEs can report their polarization capability to the eNB/gNB. Then, the eNB/gNB can divide the UEs into groups and schedule the group of UEs in appropriate polarization period interval. The UE’s polarization capability may include one or more of {linear polarization, cross linear polarization, left hand circular polarization (LHCP), right hand circular polarization (RHCP)} for both transmitting and receiving direction shall be indicated, respectively. The capability of supporting adjustable polarization for transmission and reception can also be included. The UE’s polarization capability can be reported to eNB/gNB using UECapabilityInformation-NB message or UECapabilityInformation message, respectively. To be more specific, aforementioned polarization capability can be added into UECapabilityInformation-NB or UE-NR-Capability information element, respectively.

Proposal 5: DL polarization indicated by SIB or RRC can be considered.

Proposal 6: UE polarization reporting can be supported.

  - Enhancement on Power saving for UE in IoT-NTN

In legacy NB-IoT, the power saving behavior of a UE is mainly determined by the PSM and eDRX configuration. Then, a UE will periodically enter the sleep mode or standby mode for power saving. However, in NTN case, due to the impacts of larger RTT, especially for GEO case, there will be certain gap between consecutive scheduling. In this case, an enhancement on the power saving scheme by enabling the aperiodic deactivation can be considered to achieve better power saving in NTN coverage.

Proposal 7: Enhance the legacy eDRX and PSM techniques to support aperiodic sleep.

Furthermore, in the NTN system, the service quality may not be continuous due to the constellation configuration or satellite type (e.g., cube satellite). In this this way, the service quality will be discontinues as shown in Figure 2. In legacy system, once the channel quality is poor, the UE will attempt to initialize the procedure for handover or RFL, which will be invalid and inefficient for power consumption in NTN case.

For example, in an NTN system with earth-fixed beam, the satellite uses steerable beams to cover the target areas. Once the elevation of service link (or feeder link) is too small, beam switching will occur and the service may be discontinuous. Since this case is highly predictable at the network side, the eNB can provide assistance information to UEs, such as: (1) the serving time of current and/or next satellites for a given area, and/or (2) indication of disabling radio link reconnection. For a RRC_CONNECTED NTN UE, if physical layer problem is detected (e.g., upon receiving N311 consecutive "out-of-sync" indications), the UE starts a timer (e.g., T310) to monitor the recovery of radio link. The UE will check the assistant information to decide whether to disable immediate radio link reconnection. The UE will carry out radio link reconnection until (1) its T310 timer expires, and (2) the next satellite serving time starts.

Figure 2 Illustration on the discontinuous service

Moreover, similar situation is also expected for eDRX/PSM procedure. With the invalid service, the periodically waking up of the UE for PDCCH monitoring is also not needed from power saving perspective. Since the service quality and valid duration is highly predictive from network perspective for each UE, assistance information, e.g., time duration, can be indicated to improve the power efficiency.

Proposal 8: Indication of assistance information by network to UE can be considered to optimize the legacy RLF, HO, eDRX and PSM procedure for power saving in case of discontinuous service.

- Conclusions

In this contribution, technical issues and possible solutions to support IoT over NTN is conducted with following proposals:

Proposal 1: Multiple beams per cell should be supported with potential enhancement to enable the beam management for IoT over NTN.

Proposal 2: Mechanisms to enable the beam identification in IoT over NTN should be considered via integrating beam ID into PCI or applying cover code on repetitions of PBCH.

Proposal 3: In IoT over NTN, mapping between beam and frequency resource should be studied.

Proposal 4: In IoT over NTN, enhanced repeated transmission to tackle the interrupt due to beam switch should be studied.

Proposal 5: DL polarization indicated by SIB or RRC can be considered.

Proposal 6: UE polarization reporting can be supported.

Proposal 7: Enhance the legacy eDRX and PSM techniques to support aperiodic sleep.

Proposal 8: Indication of assistance information by network to UE can be considered to optimize the legacy RLF, HO, eDRX and PSM procedure for power saving in case of discontinuous service.

- References

- RAN1 Chairman’s note, RAN1#104e, Jan. 2021

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”