3GPP TSG RAN WG1 Meeting #105                                                                  	     R1-2105215

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.4.3

Source:	Lenovo, Motorola Mobility

Title:	Enhancements on HARQ for NTN

Document for:	Discussion and decision

Introduction

In the RAN#88e plenary meeting, an updated work item for Rel.17 NR NTN was approved. The objective is to specify the following enhancements to NR NTN.

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821):

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

The following agreements on enhancements on HARQ for NTN were achieved:

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

In this contribution, detailed considerations of HARQ enhancement for NTN, especially HARQ process number and HARQ feedback enabling/disabling configuration are presented.

Discussion

In NTN, the maximum RTT (round trip time) is 541.14ms for GEO (at 35786 km altitude) and 25.76/41.75ms for LEO (at 600/1200 km altitude) transparent satellite [1]. In order to avoid reduction in peak data rates, it was agreed that the maximal supported HARQ process number is extended up to 32 to match the longer round trip delay. Another solution is to disable UL HARQ feedback to avoid stop-and-wait in HARQ procedure and rely on RLC ARQ for reliability.

HARQ process number

In NR, up to 4-bit HARQ process number field in DCI is used to indicate the HARQ process ID, which supports up to 16 HARQ processes. For NTN scenario, in order to support the number of HARQ processes up to 32, the following solutions can be considered for HARQ process number indication in DCI dynamically:

- Option 1: HARQ process field extension (up to 5 bit)

The most straightforward way is to extend the bit of HARQ process field to 5, which may increase the DCI size.

- Option2: Reusing one bit from other bit field (No DCI size extension)

Although there is no DCI size extension, the legacy DCI feature is broken and not unified design for all DCI format.

- Option 3: HARQ process ID is tied to SFN/slot number carrying the corresponding transmission/scheduling

Instead of explicitly indicate the HARQ process ID in DCI, HARQ process ID is tied to SFN/slot number, as in synchronous HARQ. For example, ‘MSB’ 1 bit of HARQ process number is derived by slot index of PDSCH/PUSCH/PDCCH (nslot) transmission, and ‘LSB 4 bit of HARQ process number is indicated by DCI to leave some flexibility of scheduling. If UE is configured with supported maximal HARQ process number 32, the HARQ process ID is shown in Figure 1.

HARQ process ID = 16 * (nslot%2) + HARQ process field value

Figure 1 HARQ process number extension

Option 1 needs more DCI bits to supported extended HARQ process number, option 2 leads to legacy feature broken and option 3 leads to large scheduling/retransmission/ delay and restrictions. We slightly prefer option 3 to support HARQ process number indication tied to SFN/slot index of PDCCH/PUSCH/PDSCH to minimize the specification impact, which is a unified design for all DCI format.

For DCI format 0-0/1-0 and DCI format 0-1/1-1, the HARQ process number field in DCI is still 4 bit, and if UE is configured with 32 HARQ process, the HARQ process number is tied to SFN/slot index of corresponding PDCCH.

For DCI format 0-2/1-2, the HARQ process number field in DCI is determined to higher layer parameter, if UE is configured with 32 HARQ process, the HARQ process number field in DCI is 5bit.

Proposal 1: The HARQ process number is tied to SFN/slot index of PDCCH/PUSCH/PDSCH for DCI format 0-0/1-0 and DCI format 0-1/1-1 if UE is configured with HARQ process number of 32.

Proposal 2: For DCI 0-2/1-2, the HARQ process number field in DCI is determined to higher layer parameter.

Regarding the HARQ soft buffer size, supporting more than 16 HARQ process number (e.g., 32 HARQ process number) will lead to larger buffer size at both gNB and UE side for data collection. However, in NTN system, the impacts of soft buffer is negligible since the current UE capability is able to support the large data rate in terrestrial network, for NTN networks. Even so, UEs with different capabilities have different soft buffer sizes. The gNB can configure different HARQ processes numbers based on UE capability.

Proposal 3: The supported number of HARQ processes is configured based on UE capability.

HARQ feedback enabling/disabling

In TR38.821, HARQ feedback enabling/disabling is proposed to solve the problem of long round-trip delay and soft buffer size. From RAN1 and RAN2 agreeements, HARQ feedback process disabling is configured per UE and per HARQ process. Regarding per HARQ process disabling configuration, a HARQ ID subset is configured to UE as HARQ feedback disabling set, then UE can assume HARQ ID belonging to this subset should be associated with no HARQ feedback, otherwise HARQ feedback is assumed to be transmitted in indicated slots.

Proposal 4: UE assumes the HARQ feedback disabling where HARQ ID belongs to the RRC configured HARQ process disabling subset.

In case HARQ feedback disabling, the enhancement to data channel transmission is discussed in SI stage [1], for example possible enhancements for slot-aggregation or blind repetitions transmission of the same TB without waiting for HARQ feedback can be considered in WI stage.

For NTN, considering the long RTT propagation time, multiple transmissions in consecutive or interlaced slots can be considered. For consecutive repetitions transmission, similar design has been proposed in URLLC with higher layer pdsch-AggregationFactor, further dynamic repetition number scheduling can be considered in NTN, especially for HARQ disabling case of reliability enhancements requirement. For interlaced transmission, the motivation is that NTN system will have residual doppler shift in most of time, so the time interlaced slot aggregation will achieve the diversity gain. The network can configure the slot interval and the repetition number to inform the UE.

Proposal 5: The multiple transmissions of same TBs in consecutive or interlaced slots can be considered when HARQ is disabled

If the HARQ is disabled by RRC signaling, it needs further study whether some of fields should be kept or not. For example, NDI, RV, DAI, TPC command for scheduled PUCCH, PUCCH resource indicator, PDSCH-to-HARQ feedback timing indicator. If blind retransmission without waiting for the HARQ feedback is supported if HARQ disabled, HARQ soft buffer should be reserved for chase combining or IR. As discussed in above, the number of slots (i.e. PDSCHs) for blind retransmission is indicated by RRC configuration pdsch-AggregationFactor. The further repetition transmission number and interlace transmission interval can be indicated in corresponding DCI when HARQ process disabling.

Proposal 6: Repetition transmission number and interlace transmission interval can be indicated in corresponding DCI when HARQ process disabling.

In last meeting email discussion, different configurations or enhancement for transmission with/without HARQ are proposed.

Further discussion on necessity to enable the different configuration on following parameter(s) for each HARQ process with/without feedback can be further discussed

- Aggregation factor

- MCS table

- Time domain resource allocation table

- Frequency resource allocation type 0 and type 1

- Block error rate target

- Physical resource block (PRB) bundling configuration

- PDSCH mapping type A and type B

From our point of view, a unified solution is preferred for with and without HARQ, there is no necessity to introduce a new MCS table, adjust the target BLER associated with each MCS table, different frequency resource allocation or PDSCH mapping. In order to lower residual BLER for transmission without HARQ, it is up to gNB implementation by configuration of different aggregation factor/repetition factor or different DCI indication if benefit identified.

Proposal 7:  A unified configuration should be considered for each HARQ process with/without feedback except aggregation factor if benefit identified.

HARQ codebook enhancement

For  HARQ-ACK codebook, if one HARQ process is configured with HARQ feedback disabling, some companies propose to redesign the HARQ-ACK reported mechanism or format in the last meeting.

Type 1 is semi-static HARQ-ACK codebook, and the size of the HARQ-ACK codebook is fixed based on configuration and should not be change dynamically. so one simple approach for NTN is option-1: no enhancement, that is, insert ACK or NACK to HARQ-ACK codebook depending on the decoding results of the PDSCHs in the positions corresponding to PDSCHs regardless HARQ feedback enabling or disabling, which can faciliate gNB further scheduling even for HARQ feedback disabling (e.g., new MCS, new repetition number).  The UE behavior is totally the same as legacy.

Type 2 is dynamic HARQ-ACK codebook, and the size of the HARQ-ACK codebook is dyanmically deteremined by corresponding DCI. Type2 is designed to save the overhead, so we should make some enhancements on dynamic HARQ-ACK codebook with disabling/enabling HARQ feedback. One simple approach is option 1: reduce codebook size with HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes. The gNB does not increase the counter DAI and total DAI for a PDCCH with HARQ feedback disabled, which is aligned with the motivation of Type 2 HARQ codebook.

Type 3 codebooks were introduced in Rel-16 NR-U, Those codebooks are used for feedback in unlicensend spectrum scenarios. So there is no need to support Type 3 codebook for NTN.

Proposal 8: For Type 1 HARQ-ACK codebook, no enhancement is needed for NR NTN.

Proposal 9: For Type 2 HARQ-ACK codebook, HARQ-ACK codebook only includes HARQ-ACK of PDSCH with HARQ feedback enabled, and counter DAI and total DAI are not increased for a PDCCH with HARQ feedback disabled.

Proposal 10: Type 3 HARQ-ACK codebook is not supported for NR NTN.

Conclusions

In this contribution, HARQ enhancement for NTN, especially HARQ process number and HARQ feedback enabling/disabling configuration are provided. The following proposals are given.

Proposal 1: The HARQ process number is tied to SFN/slot index of PDCCH/PUSCH/PDSCH for DCI format 0-0/1-0 and DCI format 0-1/1-1 if UE is configured with HARQ process number of 32.

Proposal 2: For DCI 0-2/1-2, the HARQ process number field in DCI is determined to higher layer parameter.

Proposal 3: Different numbers of HARQ processes is configured based on UE capability.

Proposal 4: UE assumes the HARQ feedback disabling where HARQ ID belongs to the RRC configured HARQ process disabling subset.

Proposal 5: The multiple transmissions of same TBs in consecutive or interlaced slots can be considered when HARQ is disabled

Proposal 6: Repetition transmission number and interlace transmission interval can be indicated in corresponding DCI when HARQ process disabling.

Proposal 7:  A unified configuration should be considered for each HARQ process with/without feedback except aggregation factor if benefit identified.

Proposal 8: For Type 1 HARQ-ACK codebook, no enhancement is needed for NR NTN.

Proposal 9: For Type 2 HARQ-ACK codebook, HARQ-ACK codebook only includes HARQ-ACK of PDSCH with HARQ feedback enabled, and counter DAI and total DAI are not increased for a PDCCH with HARQ feedback disabled.

Proposal 10: Type 3 HARQ-ACK codebook is not supported for NR NTN.

- References

3GPP TR 38.821 V1.0.0 (2019-12)

RP-201256, “WID revision: Solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#88e

R1-2102143, “Summary#4 of AI 8.4.3 for HARQ in NTN”, Moderator (ZTE), RAN1-104e