3GPP TSG RAN WG1 Meeting #105-e                                                                 	   R1-2105219

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.6.1.3

Source:	Lenovo, Motorola Mobility

Title:	Half duplex operation for RedCap

Document for:	Discussion and decision

# Introduction

In RAN#91e, the RedCap WID [1] was updated and agreed. One objective is to specify support of half duplex RedCap UEs,

- HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

One potential specification impact of HD-FDD type A operation is how the UE handles DL/UL collision. It was agreed in RAN1#104e to study following collision cases,

- For HD-FDD operation for RedCap UEs, collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

  - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

    - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

  - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

    - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

  - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

  - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

    - e.g., PUSCH, PUCCH, PRACH, SRS

  - Case 8: Dynamic or semi-static DL vs. valid RO

  - Case 9: Collision due to direction switching

In this contribution, we provide analysis on half duplex operations for RedCap UEs and focus on the DL/UL collision handling for HD-FDD type A.

# Discussion

For collision case 1/2/4, it has been agreed in RAN1#104bis-e [2] to reuse the principles of existing collision handling in Rel.15/16 NR for operation on a single carrier/single cell in unpaired spectrum.

For case 3 of collision between semi-statically configured DL reception and semi-statically configured UL transmission, it was agreed in RAN1#104b-e (as in below) that except for the case that both DL and UL are cell-specifically configured, a HD-FDD UE does not expect such collision will happen.

Agreements:

For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

- FFS: whether or not there are conditions that need to be considered

In addition, for collision case 5, a working assumption was reached as in below.

Working assumption:

- If a semi-static configured UL transmission overlaps with an SSB, down-select from the following options

  - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL

  - Option 3: Leave to UE implementation whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

For case 3 of both DL reception and UL transmission are cell-specifically configured, the collision is mostly between the configured ROs and the cell specific configured DL channels like SSB, paging occasion, and SI transmission occasions. Besides, from the discussion in RAN1#104b-e, the semi-static configured UL transmission in case 5 contains both cell specific configurations and UE specific configurations. From this point of view, the collision case 3 has some overlap with collision case 5 of semi-static configured UL transmission vs. SSB and even collision case 8 of semi-static DL vs. ROs. Therefore, the overlapped scenarios in these three cases could be jointly considered when designing the collision handling schemes.

Observation 1: Collision case 3, case 5 and case 8 have some overlap and could be jointly considered.

Following summarizes our preference for collision handling for the overlapped scenarios of case 3, case 5 and case 8,

- For case 5, if the semi-static configured UL transmission is UE specifically configured, it corresponds to case 3 of collision between UE specifically configured UL transmission and cell specifically configured DL reception. Therefore, the solution for case 3 for this scenario is reused, i.e., the UE does not expect such collision would happen. This also means option 1 in the working assumption is adopted for such scenario.

- For case 5, if the semi-static configured UL transmission is cell specifically configured (e.g., ROs), it corresponds to case 3 of collision between cell specifically UL transmission and cell specifically configured DL reception, and also case 8 of collision between semi-static DL (if DL channel is SSB) and ROs. For this scenario, it is preferred to reuse existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL transmission.

- For case 8 of semi-static DL overlap with ROs, if the DL is UE specifically configured, it corresponds to case 3 of collision between UE specifically configured DL and cell specifically configured UL. Therefore, the solution for case 3 for this scenario is reused, i.e., the UE does not expect such collision would happen. This also means option1 in the working assumption is adopted for such scenario.

Proposal 1: A HD-FDD UE does not expect the collision of a semi-static configured UL transmission overlaps with an SSB will happen, if the UL transmission is UE specifically configured.

Proposal 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between semi-static UL transmission with SSB, if the UL transmission is cell specifically configured, i.e., that SSB is prioritized over semi-static UL transmission.

Proposal 3: A HD-FDD UE does not expect the collision of a semi-static configured DL transmission overlaps with an ROs will happen, if the DL transmission is UE specifically configured.

For the remaining cases including case 5 of collision between configured SSB and dynamic scheduled UL transmission and  case 8 of collision between dynamic DL reception and ROs, it is preferred to reuse existing corresponding Rel.15/16 collision handling principles for NR TDD. That is,

- The reception of SSB is prioritized over the overlapped dynamic scheduled UL transmission.

- The transmission of PRACH is prioritized over the overlapped dynamic scheduled DL reception.

Proposal 4: Reuse existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between dynamic UL transmission with SSB, i.e., SSB reception is prioritized over dynamic UL transmission.

Proposal 5: Reuse existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between dynamic DL transmission with ROs, i.e., PRACH transmission is prioritized over dynamic DL reception.

In LTE, the HD-FDD UE does not need a prior information of DL and UL transmission pattern, but to check PDCCH in each PDCCH monitoring occasion (i.e., each subframe in LTE) if it is not overlapped with any pre-assigned UL transmission. To minimize the standard impact, such LTE principle could be reused for RedCap HD-FDD UEs to determine the transmission pattern.

Proposal 6: The principle in LTE type-A HD-FDD on the determination of transmission pattern is reused for RedCap type-A HD-FDD.

# Conclusions

As a summary, we have the following observations and proposals on HD-FDD UE operations for RedCap,

Observation 1: Collision case 3, case 5 and case 8 have overlapped scenarios and could be jointly considered.

Proposal 1: A HD-FDD UE does not expect the collision of a semi-static configured UL transmission overlaps with an SSB will happen, if the UL transmission is UE specifically configured.

Proposal 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between semi-static UL transmission with SSB, if the UL transmission is cell specifically configured, i.e., that SSB is prioritized over semi-static UL transmission.

Proposal 3: A HD-FDD UE does not expect the collision of a semi-static configured DL transmission overlaps with an ROs will happen, if the DL transmission is UE specifically configured.

Proposal 4: Reuse existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between dynamic UL transmission with SSB, i.e., SSB reception is prioritized over dynamic UL transmission.

Proposal 5: Reuse existing collision handling principles of Rel-15/16 for NR TDD to handle the potential collision between dynamic DL transmission with ROs, i.e., PRACH transmission is prioritized over dynamic DL reception.

Proposal 6: The principle in LTE type-A HD-FDD on the determination of transmission pattern is reused for RedCap type-A HD-FDD.

# References

RP- 210918, “Revised WID on support of reduced capability NR devices”, 3GPP RAN#91e

3GPP RAN1#104b-e Chairman notes.