3GPP TSG RAN WG1 #105-e												  R1-2105247

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.1.2.1

Source:	NEC

Title:               Discussion on multi-TRP for PUCCH and PUSCH

Document for:	Discussion and Decision

- Introduction

In RAN1#104b-e meeting [1], PUCCH and PUSCH enhancements based on multi-TRP were achieved as:

Working Assumption

For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition,

- Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation

  - FFS: Whether the new field is 1 bit or 2 bits

Working Assumption

For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first KxKx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx)(2N2- Kx) codepoint(s) are reserved.

Agreement

For the indication of open-loop power control parameter (OLPC) in DCI format 0_1/0_2, support enhanced open-loop power control parameter (OLPC) set indication by indicating per-TRP OLPC set.

- FFS: Details of indication.

Agreement

For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. For each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky KyTPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved.

- How to describe/capture this in 38.212 is up to the editor.

Agreement

Confirm the following Working Assumption:

For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions.

- FFS: Applicability of mapping patterns for different beam switching gaps

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows,

  - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions.

  - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions.

Agreement

Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED):

- For beam mapping /power control parameter set mapping for PUCCH repetitions,

  - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions).

  - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots.

  - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

Agreement

Confirm the following working assumption (with removing the last bullet):

For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams.

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- FFS: Support of half-half mapping.

- FFS: Additional considerations on mapping patterns (including required beam switching gaps)

Agreement

For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting,

- The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

- Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs.

- Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP.

- Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP

  - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports.

  - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

In this contribution, we provided our views on the enhancements on multi-TRP for PUCCH and PUSCH.

- Discussion

For single DCI based M-TRP PUSCH repetition schemes, it was agreed that two SRI fields corresponding to two SRS resource sets are included in DCI for both codebook and non-codebook based PUSCH transmission.

And it was hotly discussed how to support dynamic switching between single-TRP and multi-TRP transmission, and a working assumption with new field in DCI was achieved last meeting. While the number of bits for the new field needs further decision.

To support the most flexibility, there are totally three states needed for single-TRP and multi-TRP transmission, i.e. multi-TRP transmission, single-TRP transmission with the first TRP, single-TRP transmission with the second TRP. If the three states are all indicated in the new DCI field, 2 bits are needed, where one codepoint is reserved.

While as proposed in previous meetings, reserved codepoints in SRI or TPMI field can be reused to indicate the dynamic switching without additional DCI overhead, while some companies argued that in some cases, there is no reserved codepoint, which makes the solution a little complex. While in our opinion, we have already introduced second SRI field, second TPMI field, which is applied in case of multi-TRP transmission, while in case of single-TRP transmission, the second SRI field or the second TPMI field is reserved without any usage.

So there is no need to indicate all the three states in the new DCI field, we can only indicate single-TRP or multi-TRP transmission in the new DCI field, which needs only one bit. And if the DCI indicates single-TRP transmission, whether the transmission is based on the first TRP or the second TRP can be further indicated by any one of the reserved field (e.g. the second SRI field, the second TPMI field), as shown in Table 1 and Table 2.

Table 1. New DCI field to indicate single-TRP or multi-TRP

| Bit field | Description |
| --- | --- |
| 0 | Multi-TRP transmission |
| 1 | Single-TRP transmission |

Table 2. Reserved DCI field to indicate TRP index in case of single-TRP transmission

| Bit field | Description |
| --- | --- |
| i | First TRP if single-TRP transmission is indicated in Table 1. |
| i+1 | Second TRP if single-TRP transmission is indicated in Table 1. |
| … | Reserved if any. |

Based on this, only one bit is introduced for the new DCI field, and this can be applied at least for cases except one SRS resource set including only one SRS resource (i.e. 0 bit for the SRI field). And regarding the case of one SRS resource set includes only one SRS resource, there are two solutions, one solution is to introduce 2 bits for the new DCI field, as the number of SRS resources in one SRS resource set is configured by RRC, so there is no complexity or ambiguity for the DCI payload design. The other solution is keep the unified 1 bit for the new DCI field, and 1 bit SRI field is applied to indicate the TRP index as shown in Table 2. And if multi-TRP transmission is indicated, the 1 bit SRI field is reserved.

Based on the discussion, comparing directly introducing 2 bits for the new field, 1 bit overhead can be reduced in most cases.

Proposal 1: For indicating single-TRP and multi-TRP dynamic switching, the working assumption is confirmed if the bit size is configurable based on the number of SRS resources in the SRS resource set. And only one bit is introduced for the new field, if second SRI or TPMI field contains at least 1 bit, and 2 bits introduced otherwise.

For non-codebook based PUSCH transmission, a working assumption for the second SRI field design was achieved in last meeting. While when there is only one SRS resource in the SRS resource set, it’s not quite clear for the number of bits N2, and the codepoint for SRI or reserved. For example, in case of only one SRS resource in the SRS resource set, the number of bits is 1 or 0. We think this working assumption should be decided to be confirmed or revised together with working assumption for dynamic switching between single-TRP and multi-TRP.

Proposal 2: Regarding working assumption for second SRI field design in case of non-codebook based transmission, number of bits N2 should be clarified in case of only one SRS resource included in the SRS resource set. And discussion of the working assumption should be jointly considered with working assumption for indicating single-TRP and multi-TRP dynamic switching.

Furthermore, there are several issues which haven’t been discussed, the first one is the number of SRS resources in the two SRS resource sets.

To achieve network and UE deployment flexibility, there is no need to restrict same number of SRS resources in the two SRS resource sets. For example, from network perspective, different number of SRS resources can be configured for different TRPs considering TRP deployment and scheduling requirement. And from UE perspective, there may be multiple panels configured, and the number of antennas in different panels can be different.

Proposal 3: For both codebook and non-codebook based single-DCI PUSCH transmission, separate number of SRS resources in two SRS resource sets should be supported.

In this case, regarding non-codebook based uplink transmission, for example, the first SRS resource set includes X resources, and the second SRS resource set includes Y resources. If the value of X is less than Y, in other words, the maximum number of layers supported by the first SRS resource set (or first TRP) is less than the maximum number of layers supported by the second SRS resource set (or second TRP), in this case, if the first SRI field is always associated with the first SRS resource set, and based on the working assumption, the second SRI field only contains SRIs corresponding to the indicated rank of the first SRI field, then the supported number of layers for second SRS resource set (or second TRP) is restricted, in case of single-TRP transmission with second TRP. Taking one example with 1 SRS resource in the first SRS resource set, and 4 SRS resources in the second SRS resource set, and Lmax = 2, then there is no bit size for the first SRI field, and for the second SRI field, all available SRI(s) are shown in following Table 3.

Table 3. Restricted on the supported number of layers in case of single-TRP

| Bit field | SRI(s) in the second SRI field |
| --- | --- |
| 0 | 0 in case of rank = 1; 0,1, in case of rank = 2 |
| 1 | 1 in case of rank = 1; 0,2 in case of rank = 2 |
| 2 | 2 in case of rank = 1; 0,3 in case of rank = 2 |
| 3 | 3 in case of rank = 1; 1,2 in case of rank = 2 |
| 4 | 1,3 in case of rank = 2 |
| 5 | 2,3 in case of rank = 2 |

Taking the case in Table 3 for example, the issue is whether the SRIs for rank = 2 (marked with red) are available.

In case of multi-TRP transmission, it’s obvious the number of layers for two TRPs should be same, while in case of single-TRP transmission, whether the supported values of maximum number of layers can be different for different TRPs should be discussed. In our understanding, to achieve flexibility, the values can be different, i.e. the SRIs for rank = 2 in Table 3 should be supported for the second SRS resource set in case of single-TRP transmission. While based on current agreement/WA, the second SRI field only contains SRI(s) corresponding to the indicated rank of the first SRI field (in this case, only rank = 1 can be indicated by the first SRI field), which excludes the red part in Table 3. If so, even in case of single-TRP transmission, rank = 2 transmission can not be supported.

Proposal 4: For both codebook and non-codebook based single-DCI PUSCH transmission, in case of single-TRP transmission, the maximum number of layers (value of rank) supported for two TRPs can be different.

There may be several solutions, first one is to associate the first SRI field with the SRS resource set with maximum number of SRS resources for NCB and to associate the first TPMI field with the TRP with larger value of maxRank, based on this, the WA for second SRI field design for non-codebook based transmission can be confirmed and agreement for second TPMI field design for codebook based transmission can be kept.

The second solution is that the second SRI field includes SRIs among all ranks supported by the second SRS resource set, and the WA for second SRI field design can be revised as:

Working Assumption For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field in case of multi-TRP transmission. The number of bits, N2N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field supported based on the number of SRS resources in the second SRS resource set and the value of Lmax. In case of multi-TRP transmission, Ffor each rank x, the first KxKx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx)(2N2- Kx) codepoint(s) are reserved. And in case of single-TRP transmission, for each rank w, the first KwKx codepoint(s) are mapped to Kw SRIs of rank w supported based on the number of SRS resources in the second SRS resource set and the value of Lmax, the remaining (2N2-Kw)(2N2- Kx) codepoint(s) are reserved.

And for codebook based transmission, the agreement for second TPMI field design should be revised as:

Agreement For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field in case of multi-TRP transmission. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field supported based on maxRank for the second TRP. In case of multi-TRP transmission, Ffor each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky KyTPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved. And in case of single-TRP transmission, for each rank z, the first Kz codepoint(s) of the second TPMI field are mapped to Kz KyTPMI(s) of rank z supported based on maxRank for the second TRP in increasing order codepoint index, the remaining (2M2-Kz) codepoint(s) are reserved. - How to describe/capture this in 38.212 is up to the editor.

Proposal 5: To support the indication of different maximum number of layers (value of rank) for different TRPs, for NCB transmission, the first SRI field is associated with the SRS resource set with larger number of SRS resources, and for CB transmission, the first TPMI field is associated with the TRP configured with larger value of maxRank, or the agreement/WA for second TPMI/SRI field design should be revised.

In addition, the second issue is the SRS resource configuration in case of full power transmission mode. In current spec, up to 4 SRS resources can be configured in a SRS resource set for full power transmission, and the number of SRS ports in the SRS resources can be different. In case of multi-TRP transmission, full power should be supported for reliability enhancement. It should be defined that whether the full power mode should be same for two TRPs. The number of SRS resources in two SRS resource sets, and the number of SRS resources with same number of SRS ports in each set should be discussed.

Proposal 6: Whether same full power transmission mode for both TRPs and the SRS resource configuration in case of full power transmission should be discussed.

Another remaining issue is the TPC command design for PUSCH and PUCCH, this has been discussed several meetings, and proposals were listed in last meeting as:

Proposal 2.2: To support per TRP closed-loop power control for PUCCH , a second TPC field can be configured via RRC . - When the second field is configured by RRC , a second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2 (option 3). - When the second field is not configured by RRC , a single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for both PUCCH beams. - This feature is subjected to whether UE support 2 closed-loop power control processes Proposal 3.1: To support per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, adopt the same solution as with M-TRP PUCCH schemes.

These proposals can flexibly indicate one or two TPC values applied for two PUSCH/PUCCH beams, we think it’s better to end the discussion and agree the proposals.

Proposal 7: For closed-loop power control for PUSCH and PUCCH, a configurable second TPC field should be added in DCI.

- Conclusion

In this contribution, we provided our views on enhancement for PUSCH and PUCCH based on multi-TRP transmission, and we proposed that:

Proposal 1: For indicating single-TRP and multi-TRP dynamic switching, the working assumption is confirmed if the bit size is configurable based on the number of SRS resources in the SRS resource set. And only one bit is introduced for the new field, if second SRI or TPMI field contains at least 1 bit, and 2 bits introduced otherwise.

Proposal 2: Regarding working assumption for second SRI field design in case of non-codebook based transmission, number of bits N2 should be clarified in case of only one SRS resource included in the SRS resource set. And discussion of the working assumption should be jointly considered with working assumption for indicating single-TRP and multi-TRP dynamic switching.

Proposal 3: For both codebook and non-codebook based single-DCI PUSCH transmission, separate number of SRS resources in two SRS resource sets should be supported.

Proposal 4: For both codebook and non-codebook based single-DCI PUSCH transmission, in case of single-TRP transmission, the maximum number of layers (value of rank) supported for two TRPs can be different.

Proposal 5: To support the indication of different maximum number of layers (value of rank) for different TRPs, for NCB transmission, the first SRI field is associated with the SRS resource set with larger number of SRS resources, and for CB transmission, the first TPMI field is associated with the TRP configured with larger value of maxRank, or the agreement/WA for second TPMI/SRI field design should be revised.

Proposal 6: Whether same full power transmission mode for both TRPs and the SRS resource configuration in case of full power transmission should be discussed.

Proposal 7: For closed-loop power control for PUSCH and PUCCH, a configurable second TPC field should be added in DCI.

- References

- Chairman’s notes, RAN1#104b-e, April 12th – April 20th, 2021.