3GPP TSG RAN WG1 #105-e	                                                                 R1-2105262

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.3.3

Source:	NEC

Title:	Discussion on Intra-UE prioritization and multiplexing

Document for:	Discussion

# Introduction

In the TSG-RAN#88e plenary meeting, the scope of revised WID on enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR physical layer enhancements for NR URLLC was defined for Release 17 [1]. One of the objectives of this WI is intra-UE prioritization/multiplexing of traffic with different priority as outlined below.

Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

a.	Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

b.	Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline.

In this contribution, we discuss our views on issues related to intra-UE HARQ-ACK multiplexing.

# Intra-UE HARQ-ACK multiplexing

In last meeting, in case collision between two PUCCHs for HARQ-ACK of different priorities happens, supporting multiplexing HARQ-ACK of different priorities in a PUCCH was discussed and following agreements was achieved [2]:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

For the case that the total number of LP and HP HARQ-ACK bits is 2, considering that the LP HARQ-ACK is only 1-bit, multiplexing the two multiplexed HARQ-ACK bits as two HP HARQ-ACK bits on PUCCH resource for HP HARQ-ACK by joint coding is sufficient. Then the reliability of HP HARQ-ACK can be ensured and the waste of PUCCH resource can be ignored.

Proposal 1:  For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support joint coding when the total number of LP and HP HARQ-ACK bits is 2.

Another open issue related to multiplexing HARQ-ACK of different priorities on a PUCCH is the details of supporting multiplexing for two HARQ-ACK codebooks with different priorities. It was discussed in RAN1 #103-e meeting but no consensus was reached. In Rel-16, when a UE is configured with two HARQ-ACK codebooks for different services, separate configuration of HARQ-ACK codebook types and HARQ-ACK timing K1 set for the two HARQ-ACK codebooks are supported. In Rel-17, support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH to improve the performance of eMBB traffic, while when the LP and HP HARQ-ACK with following HARQ-ACK codebook type configurations, whether and how to support multiplexing for two HARQ-ACK codebooks with different priorities on a PUCCH are still under discussion.

- Case a) LP Type-1 HARQ-ACK codebook and HP Type-1 HARQ-ACK codebook

- Case b) LP Type-2 HARQ-ACK codebook and HP Type-2 HARQ-ACK codebook

- Case c) HP Type-1 HARQ-ACK codebook and LP Type-2 HARQ-ACK codebook

- Case d) HP Type-2 HARQ-ACK codebook and LP Type-1 HARQ-ACK codebook

Based on our understanding, at least case a) and case b) should be supported in Rel-17. For case a), when Type-1 HARQ-ACK codebook type is configured for both low-priority HARQ-ACK and high-priority HARQ-ACK, the details of multiplexing HARQ-ACK of different priorities on a PUCCH are still FFS. One straightforward solution is to construct two HARQ-ACK codebooks separately as Rel-16 and paste them together as a multiplexed HARQ-ACK codebook. However, when the two K1 sets separately configured for different HARQ-ACK codebooks have some same k1 values, and the same TDRA list corresponds to the two HARQ-ACK codebooks, then using the straightforward multiplexing scheme will lead to unnecessary redundancy.  One example is shown in Fig.1 below, the K1 sets configured for low-priority HARQ-ACK and high-priority HARQ-ACK are {4,3,2} and {2,1}, respectively. By using the straightforward solution, the candidate PDSCH occasions corresponding to DL slot determined by k1=2 will be generated repeatedly for two HARQ-ACK codebooks, e.g., the determined maximum number of non-overlapped candidate PDSCH occasions for the DL slot associated with k1=2 is 5, then 10 HARQ-ACK positions corresponding to the DL slot will be determined in the multiplexed HARQ-ACK codebook. So 5 bit redundancy will be generated since at most 5 PDSCH receptions can be scheduled in this DL slot.

Fig.1 Example for two overlapped K1 set configured for two HARQ-ACK codebooks

Fig.2 Example for multiplexed HARQ-ACK codebook obtained by straightforward multiplexing scheme

Observation 1: The straightforward method for multiplexing two Type-1 HARQ-ACK codebooks on a PUCCH by constructing two Type-1 HARQ-ACK codebooks separately as Rel-16 and pasting them together as a multiplexed HARQ-ACK codebook may lead to unnecessary redundancy.

To address the above issue, following enhanced solutions can be considered:

- Option 1: UE directly constructs a multiplexed HARQ-ACK codebook based on steps below.

  - Step 1: UE determines the union of the two separate HARQ-ACK timing value sets of two Type-1 CBs.

  - Step 2: UE constructs the multiplexed HARQ-ACK codebook based on the union of the two K1 sets. The other procedure for constructing the HARQ-ACK CB can follow Rel-16 rule.

- Option 2: UE constructs two HARQ-ACK codebooks of different priorities and then paste them together as follows.

  - UE constructs the high-priority Type-1 HARQ-ACK CB based on K1 set of high-priority HARQ-ACK as Rel-16, while constructs low-priority Type-1 HARQ-ACK CB based on a updated K1’ set of low-priority HARQ-ACK. The updated K1’ set is obtained by removing values in the intersection of the two separate HARQ-ACK timing K1 sets of two Type-1 CBs from K1 set of low-priority HARQ-ACK.

Considering that separate coding will be used for multiplexing more than two LP and HP HARQ-ACK bits in a PUCCH, option 2 is easier to implement than option 1, so option 2 is slightly preferred.

Proposal 2:  Support multiplexing of two Type-1 HARQ-ACK codebooks of different priorities on a PUCCH in Rel-17 as follows:

- Firstly, UE constructs the high-priority Type-1 HARQ-ACK codebook based on K1 set of high-priority HARQ-ACK as Rel-16, and constructs low-priority Type-1 HARQ-ACK codebook based on K1’ set obtained by removing values in the intersection of the two separate HARQ-ACK timing K1 sets of two Type-1 CBs from the K1 set of low-priority HARQ-ACK.

- Then, UE pastes the two HARQ-ACK codebooks together as a multiplexed HARQ-ACK codebook.

# Intra-UE HARQ-ACK/PUSCH multiplexing

In RAN1 #104-e meeting, following agreements were made [3]:

Agreements:

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group

- FFS: dynamic indication

Firstly, we discuss the collision handling for PUSCH and PUCCHs for HARQ-ACK with different priorities when simultaneous PUCCH/PUSCH transmission of different priorities over different cells is not configured.  In Rel-16, HARQ-ACK bits are allowed to be multiplexed on PUSCH only when the priority of HARQ-ACK and PUSCH are the same, i.e., multiplexing eMBB HARQ-ACK on eMBB PUSCH, multiplexing URLLC HARQ-ACK on URLLC PUSCH. In Rel-17, in order to improve the spectrum efficiency, when a PUSCH with low or high priority collides with two PUCCHs for HARQ-ACK of high priority and HARQ-ACK of low priority simultaneously, support multiplexing both low-priority HARQ-ACK and high-priority HARQ-ACK on the PUSCH when the conditions are satisfied. While the detail multiplexing scheme is still FFS. One related open issue is that if the PUSCH is scheduled by UL DCI format 0_1/0_2 with a DAI field, whether and how to apply the DAI field to the multiplexed HARQ-ACK codebook when multiplexing two HARQ-ACK codebooks with different priorities on a PUSCH is enabled. In our view, which HARQ-ACK codebook the DAI field applies to can be configured by gNB for different cases. For example, if the eMBB HARQ-ACK bits are compressed and then multiplexed with URLLC HARQ-ACK bits, e.g., bundle eMBB HARQ-ACK bits as 1-bit and append it to the URLLC HARQ-ACK codebook, the DAI field is configured as to help determine the URLLC HARQ-ACK codebook. If multiplexing all the eMBB HARQ-ACK bits and URLLC HARQ-ACK bits on PUSCH, the DAI field is configured as to help determine the eMBB HARQ-ACK codebook since the reliability of URLLC PDCCH is high.

Proposal 3: When multiplexing both low-priority HARQ-ACK and high-priority HARQ-ACK on a PUSCH scheduled by an UL non-fallback DCI with a DAI field, which HARQ-ACK codebook the DAI field is applied to should be configured by gNB.

Similarly, the collision handling for PUSCH and PUCCHs for HARQ-ACK with different priorities should be also considered when simultaneous PUCCH/PUSCH transmission of different priorities over different cells is configured. For example, when a PUCCH for eMBB HARQ-ACK on CC #1 is overlapped with a PUCCH for URLLC HARQ-ACK on CC #1 and a PUSCH for eMBB UL-SCH on CC #2, PUCCH for URLLC HARQ-ACK and PUSCH for eMBB UL-SCH can be simultaneously transmitted on CC #1 and CC #2, while whether the eMBB HARQ-ACK is multiplexed on URLLC PUCCH or on eMBB PUSCH is not clear. Other cases, e.g., collision between two PUSCHs with different priorities and two PUCCHs for HARQ-ACK with different priorities should be also studied. Therefore, configuration of simultaneous PUCCH/PUSCH transmission of different priorities over different cells should be taken into account when studying the order of prioritization or multiplexing for collision cases involving PUSCH and PUCCH with different priorities.

Proposal 4: Further study the order of prioritization or multiplexing for collision cases involving PUSCH and PUCCH with different priorities when simultaneous PUCCH/PUSCH transmission of different priorities over different cells is configured.

# Conclusion

From the discussion, we have the following observations and proposals.

Observation 1: The straightforward method for multiplexing two Type-1 HARQ-ACK codebooks on a PUCCH by constructing two Type-1 HARQ-ACK codebooks separately as Rel-16 and pasting them together as a multiplexed HARQ-ACK codebook may lead to unnecessary redundancy.

Proposal 1:  For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support joint coding when the total number of LP and HP HARQ-ACK bits is 2.

Proposal 2:  Support multiplexing of two Type-1 HARQ-ACK codebooks of different priorities on a PUCCH in Rel-17 as follows:

- Firstly, UE constructs the high-priority Type-1 HARQ-ACK codebook based on K1 set of high-priority HARQ-ACK as Rel-16, and constructs low-priority Type-1 HARQ-ACK codebook based on K1’ set obtained by removing values in the intersection of the two separate HARQ-ACK timing K1 sets of two Type-1 CBs from the K1 set of low-priority HARQ-ACK.

- Then, UE pastes the two HARQ-ACK codebooks together as a multiplexed HARQ-ACK codebook.

Proposal 3: When multiplexing both low-priority HARQ-ACK and high-priority HARQ-ACK on a PUSCH scheduled by an UL non-fallback DCI with a DAI field, which HARQ-ACK codebook the DAI field is applied to should be configured by gNB.

Proposal 4: Further study the order of prioritization or multiplexing for collision cases involving PUSCH and PUCCH with different priorities when simultaneous PUCCH/PUSCH transmission of different priorities over different cells is configured.

# References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR,” TSG-RAN#88e, June 22 – July 03 2020.

- 3GPP RAN1 NR #104b-e meeting, chairman’s notes.

- 3GPP RAN1 NR #104-e meeting, chairman’s notes.