3GPP TSG RAN WG1 #105-e															      R1-2105275

e-Meeting, May 19th – 27th, 2021

Agenda item:		8.1.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements on Beam Management for Multi-TRP

Document for:		Discussion and Decision

# Introduction

In RAN1 #104bis-e meeting [1], RAN1 made following agreements on the enhancement on BM for M-TRP.

Agreement

For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

Agreement

On CMR resource configuration for beam reporting option 2, adopt the following alternative:

- Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

  - FFS: extension to aperiodic CMR resource setting

- Each reported beam pair in a single CSI-report consists of M = 2 SSB-RI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset.

- Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above.

Agreement

On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”:

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported

  - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset.

- FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement

Adopt the following beam failure detection criteria for each BFD-RS set

- The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold

  - X is max{minimal periodicity of BFD RS in the set, 2ms}

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Agreement

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs,

- Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

  - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

  - Alt-3: Leave it up to UE implementation

- Note: PUCCH-SR resource is PUCCH resource carrying SR

- FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

In this contribution, we provide further details considering mainly the agreed high priority topics. In particular, section 2 includes beam reporting enhancements and section 3 discuss beam failure recovery related aspects.

# Enhancement on Beam Measurement and Reporting

In RAN1 104e, we have agreed at least to support Option 2 for beam measurement/reporting enhancement, and study further on supporting option 1 and option 3. Option 1 and Option 2 are based on group-based beam reporting, while option 3 is based on non-group-based beam reporting. Since two reporting have different use cases, so we will discuss them separately.

Since option 2 is already agreed to support, we don’t see further specification for option 1. And, option 3 can be further studied for non-ideal backhaul condition.

Proposal 2-1: In addition to option 2, support Option 3 for ideal/non-ideal backhaul scenario.

In the following sections, the details on option 2 and option 3 are discussed.

## 2.1	Enhanced Group-based Beam Reporting (option 2)

Issue 1: CMR association to TRP

When a group-based beam reporting is enabled, and reporting of the beam pairs should be enabled across TRP or per TRP. For that, there should be UE known association between NZP-CSI-RS resources or SSB to a TRP. We think that the SSB/NZP-CSI-RS resources can be grouped, and each group is assumed to be associated with a TRP.  To group CMR resources, we can consider the grouping of CMR resources by two CMR resource sets or two CMR resource subsets.

In RAN1 104bis-e, for M-TRP CSI, the following agreement has been made.

Agreement

On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”:

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported

  - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset.

- FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

Though the configuration either “sets” or “subsets”can be RAN2 decision, due to its impact to RAN1 signaling, it is beneficial to make a decision in RAN1.

If we use two CMR resource sets for CSI report setting, several updates in RAN1 signaling are required.

- First, UE capability for the number of CSI resources in a set should be clarified because the capability shall be aligned with the sum of the CSI resources in two CMR resource sets.

- Second, for AP-CSI reporting, aperiodic triggering states should be configured with two resource sets.

- Last, the field or interpretation of CRI field should be updated to support two CMR resource sets. For example, if we only allow reporting pair of beams, we can simply use the CRI indexing per resource set and the order of CMR resource sets can be corresponding to the order of two CRI fields. But, if we allow the beam reporting can be either beams or beam-pairs, it is required to distinguish beams from what CMR resource set.

Observation 2-1: When two CMR resource sets are used, additional specification effort is required for AP CSI triggering state, CRI indication and UE capability for maximum CSI-resources in a set.

If we use two CMR resource subsets without defining an additional CMR resource set, the existing framework of AP CSI-triggering, CRI indexing, and UE capability signaling can be reused. Because, CRI is counted per the same CMR resource set with two subsets, when assuming the same number of total CMR resources, the bitwidth of two CRI fields can increase. However, this can be reduced if we support CRI indexing per subset. In addition, for CSI report setting for M-TRP CSI acquisition in AI 8.1.4, it is agreed to configure CMRs within the same resource set. Because both beam management and CSI acquisition share a common framework for CSI reporting configuration, it is beneficial to use the same terminology between two CSI reporting configuration.

Observation 2-2: When two CMR resource subsets are used, the existing CSI framework can be reused with the consistency with CSI report setting for CSI acquisition.

Proposal 2-2: For group-based beam reporting for multi-TRP operation, support two CMR resource subsets to group CMR resources corresponding to a TRP.

For the mapping of CMR resources to two subsets, we can consider explicit or implicit methods.

For the explicit method, introduce a bitmap where its length is identical to the number of CMR resources in a set. For example, each bit-field corresponding to a CMR resource can be either “0” or “1”. This is useful for CMR of SSB resources. A common SSB bitmap can be configured in CSI-resource setting instead of having multiple bitmaps in each CSI-report setting. This is useful to avoid the repetition of adding bitmap for all report setting, especially for AP-CSI-report. For inter-cell case, SSB association to PCI may replace this SSB bitmap, and the association to a TRP can be derived by the PCI index of SSB QCLed with the CMR resources.

For CMR of NZP-CSI-RS resources, support an implicit method by utilizing the QCL relation with SSB and NZP- CSI-RS resources. Once a common bitmap for SSB resources in CSI resource setting is configured, the subset of NZP-CSI-RS resources is determined by the subset of the QCLed SSB

Observation 2-3: It is possible to utilize existing spatial QCL-typeD relationships between different RSs with the grouping of SSBs, where the UE understand the beams can be associated with a TRP.

Observation 2-4: For inter-cell M-TRP, SSB grouping by PCI is already supported.

Proposal 2-3: Support both explicit and implicit methods for mapping a subset of CMR resources to a  TRP.

- For CMR of SSB resources, support explicit method, a subset indication bitmap is included in CSI-resource Setting.

- For CMR of NZP-CSI-RS resources, support implicit method where SSB is grouped by a common SSB bitmap (or PCI index), and CSI-RS is associated with a subset/group by the QCLed SSB.

Issue 2: Number of beam pairs to report

Regarding to the number of beam pairs configured for the report, it was also showed in some company results that increased number of groups and/or beams per group could improve the overall performance. We see that having more pairs reported may improve the efficiency of reporting. Thus, we support up to 4 beam pairs to be reported.

Proposal 2-4: For enhanced group-based beam reporting for multi-TRP, support N={1,2,3,4} beam-pairs for a group-beam report.

When an UE is configured with N=4, but only 2 beam pairs are available to be reported, it is beneficial to reduce the UCI overhead by omit the remaining two. Thus we think the configured number of beam pairs should be the upper bound for the report and the real number of beams to report can be dynamically selected by UE.

Proposal 2-5: For enhanced group-based beam reporting for multi-TRP, support Alt2 (The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE)

Issue 3: UE panel/antenna related feedback

In RAN1 #104bis-e meeting, we have discussed on the following proposal.

Proposal: For potential UE panel related information feedback for beam reporting option 2, further study the following alternatives:

- Alt-1: UE reports panel ID / antenna-group ID or the reporting setting is associated with panel ID/antenna-group ID

  - the reporting setting is associated with panel ID/ antenna-group ID

- Alt-2: UE indicates if reported beams are associated to different RX spatial filters, or maximum number of supported layers corresponding to DL RS in a group, or whether two beams in a beam pair can be used for spatial multiplexing or diversity

- Other alternatives are not precluded

- NOTE: study of these alternatives with beam reporting option 1/3 is not precluded

Because the group-based beam reporting is for indicating UE’s capability of receiving two beams at the same time, we don’t see any reason to report UE’s RX panel/spatial filter information. UE will report beam-pairs and RSRP according to its receiver configuration, additional information doesn’t give more information to gNB. Though it can be helpful to understand the required timing for beam switching, this aspect should be discussed in terms of general beam management aspect, and if agreed, it can be condiered for M-TRP report.

Observation 2-5: UE Rx panel-related information should be defined for the general BM aspect first.

In addition, the information of the number of supported layers to DL RS etc is not part of beam management but CSI acquisition. And, the group-based beam reporting for M-TRP is intended for M-TRP operation, and obviously intended for spatial multiplexing but diversity reception such as delay diversity or SFN scheme. Also, we don’t have any requirement to guarantee the coherent reception of two beams from different TRP, and NCJT is not intended for the diversity reception.

Issue 4: L1-SINR and interference measurement

In RAN1 #104bis-e meeting, we have discussed on supporting of L1-SINR and interference measurement, and following proposal has been discussed.

Proposal 2.4: For beam reporting option 2, evaluate the performance, specification, and implementation aspects of L1-SINR based beam measurement/feedback, including at least the following aspects

- Physical resource for interference measurement, e.g.

  - CMR of the other reported beam within the beam pair,  and/or

  - CMR of the reported beam, and/or

  - Dedicated IMR resource

- UE behavior of interference measurement

- Note: L1-RSRP report has been agreed for option 2

When UE measures interference from a resource, the QCL assumption should be the same as CMR reception. If we assume UE has two panels and one panel is receiving a resource as CMR but the other panel is receiving the resource as IMR, there is ambiguity where what QCL assumption is made for receiving the resource as IMR. Because UE has no prior information of beam-pair to report, UE’s implicit measuring of interference from CMR of the other TRP is unclear. This is only possible only when UE is indicated with a possible beam pair to measure by dedicated IMR configuration.

From this observation, we support L1-SINR reporting only when explicit IMR is configured.

Proposal 2-6: Support the enhanced group-beam reporting for L1-SINR report only when dedicated IMR is configured.

Issue 5: Reporting criteria and fallback mechanism.

For multi-beam operation in multi-TRP deployment scenarios, it is likely that measured received signal quality of CSI resources associated with different DL TX-RX beam pair links per TRP can significantly vary among different TRPs. Therefore, it is beneficial to configure the beam reporting criterion such that it imposes UE to report only measured CSI resources within a certain power window (e.g. between maximum and minimum measured values) or above a power threshold. More specifically, with power window/difference criterion the UE determines a grouping/pairing of simultaneously received CMR such that different groups/pairs can be ranked in descending order with a power difference between absolute of maximum and minimum value of L1-RSRP/L1-SINR within a group/pair. As a result, Furthermore, from latency reduction perspective with beam domain operation, it may also be beneficial to consider additional criterion that can take into account latencies associated with multi-panel reception.  As a result of this, signalling overhead associated with reporting and latency associated with beam identification and multi-beam operation can be reduced.

Observation 2-6: It is beneficial to configure beam reporting criteria that imposes UE to rank and report only measured CSI resources being within a certain power window or above a power threshold.

Observation 2-7: From latency reduction perspective, it is beneficial to consider additional criterion that can take into account latencies associated with multi-panel reception.

Proposal 2-7: Support beam reporting criteria that imposes UE to rank and report only measured CSI resources being within a certain power window or above a power threshold.

To guarantee reliable beam-based communication in Rel-17 multi-TRP operation with enhanced group-based beam reporting, there is a need to develop a fallback mechanism/procedure that enables UE to recover from a situation where beam reporting condition is not fulfilled. For example, when UE is configured with the enhanced group-based reporting, UE is not able to identify configured N different resources transmitted from one or more TRPs according to the enhanced group-based beam reporting configuration.

Observation 2-8: To guarantee reliable multi-TRP operation with enhanced group based beam reporting, it is beneficial to develop a fallback mechanism/procedure that enables UE to recover from situation where enhanced beam reporting condition is not fulfilled.

Proposal 2-8: Consider different alternatives as fallback mechanisms, such as:

- Alt-1: use Rel-15 group reporting (with a restriction on ‘per TRP’ with predefined TRP)

- Alt-2: use Rel-15 non-group reporting (no restriction on simultaneous reception)

- Alt-3: network configures the fallback reporting (based on Alt-1 or Alt-2)

## 2.2 Enhanced non-group-based beam reporting (Option 3).

From the discussion in the previous meetings, it was noted that non-group-based beam reporting is important for supporting multi-TRP operation with non-ideal backhaul. And, it has been identified as one option (option 3) in the agreement in RAN1 #103-e meeting.

In this type of operation, each TRP may configure different CSI-report configurations at each TRP. If two CSI-reports are independent, the UE’s capability to receive multiple beams via multiple TRPs at the same time is hardly handled by the network. In order to solve the problem, it is beneficial to support UE-assisted method.

Even though it is discussed that the possibility of using group-based beam reporting for each TRP to solve the problem, this doesn’t guarantee the UE’s simultaneous reception without exchange of beam scheduling information between TRPs.

With two non-group-based beam reporting settings, simultaneous reception can be implemented by associating two reporting settings corresponding to two TRPs. If a reporting setting for L1-RSRP report is associated with another reporting setting, the reported beams (SSB or NZP-CSI-RS resources) to be reported in the reporting setting should be selected with the consideration of simultaneous reception by the UE with the SSB or CSI-RS resources reported in the associated reporting setting. From such a CSI reporting enhancement, gNB can select transmission beams to enable simultaneous reception at the UE.

Proposal 2-9: For non-group based beam reporting, support association of a reporting setting to another reporting setting to ensure the UE’s simultaneous reception from multi-TRP for multi-DCI based multi-TRP scheme,

- UE shall select beams to be reported with the consideration of the simultaneous reception from two TRP.

In order to associate two reporting settings, a higher-layer parameter indicating the associated CSI-ReportSettingID can be included in the CSI-ReportSetting.

Proposal 2-10: For the association of CSI report settings, the associated CSI-ResportSettingID is included in the CSI-ReportSetting.

There is another discussion whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously. It is also possible that UE can report the beams with/without the indication of its simultaneous reception capability.

Proposal 2-11: Study method to indicate beams simultaneous reception capability.

# Beam Failure Recovery Enhancements for Multi-TRP

Here we discuss the remaining issues related to the beam failure recovery in multi-TRP operation.

## 3.1 Simultaneous configuration of cell-specific and TRP-specifc BFR on the same cell

In RAN1#104bis-e, RAN1 has made following agreement on configuration of cell-specific and TRP-specific BFR.

Agreement

Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

One of the open issues regarding the failure detection and recovery in the mTRP is how to handle the failure of a single TRPs and the failure of both TPRs. Following scenarios can be observed based on the current agreements where UE has two sets of BFD-RS:

- TRP 0 fails (BFD-RS set q0-0)

- TRP 1 fails (BFD-RS set q0-1)

- Both TRP0 and TRP1 fail (both sets fail)

It is possible that either of the TRPs is in failure or both TRPs are in failure. Based on the current agreements, when either of the TRPs is in failure UE indicates the failed TRP with candidate beam information but it should also be considered that when both TRPs are in failure at the same time and need to be recovered.

Observation 3-1: Both TRPs have the corresponding BFD-RS sets and the sets can be in failure at the same time.

Based on the agreement on BFRQ MAC-CE in RAN1 104e, UE provides candidate beam information for the failed TRP to the network if available. For M-TRP, it should also be defined what is the UE behavior when it has detected failure of a TRP but is not able to indicate any candidate from the list of candidate beams. Candidate beam lists may contain a limited set of RS indexes and UE may not find an available candidate from that list for the failed TRP. In this case, it may not be feasible for UE to continue monitoring the CORESETs on the failed TRP e.g. before a new beam is or can be indicated.

Observation 3-2: Candidate beams are indicated from the candidate beam lists and there may not be candidates available for the failed TRPs

Proposal 3-1: Define UE behavior when no new candidate can be indicated for the failed TRP.

Furthermore, it should also be considered that when both TRPs are in failure at the same time and need to be recovered. The signaling mechanism should support providing information on both failed TRPs if new candidates are found. In case UE cannot find any candidates for both of the failed TRPs, there should a fallback option (at least SpCell) for UE to fallback to legacy BFR, e.g. Rel15/16 random access based,

Observation 3-3: If both TRPs fail but candidates are available, UE could use the TRP specific failure recovery. In case both TRPs fail but no new candidate beam can be indicated UE could switch to cell-specifc BFR (rel15/16 BFR)

Proposal 3-2: Cell-specific and TRP-specific BFR can be configured in the same serving cell (at least SpCell).

Proposal 3-3: If both TRPs fail and candidates are available, UE uses the TRP specific failure recovery. In case both TRPs fail but no new candidate beam can be indicated UE switches to cell-specifc BFR (fallback to rel15/16 BFR)

## 3.2 On number of BFD-RS of TRP specific BFR

In RAN1#104bis-e, RAN1 has made following agreement on the number of BFD-RSs in a set.

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

One current discussion point related to beam failure detection is the maximum number of BFD-RS in a set of q0. Currently, the maximum number of BFD-RS is 2 although UE can be configured with up to 3 CORESETs with the active TCI States (in non-mTRP scenario). In Rel-16 the number of CORESETs with the active TCI states in mTPR scenario was increased up to 5 CORESETs, but the number of BFD-RS remained the same.

Observation 3-4: The current maximum number of BFD-RS in set of q0 is 2 even in mTRP scenario.

Furthermore, the selection of BFD-RS to be included in the set of q0 is up to UE implementation (as opposed to e.g. selection of RLM-RS) and considering the mTRP scenario, it is not clear how the performs beam failure detection in release 16 (with a single set of q0) when more than 2 CORESETs are configured with active TCI States.

Furthermore, it should also be discussed in RAN1 that how to handle the potential increase of the maximum number of BFD-RS or how to determine the RS for failure detection with e.g. two sets of q0 if the current maximum number of BFD-RS remains the same as in Rel-15 and Rel-16.

On the proposed alternatives, Alt1 is aligned with current rel-15/16 specification (i.e. up to 2) but leaves the ambiguity that which RS UE will/has selected for failure detection in case the 2 CORESETs are configured with active TCI states for CORESETs under the same CoresetPoolIndex value.

The Alt.2 may have similar problem of ambiguity for the monitored RS for failure detection in case UE capability value 1 or 2 is selected.

Observation 3-5: Neither Alt1 or Alt2 (with capability of less 3) does not cope with the increased maxmimum number of CORESETs with active TCI states.

In case the candidate value less than 3 is accepted for the maximum number of RS per BFD-RS set there should be a rule to select the RS to be included in the sets of q0.

In rel-15 RLM, when the maximum number of CORESETs with active TCI States exceeds the maximum number of RLM-RS, UE determines the monitored RS according to a specific rule based on search space periodicity and CORESET index. Regardless whether the Alt1 or Alt2 is selected, a rule to select the RS to be included into the set of q0 should be defined. Same rule as already specified for RLM (when Lmax=4) could be used.

Proposal 3-4: For implicit configuration, adopt a mechanism to select the RS to be included into set of q0 . Similar mechanism as specified for rel-15 RLM is used if/when number of activated TCI States for CORESETs exceeds the capability of maximum number of BFD-RS for respective set of q0.

## 3.3 1-to-1 association between BFD-RS and NBI-RS set

In RAN1 # 104e meeting, RAN1 made following agreements on the M-TRP new beam identification.

Agreement

For M-TRP BFR

Support 1-to-1 association between each BFD-RS set and an NBI-RS set

- FFS: Association details

In our view the association details, the signaling part, could be left for RAN2. RAN1 can state that there is 1-1 mapping between each BFD-RS set and NBI-RS set, and that each BFD-RS has to be configured with a respective candidate beam RS list. It is up to network implementation to determine the RS for each set.

Proposal 3-5: Each failure detection resource set is associated with a respective candidate beam reference signals list (candidateBeamRSList). The association between BFD-RS set and candidate beam RS list for respective sets are configured by network. Signaling details can be left to RAN2

## 3.4 Explicit/implicit BFD-RS set association to TRP

In RAN1 #103-e meeting, RAN1 made following agreements on the BFD-RS configuration per TRP.

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

To support TRP specific failure detection, RAN1 agreed to support 2 BFD-RS sets but did not agree how to determine the RS for respective sets.  Furthermore, supporting both configuration modes (explicit and implicit) would align the configuration of TPR specific failure detection resources with Rel-15/rel16 operation.

In the explicit configuration, the network could configure the RSs that are included in each of the set of q0 (e.g. set 0/q0-0 and set 1/ q0-1) in a similar manner as in Rel-15/16.

In the implicit configuration, the RSs to be included in the failure detection resource sets would be determined by the active TCI states for respective CORESETs.

When coresetPoolIndex value is configured, to determine which of the RSs are included in each failure detection resources sets (e.g. q0-0 and q0-1), the coresetPoolIndex value associated with a respective CORESET could be used.

Proposal 3-6: Support configuration of beam failure detection resource sets (q0) per coresetPoolIndex value for M-DCI.

When the coresetPoolIndex value is not configured (e.g. S-DCI M-TRP or PDCCH repetition), it is FFS how to support implicit configuration of two sets of BFD-RS. Also, there are multiple modes that are currently being discussed in multiple sub-agenda items of M-TRP, and BFR mechanism for all M-TRP related scenarios can be discussed here. In summary, few cases can be identified as below,

- S-DCI M-TRP scenario (Rel-16 Schemes)

  - As one example, in Rel-16 PDSCH beam activation by PDSCH MAC-CE, the UE can distinguish two TCI states in a TCI codepoint are associated to different TRP. The same mechanism can be applied to support implicit configuration of two sets of BFD-RS.

- PDCCH repetition scenarios

  - SFN : Same CORESET may have multiple TCI states activated, and it is possible to support implicit configuration of two sets of BFD-RS via associating two TCI states activated in sequential order for CORESET with each BFD-RS.

  - Non-SFN : CORESETs may be linked via SSSet linking, and it is possible to support implicit configuration of two sets of BFD-RS via associating linked CORESETs with each BFD-RS.

Alternatively or additionally, one possible option for implicit mapping can also use SSB grouping suggested in section 2. Once SSB resources are grouped to be associated to different TRPs, the association of BFD-RS to a TRP can be supported based on its QCL relation to a SSB implicitly.

Proposal 3-7: Support implicit configuration of two sets of BFD-RS for the other M-TRP scenarios like M-TRP PDCCH repetition (both SFN, Non-SFN) and S-DCI M-TRP schemes (Rel-16).

## 3.5 Whether two NBI-RS sets can overlap

It has been agreed by RAN1 that for each BFD-set there is corresponding candidate beam RS set, which is configured by network. When a TRP fails, UE should indicate the failure of the TRP by referring to the failed BFD-RS set index. By referring to the index, network can determine the candidate beam RS list (NBI-RS list) that UE uses to select new candidate beam index (if any).

As the candidate beam RS list is associated with BFD-RS set and the list of RS is configured by network (as in SCell BFR) there should be no restrictions imposed for the network implementation how to configure the candite beams RSs in the respective sets.

Proposal 3-8: Configuration of candidate beam RS(s) (SSB/CSI-RS) to the NBI-RS set(s) is up to network implementation.

## 3.6 RACH-based fallback

In Rel-16, it is possible to map the SCell BFR to ‘zero’ SR configuration (as per 38.321) which means that it is not mandatory to use SR for SCell BFR. In a similar manner, the mTRP BFR should be designed so that it works regardless of the SR configuration. When the SR is not configured for the purpose of mTRP BFR, the BFR can be supported by using the CBRA procedure to send the MAC CE or use the available UL grant (if available). Even if PUCCH-SR is configured, UE can use the available UL grant for transmitting the MAC CE. Details should be up to RAN2.

Observation 3-6: UE can transmit CBRA for transmiting MAC CE for mTRP BFR regardless of PUCCH-SR configuration.

Observation 3-7: MAC CE for mTRP BFR can be transmitted on available UL grant without sending an SR.

Proposal 3-9: No restriction to perform CBRA for mTRP BFR is considered.

## 3.7 PUCCH-SR

In RAN1#104bis-e, RAN1 has made following agreement on PUCCH-SR.

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Agreement

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs,

- Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

  - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

  - Alt-3: Leave it up to UE implementation

- Note: PUCCH-SR resource is PUCCH resource carrying SR

- FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

In SCell BFR (rel-16), UE can be configured with an SR resource to indicate failure of one or more SCells. By transmitting the SR to indicate SCell failure, UE indicates the network that it requires UL resources for transmitting the (SCell) BFR MAC CE. MAC CE provides the information on the failed SCell(s), indication of candidate beam availability and the candidate beam index if available (i.e. the RSRP of at least one candidate beam in the candidate-beam-rs-list is above threshold). It should be noted that although the network may configure or assign SR ID for indicating SCell BFR, the SR ID may be associated with one or more UL logical channels thus the SR may not specifically indicate only SCell failure. Network may choose to configure the SR in a dedicated manner for SCell BFR but this would be left for NW implementation.

The SCell approach could also be valid configuration option for multi TRP BFR i.e. NW can map the PUCCH-SR for mTRP BFR to an SR ID that may have zero, one or more logical channels mapped to it i.e. the SR configuration is shared between the mapped logical channels and mTRP BFR. Network may as per implementation configure SR ID that is only used for mTRP BFR purposes or it could map the SCell

Proposal 3-10: PUCCH-SR for mTRP can be mapped to a configured SR ID that may be shared with other functionality. Details are up to RAN2.

In case of 2 PUCCH-SR resources, the respective resources and the use of the resource should map to the BFD-RS sets. When a BFD-RS set (TRP) fails and PUCCH-SR ID is associated with the failed resource set, UE can transmit the PUCCH-SR to indicate failure.

Proposal 3-11: When two PUCCH-SR resources are configured, for PUCCH-SR resource selection rule, Support Alt-2, PUCCH-SR resource associated with failed BFD-RS set.

## 3.8 BFRQ MAC-CE content

Considering the mTPR BFR procedure and current discussion, the MAC CE should be conveyed using CBRA and available UL grant in addition to the requested UL grant using the PUCCH-SR. It is noted that the mTRP BFR should work without configuration of SR ID for mTRP BFR purposes, similarly as the SCell BFR works. Thus, the MAC CE should carry information on the failed TRP (or more specifically the reference to the failed BFD-RS resource set).

The mTRP may also be used in CA scenarios and it would be required information in the MAC CE to indicate the failed CC as well when at SCell(s) are configured with beam failure detection/recovery. Also PCell indication should be supported.

Information on the new candidate beam index and the availability of candidates could follow the Rel16 approach and both indication of whether a suitable candidate beam is found/detected and the candidate beam index in the candidate beam RS list.

To provide information on the TRP specific failure, which TRP or TRPs have failed, the MAC CE could carry indication of failure of a beam failure detection resource set.

Proposal 3-12: To indicate the failed TRP, an indication of the failed beam failure detection resource set is to be carried by the MAC CE.

# Conclusion

In this contribution, we discuss remaining details related to multi-TRP/panel transmission. The following observations and proposals are made.

BM Related observations/proposals

Proposal 2-1: In addition to option 2, support Option 3 for ideal/non-ideal backhaul scenario.

Observation 2-1: When two CMR resource sets are used, additional specification effort is required for AP CSI triggering state, CRI indication and UE capability for maximum CSI-resources in a set.

Observation 2-2: When two CMR resource subsets are used, the existing CSI framework can be reused with the consistency with CSI report setting for CSI acquisition.

Proposal 2-2: For group-based beam reporting for multi-TRP operation, support two CMR resource subsets to group CMR resources corresponding to a TRP.

Observation 2-3: It is possible to utilize existing spatial QCL-typeD relationships between different RSs with the grouping of SSBs, where the UE understand the beams can be associated with a TRP.

Observation 2-4: For inter-cell M-TRP, SSB grouping by PCI is already supported.

Proposal 2-3: Support both explicit and implicit methods for mapping a subset of CMR resources to a  TRP.

- For CMR of SSB resources, support explicit method, a subset indication bitmap is included in CSI-resource Setting.

- For CMR of NZP-CSI-RS resources, support implicit method where SSB is grouped by a common SSB bitmap (or PCI index), and CSI-RS is associated with a subset/group by the QCLed SSB.

Proposal 2-4: For enhanced group-based beam reporting for multi-TRP, support N={1,2,3,4} beam-pairs for a group-beam report.

Proposal 2-5: For enhanced group-based beam reporting for multi-TRP, support Alt2 (The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE)

Observation 2-5: UE Rx panel-related information should be defined for the general BM aspect first.

Proposal 2-6: Support the enhanced group-beam reporting for L1-SINR report only when dedicated IMR is configured.

Observation 2-6: It is beneficial to configure beam reporting criteria that imposes UE to rank and report only measured CSI resources being within a certain power window or above a power threshold.

Observation 2-7: From latency reduction perspective, it is beneficial to consider additional criterion that can take into account latencies associated with multi-panel reception.

Proposal 2-7: Support beam reporting criteria that imposes UE to rank and report only measured CSI resources being within a certain power window or above a power threshold.

Observation 2-8: To guarantee reliable multi-TRP operation with enhanced group based beam reporting, it is beneficial to develop a fallback mechanism/procedure that enables UE to recover from situation where enhanced beam reporting condition is not fulfilled.

Proposal 2-8: Consider different alternatives as fallback mechanisms, such as:

- Alt-1: use Rel-15 group reporting (with a restriction on ‘per TRP’ with predefined TRP)

- Alt-2: use Rel-15 non-group reporting (no restriction on simultaneous reception)

- Alt-3: network configures the fallback reporting (based on Alt-1 or Alt-2)

Proposal 2-11: For non-group based beam reporting, support association of a reporting setting to another reporting setting to ensure the UE’s simultaneous reception from multi-TRP for multi-DCI based multi-TRP scheme,

- UE shall select beams to be reported with the consideration of the simultaneous reception from two TRP.

Proposal 2-10: For the association of CSI report settings, the associated CSI-ResportSettingID is included in the CSI-ReportSetting.

Proposal 2-11: Study method to indicate beams simultaneous reception capability.

BFR Related observations/proposals

Observation 3-1: Both TRPs have the corresponding BFD-RS sets and the sets can be in failure at the same time.

Observation 3-2: Candidate beams are indicated from the candidate beam lists and there may not be candidates available for the failed TRPs

Proposal 3-1: Define UE behavior when no new candidate can be indicated for the failed TRP.

Observation 3-3: If both TRPs fail but candidates are available, UE could use the TRP specific failure recovery. In case both TRPs fail but no new candidate beam can be indicated UE could switch to cell-specifc BFR (rel15/16 BFR)

Proposal 3-2: Cell-specific and TRP-specific BFR can be configured in the same serving cell (at least SpCell).

Proposal 3-3: If both TRPs fail and candidates are available, UE uses the TRP specific failure recovery. In case both TRPs fail but no new candidate beam can be indicated UE switches to cell-specifc BFR (fallback to rel15/16 BFR)

Observation 3-4: The current maximum number of BFD-RS in set of q0 is 2 even in mTRP scenario.

Observation 3-5: Neither Alt1 or Alt2 (with capability of less 3) does not cope with the increased maxmimum number of CORESETs with active TCI states.

Proposal 3-4: For implicit configuration, adopt a mechanism to select the RS to be included into set of q0 . Similar mechanism as specified for rel-15 RLM is used if/when number of activated TCI States for CORESETs exceeds the capability of maximum number of BFD-RS for respective set of q0.

Proposal 3-5: Each failure detection resource set is associated with a respective candidate beam reference signals list (candidateBeamRSList). The association between BFD-RS set and candidate beam RS list for respective sets are configured by network. Signaling details can be left to RAN2

Proposal 3-6: Support configuration of beam failure detection resource sets (q0) per coresetPoolIndex value for M-DCI.

Proposal 3-7: Support implicit configuration of two sets of BFD-RS for the other M-TRP scenarios like M-TRP PDCCH repetition (both SFN, Non-SFN) and S-DCI M-TRP schemes (Rel-16).

Proposal 3-8: Configuration of candidate beam RS(s) (SSB/CSI-RS) to the NBI-RS set(s) is up to network implementation.

Observation 3-6: UE can transmit CBRA for transmiting MAC CE for mTRP BFR regardless of PUCCH-SR configuration.

Observation 3-7: MAC CE for mTRP BFR can be transmitted on available UL grant without sending an SR.

Proposal 3-9: No restriction to perform CBRA for mTRP BFR is considered.

Proposal 3-10: PUCCH-SR for mTRP can be mapped to a configured SR ID that may be shared with other functionality. Details are up to RAN2.

Proposal 3-11: When two PUCCH-SR resources are configured, for PUCCH-SR resource selection rule, Support Alt-2, PUCCH-SR resource associated with failed BFD-RS set.

Proposal 3-12: To indicate the failed TRP, an indication of the failed beam failure detection resource set is to be carried by the MAC CE.

# References

- 3GPP RAN1 #104-e Chairman notes

- 3GPP RAN1 #104-bis-e Chairman notes