3GPP TSG RAN WG1 #105-e	R1-2105307

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.4.2

Source: 	Samsung

Title: 	Enhancements on UL time and frequency synchronization for NTN

Document for:	Discussion and Decision

Introduction

A new work item on “Solutions for NR to support non-terrestrial networks (NTN)” was approved in RAN#86 [1]. The following agreements pertaining to enhancements on UL time and frequency synchronization for non-terrestrial networks (NTN) were reached in RAN1#104b-e:

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3:NTA,common is the common timing offset X as agreed in RAN1 #103-e. Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported Conclusion: The orbital propagator model to be used at UE side can be left to implementation.

.

This contribution considers enhancements on UL time and frequency synchronization for NTN based on the above agreements and open issues.

Enhancements on UL time and frequency synchronization

Serving satellite ephemeris indication

The serving satellite ephemeris indication can include the satellite position and velocity at a reference time.  The altitude of the satellite can be succinctly conveyed by pre-defining a table as shown in Table 1:

| Satellite |  | Altitude level hi (km) | Altitude scalar si |
| --- | --- | --- | --- |
| type | Index i |  |  |
| LEO 300-1500 km | 0 | 300 | s0 |
|  | 1 | 400 | s1 |
|  | 2 | 500 | s2 |
|  | … | … | … |
|  | 10 | 1300 | s10 |
|  | 11 | 1400 | s11 |
| MEO 7000-25000 km | 12 | 7000 | s12 |
|  | 13 | 8000 | s13 |
|  | 14 | 9000 | s14 |
|  | … | … | … |
|  | 28 | 23000 | s28 |
|  | 29 | 24000 | s29 |
| GEO | 30 | 35786 | s30 |

Table 1: A predefined table for satellite altitude levels

The gNB indicates the altitude offset hoffset and the satellite index i, where i=0, 1, …, NmaxIndex, and NmaxIndex is a predefined integer (e.g. 30 in Table 1).  The altitude of the satellite h can be determined as:

h=hi+si∙hoffset		(1)

Here, hi is the altitude level and si is the altitude scalar.

The gNB can indicate the satellite ephemeris in system information, e.g. MIB, SIB1, other SIBs and/or a new SIB. The ephemeris information can include the following fields:

- satellite index i

- altitude offset hoffset

- satellite position, e.g. (X, Y, Z) in earth-centered earth-fixed (ECEF) coordinates

- satellite velocity, e.g. (VX, VY, VZ) in ECEF coordinates

- reference time for satellite position and velocity, e.g. system frame number and/or slot index

Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings:

- index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type

- satellite altitude offset

- satellite position

- satellite velocity

- reference time for satellite position and velocity.

In RAN1#104bis-e, two sets were discussed for ephemeris information indication as below.

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

One point for further consideration is that NTN will focus on LEO and GEO scenarios, while de-prioritizing MEO scenarios.  Since LEO and GEO orbits are not highly eccentric, this may simplify the calculations for satellite orbit propagation.

Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors, i.e., Set 1.

TA management

- General frame work for TA in NTN

In RAN1#104b-e, the following was agreed.

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3:NTA,common is the common timing offset X as agreed in RAN1 #103-e.

N_{TA,common} and N_{TA}

These two values need to reflect the propagation delay (not including the delay between the UE and the satellite).  For example, NTA,common and NTA could correspond to the feeder link delay and other processing delays. For a LEO satellite, the feeder link delay is dynamic; thus, the sum of NTA,common and NTA needs to be updated.

Figure 1 shows the feeder link delay for the altitude of 2000 km. The feeder link delay varies from about 6.5 msec to 18 msec.

Figure 2 shows 1) the feeder link delay as a function of time and 2) the expected feeder link delay estimation error when a first-order linear regression (that accounts for the drift rate) is used to estimate NTA,common  over a 2-second window. The maximal estimation error is about 2×10-5 msec, which is 40∙Tc (and much smaller than the length of the CP).

`

NTA,common and NTA can be updated based on the following options:

- Option 1

  - NTA,common: its update is supported by group common signalling.

  - NTA: Defined as the same as Rel-15/16

- Option 2

  - NTA,common: broadcasted by SIB.

  - NTA: Defined as the same as Rel-15/16

- Option 3

  - NTA,common: broadcasted by SIB with drift rate information.

  - NTA: Defined as the same as Rel-15/16.

For granularity, since this is related to the performance requirement and signaling in TA mechanism, we propose to discuss the granularity after concluding the signaling and update of NTA,common and NTA.

Figure 1: Feeder link delay as a function of elevation angle for the LEO satellite with altitude of 2000 km.

Figure 2: Feeder link delay  as a function oftime for the LEO satellite with altitude of 2000 km and its expected error with linear regression

N_{TA, UE-specific}

A UE calculates NTA,UE-specific based on its estimated position and applies it for uplink transmission. However, depending on NTA,common and NTA, the gNB may need the UE to report NTA,UE-specific to resolve ambiguity regarding TTA. Therefore, a mechanism to align TTA between the gNB and the UE is needed, e.g., a UE reports NTA,UE-specific when 1) it updates NTA,UE-specific or 2) the gNB triggers the UE to update NTA,UE-specific.

For granularity, if the UE does not report NTA,UE-specific, then the UE can just apply NTA,UE-specific as a real number, which is the distance between the satellite and the UE divided by the speed of light. If the UE reports NTA,UE-specific, it can use 16∙64∙Tc/2μ as the reporting granularity.

Proposal 3: The following options are considered for NTA,common and NTA.

- Option 1

  - NTA,common: its update is supported by group common signalling.

  - NTA: Defined as the same as Rel-15/16

- Option 2

  - NTA,common: broadcasted by SIB.

  - NTA: Defined as the same as Rel-15/16

- Option 3

  - NTA,common: broadcasted by SIB with drift rate information.

  - NTA: Defined as the same as Rel-15/16.

Proposal 4: RAN1 discusses the granularity after concluding the signaling and update of NTA,common and NTA.

Proposal 5: A mechanism to align TTA between the gNB and the UE is adopted, e.g., a UE reports NTA,UE-specific when it updates NTA,UE-specific or the gNB triggers the UE to update NTA,UE-specific with a granularity of 16∙64∙Tc/2μ.

Open/Closed Loop TA control

The following was agreed in RAN1#104-e.

Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control

NTA,UE-specific may be relatively static in certain scenarios, e.g. communication with a GEO satellite. In those instances, the gNB may only need to employ closed-loop TA control, reducing the computational burden on a UE to update NTA,UE-specific (which can be set to zero or set to its initial value). This motivates supporting closed-loop TA control (along with open-loop TA control and the combination of open&closed loop TA control, depending on the gNB configuration).

Proposal 6: Each of the following options are supported based on the gNB configuration:

- Closed-loop TA control

- Open-loop TA control

- Combination of open&closed-loop TA control

- Reporting of UE estimated TA

If a UE applies a UE-specific TA in transmitting the PRACH, rather than simply applying a common TA indicated by the network, the gNB cannot know the full TA value applied at the UE based on the estimated preamble reception timing. In our companion contribution [2], the necessity of a UE-specific K_offset is discussed for the case of large cell sizes after initial access procedure. To update a UE-specific K_offset, the network should know the full TA value applied at the UE. In order to make sure the time applied with the reported TA, the UE reports its TA value by MAC CE to the gNB.

Proposal 7: UE’s estimated TA value is reported to gNB by MAC CE.

- Common TA indication

Once RAN1 has completed the above-mentioned normative works, if RAN1 can identify any of the above-mentioned scenarios, then RAN1 should support GNSS-challenged UEs to enhance the robustness of NTN. In those scenarios:

- GNSS-challenged UEs can utilize a common TA value to perform coarse timing and frequency pre-compensation for uplink synchronization.

- The gNB should account for larger timing uncertainties in the received PRACH; for example, it can provision a longer guard time.

Also, in the previous RAN1#103-e meeting, it was agreed that in NTN, the network may broadcast a common timing offset value (while the details of this common timing offset are FFS). This common timing offset value can be set to the common TA value.

The common TA value can be indicated based on the altitude of the satellite and the additional residual propagation delay due to the slanted beam between the satellite and the ground.

Observation 1: The common TA, Tc, can be divided into the minimum common TA, Tc0, and a residual common TA, TcRes. The minimum common TA, Tc0, can be derived by UE from satellite ephemeris (or simply altitude) information without additional signalling.

The minimum common TA value can be calculated as Tc0=h/c, where h is the altitude of the satellite and c is the speed of light.

The common TA value can then be determined as:

Tc=s∙hc+ TcRes		(2)

where s can be indicated to be 2 (or a larger value), given the Rel-17 NR NTN emphasis on transparent satellite payloads. Thus, TcRes will account for the total propagation delay from the service link and the access link. In a future release, if evolved NTN considers regenerative satellite payloads, then s can be indicated to be 1 if the common TA value only includes the propagation delay from the service link.

Proposal 8: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information.

When a UE can precisely estimate, and apply, a UE-specific TA for PRACH transmission, the residual TA value would be insignificant; it would fall within the TA value signaling range in the RAR. However, for GNSS-challenged UEs, the residual TA value may differ significantly from the indicated common TA value; if so, then it cannot be signaled to the UE using the TA value signaling range in the RAR:

- In 38.213, the maximum value ofis 3846, yielding which is 1,476,864 for 15 kHz SCS.

- Multiplying NTA by the NR basic time unit, i.e., Tc = 0.509 ns, yields 0.751 ms as the maximum TA that can be indicated in the RAR; this corresponds to a maximum cell size of 225 km.

- The typical satellite beam footprint size is 100 – 1000 km for LEO and 200 – 3500 km for GEO, which may exceed the above-mentioned maximum cell size.

One solution is to extend the current TA value signaling range in the RAR for NTN. Another solution is to signal several common TA values; for example, one cell can be divided into three zones as shown in Figure 3.

Zone 2

Central Zone

Zone 1

A

B

Figure 3: Multiple common TA values for multiple zones.

For the central zone, the reference point for the common TA value is the center of the cell. For the surrounding zones 1 and 2, the reference points can be chosen as points A and B, respectively. Thus, a total of three common TA values can be utilized by the UEs; the common TA value that a particular UE utilizes can either be signaled by the satellite or derived by the UE if it knows its own location.

Proposal 9: Multiple reference points and common TA values should be considered for extremely large cells.

Enhancements to TA maintenance phase

In the previous meetings, companies discussed the mechanics of the TA maintenance phase, including the possibility of supporting a combination of open and closed control loops to that end. The UE can utilize the TA variation rate when performing TA maintenance.

The TA variation rate and Doppler shift are proportional to each other; this is shown in Figure 5 for the basic UE-satellite configuration in Figure 4.

Figure 4: Basic UE-satellite configuration.

Figure 5: TA variation rate and Doppler shift for basic UE-satellite configuration.

The TA variation rate, rTA, and the Doppler shift, fD, due to the motion of the satellite are given by

rTA=vccosω,

fD=fcvccosω		(3)

where v is the speed of the satellite, i.e., v=GMER+h, c is the speed of light, ω is the angle between the satellite velocity and the LOS from the satellite to the UE, and fc is the carrier frequency. The UE can then use (3) to determine either 1) the Doppler shift if the TA variation rate is indicated or 2) the TA variation rate if the Doppler shift is indicated. The UE can also determine both the TA variation rate and the Doppler value based on the satellite ephemeris indication if the BS also indicates either ω or cosω.

Thus, the TA variation rate and the Doppler shift can be jointly signaled to the UE by indicating one of the following:

- TA variation rate rTA

- Doppler shift fD

- ω or cosω

- Frequency offset value to be compensated

Observation 2: The gNB jointly indicates the TA variation rate and the Doppler shift.

Observation 3: Based on the indicated TA variation rate rTA (and the current TA), the UE can autonomously adjust its TA.

Observation 4: Based on the indicated Doppler shift fD (and the compensated frequency offset), the UE can determine the residual Doppler shift and pre-compensate its UL transmission.

The gNB can also use a UE-specific DCI format to indicate the TA variation or the Doppler. The following options can be considered:

- A DCI format associated with the UE-specific search space can be re-used, where the interpretation of the DCI format can be to indicate the TA variation or the Doppler if all remaining fields/bits not used for the indication have predetermined values. For instance, DCI format 0_0 or 1_0 can be reused.

- A new DCI format can be introduced for UE-specific indication, where the CRC is scrambled by the C-RNTI. The size of the new DCI format (which differs from the size of DCI formats scheduling PDSCH/PUSCH) can be L1 bits. L2 out of L1 bits can be used to indicate the TA variation or the Doppler, and the remaining (L1-L2) bits can be reserved (e.g. set to ‘0’). To fulfill the budget for a maximum of three DCI formats with C-RNTI having different sizes, it is assumed that the DCI formats scheduling PDSCH/PUSCH have two sizes (and size-matching is performed when necessary). Admittedly, this approach will limit possible schedulable DCI formats to UE.

A group-common (GC) DCI format with a corresponding RNTI (TDI-RNTI) can also be used to indicate the TA variation and/or Doppler to a set of UEs located in a certain area in the cell that are experiencing a similar TA variation rate and/or Doppler shift.

The gNB can also use the TAC MAC CE in a scheduled PDSCH reception to indicate the TA variation or Doppler. For example, the gNB can extend the size of the TAC MAC CE to indicate the TA variation or Doppler; this is shown in Figure 6.

Figure 6: Extension of TAC MAC CE for indication of TA variation and/or Doppler.

The PDSCH carrying the TAC MAC CE can be scheduled by either a UE-specific DCI or a group-common DCI with a TDI-RNTI.

Proposal 10: The gNB signals common TA drift rate to enable autonomous TA update at UE.

Proposal 11: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa.

UL frequency synchronization

In RAN1#103 and RAN1#104-e, the following agreements and conclusion were made.

Agreement: An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. - FFS: support of DL frequency compensation for the service link Doppler.

Figure 7: Center frequencies for DL at gNB, satellite, and UE with Doppler shift

In Figure 7, DL center frequencies at DL at gNB, satellite, and UE are shown. The UE can calculate frequency offset ∆f2 that occurs in service link based on the position information of the satellite. The frequency offset ∆f1 that occurs in feeder link can be pre-compensated by the gNB for DL. The other approach could be for the gNB to indicate ∆f1 to UEs. Since a UE in Rel-17 enables to calculate ∆f2 based on the GNSS information, the gNB may not need to indicate the additional offset for the UE. The UE pre-compensates the frequency offset ∆f2 before it performs PRACH transmission based on SIB and GNSS information. After RRC connection, the UE can still continue to pre-compensate ∆f2 before uplink transmission. However, similar to TA adjustment, the additional indication of frequency offset ∆f2 could be used for fine tuning for UL frequency synchronization.

Proposal 12: The gNB indicates the additional UL frequency offset value.

Conclusions

In this contribution, we discussed several enhancements on UL time and frequency synchronization for NTN.  Our proposals are summarized as follows.

Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings:

- index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type

- satellite altitude offset

- satellite position

- satellite velocity

- reference time for satellite position and velocity.

Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors, i.e., Set 1.

Proposal 3: The following options are considered for NTA,common and NTA.

- Option 1

  - NTA,common: its update is supported by group common signalling.

  - NTA: Defined as the same as Rel-15/16

- Option 2

  - NTA,common: broadcasted by SIB.

  - NTA: Defined as the same as Rel-15/16

- Option 3

  - NTA,common: broadcasted by SIB with drift rate information.

  - NTA: Defined as the same as Rel-15/16.

Proposal 4: RAN1 discusses the granularity after concluding the signaling and update of NTA,common and NTA.

Proposal 5: A mechanism to align TTA between the gNB and the UE is adopted, e.g., a UE reports NTA,UE-specific when it updates NTA,UE-specific or the gNB triggers the UE to update NTA,UE-specific with a granularity of 16∙64∙Tc/2μ.

Proposal 6: Each of the following options are supported based on the gNB configuration:

- Closed-loop TA control

- Open-loop TA control

- Combination of open&closed-loop TA control

Proposal 7: UE’s estimated TA value is reported to gNB by MAC CE.

Proposal 8: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information.

Proposal 9: Multiple reference points and common TA values should be considered for extremely large cells.

Proposal 10: The gNB signals common TA drift rate to enable autonomous TA update at UE.

Proposal 11: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa.

Proposal 12: The gNB indicates the additional UL frequency offset value.

In addition, the following observations are made.

Observation 2: The common TA, Tc, can be divided into the minimum common TA, Tc0, and a residual common TA, TcRes. The minimum common TA, Tc0, can be derived by UE from satellite ephemeris (or simply altitude) information without additional signalling.

Observation 2: The gNB jointly indicates the TA variation rate and the Doppler shift.

Observation 3: Based on the indicated TA variation rate rTA (and the current TA), the UE can autonomously adjust its TA.

Observation 4: Based on the indicated Doppler shift fD (and the compensated frequency offset), the UE can determine the residual Doppler shift and pre-compensate its UL transmission.

- References

- RP‑210908, “Revised WID: Solutions for NR to support non-terrestrial networks (NTN).”

- R1-2009748, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”.

- R1-2103241, “On Timing relationship enhancements,” Samsung, RAN1-104b-e, April 2021.