3GPP TSG RAN WG1 #105-e             	                                            R1-2105234

e-Meeting, May 10th – May 27th, 2021

Agenda Item:		8.7.2

Source:			Samsung

Title:			Discussion on DCI-based power saving techniques

Document for:		Discussion and decision

# Introduction

In RAN1#104 e-meeting [1], the following were agreed:

Agreements:

- Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration.

  - Details FFS

Agreements:

- Further study whether and how to minimize the impact to data scheduling for new transmissions and retransmissions.

  - FFS details

- Further study the application delay for PDCCH adaptation indication

Agreements:

For DCI based PDCCH skipping in active time for an active BWP (if supported), the following can be further considered,

- Explicit indication of PDCCH adaptation

  - Scheduling DCI

    - Format 1_1

    - Format 0_1

    - Format 0_2/1_2

  - Non-scheduling DCI

    - Format 2_6 in active time

    - Format 2_0

    - Format 1_1 (SCell dormancy case 2)

  - additional indication mechanism

    - By reusing Rel-16 SCell dormancy indication when CA is configured, FFS details

    - By reusing Rel-16 cross-slot scheduling indication when R16 cross-slot scheduling is configured, FFS detailds

- DCI dynamically indicates a duration/periodic interval for skipping

  - FFS: how to indicate the duration/period interval, e.g., number of slots or skipping current DRX

- PDCCH skipping for a duration indicated by minimum scheduling offset

- Others are not precluded

Agreements

- For DCI based SSSG switching in active time for an active BWP (if supported), the following can be further considered,

  - Explicit indication of PDCCH adaptation

    - Scheduling DCI based

      - Format 1_1,

      - Format 0_1,

      - Format 0_2/1_2

      - Format 1_0

    - Non-scheduling DCI supported by vivo, Samsung

      - Format 2_6 in active time

      - Format 2_0

      - Format 1_0

      - Format 1_1 (SCell dormancy case 2)

    - additional indication mechanism

      - By reusing Rel-16 SCell dormancy indication when CA is configured, FFS details

      - By associating Rel-16 cross-slot scheduling indication when R16 cross-slot scheduling is configured, FFS detailds

    - DCI dynamically indicates a duration period for the switched SSSG, UE switch back to previous/default SSSG after duration endstimer expried

  - Timer-based SSSG switching, including RRC configured a timer, UE switch back after timer expired.

  - SSSG activation/deactivation

  - FFS: Implicit SSSG switching

    - SSSG switching triggered by SR

    - SSSG switching triggered by RACH

    - Default SSSG that a UE monitors when coming out of DRX to monitor an ON duration.

- FFS: whether/how to support SSSG switching for multiple groups of cell(s).

- FFS: whether/how to support SSSG switching in active time with DCP outside active time

- FFS: whether / how to support more than 2 SSSGs,

  - FFS: number of SSSGs

- FFS: a search space set group to emulate PDCCH skipping

- Others are not precluded

Agreements:

- The following alternatives can be considered for DCI based PDCCH monitoring adaptation in active time for an active BWP for power saving

  - Alt 1: Enhancement of Rel-16 SSSG switching to support PDCCH monitoring adaptation including skipping for a duration

  - Alt 2a: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and SSSG switching

  - Alt 2b: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and PDCCH monitoring periodicity adaptation

  - Others not precluded

In this contribution, we discuss the enhancements on DCI-based power saving techniques for connected-mode UE during DRX Active Time.

# Enhancements on DCI-based power saving techniques for connected mode within DRX Active Time

## A common design for search space set group switching and PDCCH skipping

Rel-16 PDCCH monitoring indication is only applicable outside DRX Active Time. That is, the UE may or may not skip the whole DRX cycle depending on the wake-up indication from DCI format 2_6. It results in larger latency loss especially for heavy traffic since the UE needs to wait until the next DRX cycle to receive a data packet if DCI format 2_6 indicates not to wake-up. Therefore, it is well motivated to introduce power saving signal for the dynamic PDCCH monitoring indication during DRX Active Time as well. Figure 1 shows an example of dynamic PDCCH adaptation based on SS set switching. If there is a traffic SS#1 configured with shorter monitoring periodicity can be activated, while, if there is no or less traffic SS#2 configured with longer monitoring periodicity can be activated.

Figure 1. Search space set group switching

In Rel-16 NR-U, SS set group switching using group-common DCI (DCI format 2_0) has been specified. This feature can be enhanced in Rel-17 to extend its usability to more generalized scenarios. One extension from Rel-16 SSSG switching can be search space set group configuration. Rel-16 only supports switching between two search space set groups. More search space set groups can be considered to provide adaptation in multiple dimensions other than time domain, such as frequency domain or across TRPs. For example, UE can be switched to SS set group associated with a sub set of TRPs. The preferred TRPs can be reflected by associated CORESETs, i.e. CORESETPoolIndex.

Proposal 1: Support search space set group (SSSG) switching with search space set group associated with a CORESET group for PDCCH monitoring adaptation across TRPs.

Due to the high flexibility in SS set group (SSSG) switching feature, there is no motivation to introduce other power saving techniques related to PDCCH monitoring adaptation. However, it was agreed in last meeting to strive for a common design to support both SSSG switching and PDCCH skipping. In this sense, an empty SSSG can be predetermined and associated with a time duration. UE can skip PDCCH monitoring for the time duration, when the UE is triggered to switch the empty SSSG by a DCI format.

Proposal 2: Support search space set group (SSSG) switching among more than two search space set groups, including empty SSSS group for PDCCH skipping.

Unlike NR-U, the adaptation within DRX active time is triggered according to UE-specific traffic. To use group common DCI with UE group-common indication field can be inefficient in this scenarios. New signalling methods to provide UE-specific indication can be supported in Rel-17. Three types of UE-specific triggering methods can be considered as below:

- Opt-1) UE specific scheduling DCI

- Opt-2) UE-specific DCI without scheduling of PDSCH/PUSCH

- Opt-3) Group-common DCI with UE-specific indication field.

For Opt-1, the scheduling DCI format 1_1/0_1 can be used to indicate a selected SS set group. The indication can be included as a new field. Alternatively, an existing field, e.g. minimum applicable scheduling offset indicator, can reused to trigger the adaptation on both the minimum applicable scheduling offset and SS set group. However, Opt-1 is applicable only when there is traffic.

For Opt-2, the scheduling DCI format 1_1/0_1 without scheduling a PDSCH/PUSCH can be considered. Similar as dormancy behavior indication, a SS set group can be provided based on reinterpretation of scheduling information fields, such as modulation and coding scheme, new data indicator, redundancy version, HARQ process number, antenna port(s), DMRS sequence initialization. Compared with Opt-1, the adaptation can be triggered any time no matter traffic arrives or not. However, the cost of Alt-2 is high signaling overhead. The resources overhead is large, especially when many UEs are triggered.

For Opt-3, DCI format 2-6 with UE-specific indication fields can be considered. Currently, the DCI format 2-6 is only monitored outside DRX active time to provide wake-up indication for next DRX ON duration. However, the monitoring occasions are also available within DRX active time according to the configuration of the associated SS set, and NW has full flexibility to configure the SS set. Therefore, the DCI format 2-6 can be reused for SS set group switching, where the UE-specific fields can be reinterpreted as SS set group indication within DRX Active Time. In this way, both the signaling overhead and configuration overhead can be reduced.

Since the triggering field is common regardless of DCI format used, and all the three options are supported in Rel-16 for other power saving schemes. We think multiple triggering methods can be supported so that NW can choose the preferred method based as needed.

Proposal 3: Support triggering methods for SS set group switching, including

- Opt-1: the scheduling DCI format with PDSCH/PUSCH

- Opt-2: group-common DCI format, e.g., DCI format 2-6

The adaptation can also be triggered by UEs based on feedback from UEs. In Rel-16, UE assistance information for power saving is introduced. It’s straightforward to extend the UE assistance information to include preferred search space set group.

Propose 4: Support UE assistance information of preferred search space set group.

Other extensions from Rel-16 include application delay and fallback operations. These can be discussed after determination of signaling method or search space set group configuration.

## 2.2 PDCCH skipping based on minimum scheduling offset

PDCCH skipping for a certain duration or DRX cycle was discussed as another candidate solution for adaptation on PDCCH monitoring. As mentioned in 2.1, DCI-based PDCCH skipping can be achieved by a common design based on SS set group switching. Other than DCI-based solution, there is one case PDCCH skipping could be beneficial. In Rel-16, cross-slot scheduling based power saving technique has been specified. By adapting minimum scheduling offset value, a UE can achieve power saving gain by means of 1) no PDSCH buffering, 2) micro-sleep after receiving PDCCH, 3) relaxing PDCCH processing timeline, etc. However, if the PDCCH monitoring periodicity is less than the minimum scheduling offset, UE cannot fully achieve these power saving benefits. This is because of that (i) a UE has to start buffering PDSCH samples due to the accumulated PDCCH monitoring occasions over time as illustrated in Figure 2 and (ii) the micro-sleep duration after receiving PDCCH is limited by the next PDCCH monitoring occasions other than the minimum scheduling offset. Therefore, joint adaptation on PDCCH skipping should be considered to achieve the power saving gain properly for cross-slot scheduling. In other words, the UE can skip the PDCCH monitoring occasions for the time duration indicated by minimum scheduling offset. This implicit mechanism does not require any additional L1 signaling as well as much specification effort.

Figure 2. Example of cross-slot scheduling based power saving technique in Rel-16

Proposal 5: Support PDCCH skipping for a duration indicated by minimum scheduling offset.

## 2.3 Enhancements for processing time relaxation

NR Rel-16 supports adaptation on minimum scheduling offset K0/K2 based on an indication carried in a DCI format 1_1 or 0_1. Therefore, UE can reduce power consumption by relaxing PDCCH processing, avoid buffering PDSCH reception, and operates in micro sleep mode during the active time in RRC_CONNECTED state. However, in order to achieve power saving from relaxed processing, it’s essential to consider relaxation on both PDCCH processing timeline and PDSCH reception and ACK/NACK feedback timeline, so that UE can lower the clock rate for all DL processing modules.  For example, a minimum PDSCH processing time, KminPDSCH , can be defined as minimum time offset between a first symbol of a PDSCH reception providing one or more transport blocks and a first symbol of a PUCCH transmission with HARQ-ACK information in response to the reception of the transport blocks.

To reduce system overhead on signaling processing, the minimum applicable scheduling offset indicator can be reused to provide joint indication of KminPDSCH and K0min/K2 min, as illustrated in Figure 3.

Figure 3: Joint adaptation on PDSCH processing time and minimum scheduling offset

Proposal 6: Support PDSCH processing time relaxation based on minimum scheduling offset.

# Conclusion

This contribution discussed potential enhancements for DCI-based power saving techniques during DRX Active Time and following proposals were made:

Proposal 1: Support search space set group (SSSG) switching with search space set group associated with a CORESET group for PDCCH monitoring adaptation across TRPs.

Proposal 2: Support search space set group (SSSG) switching among more than two search space set groups, including empty SSSS group for PDCCH skipping.

Proposal 3: Support triggering methods for SS set group switching, including

- Opt-1: the scheduling DCI format with PDSCH/PUSCH

- Opt-2: group-common DCI format, e.g., DCI format 2-6

Propose 4: Support UE assistance information of preferred search space set group.

Proposal 5: Support PDCCH skipping for a duration indicated by minimum scheduling offset.

Proposal 6: Support PDSCH processing time relaxation based on minimum scheduling offset.

# References

- [1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #104-e