3GPP TSG RAN WG1 #105-e	                                                        R1-2105326

e-Meeting, May 10th  – May 27th, 2021

Agenda Item:		8.8.1.2

Source:				Samsung

Title:					TB processing over multi-slot PUSCH

Document for:	Discussion

# Introduction

PUSCH was identified as the bottleneck for NR coverage. The following enhancements were agreed in Coverage enhancement WI and updated in [1] to improve coverage of PUSCH:

- Specification of PUSCH enhancements [RAN1, RAN4] - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1] - Increasing the maximum number of repetitions up to a number to be determined during the course of the work. - The number of repetitions counted on the basis of available UL slots. - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1] - TBS determined based on multiple slots and transmitted over multiple slots. - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4] - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4] - Potential optimization of DMRS location/granularity in time domain is not precluded - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

This contribution discusses the support of TB processing over multi-slot PUSCH.

# Discussion

Type A and Type B PUSCH repetition are supported by NR system. Two repetition schemes have different mechanism to determinate time domain resource. Type A PUSCH repetitions occupy the same symbols in each slots, while multiple nominal repetitions in Type B PUSCH repetition occupy consecutive symbols, and segments into actual repetitions over invalid UL slots and slot boundary. In both Type A and Type B PUSCH repetition, each (actual) repetition contains all the information of a TB, although the actual code rate in each actual repetition may be different for Type B PUSCH repetition. For the method that TB processing over multi-slot, the information of a TB can span over multiple slots. This paper discusses several key aspects to support this method of TB over multi-slot.

## Time domain resource allocation

The following agreement was made for TDRA related issue:

Agreement:

- Consider one or two of the following options as starting points to design time domain resource determination of TBoMS

  - PUSCH repetition type A like TDRA, i.e., the number of allocated symbols is the same in each slot.

  - PUSCH repetition type B like TDRA, i.e., the number of allocated symbols in each slot can be different

Both Type A and Type B PUSCH repetition use TDRA table to indicate time domain resource allocation, in which slot, starting symbol, and the number of symbols of one repetition are indicated. In Rel-16, repetition number can also be configured in TDRA table.

There was discussion on whether support repetition type A like and/or repetition type B like. From our understanding, the purpose of the proposals related to this issue: which is for resource determination rather than the actual transmission scheme. Therefore, from resource determination perspective, repetition type B-like manner could certainly be better than the repetition type A-like due to the more flexible usage of the symbols. This is extremely helpful in TDD case whether the UL resource is quite limited for a given time period. With all the UL signals needs to be considered (e.g., PRACH, PUCCH), it will be likely the possibility to have same number of symbols in the UL slots are questionable. It is also the reason to motivate the discussion on how to use the symbols in special slot. Comparing with designing new operation on handling symbols in special slot for repetition type A-like manner, it’s more convenient and beneficial to just reuse the type B-like TDRA manner.

Proposal 1: both PUSCH repetition type A and type B like TDRA are supported.

In addition to the TDRA, one important information is the number of slot carrying a single TB. There could be two options as well:

- Option 1: Indicating number of slot for one TB based on Type A and/or Type B PUSCH

With this method, UE can determinate the time domain resource, i.e., symbol(s), based on existing method. Additionally, number of slots (as well as repetitions) occupied by one TB can be indicated to UE. Shown as Figure 1, time domain resource is indicated by TDRA field in DCI as starting symbol S, number of symbols L in Slot n. In addition, 2 slots and 2 repetitions is used to transmit a single TB. For Type A repetition, PUSCH occupies L symbols starting from symbol S in Slot n and Slot n+1. 2L symbols are used to transmit one TB. On top of it, repetition is further configured, which occupies same symbols in Slot n+2 and Slot n+3.  Similarly for Type B PUSCH repetition, UE can determinate occupied symbols in each slot, and span one TB over multiple slots/repetitions.

This method can be supported on top of current Type A and Type B repetition. An extra parameter for the number of slot or repetition for on TB needs to be configured additionally. Repetition of a TB can be additional supported, to balance the decoding latency and performance. The benefit of this method is that, the new scheme of TB over multi-slot can better coexistence with PUSCH scheduled by Type A or Type B PUSCH repetition. gNB scheduler can be reused, and it is also easy for UE to reuse current scheme as much as possible.

Figure 1

- Option 2: Directly indicating a number of symbol L that can be larger than 14

In existing system, i.e., NB-IoT system, one TB over multiple subframes is already supported. The starting subframe and the number of subframes are directly indicated to UE. Different from NB-IoT, in NR, one transmission can only occupy several symbols in a slot. Beside the starting slot, the starting symbols and number of symbols needs to be indicated. Supporting the number of symbol L >14 was discussed in Rel-16 URLLC when designing Type B PUSCH repetition. To support TB transmit over multi-slot, we can directly extend the range of symbol number L, shown as Figure 2. To reduce the transmission latency, consecutive valid uplink symbols can be used. Different from Type B PUSCH repetition, one repetition can (effectively) mapping cross the slot boundary and invalid uplink symbols.  To support low code rate and decent TBS, which can avoid unnecessary RLC segmentation, the total number of symbols occupied in time domain might be large. In order to reduce the overhead, a symbol group can be used for time domain resource allocation.

In addition, similar as Option 1, repetition can be supported for TB over multi-slot.

Figure 2

Proposal 2: Consider following two options for time domain resource for a single TB in TBoMS:

- Option 1: Indicating number of slot for one TB based on Type A and/or Type B PUSCH

  - Number of occupied repetition/slots can be configured.

- Option 2: Directly indicating a number of symbol L that can be larger than 14.

  - A symbols group can be considered

- Other options are not precluded.

Proposal 3: Repetition is supported for TB over multi-slot.

## DMRS location

DMRS location is determinated based on the starting symbol of a slot or PUSCH. For Type B PUSCH repetition, the DMRS is inserted per actual repetition. For Type A PUSCH repetition scheme supported by Option 1, current two DMRS type can be reused. For Type B PUSCH repetition like scheme supported by Option 1 or new resource allocation method as Option 2, time domain location of DMRS can be determianted per TOT, or per slot. On the other hand, the support of joint channel estimation over multiple transmissions/slots also need to be considered.

Proposal 4: Further study the following method for time domain location of DMRS considering the joint channel estimation over multi-slot and transmissions:

- DMRS time domain location is determined per TOT

- DMRS time domain location is determined per slot

## Frequency domain resource allocation

TB over multi-slot is introduced to improve the coverage of PUSCH. Since uplink transmission is limited by transmission power, boosting the transmission power into a narrower bandwidth can help to improve coverage. On the other hand, because on TB can be transmitted over multiple slots. There is no need to occupy more frequency domain resource to achieve a lower code rate. There is no need to support more PRB in frequency domain, and single PRB might be enough. On the other hand, restricting the PRB number in frequency domain can reduce the DCI size, which is benefit for coverage of PDCCH.

Proposal 5: The maximal number of PRB allocated in time domain is reduced for TB over multi-slot.

Agreements:

One or two of the following approaches will be considered as a starting point to decide howNinfo NInfo for TBoMS is calculated (aiming for down selection in RAN1 #104-bis-e):

- Approach 1: Based on all REs determined across the symbols or slots (FFS whether symbols or slots are used) over which the TBoMS transmission is allocated

- Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1.

  - FFS: the definition of K

Note: L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios.

Agreements:

One or two of the following options will be considered (aiming for down-selection in RAN1#104b-e) to calculate NohPRB for TBoMS:

- Option 1: NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

- Option 2: NohPRB is calculated depending on both xOverhead and the number of symbols or slots (FFS whether symbol or slot are used) over which the TBoMS transmission is allocated.

  - FFS: if either the number of symbols or the number of slots is used.

  - FFS: if xOverhead is separately configured from the one in Rel-15/16.

FFS: impacts and further details if repetitions of TBoMS is supported.

FFS: whether the symbols allocated over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed.

TBS determination

During last meeting, the following agreements are concluded for TBS related aspects.

For PUSCH, TBS is determinated based on the code rate, modulation order, RE number in a slot and the number of PRB in frequency domain. Similarly, the number of TBS can also determinated based on the total number of REs, code rate and modulation. However, when calculating the RE number, REs in all the symbols over multi-slot need to be counted as illustrated in approach 1, excluding DMRS and overhead configured by high layer. In Rel-15/16,  NohPRB represents the overhead within a PRB, which is configured by gNB. Therefore, when calculating the total RE over multi-slot, the overhead of all the slots needs to be used. Regarding the options to derive NohPRB, option 1 seems simple and straightforward.

Proposal 6: NInfo for TBoMS is calculated based on all REs in all slots for the TB. NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

## TOT, Rate-matching and interleaver operation

Working Assumption

The concept of transmission occasion for TBoMS (TOT) is utilized for the purpose of discussion, where a TOT is constituted of time domain resources which may or may not span multiple slots

- FFS: details, whether multiple slots which constitute a TOT are consecutive or non-consecutive physical slots for UL transmissions

- FFS: other details.

- FFS: whether such concept will be specified or not.

Agreements:

For the definition of a single TBoMS, down select among the following options:

- Option 1: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using a single RV.

  - FFS: whether and how the single RV is rate matched across the TOT, e.g., continuous rate-matching across the TOT, rate matched for each slot and so on.

- Option 2: Only one TOT is determined for a TBoMS. The TB is transmitted on the TOT using different RVs.

  - FFS: how RV index is refreshed within the TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

- Option 4: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using different RVs.

  - FFS: whether and how RV index is refreshed within one TOT, e.g. after each slot boundary, at every jump between two non-contiguous resources, if any, and so on.

- FFS: the exact TBS determination procedure.

- FFS: whether a single TBoMS can be repeated or not.

- FFS: other implications, e.g., power control, collision handling and so on.

During last meeting, the concept of TOT for TBoMS transmission was raised and its potential relation with the rate matching operation was summarized in below agreements.

For the understanding of concept of TOT, as listed in the working assumption, it’s not yet decided it could include UL resources those are consecutive only, or could be non-consecutive. Thus, it impacts the understanding of the coming options stated as “Only one TOT is determined for a TBoMS” (referred to single TOT case) and “Multiple TOTs are determined for a TBoMS” (referred to multi-TOT case) as shown in Fig.3.

For single TOT case, the general understanding is that it contains all the possible UL resources for one TBoMS. As special case is that companies may understand it together with the slot number determination of one TBoMS which is only for these consecutive slots.

For multi-TOT, there are also two cases: (a) is that the consecutive UL resources are regarded as one TOT and separated with another TOT by other signals like DL signals or invalid symbols etc; while (b) is that the UL resource in one slot is regarded as one TOT and separated with another TOT by slot boundary as well as other signals mentioned above.  So the difference is that the multi-TOT (b) considers the impact of slot boundary, which is the closest design to what we have for current NR system.

Observation 1: TOT did not span over multiple slot that is suitable for current NR design.

Figure 3

Another issues mentioned in the options are the TB transmission over TOT(s) with single RV or different RV, which is actually related to the RM operation to using single RV for all TOTs or different RV for different TOTs.  As we discussed, for TB over multi-slot, by normal extension from Rel-16 PUSCH, there is a possibility that the coded bits corresponding to one CB would be mapped across the resources of more than one slot. There could be two options for the rate matching operation, namely continuous rate matching (C-RM) and segmented rate matching (S-RM). For the C-RM, the starting positioning of output bit sequence for a given slot is from the end of output bit sequence for previous slot. While for the S-RM, the starting position is based on the RV configured for PUSCH in this slot, like in current operation.

Figure 4

As illustrated in Figure 3, a mother code of 1/3 is assumed and in two cases of relatively low coding rate and high coding rate, we could see if with enough number of RV, and the output bits for a slot is larger than the distance of two RV, the coded bit length could be fully covered. Thus the decoding performance should be similar. However, if the above condition is not fulfilled, the decoding performance could be degraded for S-RM. Whether the RV number is enough, or the coded length is enough, it could be highly related to design of TBoMS, e.g., number of slot for a TBoMS etc. General speaking, in the case of CovEnh situation, in which the coding rate is relatively small, it’s likely that the S-RM should have similar performance to C-RM.

Observation 2: In case of CovEnh, the performance difference between C-RM (single RV based) and S-RM (multi-RV based) may be not significant.

Thus, consider the impact to the current NR implementation and benefits we can get from the TBoMS, the slot based TOT (i.e., multi-TOT) with different RV are slightly preferred.

Proposal 7: Option 4 is slightly preferred for the definition of a single TBoMS.

Besides, the RM operation provides the set of coded bits as output for the interleaver. The interleaver operates on the RM output to provide a mapping for the coded bits onto the symbols to be transmitted in each slot. Two possible operations for interleaving, one is interleaving among all slots, or interleaving per slot. It is natural to see the interleaving per slot is more aligned with current NR operation and implementation.

Proposal 8: slot based interleaving is supported for TBoMS.

## PUCCH vs TBoMS PUSCH

In the case that a PUCCH is overlapped with PUSCH, e.g., HARQ-ACK, the UCI will be multiplexed in PUSCH by current NR behavior instead of parallel transmission of both signals. Thus, first discussion point is whether we allow PUSCH and PUCCH to be transmitted in case the overlapping happened.

To allow parallel transmission of PUCCH and PUSCH (which is allowed in LTE system), it matters to the power allocation at UE side. Based on current power priority rules, UE will usually prioritize the PUCCH transmission over PUSCH which results in a potential power reduction of PUSCH transmission. Considering the scenario of Coverage Enhancement, usually the UE will transmit with full power already, then PUSCH transmission will be power reduced almost for sure.  Moreover, further splitting the power for multiple channels may be not desirable as it may result that all the UL transmissions are lack of power and fail in the coverage requirement.

Proposal 9: Parallel transmission of PUCCH and TBoMS PUSCH is not preferred due to power splitting during CE situation.

Instead, UCI multiplexing on TBoMS PUSCH could be then a more suitable choice for coverage enhancement case. To do UCI multiplexing in TBoMS PUSCH, several aspects to be considered, e.g., the REs for UCI transmission for all slots, the actual RE for UCI transmission per slot, etc. Using PUSCH repetition type B as the example, the eventually UCI coded modulated symbol number is the comparison outcome among: target UCI RE number based on nominal repetition RE, available UCI RE number based on nominal repetition RE and the available RE number based on actual repetition RE. More specifically, the minimum one among these three RE numbers will be chosen. The different part for TBoMS PUSCH is, now the RE number for one TBoMS PUSCH transmission is the total number over multiple slots, instead of one single slot. One possible direction is that we still follow current calculation method, i.e., using nominal repetition REs, we need to tackle the codeblock size used in the calculation, which now is counted for all slot instead of single slot. Thus the calculated target UCI RE number might be not accurately reflects the situation, e.g., become too small if we still use total RE per nominal repetition but CB size for all slots. Another possible direction is to regard the TBoMS PUSCH as a complete part so that we calculate the target UCI REs based on the total available REs in all slots, and then multiplexing the UCI bits in the slots corresponding to the TBoMS PUSCH. Another aspect is about the timeline determination, whether there is an impact should be carefully studied.

Proposal 10: UCI multiplexing in TBoMS PUSCH is supported in Rel-17 CE, RAN1 further study the details.

# Conclusion

This paper discusses the mechanism to support TB over multi-slot. The following proposals are made:

Proposal 1: both PUSCH repetition type A and type B like TDRA are supported.

Proposal 2: Consider following two options for time domain resource for a single TB in TBoMS:

- Option 1: Indicating number of slot for one TB based on Type A and/or Type B PUSCH

  - Number of occupied repetition/slots can be configured.

- Option 2: Directly indicating a number of symbol L that can be larger than 14.

  - A symbols group can be considered

- Other options are not precluded.

Proposal 3: Repetition is supported for TB over multi-slot.

Proposal 4: Further study the following method for time domain location of DMRS considering the joint channel estimation over multi-slot and transmissions:

- DMRS time domain location is determined per TOT

- DMRS time domain location is determined per slot

Proposal 5: The maximal number of PRB allocated in time domain is reduced for TB over multi-slot.

Proposal 6: NInfo for TBoMS is calculated based on all REs in all slots for the TB. NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated and can be configured by xOverhead as in Rel-15/16.

Observation 1: TOT did not span over multiple slot that is suitable for current NR design.

Observation 2: In case of CovEnh, the performance difference between C-RM (single RV based) and S-RM (multi-RV based) may be not significant.

Proposal 7: Option 4 is slightly preferred for the definition of a single TBoMS.

Proposal 8: slot based interleaving is supported for TBoMS.

Proposal 9: Parallel transmission of PUCCH and TBoMS PUSCH is not preferred due to power splitting during CE situation.

Proposal 10: UCI multiplexing in TBoMS PUSCH is supported in Rel-17 CE, RAN1 further study the details.

# References

[1] RP-210855  New WID on NR coverage enhancements