3GPP TSG RAN WG1 #105-e		                          R1-2105420

e-Meeting, May 10th –27th, 2021

Agenda Item:	8.2.3

Source: 	LG Electronics

Title: 	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

The following WID objectives related to PUCCH were made in RAN#90-e meeting [1]:

- Physical layer aspects including [RAN1]: - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

In this contribution, we discuss and provide views on the enhancements for PUCCH format 0/1/4 to support NR above 52.6 GHz, in terms of the resource allocation and the sequence design under the regulatory requirement.

- Discussions

- Frequency domain resource mapping for enhanced PF 0/1/4

Agreement: For enhanced (multi-RB) PUCCH Formats 0/1/4 for 120/480/960 kHz SCS, support allocation of N_RB contiguous RBs - FFS: Values of N_RB for each SCS - For 480/960 kHz SCS, all REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - For 120 kHz SCS, further discuss the following two alternatives: - Alt-1: All REs within each RB are mapped - Note: PRB and sub-PRB interlaced mapping is not considered further - Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping) Agreement: User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

In RAN1#104-e meeting [2], it was agreed to support allocation of N_RB contiguous RBs for enhanced PUCCH format 0/1/4 for 120/480/960 kHz SCS to achieve higher transmit power. For 120 kHz SCS, two different RB mapping methods are captured. The reason for using the PRB-based interlaced structure in NR-U was to boost transmission power while complying with the OCB requirement. However, in 60GHz band, the gain is limited even if the sub-PRB interlaced mapping is used because of the large SCS, and the OCB requirement that is different from the NR-U. Moreover, it was agreed in RAN1#104b-e meeting [3] that user-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion. Therefore, the PRB and sub-PRB interlaced mapping should not be considered further for 120 kHz SCS.

Proposal #1: For enhanced PUCCH formats 0/1/4 for 120 kHz SCS, the PRB and sub-PRB interlaced mapping should not be considered further.

- The number of RBs for enhanced PF 0/1/4

Agreement: - The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least: - 12 RBs for 120 kHz SCS - 3 RBs for 480 kHz SCS - 2 RBs for 960 kHz SCS - FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions Agreement: Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4: - Alt-1: - For enhanced PF0/1 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS - For enhanced PF4 - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers. - Alt-2: - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

For the allowed values of NRB within the [min/max] range, the positive integer values between the min/max NRB can be used for PF0/1 while the allowed values of NRB for PF4 can be obtained by applying the DFT constraint (i.e., NRB = 2a×3b×5c where a, b, c are positive integer value including 0). Since the regulatory requirements may differ from the region and gNB may need to adjust the number of PRB according to network configuration, the values of NRB after the RRC connection can be configured based on the allowed values of NRB defined in the specification for each PUCCH format/resource by the gNB (UE-dedicated RRC signalling).

Proposal #2: For the allowed values of NRB, the positive integer values between the min/max NRB can be used for PF0/1 while the allowed values of NRB between the min/max NRB for PF4 can be obtained by applying the DFT constraint.

Proposal #3: The values of NRB after the RRC connection can be configured based on the allowed values of NRB defined in the specification for each PUCCH format/resource by the gNB (UE-dedicated RRC signalling).

- Sequence type for PUCCH format 0/1

Agreement: - For enhanced PF0/1, support Type-1 low PAPR sequences. Further study and strive to select one of the following alternatives: - Alt-1: A single sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts for PF0/1 are defined in the same way as Rel-16 for the case that useInterlacePUCCH-PUSCH is not configured. - Alt-2: A single sequence of length equal to the number of mapped REs per RB of the PUCCH resource is used, and the sequence is repeated in each RB. At least the following scheme is considered for PAPR/CM reduction: - Cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured - At least the following aspects should be considered in the study - Coverage (maximum isotropic loss (MIL)), including - Required SNR to fulfil PUCCH detection criterion - PAPR/CM as a function of N_RB - Specification impact

In Rel-16 NR-U, enhanced PUCCH format 0/1 is configured such that CS offset (i.e., the step size ∆ = 5) is applied for each PRB in a single interlace in addition to the starting CS value in NR in order to improve PAPR/CM performance. However, since the contiguous mapping is preferred (rather than interlaced mapping) in above 52.6 GHz, the sequence repetition other than L=12 CGS with different CS offset value can be also considered when the PUCCH sequence repetition based mapping is used for multi-PRB based PUCCH format 0/1.

In RAN1#104b-e meeting [3], there were discussions and observations on the performance for Alt-1 and Alt-2 in terms of the PAPR/CM and the MIL. Considering the coverage of PUCCH format 0/1, the multi-PRB (i.e., NRB larger than 1) based PUCCH should be also supported for the initial PUCCH resource. In addition, at least for 120 kHz SCS, the PAPR/CM performance of Alt-2 for the specific RB range (e.g., NRB around 12-16) is better than that of Alt-1. In this regard, at least Alt-2 should be supported. For PAPR/CM reduction schemes, the step size ∆ = 5 can be considered for the cycling of cyclic shifts across RBs in a similar way as for Rel-16 for PF0/1 for the case that useInterlacePUCCH-PUSCH is configured.

Proposal #4: Considering better MIL performance and improved coverage of multi-PRB based initial PUCCH for the specific RB range (e.g., NRB around 12-16), support Alt-2 (a single sequence of length equal to the number of mapped REs per RB with the step size ∆ = 5 for the cycling of cyclic shifts across RBs) for the sequence type for enhanced PUCCH format 0/1 in 60 GHz.

- Considerations for initial PUCCH resource set

(TS 38.213) Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

In NR, each initial PUCCH resource set consists of 16 PUCCH resources, and the number of PRBs required to create 16 resources was determined according to the number of initial CS indexes. For example, for PUCCH resource set indexes of 0, 3, 7, and 11, 8 PRBs (i.e., 8 FDM resources) are required to provide 16 PUCCH resources due to a single RB allocation design for PUCCH formats 0 and 1.

Considering that the coverage of initial PUCCH and the multi-RB PF 0/1 introduced to increase PUCCH transmission power, a number of RBs greater than 1 should be supported even for the initial PUCCH resource and the PRB offset value also needs to be scaled by NRB. The value of NRB for initial PUCCH resource can be determined considering the available number of RB in the initial bandwidth part and the required number of FDM resources for each PUCCH resource set. As mentioned in Section 2.2, the maximum values of NRB for PF0/1 for each SCS can be predefined in the specification and these values can be used for the initial PUCCH resource directly. Alternatively, the value of NRB can be configured by RRC signalling (e.g., SIB1). In all alternative methods, the potential shortage of PUCCH resources for the initial PUCCH resource set resulting from using multi-PRB to transmit PF 0/1 should be addressed.

Proposal #5: A number of RBs greater than 1 should be supported even for the initial PUCCH resource and the PRB offset value also needs to be scaled by NRB.

Proposal #6: To determine the value of NRB for the initial PUCCH resource, the following options can be considered:

- Opt.1: Directly use the predefined maximum value of NRB for PF 0/1 in the specification.

- Opt.2: Use the value of NRB configured through RRC signalling (e.g., SIB1) by gNB.

- Opt.3: Calculate the value of NRB based on the size of the initial BWP and the required number of FDM resources for each PUCCH resource set.

In RAN1#104b-e meeting [3], it was agreed that the maximum values for the configured number of RBs for enhanced PF0/1/4 are at least 12 RBs for 120 kHz SCS. If the maximum value of NRB for 120 kHz (i.e., NRB=12) is directly applied to PUCCH formats 0 and 1 in 60 GHz band, the available number of PRBs in the initial bandwidth part may not suffice to provide 16 PUCCH resources per each PUCCH resource set. For example, the total available number of PRBs can be 132 when the channel bandwidth of the initial bandwidth part is 200 MHz for 120 kHz subcarrier spacing. However, when NRB=12 is used to transmit a single PUCCH resource, the minimum required number of PRBs to provide 16 PUCCH resources is 192, which exceeds the number of PRBs available in initial bandwidth part.

One way to solve this problem is to use only PUCCH resources that do not overlap in the frequency domain, starting with the PUCCH resource allocated to the lowest frequency RB. For example, 11 PUCCH resources (i.e., 0 <  < 10) can be expected to be configured as valid resources if NRB=12 is used to transmit a single PUCCH resource when 132 PRBs are available in the initial bandwidth part with 200 MHz in 120 kHz subcarrier spacing.

Another way is that similar to NR-U, the additional CDMed or TDMed PUCCH resources can be provided until 16 PUCCH resources can be available. For example, a method of applying additional starting symbol index or OCC index based on the value of (pre-defined or configured) NRB can be considered.

There are other consideration points in addition to the potential shortage of PUCCH resources for the initial PUCCH resource set. Considering the available number of RBs in the initial BWP and the multi-RB is used to transmit the initial PUCCH resource, the diversity gain of frequency hopping may not be enough. Therefore, it is necessary to discuss how to configure the hopping distance to obtain hopping gain equally for each PUCCH resource.

Proposal #7: To address the potential shortage of PUCCH resources for the initial PUCCH resource set resulting from using multi-PRB to transmit PUCCH formats 0 and 1, consider the following alternatives:

- Alt. 1: Use only valid resources in the frequency domain

- Alt. 2: Support additional starting symbol and OCC index

Proposal #8: Considering the available number of RBs in the initial BWP and more than 1 RB allocated for an initial PUCCH resource, discuss how to configure the hopping distance to obtain hopping gain equally for each initial PUCCH resource.

- Conclusions

In this contribution, the enhancements for PUCCH format 0/1/4 to support NR above 52.6 GHz were discussed, and the followings are proposed.

Proposal #1: For enhanced PUCCH formats 0/1/4 for 120 kHz SCS, the PRB and sub-PRB interlaced mapping should not be considered further.

Proposal #2: For the allowed values of NRB, the positive integer values between the min/max NRB can be used for PF0/1 while the allowed values of NRB between the min/max NRB for PF4 can be obtained by applying the DFT constraint.

Proposal #3: The values of NRB after the RRC connection can be configured based on the allowed values of NRB defined in the specification for each PUCCH format/resource by the gNB (UE-dedicated RRC signalling).

Proposal #4: Considering better MIL performance and improved coverage of multi-PRB based initial PUCCH for the specific RB range (e.g., NRB around 12-16), support Alt-2 (a single sequence of length equal to the number of mapped REs per RB with the step size ∆ = 5 for the cycling of cyclic shifts across RBs) for the sequence type for enhanced PUCCH format 0/1 in 60 GHz.

Proposal #5: A number of RBs greater than 1 should be supported even for the initial PUCCH resource and the PRB offset value also needs to be scaled by NRB.

Proposal #6: To determine the value of NRB for the initial PUCCH resource, the following options can be considered:

- Opt.1: Directly use the predefined maximum value of NRB for PF 0/1 in the specification.

- Opt.2: Use the value of NRB configured through RRC signalling (e.g., SIB1) by gNB.

- Opt.3: Calculate the value of NRB based on the size of the initial BWP and the required number of FDM resources for each PUCCH resource set.

Proposal #7: To address the potential shortage of PUCCH resources for the initial PUCCH resource set resulting from using multi-PRB to transmit PUCCH formats 0 and 1, consider the following alternatives:

- Alt. 1: Use only valid resources in the frequency domain

- Alt. 2: Support additional starting symbol and OCC index

Proposal #8: Considering the available number of RBs in the initial BWP and more than 1 RB allocated for an initial PUCCH resource, discuss how to configure the hopping distance to obtain hopping gain equally for each initial PUCCH resource.

- References

RP-202925, “Revised WID: Extending current NR operation to 71 GHz”, RAN#90-e, CMCC.

RAN1#104-e Chairman’s note

RAN1#104b-e Chairman’s note