3GPP TSG RAN WG1 #105-e                               R1-2105427

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.3.2

Source: 	LG Electronics

Title: 	Discussion on unlicensed band URLLC/IIOT

Document for:	Discussion and decision

- Introduction

In RAN1#104bis-e, following agreements were made for unlicensed band URLLC, in terms of UE-initiated COT based FBE operation and URLLC/NR-U CG harmonization [1].

Agreement: - Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode. Agreement: - For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. Agreement: - For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell: - Do not support any additional period value Agreement: - For semi-static channel access mode, the starting point of first UE FFP for a serving cell - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement). Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type B when using based on NR-U Rel-16 based CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance Agreement: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreement: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. Agreement: - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

In this contribution, based on the above agreements, we discuss and provide our views on some of the issues decided to be focused in this meeting.

- Support of UE-initiated COT for FBE based URLLC

- Configured UL transmission based on UE/gNB-initiated COT

On the UE behaviour/assumption for a configured UL transmission aligned with a UE FFP boundary, following two alternatives are currently on the table.

- Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Figure 2. An example of FFP overlapping between gNB and UE.

Figure 3. Another example of FFP overlapping between gNB and UE.

Figures 1 and 2 show some examples of FFP overlapping between gNB and UE where some configured UL (i.e. C-UL) resource are configured at the UE FFP boundary or within UE FFP. Firstly in Figure 1, if the transmission of C-UL #1 is assumed as UE-initiated COT, then gNB is not allowed to transmit DL signal/channel in the idle period of FFP-u #1, but if the C-UL #1 transmission is assumed as shared-gNB COT, then the gNB can transmit DL signal/channel in the idle period of FFP-u #1. Secondly in Figure 2, if the transmission of C-UL #1 is assumed as UE-initiated COT, then UE can also transmit C-UL #3 within FFP-u #2, but if the C-UL #1 transmission is assumed as shared-gNB COT, then the UE is not allowed to transmit C-UL #3 in outside of FFP-g #1.

With the above observation, considering the case where gNB initiates COT but UE fails to detect the DL signal corresponding to the gNB-initiated COT, Alt-a might be able to cause unexpected transmission from/to UE/gNB, and then only focusing the aligned/reliable interaction between UE and gNB, Alt-b might be better. However, in case of Alt-b, LBT behaviour would be enforced to the UE for every configured UL transmission aligned with a UE FFP boundary, and COT management in gNB side during FFP-g period might be less flexible due to ensuring LBT gap before the configured UL.

During the previous RAN1#104bis-e meeting, there were some arguments to compare Alt-a and Alt-b in terms of UE complexity/power consumption, LBT requirement, and gNB operation flexibility.

Regarding UE complexity/power consumption, it should not be considered as an issue with Alt-a since similar behaviour in terms of UE detection on gNB’s COT was already agreed to apply for the CG not aligned with UE FFP boundary. Another reason is that considering the case where UE FFP parameters are not configured yet for UE, the UE would, anyhow, need to perform Rel-16 FBE behaviour only based on sharing of gNB’s COT for which the UE is required to detect the gNB’s COT in advance for transmission of a CG. Besides, the power saving gain obtainable with Alt-b only from the CG aligned with UE FFP boundary is not seen considerable to pursue at the sacrifice of benefits from no LBT based UE transmission, gNB flexibility in COT operation, and so on.

Regarding LBT requirement, if gNB would like to allow for UE to transmit the CG based on Alt-b, the gNB would be required to always reserve LBT gap for the UE before the CG aligned with UE FFP boundary (by stopping gNB’s COT transmission). On the other hand, in case with Alt-a, the gNB could continue its COT until right before the CG without gap so that the UE could transmit the CG without separate LBT (and by reducing overhead for LBT gap reservation), while a separate LBT is required by the UE for the CG (with reservation of LBT gap) in case with Alt-b. Note that in the former case, successful transmission of the CG could be guaranteed by avoiding the risk of LBT failure, while the CG transmission would not be possible if LBT is not passed in the latter case.

Regarding gNB operation flexibility, the gNB’s behaviour/management during its COT would be different from Rel-16 FBE with Alt-b since the gNB is required to always ensure/reserve X-usec gap before the CG aligned with UE FFP boundary, even though the X-usec entirely belongs to its own COT duration. Moreover, whether ensuring/reserving X-usec gap is required or not would also be different between the CG aligned with UE FFP boundary and the CG not aligned with UE FFP boundary, even though those two CGs are all within the gNB’s COT duration. Related to this aspect of gNB operation flexibility, there was some argument that subsequent UL transmission after the CG aligned with UE FFP boundary could be possible with Alt-b under particular configuration where a large value is configured as UE FFP offset (relative to gNB FFP). However, such configuration should be careful and would be involved with risk due to potential UE-to-gNB interference on gNB’s COT initiation for gNB FFP. Therefore, rather than facing such dangerous situation, it would be better for gNB to configure more proper UE FFP offset, for example, to smaller value so that subsequent UL would not have overlapping with the idle period.

For the above reasons, Alt-a is preferred as long as there would be no critical problem potentially caused by the above misalignment between UE and gNB.

Proposal #1: Alt-a is preferred for the configured UL transmission aligned with UE FFP boundary, with consideration of LBT burden at UE and COT management at gNB.

- Scheduled UL transmission based on UE/gNB-initiated COT

On the UE behaviour/assumption for a scheduled UL transmission (aligned or not aligned with a UE FFP boundary), following two alternatives are currently on the table.

- Alt-a: Determination based on the content in the scheduling DCI.

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

- Alt-b: Determination based on the rules applied for a configured UL transmission.

Regarding this issue, it seems that the decision between Alt-a and Alt-b requires consideration on the case of multi-carrier transmission as well as the case of cross-FFP scheduling.

On the case of multi-carrier transmission, for example with Alt-a, in case when a CG PUSCH is configured in CC #1 and a DG PUSCH is scheduled in CC #2 at the same timing, there could be the case where the COT initiator indicated by DCI for the DG PUSCH is different from the COT initiator assumed by UE for the CG PUSCH (e.g. gNB-initiated COT is indicated by DCI for the DG, but UE-initiated COT is assumed for the CG), and thus the relevant UE behaviour would need to be discussed/determined.

On the case of cross-FFP scheduling, in case with Alt-a whether/how the gNB ensures the COT initiator indicated by DCI for the DG PUSCH scheduled in the next FFP need to be discussed/decided, according to whether the DG PUSCH is aligned with FFP boundary or not and/or whether the DG PUSCH is scheduled by a DCI format without the field indicating the COT initiator (if the DCI format exists).

For the above reasons, in order to pursue a unified/consistent UE behaviour for any type of UL transmission and to avoid complicated design for various cases such as multi-carrier transmission and cross-FFP scheduling, Alt-b is preferred. Note that with Alt-b, all the FFS points in the above as well as the FFS points under the agreement on cross-FFP scheduling could be resolved directly without consuming further argument and discussion time.

Proposal #2: Alt-b is preferred for a scheduled UL transmission, for unified/consistent UE behaviour on any UL transmission including multi-carrier transmission and cross-FFP scheduling.

- Harmonization of CG features in Rel-16 NR-U and URLLC

On the harmonization of Rel-16 NR-U CG feature and Rel-16 URLLC CG feature, following two options are currently on the table.

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

Regarding this issue, it seems there is no essential reason to decouple three features of CG-UCI based procedure, CG-DFI based procedure, and CG-retransmission timer except for some UCI/DCI overhead saving, but it is not the purpose of this CG harmonization. Moreover, decoupling those features would cause UE complexity and specification impacts (and require some handling) to implement various combinations of CG-UCI, CG-DFI, and CG-timer.

For example, if CG-retransmission timer is configured by disabling CG-UCI based procedure, there might be ambiguity between UE and gNB (due to NDI absence) on whether a CG PUSCH is a new transmission or a retransmission for the same HARQ process ID. For another example, if CG-UCI based procedure is enabled without configuring CG-retransmission timer and CG-DFI based procedure, it might cause unnecessary overhead even under controlled environment.

For the above reasons, focusing on main purpose/use case rather than just introducing any workable variants, Option 1 is preferred with consideration of UE complexity and specification impacts.

Proposal #3: Option 1 is preferred for the harmonization of NR-U CG and URLLC CG, with consideration of UE complexity and specification impacts.

- Conclusions

In this contribution, UL enhancements for URLLC in unlicensed controlled environments were discussed, and the followings are proposed.

Proposal #1: Alt-a is preferred for the configured UL transmission aligned with UE FFP boundary, with consideration of LBT burden at UE and COT management at gNB.

Proposal #2: Alt-b is preferred for a scheduled UL transmission, for unified/consistent UE behaviour on any UL transmission including multi-carrier transmission and cross-FFP scheduling.

Proposal #3: Option 1 is preferred for the harmonization of NR-U CG and URLLC CG, with consideration of UE complexity and specification impacts.

- References

RAN1 chairman’s notes, RAN1#104bis-e