3GPP TSG RAN WG1 #105		R1-2105457

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Discussion on CSI-RS Resource counting for UE feature 2-33

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

There is ambiguity regarding whether CSI-RS resources used for L1-RSRP are counted as active resources defined in 38.214 as following. This may cause mis-aligned understanding for the interpretation when UE reports UE feature 2-33.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the PUSCH containing the report associated with this aperiodic CSI-RS. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signaling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

In this paper, we try to further clarify the understanding.

- Discussion

For UE feature 2-33 define the RS counting limitations as following:

| 2-33 CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC |
| --- | --- |

Regarding the “max # simultaneous NZP-CSI-RS resources in active BWPs”, the following agreement was achieved:

Agreement

- …….Previous parts omitted………

- Simultaneous resources in FG 2-36, 2-40, 2-41, 2-43, 2-33

  - In any slot, the number of “active” CSI-RS resources/ports should not exceed the numbers reported in FG2-36, 2-40, 2-41, 2-43, 2-33

  - An NZP CSI-RS resource is active in a duration of time defined as follows:

    - For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS.

    - For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied.

    - For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released.

In Rel-15 discussion, the background of this agreement is for CSI feedback other than L1-RSRP. But when captured in specification, such background information is not reflected.

Also, during previous discussion in RAN1 #101e, the majorities’ understanding is that this only covers the CSI-RS associated with reporting settings configured with 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI'.

Yet, there is another understanding that this covers all the CSI-RS resources. Without common understanding on this issue, there would be under-reporting or degraded performance issues if the corresponding configured CSI-RS is over the actual processing capability UE can handle.

Based on above discussion, we have the following proposal.

Proposal 1: Clarify that for UE capability csi-RS-IM-ReceptionForFeedback and csi-RS-IM-ReceptionForFeedbackPerBandComb  reporting, only the following CSI-RS are considered when counting active resources defined in 38.214:

  - CSI-RS associated with reporting settings configured with reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI'

- Conclusion

In this contribution, the following is proposed to clarify counting of CSI-RS for UE feature 2-33.

Proposal 1: Clarify that for UE capability csi-RS-IM-ReceptionForFeedback and csi-RS-IM-ReceptionForFeedbackPerBandComb  reporting, only the following CSI-RS are considered when counting active resources defined in 38.214:

  - CSI-RS associated with reporting settings configured with reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI'

- References