3GPP TSG RAN WG1 #105-e	                                              R1-2105458

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Discussion on PUSCH skipping with overlapping UCI on PUCCH in Rel-16

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #104b-e, PUSCH skipping with overlapping UCI on PUCCH for both DG and CG was discussed in RAN1, including the PUSCH skipping with repetitions. However, there is no conclusion yet. In this contribution, we will discuss the possible solution for the case of PUSCH skipping with PUSCH repetitions, when there is no intra-UE prioritization in Rel-16. The discussions on UE behavior for PUSCH skipping with intra-UE prioritization in Rel-16 can be found in our companion contribution [1].

- Discussions on PUSCH skipping with repetitions

  - DG PUSCH skipping with PUSCH repetitions

RAN1 has discussed intensively for UE behavior for PUSCH skipping with overlapping with UCI in case of PUSCH with repetitions during RAN1#104-e. However, we have not achieved consensus for this issue yet.

During the discussions, following options were identified to address this issue.

- Option 1: When there’s a UCI to be multiplexed on any of the repetitions of the DG PUSCH, MAC generates MAC PDU for the DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI can be multiplexed on the DG PUSCH. - MAC generate MAC PDU for all DG PUSCH repetitions - Note: the UCI multiplexing timeline condition for the first repetition of DG PUSCH should be ensured - Option 2: - When there’s UCI overlapping with the first PUSCH repetition of the DG PUSCH, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. - UE does not expect when a UCI is overlapping with the repetitions other than the first PUSCH repetition. - Option 3: When a PUCCH is overlapped with the first PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - When a PUCCH is overlapped with the repetitions other than the first PUSCH repetition, if there is no PDU including data delivered from MAC, the DG PUSCH can be skipped. UCI is transmitted on the PUCCH. - Option 3’: When a PUCCH is overlapped with the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - When a PUCCH is overlapped with the repetitions other than the first X PUSCH repetition, if there is no PDU including data delivered from MAC, the DG PUSCH can be skipped. UCI is transmitted on the PUCCH. - The value of X can be 1 or 2 - Option 4: Rel-16 PUSCH skipping and PUSCH repetitions are not allowed to be enabled together (error case is defined). - Option 5: When PUSCH repetition is configured, - if a PUSCH repetition overlaps with PUCCH, MAC generates PDU for the repetition, - otherwise, MAC does not generate PDU for the repetition if there is no data for the DG PUSCH. - Note: it requires the MAC layer can decide whether to generate a MAC PDU for the repetition depending on whether it overlaps with PUCCH, which is different from current MAC behaviour. - Option 6: When PUSCH repetition is configured, - MAC layer behavior: For a PUSCH repetition, MAC always generate a PDU. If MAC has data in buffer, generate a real PDU; otherwise, generate a dummy PDU. And MAC use 1-bit to tell the PHY the PDU is a dummy PDU or real PDU. The 1-bit can be UE internal implementation between MAC and PHY, no need to specify it. - PHY layer behavior: Each PUSCH repetition independently check it overlap with a PUCCH or not. - If it overlaps with a PUCCH, that PUSCH repetition cannot be skipped. - If it does not overlap with any PUCCH, - if the MAC PDU is a dummy PDU, PHY can skip this PUSCH repetition - If the MAC PDU is a real PDU, PHY cannot skip this PUSCH repetition. - Option 7: When a PUCCH is overlapped with any of the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - UE does not expect a UCI is overlapping with the repetitions other than the first X PUSCH repetition - The value of X is 2

Among the above options, there are both pros and cons for each option. It may be difficult to find out a perfect solution. In principle, the goal is to figure out a solution that lead to a deterministic behavior in terms of PUSCH transmissions from the UE and avoid any additional blind detection efforts at the gNB. Since these options have been discussed intensively, it may be better to consider a compromised solution to address the PUSCH skipping with repetitions issue.

Firstly, it should be pointed out that, in Rel-15, the time restriction of DCI triggering UCI overlapping with PUSCH is specified, i.e. the DCI triggering the UCI should be before the DCI scheduling the PUSCH overlapping with the UCI. From our perspective, for all the above options, such restriction should be followed. Therefore, the DCI triggering UCI will come “early enough” in case of PUSCH repetitions, which will ensure sufficient time for UCI multiplexing on any of the PUSCH repetitions. We are open to further discuss whether additional UE processing time budget is needed.

38.213 9 UE procedure for reporting control information A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

Figure 1: Time condition for DG PUSCH and the overlapping PUCCH

By this time restriction of gNB scheduling, the DCI triggering UCI will not come “too late”. However, it will cause delay of PUCCH. Whether and how to address the issue of time restriction of gNB scheduling for PUSCH repetitions seems a general issue for the case with and without UL skipping. Note that there are already some discussions in Rel-17 TEI trying to address this issue (e.g. R1-2103745).

So we suggest to try to solve the issue of UL skipping with repetition under the time restriction defined in Rel-15. For the issue of gNB scheduling restriction due to UCI multiplexing on DG-PUSCH with large number of repetitions, we can further discussion how to address this issue in a separate discussion.

For the above options, we analyze the main concerns for each option. Then we can try to find a direction way forward.

- For all the options except option 4, the UE behavior is the same when PUCCH is overlapped with the first PUSCH repetition, i.e. MAC generates MAC PDU for PUSCH and the UCI is multiplexed on the PUSCH

- When PUCCH is overlapped with the repetitions other than the first PUSCH repetition, different behaviours are observed for different options.

  - Option 1 may require (potentially) increased UE implementation complexity.

  - Option 3 may lead to gNB blind detection issue for UCI reception and require gNB reliable detection on the first repetition, which may be difficult when number of repetitions is large. Besides, UE needs to implement two different behavior for PUSCH with and without repetitions cases.

  - Option 2 and option 4 may put too much limitation on gNB scheduling.

  - Option 5 and option 6 require MAC can generate PDU for any repetition, which introduce non-trivial specification impact.

  - Option 3’ is a compromised solution based on option 3. However, the gNB BD issue still exists.

  - Option 7 is a good direction for compromise. In fact, it is doubtful that UL skipping and large number of repetitions are configured to be enabled simultaneously. To avoid too much increased UE implementation complexity, gNB can configure small number of repetitions when UL skipping is enabled.

So we propose an alternative solution. There is no blind detection issue for gNB and the increased UE implementation complexity could be limited. Besides, the restriction on the scheduling flexibility for gNB can be also relaxed.

Proposal 1: For DG PUSCH in Rel.16, for non-CA and CA cases, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is applied,

- When a PUCCH is overlapped with any of the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped.

  - UE does not expect to be configured with K > X when UL skipping is configured.

    - X can be 2 or 4.

  - CG PUSCH skipping with PUSCH repetitions

For CG PUSCH with repetitions, MAC determines whether to generate a PDU for the PUSCH at any transmission occasion that may be used for initial transmission of a TB. If a configured grant configuration is configured with startingFromRV0-r16 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, initial transmission of a transport block may start at any of the transmission occasions of the K repetitions that are associated with RV=0, except the last transmission occasion when RV sequence is {0,0,0,0} and K≥8.

Observation 1: For CG PUSCH with repetitions, MAC determines whether to generate a PDU for the PUSCH at any transmission occasion that may be used for initial transmission of a TB.

Similar to DG PUSCH skipping with repetition, the unified solution can be adopted for CG PUSCH skipping with repetitions. I.e. when a PUCCH is overlapped with any of the first X PUSCH repetition of CG transmission, MAC generates MAC PDU for CG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the CG PUSCH. All of the PUSCH repetitions are not skipped. UE does not expect to be configured with number of repetitions K > X for CG when UL skipping is configured. X can be 2 or 4.

Proposal 2: For CG PUSCH skipping with repetitions in Rel.16, same solution as DG PUSCH skipping with repetitions is applied.

for CG PUSCH, there is no time restriction of gNB scheduling as DG PUSCH case in current spec. So for CG with repetitions, a timeline would be needed to ensure the DCI triggering UCI comes “early enough” to trigger PDU generation for PUSCH for UCI multiplexing.

Follow timeline can be defined for the CG PUSCH with repetitions when there’s UCI overlapping with any of the CG PUSCH repetitions.

- When UE is scheduled a PUCCH overlapping with a CG PUSCH transmission occasion corresponding to any transmission opportunity, the DCI scheduling the PUCCH needs to be received before [Tproc,2mux] before the CG PUSCH transmission occasion corresponding to an initial transmission opportunity

Figure 2: Time condition for CG PUSCH and the overlapping PUCCH

With such solution, it can be ensured that there is sufficient time for UE when determining UCI multiplexing and generating MAC PDU for the CG PUSCH with repetitions, in case when there’s UCI overlapping with the non-initial transmission occasions of the repetitions of the CG PUSCH.

Proposal 3: When CG PUSCH skipping is configured and when PUSCH repetition is applied, UE is expected that a DCI scheduling a PUCCH is received before [Tproc,2mux] before the first symbol of the latest CG PUSCH transmission occasion corresponding to an initial transmission opportunity, when the PUCCH is overlapping with a CG PUSCH transmission occasion corresponding to non-initial transmission opportunity.

- Conclusion

In this contribution, we provide our views on the remaining issue and solution for PUSCH skipping. The observations and proposals are summarized below.

Proposal 1: For DG PUSCH in Rel.16, for non-CA and CA cases, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is applied,

- When a PUCCH is overlapped with any of the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped.

  - UE does not expect to be configured with K > X when UL skipping is configured.

    - X can be 2 or 4.

Proposal 2: For CG PUSCH skipping with repetitions in Rel.16, same solution as DG PUSCH skipping with repetitions is applied.

Proposal 3: When CG PUSCH skipping is configured and when PUSCH repetition is applied, UE is expected that a DCI scheduling a PUCCH is received before [Tproc,2mux] before the first symbol of the latest CG PUSCH transmission occasion corresponding to an initial transmission opportunity, when the PUCCH is overlapping with a CG PUSCH transmission occasion corresponding to non-initial transmission opportunity.

- References

- R1-2105467, “Maintenance on eCG enhancement and intra-UE prioritization”, vivo, RAN1 #105-e.