3GPP TSG-RAN WG1 Meeting # 105-e	R1-2105468

# e-Meeting,  May 10th - May 27th , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft] CR on the number of SRS resource set configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-15, only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to 'codebook' for codebook based transmission and only one SRS resource set to 'nonCodebook' for non-codebook based transmission. In Rel-16, two SRS resource sets can be configured, with each one of the sets used for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 respectively. However, the text in spec current version of spec seems to restrict only one SRS resource set for both usage ‘codebook’ and ‘non-codebook’, which is ambiguous. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 6.1.1.1, it is clarified that one SRS resource set can be configured for codebook based PUSCH transmission scheduled by DCI format 0-1 or DCI format 0-2, respectively. - In section 6.1.1.2, it is clarified that only one SRS resource set configured for non-codebook based PUSCH transmission scheduled by DCI format 0-1 or DCI format 0-2, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description in the spec is incorrect on the number of SRS resource set(s) configured for cobebook and non-codebook based transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Chapter 6.1.1.1 and 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.1.1	Codebook based UL transmission

For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2  or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubsetDCI-0-2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRank-ForDCIFormat0_2 for PUSCH scheduled with DCI format 0_2.

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent'.

A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'.

A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2 set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two.

For codebook based transmission, only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubsetDCI-0-2 set to 'fullAndPartialAndNonCoherent' simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'.

6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder and transmission rank based on the SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource(s) identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI.

For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured.

-	If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols.

-	If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'.

-	If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet.

The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) and their corresponding PUSCH layers {0 … ν-1} given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3 in increasing order.

The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as .

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS 38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-ResourceSet for SRS resource set.

For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to 'nonCodebook'.