3GPP TSG RAN WG1 #105-e	R1-2105495

e-meeting, May 10th –27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Initial access aspects for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.1

Document for:	Discussion

# 1	Introduction

In RAN#91-e [1] revised WI has been approved with the following objectives related to initial access for NR operation from 52.6 to 71 GHz:

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS (480kHz, 960kHz) for initial access related signals/channels in initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

Following agreements related to initial access were made in RAN1#104-bis-e [2]:

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst (DB) and discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - Definition of DB is the same as in Rel-16 37.213 Section 4.0

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DB/DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

In this contribution, we provide our view on the following aspects:

- Remaining aspects of numerology for SSB and PRACH

- Coverage enhancements during initial access

- SSB beam switching gaps

- DRS enhancements for LBT-based channel access

# 2	Discussion

## SSB and PRACH numerology

In NR Rel 15/16, for FR2, the SCS of SSB is always similar or higher than that used for CORESET0. In general, smaller SCS such as 120kHz and 240kHz for SSB could offer better coverage than higher SCS due to the longer symbols. However, if 480kHz or 960kHz SCS are used for UL data channel while small SCS are used for SSB, there could be timing accuracy issue for UL transmission in initial access due to the small SSB frequency allocation since the timing estimation error scales up as BW decreases which leads to tight timing error requirements. Therefore, to guarantee enough time synchronization accuracy for UL transmission during the initial access, supporting 480 kHz SCS, 960kHz SCS for SSB needs to be considered. It is beneficial to support matched numerology for both data channel and SSB for easy implementation and less numerology multiplexing complexity for both initial access and non-initial access cases. Furthermore, it would be also beneficial to support same high numerology for PRACH as well to allow for single numerology system operation.

Proposal 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, support the same numerologies of data channel for SSB including 480kHz and 960kHz for both initial access and non-initial access cases

Proposal 2: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, support both the numerologies of 480kHz and 960kHz for PRACH transmission

## 2.2 Enhancement to cell coverage during initial access

For beyond 52.6GHz band, the cell coverage especially during the initial access can be limited due to the severe attenuation at high frequencies and due to the use of low gain wide beams that depend on the SSB beams. UL initial access channels and signals such as PRACH and Msg3/MsgA for example in Rel-16 are expected to use the same Tx spatial filter as the Rx spatial filter used to receive the SSB beams at the UE. Generally, these beams are coarser than those used for control/data transmission in connected mode, and hence the coverage of these messages would be a bottleneck at high frequencies. Furthermore, if higher SCS values (in comparison to NR Rel-16) are agreed to be supported for initial access messages, then the cell radius would be further reduced as SCS increases due to the short symbols and CP. Therefore, the SCS needs to be carefully selected for PRACH in order to satisfy the coverage requirements for different scenarios. One potential solution to enhance the detection performance of PRACH, is the use of repetition or multi beam transmission for each RACH attempt. Adapting the SCS for each RACH attempt along with power ramping could also be considered to reduce the number of RACH attempts.

In Rel-16, PUSCH repetitions are already supported for enhancing UL coverage. However, it is used in connected mode and not applied for initial access message such as Msg3 or MsgA. To enhance the link budget and hence the cell coverage during the initial access for NR beyond 52.6GHz, repetition of Msg3/MsgA can also be considered. Furthermore, ways for early termination of Msg3/MsgA repetition in case of correct reception needs to be considered.

Proposal 3: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for initial access, coverage enhancement of channels and signals used for initial access should be considered for NR beyond 52.6 GHz.

## 2.3 Switching gaps between contiguous SSBs

For SSB beam switching/sweeping procedure with SCS up to 240kHz, no time gaps between the contiguous SSB candidates are required and the beam switching occurs during the CP of the next OFDM symbol. RAN4 (TS38.817-2) specified the required beam switching time considering the analogue phase shifters delay of about 100ns. So, for SCS of 240kHz, the CP is long enough to handle the beam switching delay. However, for beyond 52.6GHz bands if SSB used high SCS values similar to data channels, i.e. 960kHz, a beam switching issue would appear between the contiguous SSB transmissions since the CP length would not be enough for beam switching, and an extra gap needs to be added to prevent performance degradation.

Observation 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, beam switching issue would appear between the contiguous SSB beams since the CP length would not be enough for beam switching, and an extra gap might be needed to prevent performance degradation.

One possibility could be introducing a symbol gap to accommodate the beam switching delay. However, it might not be a spectral efficient solution as using the entire symbol gap might be unnecessary, depending upon the SCS value. Other possibility that can be considered is to adapt the length of a gap between two contiguous SSBs that are on two different beams. Another possibility is to add a post prefix at the end to the last OFDM symbol of the SSB and the last PDSCH-SIB symbol in case of pattern 2/3.

Proposal 4: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, then to allow the beam switching between contiguous SSBs, a gap (for example a symbol gap or post prefix) should be supported before beam switching.

## 2.4 SSB (DRS) enhancements for LBT-based access

Transmission of SSB and CORESET 0 can be configured to be beam swept in different spatial direction in a DRS window. This can be achieved by gNB by performing directional LBT prior to the transmission of beam swept DRS at different time instant within the DRS window. For example, each directional LBT is performed prior to the beam swept SSB/PBCH/CORESET 0 transmission based on the bitmap signaled in ssb-PositionsInBurst and DRS is transmitted when LBT is successful. Base on this approach, QCL-ed SSB beams may also be transmitted within a MCOT initiated by a directional LBT prior to the transmission of SSB. The relationship between the sensing beam and the DRS transmission beam is same as that of the PUSCH transmission.

Proposal 5: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, potential enhancements related to periodic transmission of DRS such as SSB/PBCH/CORESET#0 are needed including:

- performing directional LBT prior to the transmission of SSB according to the ssb-PositionsInBurst

- directional LBT on multiple beams at the same time at the beginning of the DRS window

- Cat 2 LBT (depending on the gap) before actual transmission

# 3	Conclusion

Here we summarize the observations and proposals from the above sections:

Observation 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, beam switching issue would appear between the contiguous SSB beams since the CP length would not be enough for beam switching, and an extra gap might be needed to prevent performance degradation

Proposal 1: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, support the same numerologies of data channel for SSB including 480kHz and 960kHz for both initial access and non-initial access cases

Proposal 2: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, support both the numerologies of 480kHz and 960kHz for PRACH transmission

Proposal 3: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for initial access, coverage enhancement of channels and signals used for initial access should be considered for NR beyond 52.6 GHz.

Proposal 4: For supporting NR from 52.6 GHz to 71 GHz in Rel. 17, if higher subcarrier spacings (numerologies) are adopted for SSB, then to allow the beam switching between contiguous SSBs, a gap (for example a symbol gap or post prefix) should be supported before beam switching.

Proposal 5: For NR operation in unlicensed bands between 52.6 GHz and 71 GHz, potential enhancements related to periodic transmission of DRS such as SSB/PBCH/CORESET#0 are needed including:

- performing directional LBT prior to the transmission of SSB according to the ssb-PositionsInBurst

- directional LBT on multiple beams at the same time at the beginning of the DRS window

- Cat 2 LBT (depending on the gap) before actual transmission

# 4	References

[1]	3GPP RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC

[2] 	3GPP RAN1#104-bis-e. “Chairman Notes