3GPP TSG RAN WG1 #105-e	                    R1-2105496

e-meeting, May 10th –27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.3

Document for:	Discussion

# 1	Introduction

Following agreements related to PUCCH formats for NR operation from 52.6 GHz to 71 GHz were made in RAN1#104-bis-e [1]:

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Agreement:

Down select to one of the following two alternatives for the configuration of the number of RBs, , for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max()] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max()] for each SCS that fulfil the requirement  where  is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of  can be configured

  - FFS: Which values of  are supported values in the range [1 .. max()]

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16.

Agreement:

For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e).

Proposal:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs:

The following additional set of values are to be considered in addition to the already agreed evaluation assumptions in Table 2 and 3 in Section 2.3 of R1-2102127:

- UE_EIRP = 40 dBm

- UE_P = 25 dBm

- TxBF = 0 dBi

Agreement:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

Agreement:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

In this contribution, we discuss and present our views on the following topics for PUCCH format 0/1/4 enhancements

- Frequency Domain Resource Mapping including the number of required RBs

- PUCCH Format 0/1 Sequence Type

- PUCCH Format 4 DFT Precoding options

- Link level simulation for PUCCH Format 0/1/4 and link budget calculation

# 2	Enhancements for PUCCH

## 2.1 Enhancements for PUCCH formats 0/1

In order to satisfy the PSD requirements, PUCCH formats 0/1 need to be mapped to multiple RBs depending on the configured SCS and the maximum PSD value for a certain region. One possibility to utilize increased number of RBs (more than one) is by simply applying frequency domain repetition of the generated PUCCH. Depending on the configured SCS and the PSD requirement for a certain region, number of RB is configured to repeat the single RB generated PUCCH resources in frequency domain. Another potential option is to use long base sequence for generating the PUCCH signal, i.e. multiple of the conventional length-12 sequence.  Since the orthogonal sequences require the number of REs of a PUCCH to be larger than or equal to 2k, where k denotes the number of information bits of the UCI, increasing the number  of REs for mapping one/two bits does not affect the orthogonality between the sequences. In other words, longer base sequence allows for multiple RB mapping of small UCI information without affecting phase rotation separations of the information bits for the UE, i.e. the UCI information of one bit ACK/NACK or SR can still be seperated with π phase rotation and two bits informations is seperated with π/2. For simultaneous ACK/NACK and SR, the phase rotation separation is π/4 for one bit ACK/NACK and π/6 for two bits. Using a longer base sequence would allow for multiplexing higher number of UEs using the same time-frequency resources and different cyclic shifts of the same base sequence. An alternative solution is to use multiple (same) base sequence of length N, where the number of base sequences is equal to the number of RBs configured for the PUCCH format, if the base sequence length is 12. If the base sequence length is longer for example 24, then one base sequence is mapped to 2 RBs and if 2 base sequences of length 24 are configured, then the total number of RBs is 4. Therefore, generally, total number of RBs can be a multiple of (length of base sequence * number of base sequences)/12. The information bit for a UE is transmitted depending upon the combination of phase rotation on each of the multiple base sequence on each of the multiple RBs. For example, if UE is configured to use 2 RBs for a PUCCH format, and if the PUCCH format is required to transmit only one bit of information i.e. ACK or NACK, then the ACK is indicated if the phase rotation is 0 on each base sequence mapped to each of the RBs and NACK is indicated if the phase rotation is 0 on the 1st base sequence mapped to the first RB (with minimum index) and the phase rotation is π on the 2nd base sequence mapped to the second RB (with minimum index+1). In case of 2 bits UCI, then each of the base sequence is used independently for indication. For example, the 1st bit is indicated by phase rotation of the 1st base sequence mapped to the first RB (such as LSB mapped to RB with minimum index) and the 2nd bit is indicated by the phase rotation of the 2nd base sequence mapped to the second RB (such as MSB mapped to RB with minimum index+1).

Proposal 1: For NR operation between 52.6 GHz and 71 GHz, PUCCH format 0 transmitted with multiple number of (same) base sequences with different phase shifts should be supported for mapping to multiple RBs

### 2.1.1 Link level simulation for enhancing PF 0

We performed simulation for PUCCH formats 0 with multiple RBs. For PF0, we considered two alternatives: repetition of length 12-based sequence and long sequence (12*N, where N > 1) spanning multiple RBs. For one RB and two RBs, the two alternatives use identical sequence based on low-PAPR sequence type1 CGS based [3GPP TS 38.211 V16.4.0, clause 5.2.2.2]. For more than two RBs, the low-PAPR sequence type 1 ZC based [3GPP TS 38.211 V16.4.0, clause 5.2.2.2] is used. The simulation assumptions used for the evaluation are collected in the Appendix section, Table A.

Figure 1 and Figure 2 show simulation results for BLER and ACK 2 Error performance for PUCCH format 0 for different mapping options and different number of RBs. More results are listed in the Appendix.

- (b)

Figure 1: Performance of PF0 with multiple RBs ; (a) with repetition, (b) with long sequence

Figure 2: Performance comparison between 120kHz/960kHz for PF0 with multiple RBs

| SNR[dB] points for Ack 2 Error (1%) | 1RB | 2RBs | 3RBs | 4RBs | 8RBs | 16RBs |
| --- | --- | --- | --- | --- | --- | --- |
| PF0, Repetition, 1bit, 1 symbol, 120kHz | 8.2 | 7.1 | 5 | 3.6 | 0 | -3 |
| PF0, LongSeq, 1bit, 1 symbol 120kHz | 8.2 | 5.9 | 4 | 2.9 | 0.1 | -2 |
| PF0, LongSeq, 1bit, 1 symbol, 960kHz | 9 | 7 | 4.9 | 3.8 |  |  |
| PF0, LongSeq, 1bit, 2 symbols 120kHz | 3 | -1 | -2.3 | -3.2 |  |  |
| PF0, LongSeq, 1bit, 2 symbols 960kHz | 3 | -1.3 | -3 | -4.4 |  |  |

Based on the link level simulation and the resulted SNR points for different detection requirements with the assumed power parameters listed in Table A and the additional agreed parameters (UE_EIRP = 40 dBm, UE_P = 25 dBm, TxBF = 0 dBi), the calculated Maximum Isotropic Loss for PF0 in the three regions and for both RB mapping alternatives with 1bit payload are captured in the following figures.

South Korea regulatory power limits for <300m

Considering South Korea regulation with the following parameters: Pmax_EIRP = 27 dBm – TxBF, Pmax_PSD = 13 dBm/MHz + max(0, 10*log10(BW)) – TxBF, Pmax = min(Pmax_PSD, Pmax_EIRP)

Figure 3: MIL under South Korea power limits for PF0

For format 0 with South Korea power limits, increasing the number of RBs from 1 to 16 with long sequence provides a gain of MIL with round 8dB between 132dB to 141dB. For repetition, due to the high CM, the achieved gain is reduced to 138.3dB. MIL is slightly less for 20ns comparing to 5 and 10ns. Two symbols give larger coverage for PF0.

US regulatory power limits

Considering US regulation with the following parameters:  Pmax_EIRP = 40 dBm – TxBF, Pmax_P = 27 dBm – max(0, 10*log10(100 / BW)), Pmax = min(Pmax_P, Pmax_EIRP)

Figure 4: MIL under US power limits for PF0

For format 0 with US power limits, increasing the number of RBs from 1 to 4 with long sequence provides a gain of MIL with round 6dB. For repetition, due to the high CM, the achieved gain is slightly reduced. The coverage is slightly less for 20ns comparing to 5 and 10ns. Two symbols give larger coverage for PF0.

Europe regulatory power limits

Considering Europe regulation with the following parameters:  Pmax_EIRP = 40 dBm – TxBF, Pmax_PSD = 23 dBm/MHz + max(0, 10*log10(BW)) – TxBF, Pmax = min(Pmax_PSD, Pmax_EIRP)

Figure 5: MIL under Europe power limits for PF0

For format 0 with Europe power limits, increasing the number of RBs from 1 to 4 with long sequence or repetition, the gain varies between different number of RBs. Increasing the number of RBs doesn’t always increase the coverage. The coverage is slightly less for 20ns comparing to 5 and 10ns. Two symbols give larger coverage for PF0 comparing to 1 symb.

Observation 1: For mapping of the RBs for PUCCH format 0 with long sequence, detection probability and the coverage become better with increasing the number of RBs depending on the sequence length under both South Korea and US power limits, while for Europe power limits, the coverage gain varies between different number of RBs.

### 2.1.2 Link level simulation for enhancing PF 1

Figure 6 show simulation results for BLER and ACK 2 Error performance for PUCCH format 1 for different mapping options and different number of RBs 1bit payload. More results are listed in the Appendix.

Figure 6: Performance for PUCCH format 1 with one or multiple RBs for 120kHz

Based on the link level simulation and the resulted SNR points for different detection requirements with the assumed power parameters listed in Table A and the additional agreed parameters (UE_EIRP = 40 dBm, UE_P = 25 dBm, TxBF = 0 dBi), the calculated the Maximum Isotropic Loss for PF1 in the three regions and for both RB mapping alternatives with 1bit payload are captured in the following figures.

South Korea regulatory power limits for <300m

Figure 7: MIL under South Korea power limits for PF1

For PUCCH format 1 with South Korea power limits, increasing the number of RBs from 1 to 4 with long sequence provides a gain of MIL with round 3.5dB coverage gain between 151.2 to 154.7dB. For repetition, due to the high CM, the achieved gain is slightly reduced. For repetition, due to the high CM, the achieved gain is slightly reduced. There was no difference observed between different delay spread values.

US regulatory power limits for <300m

Figure 8: MIL under US power limits for PF1

For PUCCH format 1 with US power limits, increasing the number of RBs from 1 to 4 with long sequence provides a gain of MIL with round 4.2dB coverage gain. For repetition, due to the high CM, the achieved gain is slightly reduced. There was no noticeable difference observed between different delay spread values.

Europe regulatory power limits

Figure 9: MIL under Europe power limits for PF0

For format 1 with Europe power limits, increasing the number of RBs from 1 to 4 with long sequence or repetition, the gain varies between different number of RBs. Increasing the number of RBs doesn’t always increase the coverage. The coverage is less for 20ns comparing to 5 and 10ns. Two symbols give larger coverage for PF0 comparing to 1 symb.

Observation 2: For mapping of the RBs for PUCCH format 1 with long sequence, detection probability and the coverage become better with increasing the number of RBs depending on the sequence length under both South Korea and US power limits, while for Europe power limits, the coverage gain varies between different number of RBs.

The link level simulation results show that PUCCH format 1 achieves better coverage than PUCCH format 0 and show an increase of the coverage with increasing the number of RBs for PUCCH formats 0/1 for both alternatives. However simple frequency domain repetition shows significant increase of PAPR and CM comparing to long sequence-based approach which results in reduced link budget. Therefore, for enhancing PUCCH formats 0/1 to allow for both coverage and satisfying PSD limitation, Rel15 based low-PAPR type sequence with different lengths that cover multiple RBs should be prioritized.

Proposal 2: For NR operation between 52.6 GHz and 71 GHz, Rel 15 based long sequence should be considered for PUCCH formats 0/1 for mapping to multiple RBs.

Currently, PUCCH format 0 is transmitted either in one or two OFDM symbols for feedback with low latency purpose. This depends on the size of the UCI payload whether it is one or two bits. In order to enable 2 PRBs for mapping PUCCH sequence and enhance the total transmitted power under PSD limitation, may be only one symbol is used even for two bits UCI and PUCCH sequence is mapped on two PRBs. If two symbols are configured for transmission, then repetition on the second symbol can be performed.

Proposal 3: For NR operation between 52.6 GHz and 71 GHz, PUCCH format 0 should be enhanced to support 2-bit transmission with 1 symbol by mapping to 2 RBs.

## 2.2 Enhancements to PUCCH format 4

PUCCH format 4 is a long format that spans several symbols (4 -14 OFDM symbols), however, it is mapped to a single RB. The BPSK/QPSK modulated information are spread using DFT with length 12 for the number of OFDM symbols configured for PUCCH excluding RS symbols which are TDMed with PUCCH symbols. In order to satisfy the PSD requirements, PUCCH formats 0/1 need to be mapped to multiple RBs depending on the configured SCS and the PSD required value for a certain region. One possibility to utilize increased number of RBs (more than one) is by simply applying frequency domain repetition of the generated PUCCH with 12-based DFT. Depending on the configured SCS and the PSD requirement for a certain region, number of RB is configured to repeat the single RB generated PUCCH resources in frequency domain, the other option is to increase the length of DFT that spans the required number of RBs.

### 2.2.1 Link level simulation for enhancing PF 4

We performed simulation for PUCCH format PF4 with two mapping options, using either multiple DFTs over, each over one RB and repeated over multiple RBs or a single DFT that spans the multiple RBs. The simulation assumptions used for the evaluation are collected in the Appendix Table  A.   For long DFT, the CR is changing based on the number of RBs used for PF4. The below table lists the effective code rate for different payload and number or RBs

| Payload | #RBs | CR |
| --- | --- | --- |
| 22 (Polar) | 1 | 0.3056 |
|  | 2 | 0.1528 |
|  | 3 | 0.1019 |
|  | 4 | 0.0764 |
| 11 (RM) | 1 | 0.1528 |
|  | 2 | 0.0764 |
|  | 3 | 0.0509 |
|  | 4 | 0.0382 |
| 4 (RM) | 1 | 0.0556 |
|  | 2 | 0.0278 |
|  | 3 | 0.0185 |
|  | 4 | 0.0139 |

In below table  we show the calculated CM of PF 4 for both alternatives

CM [dB] for different numbers of RBs

|  | Repetition with 12-based DFT | Long DFT |
| --- | --- | --- |
|  | PF4 | PF4 |
| 1RB | 0.8300 | 0.8300 |
| 2RBs | 2.4700 | 0.8870 |
| 3RBs | 4.2800 | 1.0900 |
| 4RBs | 5.7300 | 1.2580 |

Observation 3: PUCCH format 4 with repetition of single RB DFT over multiple RBs shows significant increase of CM comparing to single (longer) DFT that spans all the allocated RBs.

Figure 10 shows simulation results for BLER performance for PUCCH format 4 for different mapping options, different SCS and different number of RBs.

Figure 10: BLER Performance for PUCCH format 4 with one or multiple RBs

Based on the link level simulation and the resulted SNR points for different detection requirements with the assumed power parameters listed in Table A and the additional agreed parameters (UE_EIRP = 40 dBm, UE_P = 25 dBm, TxBF = 0 dBi), the calculated the Maximum Isotropic Loss for PF4 in the three regions and for both RB mapping alternatives are captured in the following figures.

Figure 11: MIL for PF4

Observation 4: For both alternatives (single 12 SCs DFT or long DFT) of RB mapping for PUCCH format 4, the detection probability and coverage become better with increasing the number of repetitions or increasing the length of DFT covering multiple RBs under South Korea and US power limits. Around 3dB gain can be achieved for long DFT from 1RB to 4RBs. The coverage increases for low payload size.

The link level simulation results show an increase of the coverage with increasing the number of RBs for PUCCH format 4 for both repetition of single 12 SCs DFT and multi-RBs-based DFT. Since the PAPR and CM comparing significantly increase with number of repeated RBs comparing with long DFT precoding, we conclude that for enhancing PUCCH formats 0/1/4 to allow for both coverage and satisfying PSD limitation, long DFT precoding that spans all allocated RBs should be considered for PUCCH format 4.

Proposal 4: For NR operation between 52.6 GHz and 71 GHz, Rel 15 long DFT precoding that spans all RBs (greater than 1) should be considered for PUCCH format 4 for mapping to multiple RBs.

## 2.3 Indication of the number of PRBs and the configuration type

If both alternatives (long sequence/Long DFT and repetition) are agreed to be supported, the UEs in ideal mode (before receiving the dedicated PUCCH configuration) can be indicated with the number of RBs and in addition the RB configuration type for PUCCH formats 0/1/4 using the predefined Table 9.2.1-1 [3GPP TS 38.213 V16.4.0] with including additional values of the number of RBs and RB configuration/mapping type for the three supported SCSs. For UEs in RRC connected mode, the indication of the number of RBs and configuration/mapping type can be signaled by UE specific PUCCH resource configuration RRC message.

Proposal 5: For NR operation between 52.6 GHz and 71 GHz, if both RB configuration options including long sequence/DFT and repetition are agreed to be supported for PUCCH formats 0/1/4, number of RBs and RB configuration/mapping type needs to be added to the PUCCH resource sets before dedicated PUCCH resource configuration

Proposal 6: For NR operation between 52.6 GHz and 71 GHz, if both RB configuration options including long sequence/DFT and repetition are agreed to be supported for PUCCH formats 0/1/4, number of RBs and RB configuration/mapping type needs to be signaled to the UE in the dedicated RRC message for RRC connected UEs

Long sequence alternative showed a better coverage enhancement of PUCCH transmission since it gives better MIL tolerance due to its low PAPR comparing with the frequency domain repetition. On the other hand, frequency domain repetition of PUCCH formats 0/1/4 gives a more chance for FDM multiplexing of UEs than long sequence option. Therefore, to achieve a certain trade-off between the coverage gain (e.g. for UEs in the cell edge scenario) and the UE multiplexing gain (e.g. for large number of UEs in good coverage scenario), a combination of both options,  as shown in Fig12, may enhance the flexibility of the system for the different scenarios.

Figure 12: Combination of RB configuration/mapping alternatives

The number of RBs and the combination factor, that controls the combination options between the long sequence /long DFT and the frequency domain repetition, can be signaled to the UE using RRC dedicated PUCCH configuration.

Proposal 7: For NR operation between 52.6 GHz and 71 GHz, if both RB configurations options are agreed to be supported for PUCCH formats 0/1/4, a combination of repetition and long sequence could be supported for mapping on multiple RBs. The combination factor can be signaled by RRC UE dedicated PUCCH resource configuration

# 3	Conclusion

Here we summarize the observations and proposals from the above sections:

Observation 1: For mapping of the RBs for PUCCH format 0 with long sequence, detection probability and the coverage become better with increasing the number of RBs depending on the sequence length under both South Korea and US power limits, while for Europe power limits, the coverage gain varies between different number of RBs.

Observation 2: For mapping of the RBs for PUCCH format 1 with long sequence, detection probability and the coverage become better with increasing the number of RBs depending on the sequence length under both South Korea and US power limits, while for Europe power limits, the coverage gain varies between different number of RBs.

Observation 3: PUCCH format 4 with repetition of single RB DFT over multiple RBs shows significant increase of CM comparing to single (longer) DFT that spans all the allocated RBs.

Observation 4: For both alternatives (single 12 SCs DFT or long DFT) of RB mapping for PUCCH format 4, the detection probability and coverage become better with increasing the number of repetitions or increasing the length of DFT covering multiple RBs under South Korea and US power limits. Around 3dB gain can be achieved for long DFT from 1RB to 4RBs. The coverage increases for low payload size.

Proposal 1: For NR operation between 52.6 GHz and 71 GHz, PUCCH format 0 transmitted with multiple number of (same) base sequences with different phase shifts should be supported for mapping to multiple RBs

Proposal 2: For NR operation between 52.6 GHz and 71 GHz, Rel 15 based long sequence should be considered for PUCCH formats 0/1 for mapping to multiple RBs.

Proposal 3: For NR operation between 52.6 GHz and 71 GHz, PUCCH format 0 should be enhanced to support 2-bit transmission with 1 symbol by mapping to 2 RBs.

Proposal 4: For NR operation between 52.6 GHz and 71 GHz, Rel 15 long DFT precoding that spans all RBs (greater than 1) should be considered for PUCCH format 4 for mapping to multiple RBs.

Proposal 5: For NR operation between 52.6 GHz and 71 GHz, if both RB configuration options including long sequence/DFT and repetition are agreed to be supported for PUCCH formats 0/1/4, number of RBs and RB configuration/mapping type needs to be added to the PUCCH resource sets before dedicated PUCCH resource configuration

Proposal 6: For NR operation between 52.6 GHz and 71 GHz, if both RB configuration options including long sequence/DFT and repetition are agreed to be supported for PUCCH formats 0/1/4, number of RBs and RB configuration/mapping type needs to be signaled to the UE in the dedicated RRC message for RRC connected UEs

Proposal 7: For NR operation between 52.6 GHz and 71 GHz, if both RB configurations options are agreed to be supported for PUCCH formats 0/1/4, a combination of repetition and long sequence could be supported for mapping on multiple RBs. The combination factor can be signaled by RRC UE dedicated PUCCH resource configuration

# 4	References

[1] 	3GPP RAN1#104-bis-e. “Chairman Notes”

# 5	Appendix

## 5.1 Simulation assumptions

Table A: Simulation Assumptions

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 60 GHz |
| Subcarrier Spacing [kHz] | 120, 480, 960 kHz |
| Number of usable RBs per carrier | 256 for 120 kHz SCS (corresponds to ~400 MHz carrier) 256 for 480 kHz SCS (corresponds to ~1600 MHz carrier) 160 for 960 kHz SCS (corresponds to ~2000 MHz carrier) |
| PUCCH Frequency Hopping | On, (hopping with half of the allocated PUCCH RBs) |
| CP Type | Normal CP |
| Channel Model | TDL-A, 5ns, 10ns, 20ns |
| BS Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,2} |
| UE Antenna Configuration (Mg,Ng,M,N,P) | {1,1,1,1,1} |
| Mobility | 3 km/hr |
| Channel Estimation | Realistic channel estimation |
| PUCCH Format | PF0, PF1, PF4 |
| Number of OFDM symbols used for PUCCH resource | 1,2 for PF0 8,10 for PF4 |
| Sequence construction details | Rel15, Low-PAPR Sequence type 1 for PF0/1 |
| PUCCH payload encoder type | 4, 11 (with RM coding), 22(with Polar coding) for PF4, 1,2 for PF0/1 |
| #RBs | 1, 2, 3, 4, 8, 16 |
| UE Tx Beamforming gain | TxBF = 0 dBi |
| BS Rx Beamforming gain | RxBF = 20 dBi |
| UE Power Limitations | Maximum EIRP: UE_EIRP = 40 dBm Maximum conducted power UE_P = 25 dBm |
| Noise | BS Noise Figure, NF = 7 dB Noise PSD = -174 dBm/Hz |
| Considered region | South Korea regulation for <300m, US, Europe |

## 5.2 Additional results

| SNR points [dB] for 1% and 0.1% |  | 1RB | 2RBs | 3RBs | 4RBs |
| --- | --- | --- | --- | --- | --- |
| PF0 | 120kHz, 20ns, 1sym, 1bit, LSeq (1%) | 9 | 6.3 | 6 | 2.6 |
|  | 120kHz, 20ns, 1sym, 1bit, rep. (1%) | 10 | 6.7 | 3.6 | 2.5 |
|  | 120kHz, 10ns, 1sym, 1bit, LSeq(1%) | 8.2 | 5.5 | 4 | 2 |
|  | 120kHz, 10ns, 1sym, 1bit, rep(1%) | 8.3 | 7 | 5 | 4 |
|  | 120kHz, 10ns, 2sym, 1bit, LSeq (1%) | 2.5 | -1 | -2.5 | -3 |
|  | 120kHz, 10ns, 2sym, 1bit, LSeq (0.1%) | 8.2 | 5 | 3.2 | 2.3 |
|  | 480Khz, 20ns, 1sym, 1bit, LSeq (1%) | 10 | 7.5 | 6.8 | 4 |
|  | 480Khz, 20ns, 1sym, 1bit, rep. (1%) | 10 | 4.7 | 2 | 0 |
|  | 480kHz, 5ns, 1sym, 1bit, LSeq (1%) | 9 | 6.5 | 4 | 3 |
|  | 480kHz, 5ns, 1sym, 1bit, rep. (1%) | 9 | 5.5 | 4 | 2.4 |
|  | 960kHz, 10ns, 1sym, 1bit, rep (1%) | 11 | 4.3 | 2.3 | 1.2 |
|  | 960kHz, 10ns, 1sym, 1bit, LSeq (1%) | 9 | 6.8 | 4.9 | 4 |
| PF1 | 120kHz, 10ns, 10sym, 1bit, LSeq (1%) | -10 | -12.7 | -14 | -14.7 |
|  | 120kHz, 10ns, 10sym, 1bit, rep (0.1%) | -6.5 | -9.5 | -10.3 | -11.4 |
|  | 120kHz, 10ns, 10sym, 1bit, rep (1%) | -9.5 | -12 | -13.9 | -15.8 |
|  | 120kHz, 20ns, 8sym, 1bit, LSeq(1%) | -9.5 | -12.8 | -14.8 | -15.9 |
|  | 120kHz, 20ns, 8sym, 1bit, LSeq(.1%) | -5.6 | -9 | -10.5 | -12.3 |
|  | 480kHz, 20ns, 8sym, 1bit, LSeq (1%) | -9 | -11.3 | -12 | -12.2 |
|  | 480kHz, 20ns, 8sym, 1bit, LSeq (.1%) | -5.9 | -7.2 | -8 | -9.2 |
|  | 480kHz, 20ns, 8sym, 1bit, rep(1%) | -9.5 | -11.7 | -13.4 | -14 |
|  | 480kHz, 20ns, 8sym, 1bit, rep (.1%) | -6.3 | -9 | -10.5 | -10.9 |
|  | 960kHz, 10ns, 10sym, 1bit (1%) | -9 | -12 | -13.9 | -15 |
|  | 960kHz, 10ns, 10sym, 1bit (.1%) | -6 | -9.3 | -10 | -12 |
|  | 960kHz, 20ns, 8sym, 1bit, LSeq (1%) | -9.5 | -12 | -13 | -14.1 |
|  | 960kHz, 20ns, 8sym, 1bit, LSeq (.1%) | -5.3 | -8.2 | -9 | -10.5 |
|  | 960kHz, 10ns, 8sym, 1bit, LSeq(1%) | -9.5 | -12 | -12.8 | -14.4 |
|  | 960kHz, 10ns, 8sym, 1bit, LSeq(.1%) | -5.5 | -8.2 | -9 | -10.5 |
| PF4 | 960kHz, 10ns 8sym, 22bits, L_DFT | 3 | -.5 | -2 | -3 |
|  | 960kHz, 10ns 8sym, 22bits, rep | 3 | .2 | -1 | -1.3 |
|  | 120kHz, 10ns, 8sym, 22bits, L_DFT | 3 | -.9 | -2.7 | -4.1 |
|  | 120kHz, 10ns, 8sym, 22bits, rep | 3 | .4 | -1.2 | -2 |
|  | 960kHz, 10ns, 10sym, 11, L_DFT | -2.2 | -5.1 | -6.4 | -7.2 |
|  | 120kHz, 10ns, 10sym, 11, L_DFT | -2 | -5 | -6.7 | -7.8 |
|  | 120kHz, 10ns, 10sym, 11, rep | -1.8 | -4 | -6.6 | -7.5 |
|  | 480kHz, 20ns, 8sym, 22bits, L_DFT | 3 | 0 | -1.9 | -2.6 |
|  | 480kHz, 20ns, 8sym, 22bits, rep | 2.7 | 0 | -1 | -2 |
|  | 480kHz, 5ns, 8sym, 22bits, L_DFT | 2.6 | -1.4 | -3 | -4 |
|  | 480kHz, 5ns, 8sym, 22bits, rep | 2.7 | .6 | -1.5 | -2.4 |
|  | 120kHz, 20ns, 8sym, 22bits, L_DFT | 2.5 | -1 | -2.9 | -4 |
|  | 120kHz, 20ns, 8sym, 22bits, rep | 2.5 | -.6 | -1.9 | -2.5 |