3GPP TSG RAN WG1 Meeting #105-e	R1-2105520

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on UL skipping

Document for:	Discussion and decision

Introduction

In RAN1 #104bis-e, the handling of PUSCH skipping with repetition overlapping with PUCCH was intensively discussed, but no agreement was reached. Up to 7 options for DG PUSCH were listed in [1] and additional option 3’ was captured for further discussion. In this contribution, analysis for these options are provided and solutions for both CG and DG PUSCH skipping with repetition issues are also proposed.

- Option 1: When there’s a UCI to be multiplexed on any of the repetitions of the DG PUSCH, MAC generates MAC PDU for the DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI can be multiplexed on the DG PUSCH. - MAC generate MAC PDU for all DG PUSCH repetitions - Note: the UCI multiplexing timeline condition for the first repetition of DG PUSCH should be ensured - Option 2: - When there’s UCI overlapping with the first PUSCH repetition of the DG PUSCH, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. - UE does not expect when a UCI is overlapping with the repetitions other than the first PUSCH repetition. - Option 3: When a PUCCH is overlapped with the first PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - When a PUCCH is overlapped with the repetitions other than the first PUSCH repetition, if there is no PDU including data delivered from MAC, the DG PUSCH can be skipped. UCI is transmitted on the PUCCH. - Option 3’: When a PUCCH is overlapped with the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - When a PUCCH is overlapped with the repetitions other than the first X PUSCH repetition, if there is no PDU including data delivered from MAC, the DG PUSCH can be skipped. UCI is transmitted on the PUCCH. - The value of X can be 1 or 2 - Option 4: Rel-16 PUSCH skipping and PUSCH repetitions are not allowed to be enabled together (error case is defined). - Option 5: When PUSCH repetition is configured, - if a PUSCH repetition overlaps with PUCCH, MAC generates PDU for the repetition, - otherwise, MAC does not generate PDU for the repetition if there is no data for the DG PUSCH. - Note: it requires the MAC layer can decide whether to generate a MAC PDU for the repetition depending on whether it overlaps with PUCCH, which is different from current MAC behaviour. - Option 6: When PUSCH repetition is configured, - MAC layer behavior: For a PUSCH repetition, MAC always generate a PDU. If MAC has data in buffer, generate a real PDU; otherwise, generate a dummy PDU. And MAC use 1-bit to tell the PHY the PDU is a dummy PDU or real PDU. The 1-bit can be UE internal implementation between MAC and PHY, no need to specify it. - PHY layer behavior: Each PUSCH repetition independently check it overlap with a PUCCH or not. - If it overlaps with a PUCCH, that PUSCH repetition cannot be skipped. - If it does not overlap with any PUCCH, - if the MAC PDU is a dummy PDU, PHY can skip this PUSCH repetition - If the MAC PDU is a real PDU, PHY cannot skip this PUSCH repetition. - Option 7: When a PUCCH is overlapped with any of the first X PUSCH repetition, MAC generates MAC PDU for DG PUSCH and delivers the MAC PDU(s) to PHY and the UCI is multiplexed on the DG PUSCH. All of the PUSCH repetitions are not skipped. - UE does not expect when a UCI is overlapping with the repetitions other than the first X PUSCH repetition - The value of X can be 1 or is 2

UL skipping for PUSCH with repetitions

It has been agreed in RAN1 that for single slot PUSCH, if UE capability of UL skipping is supported by UE and meanwhile PUSCH overlaps with PUCCH(s), a MAC PDU is generated for UCI multiplexing although there is no actual data in MAC layer. However, for multi-slot PUSCH, principle to handle the overlapping of PUCCH and PUSCH skipping are not decided yet.

DG PUSCH skipping with repetitions

Form the list of options in section 1, option 1 requires UE to look up the repetitions of PUSCH one by one, if any one of repetitions has overlapping with PUCCH and UCI from the PUCCH is going to be piggybacked on that PUSCH repetition, then MAC layer should generate a PDU and transport it to PHY layer. However, this criteria requests UE to check the overlapping of PUCCH with each PUSCH repetition before the starting of PUSCH transmission, which would impact the UE processing order and bring more UE implementation complexity. On the other hand, always transmit all PUSCH repetitions will also consume more power from UE side.

Option 3 is the one supported by majority in the last meeting. It requires UE to check the overlapping with the first repetition of PUSCH only. If the first repetition overlaps with any PUCCHs and UCI would be multiplexed on the PUSCH, a MAC PDU is generated and delivered to physical layer. On the contrary, if gNB finds no data is received in the first repetition, gNB could understand the whole UL transmission may be skipped and the UCI is carried on PUCCH. Therefore, gNB does not need to decode PUCCH and PUSCH both. Unlike option 1, this option only considers the overlapping for the first repetition and avoids to introduce additional UE complexity for UE, especially multiple carriers are configured. Nevertheless, the reliability of this option depends on whether gNB could detect and confirm the DTX of PUSCH. In real deployment, gNB might be not always sure whether there is an actual PUSCH transmission or not based on the first repetition only. So gNB is potential to detect both PUSCH and PUCCH in the follow-up slots. For enhancing the reliability of PUSCH DTX detection, one more PUSCH repetition can be checked in further, i.e. overlapping status of first or first two PUSCH repetitions with PUCCH are used to determine whether a padding MAC PDU is generated. If the PUSCH skipping is enabled and PUCCH overlaps with first or first two PUSCH repetitions (one or both first two repetitions), and UCI would be multiplexed on any of the repetitions, then a MAC PDU is generated. That is the option 3’ which aims to have a balance on gNB detection performance and UE complexity.

A common concern for both option 5 and option 6 is the identity of a padding MAC PDU. Option 5 and 6 rely on UE to judge (by MAC or PHY layer) the overlapping status and to generate a padding MAC PDU. However, the actual MAC PDU information is not informed by gNB and gNB does not know the UL-SCH in the PUSCH is just for padding or real data. If less repetitions are transmitted, gNB may fail to decode the UL-SCH decoding. Therefore, further retransmissions may be indicated. Considering from system performance perspective, retransmitting a padding MAC PDU would waste UL resource and consume more power of UE, which should be avoided. On the other hand, option 5 is not aligned with current MAC PDU generation procedure and redefines to generate MAC PDUs repetition by repetition. More standard efforts and research are needed in both RAN1 and RAN2 for this new procedure and the option is not preferred in such late CR stage of Rel-16.

For option 2 and option 7, they introduce more scheduling restrictions on gNB which is not desirable in the real deployment, so they should not be considered as well.

Based on above analysis, option 3 is more suitable for handling the DG PUSCH skipping issue, and option 3’ can be the second candidate option to balance the NW performance and UE complexity, so a proposal is made as below:

Proposal 1: For handling the issue of DG PUSCH skipping with repetition, one of following options is used based on further discussion by RAN1:

- Option 3 (first priority)

- Option 3’ (second priority)

CG PUSCH skipping with repetitions

It is preferred to have a uniform solution for both DG PUSCH and CG PUSCH, but the condition to generate MAC PDU for CG and DG is not always same. As shown in Figure 1, since the UL grant comes after the DL grant, when the UL grant arrives, UE could know the second PUSCH repetition is going to overlap with the PUCCH, and HARQ from the PUCCH needs to be multiplexed on that PUSCH repetition. So the option 1 for DG PUSCH, i.e. to check the overlapping with every PUSCH repetition, can be utilized. However, for the CG PUSCH transmission, same condition may not work. Since UL grant is absent and UE cannot predict whether there is a PUCCH would overlap with any repetitions of PUSCH before the starting of PUSCH transmission. Hence, UE does not know whether to generate a padding MAC PDU. The example in Figure 2 can illustrate this issue more clearly. In slot n, UE starts to transmit PUSCH, but in that slot, UE cannot anticipate a PUCCH will be scheduled in slot n+5 and the PUCCH would overlap with the PUSCH repetition 8 in slot n+7, and UE does not know either HARQ information needs to be carried on that repetition. So possible UE behavior is skipping the PUSCH due to no MAC PDU is delivered and HARQ is transmitted on PUCCH instead of PUSCH in slot n+7. From this point of view, option 1 cannot be applied to CG PUSCH repetition and option 3 is more suitable.

Figure 1. The overlapping between DG PUSCH with repetition and PUCCH

Figure 2. The overlapping between CG PUSCH with repetition and PUCCH

For CG PUSCH repetitions, UE could start to transmit them at every occasion with RV=0. Based on the above analysis, option 3 can be also used for CG PUSCH skipping with repetition, but the understanding of first repetition has two angles. First, to have less UE implementation complexity, the first repetition of CG PUSCH could be the first occasion with RV=0. Second, as per the each occasion with RV=0 is allowed to have PUSCH transmission, first repetition could be any occasion with RV=0. Therefore, following proposal is made.

Proposal 2: For handling the issue of CG PUSCH skipping with repetition, option 3 for DG PUSCH is reused and the first PUSCH repetition is defined as one of the following options based on further RAN1 discussion:

- Option 1 (first priority): First repetition with RV=0

- Option 2 (second priority): Any repetition with RV=0

Conclusions

Based on the discussion above, following proposals are obtained:

Proposal 1: For handling the issue of DG PUSCH skipping with repetition, one of following options is used based on further discussion by RAN1:

- Option 3 (first priority)

- Option 3’ (second priority)

Proposal 2: For handling the issue of CG PUSCH skipping with repetition, option 3 for DG PUSCH is reused and the first PUSCH repetition is defined as one of the following options based on further RAN1 discussion:

- Option 1 (first priority): First repetition with RV=0

- Option 2 (second priority): Any repetition with RV=0

- References

R1- 2104073, “Summary of [104b-e-NR-7.1CRs-05] discussion on PUSCH skipping with UCI overlapping (Rel16)”, Vivo, 3GPP TSG RAN WG1 #104b-e, April 12th – 20th, 2021