3GPP TSG RAN WG1 #105-e                                                                                                  R1-2105521

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.14.3

Source:	Huawei, HiSilicon

Title:	Initial evaluation results for XR and Cloud Gaming

Document for:	Discussion and Decision

# Introduction

In RAN1#104-e and RAN1#104b-e, some agreements on XR (eXtended Reality) and CG (Cloud Gaming) have been made such as traffic model, KPIs, evaluation assumptions/methodologies, etc. [1] [2]. Based on these, RAN1 is to carry out the evaluation of XR and CG in NR under various scenarios and traffic conditions.

In this contribution, we provide our initial system level evaluation results based on the agreed evaluation assumptions/methodologies. The simulation results are also filled into the template excel as attached.

# Simulation Setting

## Traffic Model

### Single-stream model

We consider the statistical traffic models with different parameters for XR and CG services. The packet size and the jitter of the packet arrival time follow the truncated Gaussian distribution. According to the agreements in RAN1#104-e [1] and RAN1#104b-e [2], the details of the single-stream model for downlink (DL) are summarized in Table 1.

Table 1. The single-stream traffic model for DL

| Application |  | VR/AR | CG |
| --- | --- | --- | --- |
| Frame per second |  | 60 FPS |  |
| Average data rate |  | 30, 45Mbps | 8, 30Mbps |
| Packet size | Distribution | Truncated Gaussian Distribution |  |
|  | Mean | 62.5K, 93.75K Bytes | 16.67K, 62.5K Bytes |
|  | STD | 0.105*mean |  |
|  | Max | 1.5*mean |  |
|  | Min | 0.5*mean |  |
| Jitter | Distribution | Truncated Gaussian Distribution |  |
|  | Mean | 0 ms |  |
|  | STD | 2 ms |  |
|  | Range | [-4, 4] ms |  |

### Multi-stream model

Agreement:

In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

- Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria for being satisfied.

- Companies should strive to align the parameter values for the options chosen as much as possible

- FFS: Whether audio stream is separate or aggregated with the data stream in option 2 (Intention of option 2 is not to create a 3 stream option)

In RAN1#104b, three options of DL multi-stream model are agreed to be optionally evaluated [2].

In this paper, the initial evaluation results for Option 1B: GOP-based traffic model are provided.

- Option 1B: GOP-based traffic model: stream#1 and stream#2 refer to I-stream and P-stream. Here, we consider the GOP size as K = 8. The average size ratio between I frame and P frame is α = 1.5.

The detailed description of the above multi-stream model are given in our companion paper [4]. We summarize the parameters of Option 1B: GOP-based traffic model in Table 2.

Table 2. Option 1B: GOP-based traffic model

| Stream |  | Stream#1: I-stream | Stream#2: P-stream |
| --- | --- | --- | --- |
| Total average data rate |  | 30 Mbps |  |
| Average data rate per stream |  | 5.29 Mbps | 24.71 Mbps |
| Number of packets at a time |  | 1 or 0 | 0 or 1 |
| Packet size | Distribution | Truncated Gaussian Distribution |  |
|  | Mean | 88.24K Bytes | 58.82K Bytes |
|  | STD | 0.105*mean |  |
|  | Max | 1.5*mean |  |
|  | Min | 0.5*mean |  |
| Jitter | Distribution | Truncated Gaussian Distribution |  |
|  | Mean | 0 ms |  |
|  | STD | 2 ms |  |
|  | Range | [-4, 4] ms |  |

## KPI

For the DL single-stream model, according to the agreement in RAN1#104b-e [2], we choose the following (packet success rate (PSR), PDB) combinations to evaluate the capacity.

- KPI of  DL single-stream model for VR/AR and CG service

| (PSR X%, PDB /ms) of DL single stream |  |
| --- | --- |
| VR/AR | CG |
| (99, 7) | (99, 12) |
| (99, 10) | (99, 15) |
| (95, 13) | (95, 18) |

For the DL multi-stream model, as discussed in our companion paper [5], we use the following (PSR, PDB) combinations as KPI.

- KPI of  DL multi-stream model for VR/AR service

(PSR X%, PDB /ms) of DL {I-stream, P-stream}

VR/AR

{(99.5, 7), (95, 7)}

{(99.5, 10), (95, 10)}

{(95.5, 13), (90, 13)}

## Details and Parameters

We present the evaluation results for FR1 Dense Urban and Urban Macro scenarios. Both DL and UL transmission is dynamically scheduled in our simulation. In the uplink slot, users send ACK/NACK over PUCCH according to the DL decoding result. The overall simulation assumptions are listed in Appendix A.

# Evaluation Results for DL Capacity

According to the agreement in RAN1#103-e [3], system capacity is defined as the maximum number of users per cell with at least 90% (baseline) of users being satisfied. Therefore, 90% user satisfaction ratio is considered in our evaluation. The evaluation is performed over different average numbers of users per cell, i.e., 1, 3, 5, 7, 10, and 15. Note that the average number of user per cell means the average load per cell. The actual number of users for each cell may be unequal. In the simulation, a drop-packet-upon-timeout mechanism is adopted. That is, if the serving time of a packet exceeds the PDB, the packet will be dropped. The initial system level evaluation results for capacity of VR/AR and CG in Dense Urban and Urban Macro scenarios are shown as follows.

## Single-stream model

### Results under different (PSR, PDB)

Figure 1 gives the capacity results of VR/AR with 30Mbps for different (PSR, PDB) combinations in Dense Urban SU-MIMO scenario. It is observed that the network can support 6.8 users per cell with (PSR=95%, PDB= 13ms). If the requirement is more stringent, i.e. (PSR=99%, PDB= 7ms), the network capacity decreases to 2.3 users per cell. Besides, we can see that the network capacity reduces as the average number of users per cell increases, this is mainly due to the fact that more users per cell results in larger queuing delay.

Figure 1. Capacity results of the single-stream model with different (PSR, PDB)

Observation 1: As shown in Table 3, higher requirement of reliability and latency results in lower network capacity.

Table 3. Capacity of the DL single-stream model with different (PSR, PDB) in FR1 Dense Urban

| (PSR, PDB) | Average number of supported users per cell |
| --- | --- |
| (99, 7) | 2.3 |
| (99, 10) | 5.1 |
| (95, 13) | 6.8 |

### Impact of frame level integrated transmission

Agreement:

For XR/CG capacity evaluation, a packet is considered as lost when it has exceeded the PDB, such that it will be added to the PER and the data of the packet is discarded.

- It is up to company to report the details for the packet when it has exceeded the PDB, e.g.

  - Option 1: The packet exceeding the delay is still delivered to the other side

  - Option 2: The packet (including the non-transmitted part) is discarded at the transmitter (at the gNB for DL packets and at the UE for UL packets)

  - Other options are not precluded

- Note: This is for the purpose of evaluation

In RAN1#104b-e, the following agreements on transmission scheme in XR/CG capacity evaluation were achieved [1].

If the queuing time of non-transmitted part for a packet exceeds the PDB constraint, this packet will anyway be counted as error packet, so sending this part is useless to improve the PSR performance of users. Therefore, in order to save the network resource, each packet (frame) should be delivered as complete as possible. For example, consider a case where the current transmitted proportion of a packet of user 1 and 2 is 99%, 10% respectively, then the network should try to finish the delivery for user 1 first since the small number of the remaining bits for user 1 is easier to be transmitted successfully. Inspired by this characteristic of sending XR/CG packet, we further consider a transmission scheme, which incorporates the packet integrity into delivery and prioritize scheduling user whose packet is close to completion:

- Option 3 (FLIT, Frame Level Integrated Transmission): based on the scheme in option 2, we take the proportion of the already transmitted part for each packet into consideration. Define A as the number of bytes of the already transmitted part for a packet, and B as the total packet size. The packets with larger ratio A/B, are prioritized to be scheduled. Thus, the scheduling priority of each packet is multiplied with a factor which monotonically increases with the ratio A/B, i.e.

Priority=PF_value*f(A/B),

where PF_value is the conventional proportional fair scheduling factor, and f is a monotonically increasing function such as linear function, exponential function, etc. More details of FLIT can be found in our companion paper [6].

In this subsection, we choose option 2 as a baseline to compare the performance of FLIT. As shown in Figure 2, FLIT can improve the capacity from 5.1 users per cell to 6.2 users per cell.

Figure 2. Capacity results of the single-stream model with different transmission schemes

Observation 2: Frame level integrated transmission (FLIT), which prioritize scheduling user whose packet is close to completion, can improve the network capacity about 21.6% as shown in Table 4.

Table 4. Capacity of the DL single-stream model of different transmission schemes in Dense Urban

| Transmission scheme | Option 2 | Option 3 (FLIT) |
| --- | --- | --- |
| Average number of supported users per cell | 5.1 | 6.2 |
| Capacity improvement of option 3 over option 2 | 21.6% |  |

### Results in Dense Urban

This part shows the network capacity of applications with different data rates. From Figure 3, it can be generally observed that the capacity decreases with the data rate. For example, the network can support 5.1 VR/AR users with data rate of 30Mbps in SU-MIMO setting while it can support 2 VR/AR users with data rate of 45Mbps. Besides, we can see that for the same data rate, the more tight PDB constraint results in less capacity. For example, the network can support 5.1 VR/AR (30Mbps) users with PDB=10ms in SU-MIMO setting while it can support 7.3 CG (30Mbps) users with PDB=15ms.

- SU-MIMO

- MU-MIMO

Figure 3. Capacity results of the single-stream model in Dense Urban

Observation 3: In FR1 Dense Urban, the network capacity of the DL single-stream model is summarized in Table 5.

Table 5. Capacity of the DL single-stream model in FR1 Dense Urban

| Average number of supported users per cell | VR/AR @30Mbps (PSR=99%, PDB=10 ms) | VR/AR @45Mbps (PSR=99%, PDB=10 ms) | CG @8Mbps (PSR=99%, PDB=15 ms) | CG @30Mbps (PSR=99%, PDB=15 ms) |
| --- | --- | --- | --- | --- |
| SU-MIMO | 5.1 | 2.0 | >30 | 7.3 |
| MU-MIMO | 12.0 | 5.5 | >30 | 15.6 |

### Results in Urban Macro

Similar to Dense Urban scenario, from Figure 4, we can see that 1) the network capacity decreases with data rate, and 2) the more tight PDB results in lower network capacity. In addition, by comparing Figure 3 and Figure 4, it is concluded that the network capacity in Urban Macro scenario is generally less than that in Dense Urban scenario due to the larger pathloss in Urban Macro case. It is also observed that MU-MIMO can effectively increase the network capacity.

- SU-MIMO

- MU-MIMO

Figure 4. Capacity results of the single-stream model in Urban Macro

Observation 4: In FR1 Urban Macro, the network capacity of the DL single-stream model is summarized in Table 6.

Table 6. Capacity of the DL single-stream model in FR1 Urban Macro

| Average number of supported users per cell | VR/AR @30Mbps (PSR=99%, PDB=10 ms) | VR/AR @45Mbps (PSR=99%, PDB=10 ms) | CG @8Mbps (PSR=99%, PDB=15 ms) | CG @30Mbps (PSR=99%, PDB=15 ms) |
| --- | --- | --- | --- | --- |
| SU-MIMO | 4.3 | 1.6 | >30 | 6.5 |
| MU-MIMO | 9.5 | 4.0 | >30 | 11.5 |

Observation 5: In FR1 Dense Urban and Urban Macro, MU-MIMO can effectively increase the network capacity.

## Multi-stream model

This sub-section gives our evaluation results of the multi-stream model in FR1 Dense Urban SU-MIMO scenario.

### Impact of the size ratio between I frame and P frame

Figure 5 shows the capacity results of the multi-stream option 1B (GOP-based traffic model) at different size ratios between I frame and P frame, i.e. α=1 and 1.5. Here, we also plot a capacity result of single-stream model which is obtained from the simulation results of the multi-stream model with α=1 by using KPI (PSR=99%, PDB=10ms).

It can be observed that the capacity of multi-stream model with α=1  (green line) is less than that of single-stream model (red line). The decrease of network capacity is due to the fact that in the single-stream model, we do not distinguish I/P frame and calculate the PSR of all packets from two streams, and a user is deemed to be satisfied if NI+NPNall_I+Nall_P≥99% , where NI, NP and Nall_I,Nall_P denote the numbers of successful I/P packets, and the numbers of all I/P packets of a UE, respectively. But in the multi-stream model, the PSR’s of I stream and P stream are counted separately and a user is deemed satisfied if both streams are satisfied, i.e. NINall_I≥99% and NPNall_P≥99%, which is more stringent than that in single-stream case.

In addition, it is observed that the network capacity for α=1.5 is worse than that for α=1 since the larger the packet size of I frame, the harder it can transmitted successfully within the PDB.

Figure 5. Capacity results of the multi-stream option 1B at α=1 and 1.5

Observation 6: The network capacity of the DL multi-stream Option 1B decreases as the size ratio between I frame and P frame becomes larger.

### Benefit of prioritizing the transmission of the more important stream

In this subsection, we consider two types of scheduling schemes for the multi-stream model:

- Scheme 1 (S1): Proportional fair (PF). The scheduling priority of each user is calculated as the ratio of the instantaneous data rate, indicated by channel quality indicator, over the historical data rate. The ratio is also named as the PF value. Larger PF value means higher scheduling priority.

- Scheme 2 (S2): Based on the PF scheme, the packets of the more important stream (i.e., I-stream in our simulation) are prioritized to be scheduled. That is, if the currently transmitting packet of a user belongs to Stream#1 (I-frame), the PF value of the user is scaled up with a factor.

From Figure 6, it is shown that the capacity of the multi-stream option 1B by baseline scheduler S1 is 3.6 users per cell, while the network can support 5.8 users per cell by using enhancement scheduler S2.

Figure 6. Capacity results of the multi-stream option 1B at α=1.5

Observation 7: The network capacity of the DL multi-stream Option 1B can be improved by prioritizing the transmission of the more important stream as shown in Table 7.

Table 7. Capacity of the DL multi-stream Option 1B in FR1 Dense Urban

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) |
| --- | --- | --- |
| Average number of supported users per cell | 3.6 | 5.8 |
| Capacity improvement of S2 over S1 | 61.1% |  |

# Evaluation Results for DL UE power consumption

In this section, the initial system level evaluation results for DL UE power consumption of VR/AR in Dense Urban scenario are provided. The power consumption is evaluated based on single-stream model. In the evaluation, the traffic model is listed as following:

- VR/AR: DL, 30Mbps @60fps, PDB=10ms, PSR=99%

In this section, baseline (no C-DRX configuration) and C-DRX are evaluated. The existing integer C-DRX cycle cannot align with the XR traffic arrival. 10ms is the minimum value of long DRX cycle configuration supported by the current specification and it is considered to be evaluated for XR traffic. To be close to the periodicity of XR traffic, 16ms of DRX cycle is also considered. The parameters of C-DRX are shown in Table 8.

Table 8. C-DRX configurations

| DRX cycle (ms) | onDuration Timer (ms) | Inactivity Timer(ms) |
| --- | --- | --- |
| 10 | 5 | 2 |
| 10 | 8 | 4 |
| 16 | 5 | 4 |
| 16 | 8 | 4 |

In the evaluations, N=5 UEs on average are assumed within a cell. In our evaluations, the power consumption of PDCCH-only, PDCCH+PDSCH, PUCCH for HARQ-ACK and different states of sleep are included. The power model of PDCCH-only, PDCCH+PDSCH and sleep state in TR 38.840 are reused. In our evaluations, it is assumed that power value for S slot is same as that for DL slot. For simplicity, it is assumed that the power value of PUCCH for HARQ-ACK is the value of long PUCCH at 23dBm. Based on the assumptions and C-DRX configurations mentioned above, the following observations are provided.

Observation 8: For DL VR/AR@30Mbps in dense urban, the power saving gains are summarized in Table 9.

Table 9. Power saving gain for dense urban, DL VR/AR@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 4.5/ 5 |
| C-DRX(10,5,2) | 9.76% | 0.9/ 5 |
| C-DRX(10,8,4) | 3.39% | 4.2/ 5 |
| C-DRX(16,5,4) | 28.50% | 0/ 5 |
| C-DRX(16,8,4) | 13.88% | 0/ 5 |

Observation 9: For DL VR/AR@30Mbps in dense urban, the network capacity of are summarized in Table 10.

Table 10. Capacity for dense urban, DL VR/AR@30Mbps

|  | Baseline | C-DRX(10,5,2) | C-DRX(10,8,4) | C-DRX(16,5,4) | C-DRX(16,8,4) |
| --- | --- | --- | --- | --- | --- |
| Average number of supported users per cell | 5.1 | 0 | 4.0 | 0 | 0 |

# Conclusions

In this contribution, initial system level evaluation results of the scenario of FR1 Dense Urban and the scenario of FR1 Urban Macro for XR and CG service are discussed with the following observations:

Observation 1: As shown in Table 3, higher requirement of reliability and latency results in lower network capacity.

Table 3. Capacity of the DL single-stream model with different (PSR, PDB) in FR1 Dense Urban

| (PSR, PDB) | Average number of supported users per cell |
| --- | --- |
| (99, 7) | 2.3 |
| (99, 10) | 5.1 |
| (95, 13) | 6.8 |

Observation 2: Frame level integrated transmission (FLIT), which prioritize scheduling user whose packet is close to completion, can improve the network capacity about 21.6% as shown in Table 4.

Table 4. Capacity of the DL single-stream model of different transmission schemes in Dense Urban

| Transmission scheme | Option 2 | Option 3 (FLIT) |
| --- | --- | --- |
| Average number of supported users per cell | 5.1 | 6.2 |
| Capacity improvement of option 3 over option 2 | 21.6% |  |

Observation 3: In FR1 Dense Urban, the network capacity of the DL single-stream model is summarized in Table 5.

Table 5. Capacity of the DL single-stream model in FR1 Dense Urban

| Average number of supported users per cell | VR/AR @30Mbps (PSR=99%, PDB=10 ms) | VR/AR @45Mbps (PSR=99%, PDB=10 ms) | CG @8Mbps (PSR=99%, PDB=15 ms) | CG @30Mbps (PSR=99%, PDB=15 ms) |
| --- | --- | --- | --- | --- |
| SU-MIMO | 5.1 | 2.0 | >30 | 7.3 |
| MU-MIMO | 12.0 | 5.5 | >30 | 15.6 |

Observation 4: In FR1 Urban Macro, the network capacity of the DL single-stream model is summarized in Table 6.

Table 6. Capacity of the DL single-stream model in FR1 Urban Macro

| Average number of supported users per cell | VR/AR @30Mbps (PSR=99%, PDB=10 ms) | VR/AR @45Mbps (PSR=99%, PDB=10 ms) | CG @8Mbps (PSR=99%, PDB=15 ms) | CG @30Mbps (PSR=99%, PDB=15 ms) |
| --- | --- | --- | --- | --- |
| SU-MIMO | 4.3 | 1.6 | >30 | 6.5 |
| MU-MIMO | 9.5 | 4.0 | >30 | 11.5 |

Observation 5: In FR1 Dense Urban and Urban Macro, MU-MIMO can effectively increase the network capacity.

Observation 6: The network capacity of the DL multi-stream Option 1B decreases as the size ratio between I frame and P frame becomes larger.

Observation 7: The network capacity of the DL multi-stream Option 1B can be improved by prioritizing the transmission of the more important stream as shown in Table 7.

Table 7. Capacity of the DL multi-stream Option 1B in FR1 Dense Urban

| Scheduling scheme | S1 (no prioritization between I/P streams) | S2 (prioritize I-stream) |
| --- | --- | --- |
| Average number of supported users per cell | 3.6 | 5.8 |
| Capacity improvement of S2 over S1 | 61.1% |  |

Observation 8: For DL VR/AR@30Mbps in dense urban, the power saving gains are summarized in Table 9.

Table 9. Power saving gain for dense urban, DL VR/AR@30Mbps

| Power saving scheme | mean PS gain when all UEs are considered | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- |
| Baseline | - | 4.5/ 5 |
| C-DRX(10,5,2) | 9.76% | 0.9/ 5 |
| C-DRX(10,8,4) | 3.39% | 4.2/ 5 |
| C-DRX(16,5,4) | 28.50% | 0/ 5 |
| C-DRX(16,8,4) | 13.88% | 0/ 5 |

Observation 9: For DL VR/AR@30Mbps in dense urban, the network capacity of are summarized in Table 10.

Table 10. Capacity for dense urban, DL VR/AR@30Mbps

|  | Baseline | C-DRX(10,5,2) | C-DRX(10,8,4) | C-DRX(16,5,4) | C-DRX(16,8,4) |
| --- | --- | --- | --- | --- | --- |
| Average number of supported users per cell | 5.1 | 0 | 4.0 | 0 | 0 |

# References

Chairman notes, RAN1#104-e, February, 2021.

Chairman notes, RAN1#104b-e, April, 2021.

Chairman notes, RAN1#103-e, November, 2020.

R1-2104238, “Traffic model for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#105-e, May 10th – 27th, 2021

R1-2104239, “Evaluation methodology for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#105-e, May 10th – 27th, 2021

R1-2105522, “Challenges and potential enhancements for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#105-e, May 10th – 27th, 2021

# Appendix A: System Simulation Parameters

Table A-1. System level simulation assumption for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Dense Urban | Scenario-2: Urban Macro |
| Layout | 21cells with wraparound |  |
| Channel Model | UMa |  |
| Carrier frequency | 4.0 GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | [DDDSU DDDSU] Detailed S slot format is 10D:2F:2U. |  |
| Inter Site Distance | 200m | 500m |
| BS Antenna Height | 25m |  |
| BS Antennas | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |
| CSI-RS Port | 32 |  |
| UE Antenna Height | General equation: hUT=3(nfl – 1) + 1.5 - nfl for outdoor UEs: 1 - nfl for indoor UEs: nfl ~ uniform(1, Nfl ) where Nfl ~ uniform(4,8) |  |
| UE Antennas | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |  |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| TX Power | BS : 44 dBm per 20MHz | BS : 49 dBm per 20MHz |
| UE MAX Power | 23dBm |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | SU-MIMO Proportional Fair/MU-MIMO Proportional Fair |  |
| MCS | Up to 256QAM |  |
| UE distribution | 80% indoor, 20% outdoor |  |
| UE speed | 3 km/h |  |
| Channel Estimation | Realistic |  |
| Downtilt | 12 degree |  |
| CSI acquisition | CSI-RS: Period=5ms, Density=1, feedback delay=1ms, 4bit quantization, and modelling the error SRS: Period=5ms |  |
| Transmission scheme | Close loop rank adaptation |  |
| PHY processing delay | UE PDSCH processing Capability #1 |  |
| PDCCH overhead | 2 symbols |  |
| Target BLER | 10% |  |
| Max HARQ transmission | 4 |  |

Note 1: For frame structure, U symbol of S slot is not used for uplink transmission.