3GPP TSG RAN WG1 #105-e	R1-2105531

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on UCI enhancements and SPS

Document for:	Discussion and Decision

Introduction

This contribution discusses following issues:

- UCI enhancement: Conflict between the first PUCCH repetition and semi-static configuration

- UCI enhancement: Discussion of the same PUCCH resource for both CSI/SR and HARQ-ACK

  - Note: This issue was discussed in SPS enhancement during last meeting, but we think it is more related to UCI enhancement.

- SPS enhancement: SPS PDSCH release and SPS receptions with slot aggregation

- SPS enhancement: PUCCH-CSI-ResourceList where SPS HARQ-ACK multiplexed

Discussions

  - Remaining issues on UCI enhancements

Conflict between the first PUCCH repetition and semi-static configuration

In the RAN1 #104b-e meeting, for the issue on “Conflict between the first PUCCH repetition and semi-static configuration”, the following potential conclusion has been proposed [1]:

Potential conclusion: It is clarified that, according to the running R15 and R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For PUCCH repetition, it is beneficial for the UE implementation to have a unified behavior for the first and for other repetitions. For scheduled PUCCH repetition, we don’t see the need to define an exceptional rule. For a first scheduled PUCCH repetition, if all the candidate values in the K1 set point to a DL slot/sub-slot, then a collision occurs for the first repetition. In such case, it should defer to the next slot/sub-slot which contains an available PUCCH resource.

In the above potential conclusion, it is concluded that the PUCCH repetition should be postponed to the next available UL slot. The term “UL slot” means one slot consists of only UL symbols. But for the deferral of PUCCH repetition, it can be deferred to a slot consisting of flexible symbols or DL symbols, as long as all the symbols of the PUCCH resource for deferred PUCCH repetition are UL symbols. Thus, we propose to change “UL slot” to “slot” in the conclusion.

Based on the above discussion, we support the potential conclusion.

Proposal 1: Conclude that,

It is clarified that, according to the running R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Discussion of the same PUCCH resource for both CSI/SR and HARQ-ACK

During last meeting in the SPS enhancement thread, one discussion was whether the PUCCH resource configured in PUCCH-CSI-ResourceList is sub-slot based or not. Based on companies’ comments, it is a common understanding that the resource in PUCCH-CSI-ResourceList is slot-based. Therefore, the following TP was proposed to be adopted for TS 38.213, but not agreed.

TS 38.213 - 9.2.1 PUCCH Resource Sets <omit text unchanged> … If a UE is provided subslotLengthForPUCCH in a PUCCH-Config, the first symbol of a PUCCH resource in PUCCH-Config excluding PUCCH resource in multi-CSI-PUCCH-ResoruceList, pucch-CSI-ResourceList and SchedulingRequestResourceConfig, for multiplexing HARQ-ACK in a PUCCH transmission is relative to the first symbol of the subslotLengthForPUCCH symbols [12, TS 38.331]. For the remaining cases, the first symbol of a PUCCH resource is relative to the first symbol of a slot with Nsymslot symbols [4, TS 38.211]. … <omit text unchanged>

Regarding the TP, a concern that was raised is that it would prevent a PUCCH-Resource ID to be configured for both CSI and HARQ-ACK, which is allowed in Rel-15. We think it is still up to gNB’s implementation in Rel-16 to decide when sub-slot is configured. For example, if the startingSymbolIndex of one PUCCH resource is less than the subslotLengthForPUCCH and the PUCCH resource is confined within the sub-slot, such PUCCH resource can be configured for both CSI and HARQ-ACK. If the startingSymbolIndex of one PUCCH resource is equal to or larger than the subslotLengthForPUCCH or if one PUCCH resource is across the sub-slot boundary, such PUCCH resource can only be configured for CSI.

Based on the above discussion, we think whether to configure a PUCCH resource with the same PUCCH-Resource ID for both HARQ-ACK and CSI when sub-slot is configured can also be left for gNB’s implementation, and no spec change is needed.

Proposal 2: Conclude that

Whether to configure a PUCCH resource with the same PUCCH-Resource ID for both HARQ-ACK and CSI when sub-slot is configured can be left for gNB’s implementation, and no spec change is needed.

  - Remaining issues on SPS enhancements

Discussion of SPS PDSCH release and SPS receptions with slot aggregation

In the RAN1 #104b-e meeting, it has been discussed whether to support “the case that SPS release is received in a slot where SPS PDSCH is configured to be received for the SPS configuration corresponding to the SPS release if the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs” of single SPS configuration.

With the following conclusion it was agreed to not support this case:

Conclusion The following is not supported: - The case that SPS release is received in a slot where SPS PDSCH is configured to be received for the SPS configuration corresponding to the SPS release if the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs.

Another but still open issue about SPS PDSCH release and SPS receptions is how to handle SPS release when SPS PDSCH reception is configured with repetition. It could not be discussed during the RAN1#104b-e meeting due to lack of time.

If repetition is configured for SPS PDSCH, different UE behaviors can be discussed.

For UE behavior 1, which is shown in Figure 1 below, the UE can receive SPS release freely in the slot that does not include last occasion of SPS PDSCH and in the slot that includes the last occasion of SPS PDSCH, the UE can receive SPS release only before the end of the SPS PDSCH reception. However, if the UE receives SPS release in a slot other than the first slot, it will drop previous receptions and clean the HARQ process, which is waste of UE power both from the perspective of SPS release DCI monitoring and SPS PDSCH reception.

Figure 1. UE behaviour 1 with 1 slot periodicity and 4 slot aggregation.

For UE behaviour 2, as shown in Figure 2 below, the UE can receive SPS release only before the end of the reception of the first SPS occasion among all the repetitions.

Figure 2. UE behaviour 2 with 1 slot periodicity and 4 slot aggregation.

Between UE behaviour 1 and 2, we prefer behaviour 2. It is simpler for the UE implementation, consumes less power and there is no use case that would benefit from sending the release DCI later than the first repetition.

In a previous meeting, one concern for behaviour 1 was the joint release. In our view, the system is not broken, since separate releases can be used instead or the gNB can handle it by proper configuration for different SPS configurations.

Proposal 3: If repetition is configured for SPS PDSCH, the UE can receive SPS release only before the end of the SPS PDSCH in the first slot among all the repetitions and the timeline for single-slot SPS PDSCH is applied in the slot.

PUCCH-CSI-ResourceList where SPS HARQ-ACK multiplexed

In the RAN1 #104b-e meeting, for the issue on “PUCCH-CSI-ResourceList where SPS HARQ-ACK multiplexed”, it was concluded that the sub-slot for SPS HARQ-ACK transmission during the UL multiplexing procedure should not be changed. The following proposal was reached.

Latest Proposal 3-1: For the multiplexing among overlapping channels with same a given priority index, if a UE is provided subslotLengthForPUCCH for the HARQ-ACK codebook of the given priority index, UE does not expect that the HARQ-ACK corresponding only to SPS PDSCH(s) in one sub-slot is moved to a different sub-slot after multiplexing. - Note: It is up to the editor to decide whether/how to capture the proposal in the spec if agreed.

For scheduled PDSCH reception, the sub-slot for HARQ-ACK transmission is indicated by the K1 contained in the corresponding DCI even after UL multiplexing. For HARQ-ACK transmission corresponding only to SPS PDSCH(s), it is natural to obey the same rule, i.e. that the HARQ-ACK transmission should always be sent in the original sub-slot. There is no need to make changes in the specification. We propose the following conclusion:

Proposal 4: Conclude that,

For the multiplexing among overlapping channels with a given priority index, if a UE is provided subslotLengthForPUCCH for the HARQ-ACK codebook of the given priority index, UE does not expect that the HARQ-ACK corresponding only to SPS PDSCH(s) in one sub-slot is moved to a different sub-slot after multiplexing.

Conclusions

Based on the above discussions, the following proposals are given:

For Conflict between the first PUCCH repetition and semi-static configuration

Proposal 1: Conclude that,

It is clarified that, according to the running R16 specification, a PUCCH repetition (including the first PUCCH repetition) is postponed to the next available UL slot if the PUCCH repetition collides with SSB symbols or symbols indicated as DL by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

For Discussion of the same PUCCH resource for both CSI/SR and HARQ-ACK

Proposal 2: Conclude that

Whether to configure a PUCCH resource with the same PUCCH-Resource ID for both HARQ-ACK and CSI when sub-slot is configured can be left for gNB’s implementation, and no spec change is needed.

For Discussion of SPS PDSCH release and SPS receptions with slot aggregation:

Proposal 3: If repetition is configured for SPS PDSCH, the UE can receive SPS release only before the end of the SPS PDSCH in the first slot among all the repetitions and the timeline for single-slot SPS PDSCH is applied in the slot.

For PUCCH-CSI-ResourceList where SPS HARQ-ACK multiplexed

Proposal 4: Conclude that,

For the multiplexing among overlapping channels with a given priority index, if a UE is provided subslotLengthForPUCCH for the HARQ-ACK codebook of the given priority index, UE does not expect that the HARQ-ACK corresponding only to SPS PDSCH(s) in one sub-slot is moved to a different sub-slot after multiplexing.

- References

- R1-2103867, Summary#1 of email thread [104b-e-NR-L1enh-URLLC-02],	Moderator (OPPO), RAN1#104b-e

- R1-2104126, Final Summary of [104b-e-NR-L1enh-URLLC-05] on other aspects for URLLC and IIoT,	Moderator (LG Electronics), RAN1#104b-e