3GPP TSG RAN WG1 #105-e                                          	       R1-2105563

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.5.3

Source:	Xiaomi

Title:                     Accuracy improvements for DL-AoD positioning solutions

Document for:	Discussion and Decision

Introduction

In RAN-91 meeting, the work item of NR Positioning enhancements [1] was approved. The detailed specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]:

  - DL NR positioning methods and RAT-independent positioning methods

    - Support of UE positioning measurements for UEs in RRC_INACTIVE state

    - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state

Note: this work will be coordinated with the SDT WI.

  - As 2nd priority:

    - UL and DL+UL NR positioning methods

    - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

There are some agreements in RAN1-104e meeting [2].

Agreement:

- For both UE-based and UE-assisted DL-AOD study the following enhancements that enable the UE to measure and report (for UE-assisted) information related to the first arriving path

  - Option 1: Information corresponds to PRS-RSRP of the first arriving path

  - Option 2: Information corresponds to the angle of departure of the first arriving path

  - Option 3: Information corresponds to the arrival time of the first path

  - Option 4: Information corresponds to phase of the CIR corresponding to the first arriving path

  - Option 5: Information corresponds to received signal value (amplitude and phase of the channel estimated from the first path which can be achieved as a combination of option 1 and option 4) of the first arriving path

- FFS: Reporting of additional path to the first arriving path.

- FFS: Measurement definition details

- FFS: additional assistance data to support these enhancements

- FFS: how the “first path” is selected among PRS resources in a PRS resource set

- Note 1: Supporting multiple options as well as none of the options above is not precluded.

There are some agreements in RAN1-104b e-meeting [3].

Agreement:

Regarding support of angle calculation enhancement for DL-AoD:

- Support gNB providing the beam/antenna information to the LMF.

  - The gNB beam/antenna information can be provided to the UE for UE-based DL-AoD

  - FFS: the details of contents of the beam/antenna information

  - FFS: the details of how to provide the beam/antenna information.

  - Note: The antenna information is related to reducing the overhead of beam information

- Send an LS to RAN2/RAN3 regarding the option of angle report from gNB to LMF for UE-A DL-AoD requesting them to consider this option in Rel-17.

Agreement:

Support the following enhancements under UE capability for both UE-B and UE-A DL-AOD positioning method

- Enhancing the signaling to UE for the purpose of PRS resource(s) measurement and (for UE-A) report

  - FFS: The detailed signaling (e.g, the boresight direction for UE-A DL-AoD, further spatial information of PRS resources, processing prioritization of PRS resources)

- FFS: The following options

  - Option 1: Enhancing the reporting to include the measurements of adjacent beams PRS resources that related with each other indicated by the assistance data.

  - Option 2: UE can be requested to measure and report on specific PRS resources

Agreement:

- For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options:

  - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

    - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP]

  - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

    - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP]

  - Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced.

  - FFS: details of signaling

- FFS: Applicability of this agreement to other Positioning methods

In this contribution, we want to discuss about accuracy improvements for DL-AoD positioning solutions.

Improvements for DL-AoD positioning solutions

Report DL TDoA and DL PRS-RSRP for DL AoD

Rel-16 defines the DL PRS-RSRP, DL RSTD and the UE Rx-Tx time difference measurements. And different positioning solutions depends on different measurements. For TDOA and RTT, which are time difference based approaches, and their measurements are based on first detected path in time. However, for AoD, it depends on PRS RSRP measurements and the angle of departure. In fact, the PRS RSRP equals to the total received power of the PRS. The total received power may contain the power of one or more paths and each path related to different transmission beams at TRP side.  If the total received power contains the power of more paths, which includes the first path, the second path, and the third path and so on, in this case when non-first paths are received with higher power than that of the first path, the estimated result for angle of departure would be biased by these non-first paths. Which means that the estimated result for angle of departure is more likely to be the angle of the non-first path. In order to solve this problem, it is beneficial to estimate the first path by DL-TDOA firstly and then estimated the angle of the first path. So as for the enhancement on measurement and report for DL AoD, it is better to report the DL TDoA together with the DL PRS-RSRP of the first path. Based on the measurement report of the first path, the angle can be estimated with much higher accuracy.

Proposal 1: Report DL TDoA together with DL PRS-RSRP for DL AoD.

Beam management for DL AoD

In order to perform positioning PRS measurement with short latency and high accuracy, it is better to perform beam management before positioning PRS configuration. In order to reduce the PRS signaling overhead and the PRS measurement time, the PRS can be configured according to the beam measurement report. For example, only a number of PRSs with higher DL PRS-RSRP or with earlier arrival time should be configured with different beam direction. Thus the number of PRS to be measured will be reduced, which can reduce the PRS resource overhead and the PRS measurement time. In addition, with a limited number of beams, the range of the angle can be estimated roughly at the first time.

From another aspect, in order to improve the positioning accuracy, it is very important to find the LOS path or the first arrival path. Since only the first arrival path can indicate the accurate distance between the UE and the TRP. But during beam management procedure, it is possible that the beam with the strongest L1-RSRP/L1-SINR is not the one refer to the first arrival path. Thus the beam refer to the first arrival path may not be included in the beam measurement report. If gNB configures PRS resources based on beam measurement report, there may be no PRS resource with beam refer to the first arrival path. That will decrease the positioning accuracy. If gNB configure more PRS resources, e.g., based on gNB Tx sweeping, the signaling overhead and the latency for positioning measurement will be increased. In order to solve this problem, we suggest to find the first arrival path during beam management procedure and report to the gNB. For example, the arrival time of each beam can be included in the beam measurement report. Based on the enhanced beam measurement report, gNB can configure PRS resources with earlier arrival time. Thus the latency for positioning measurement will be reduced.

Proposal 2: To indicate the first arrival path by reporting the arrival time of each beam in beam measurement report.

As for beam management of neighboring cell, in addition to UE based beam measurement, gNB based beam measurement should also be supported for neighboring cell beam management. For UE based beam measurement, it is preferred to configure cell specific reference signal for beam management of neighboring cell, e.g., SSB. In this case, the signaling overhead in neighboring cell and beam management latency can be reduced. And it is also better to configure a cell specific reference signal with smaller transmission period. While for neighboring gNB based beam measurement, UE need to transmit the beam measurement reference signal, such as SRS. It is preferred to reuse the SRS for other use cases if possible, for example, reuse beam management SRS for serving cell. In this case, there will be no more impact on UE side. As same as UE based beam measurement, it is better to consider the arrival time of the first path for each beam when performing beam measurement at neighboring gNB side.

Information related to the first arriving path

There are five Options agreed in RAN1-104e meeting. For Option 2, we are still wondering how does UE to obtain the angle of departure of the first arriving path? In our understanding, it is the knowledge of gNB. For Option 4, it is not clear for what unit in frequency domain the phase is? Is it for the whole bandwidth or for each subband? For Option 5, why to list an option which can be obtained by the combination of other two options? Thus we prefer Option 1 and Option 3.

Proposal 3: Prefer Option 1 and Option 3 on information related to the first arriving path.

Adjacent PRS resources measurement/report

As discussed in RAN1-104e meeting, the only problem is how to provide the information on which PRS resources are adjacent PRS resources. From our point of view, the straight forward way to solve this problem is to predefine the adjacent PRS resources by resource index. With this method, no additional signaling is needed to provide this information.

Proposal 4: Adjacent PRS resources can be predefined by resource index.

Angle error for DL AoD

DL AoD depends on PRS RSRP measurements and the angle of departure. The angle of departure is the angle related to the gNB’s transmission beam of the DL PRS. But during the transmission, each gNB may introduce an uncertain error in their beam based transmission. With the estimated angle of departure based on the beam error, it is difficult to get a location with higher accuracy.

In order to improve the accuracy of DL AoD, it is necessary to estimate the angle error of the transmission beam and cancel it. Since different DL PRS is configured with different transmission beam, and the gNB needs to change the transmission beam over time. Thus the angle error with different transmission beam may be different and varied over time. It is difficult to estimate the angle error of each beam with short latency. One possible way is to use a reference node, whose accurate location is known. The reference node will perform the same measurement on DL PRS and feedback the measurement results together with its accurate location to the LMF. The LMF can calculate the estimated location of the reference node, and obtain the angle error based on the comparison of the estimated location and the accurate location of the reference node. Then, during the location estimation of required UE, LMF can calculate an accurate angle of departure by canceling the angle error.

Proposal 5: Estimate the angle error by a reference node whose accurate location is known.

Uncertainty window indication for DL AoD

In order to reduce the measurement time and improve the accuracy, similar to UL AoA, an expected uncertainty window for DL AoD was discussed in RAN1-104b e-meeting. First we think it is necessary to indicate of expected uncertainty window for DL AoD in the case of no beam measurement report. With the indication of expected uncertainty window, UE can only measure the PRS resource within a limited receiving angle window, which can save UE’s power consumption and PRS measurement time. Then for Option 1 and Option 2, if it is a LoS path, there is no difference for Option 1 and Option 2. But for NLoS Path, without beam measurement report, gNB does not know DL AoA. Thus gNB can only provide DL AoD to UE through LMF. But UE does not know how to use this information without beam measurement. Thus we slightly prefer Option 1.

Proposal 6: Slightly prefer Option 1 for LoS path.

- Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE.

Conclusion

In this contribution, we discussed about accuracy improvements for DL-AoD positioning solutions. Based on above discussion, we provide the following proposals.

Proposal 1: Report DL TDoA together with DL PRS-RSRP for DL AoD.

Proposal 2: To indicate the first arrival path by reporting the arrival time of each beam in beam measurement report.

Proposal 3: Prefer Option 1 and Option 3 on information related to the first arriving path.

Proposal 4: Adjacent PRS resources can be predefined by resource index.

Proposal 5: Estimate the angle error by a reference node whose accurate location is known.

Proposal 6: Slightly prefer Option 1 for LoS path.

- Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE.

References

- RP-210903 Revised WID on NR Positioning Enhancements

- 3GPP RAN1-104e meeting chairman notes

- 3GPP RAN1-104b e-meeting chairman notes