3GPP TSG RAN WG1 #105-e                                          	       R1-2105566

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.5.6

Source:	Xiaomi

Title:                     On-demand PRS and positioning for in-active state UE

Document for:	Discussion and Decision

Introduction

In RAN-91 meeting, the work item of NR Positioning enhancements [1] was approved. The detailed specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]:

  - DL NR positioning methods and RAT-independent positioning methods

    - Support of UE positioning measurements for UEs in RRC_INACTIVE state

    - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state

Note: this work will be coordinated with the SDT WI.

  - As 2nd priority:

    - UL and DL+UL NR positioning methods

    - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]:

  - The assistance information that will be used to support integrity determination

  - The information that will be used to provide the positioning integrity KPIs and integrity results

  - Support of integrity for UE-based and UE-assisted A-GNSS positioning.

Note: This objective is applicable to NR and E-UTRA.

- Support the following enhancements of A-GNSS positioning  [RAN2, RAN3, RAN4]

  - Specify support for BDS B2a signal

  - Specify support for BDS B3I signal

  - Specify support for NavIC to NR

Note: This objective is applicable to NR and E-UTRA.

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

- Discuss and specify new as well as the impact on the existing RAN4 requirements for positioning and other RRM measurements and corresponding procedures [RAN4]

- Define extensions of signalling, protocol and procedure for NR positioning enhancement, as needed [RAN3]

In this contribution, we focus on on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions.

On-demand PRS

Initiation of on-demand PRS transmission and reception

In order to improve the network efficiency and reduce the latency, on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE based and UE-assisted positioning solutions will be specified in Rel-17. Different type of on-demand DL PRS should be included for different scenarios, i.e., periodical PRS, Semi-persistent PRS and aperiodic PRS.

Proposal 1: on-demand PRS should support periodical transmission, semi-persistent transmission and aperiodic transmission.

As for the initiation of on-demand PRS transmission, both UE initiated request and LMF (network) initiated request will be specified. For the case of UE initiated, two procedures need to be considered. One is UE initiated request of on-demand DL PRS transmission to LMF, which belongs to RAN2’s work scope. The other one is UE initiated request of on-demand DL PRS transmission to gNB, and some mechanism should be discussed in RAN1. In this contribution, we will mainly focus on the second procedure and discuss on how to initiate the request of on-demand DL PRS transmission.

As for mechanism of UE initiated request to gNB, there are two options.

- Option 1-1: indicate the on-demand PRS transmission request by transmission of random access preamble on PRACH.

  - Both contention free based random access and contention free based random access should be considered. And also both 2-step RACH and 4-step RACH can be used.

  - With this option, UE can further indicate the recommending PRS configuration parameter set ID by PUSCH in Msg 3 or Msg A.

- Option 1-2: indicate the on-demand PRS transmission request by transmission of SR on PUCCH.

  - With this option, UE can further indicate the recommending PRS configuration parameter set ID by PUSCH scheduled by gNB responding to the SR.

While the mapping between PRS configuration parameter set ID and related PRS configuration parameter set need to be known by UE in advance, which can be transmitted to the UE by LMF or serving gNB. And the PRS configuration parameter can include the bandwidth, comsize, number of symbols, periodicity and repetition factor (if periodical PRS and semi-persistent transmission is requested).

But based on the request from UE to gNB, gNB further need to send request to LMF for indicating PRS resource of neighboring TRPs. From this perspective, the latency will be not reduced a lot. Thus we can take it as low priority.

For the DL PRS transmission request initiated by LMF and gNB, here we will focus on the request initiated by gNB. As for the case of initiated by gNB, different type of DL PRS transmission can be initiated by different mechanism.

As for mechanism of gNB initiated request to UE, there are three options.

- Option 2-1: indicate the on-demand DL PRS transmission by RRC signaling.

  - This can be applied for periodical on-demand DL PRS transmission. Configuration parameters for periodical on-demand DL PRS transmission can be configured in RRC signaling.

- Option 2-2: indicate the on-demand DL PRS transmission by MAC CE.

  - This can be applied for semi-persistent on-demand DL PRS transmission. Multiple configuration parameter sets for semi-persistent on-demand DL PRS transmission can be configured in RRC signaling and one of them will be activated by MAC CE.

- Option 2-3: indicate the on-demand DL PRS transmission by DCI.

  - This can be applied for semi-persistent and aperiodic on-demand DL PRS transmission. Multiple configuration parameter sets for semi-persistent and aperiodic on-demand DL PRS transmission can be configured in RRC signaling and one of them will be activated by DCI. If the number of configuration parameter sets is larger than the number of codepoints in DCI, MAC CE can be used to activate some of configuration parameter sets configured in RRC signaling.

As the same time, gNB sends triggering indication of on-demand DL PRS transmission to neighboring gNB directly or through LMF.  After above procedure, measurement for positioning can be started.

Proposal 2: gNB initiated of on-demand PRS transmission can be supported by RRC, MAC CE and DCI.

PRS measurement report transmission

After PRS measurement, UE need to report the measurement results. In order to reduce the latency, we suggest that UE sends measurement report to gNB. Because of the higher overhead of the measurement report, it is better to send it by PUSCH. Both configured grant PUSCH and DCI scheduled PUSCH can be supported. For example, periodical PRS measurement report can be sent by configured grant PUSCH. While semi-persistent and aperiodic PRS measurement report can be sent by DCI scheduled PUSCH. And configured grant PUSCH can be triggered by RRC only or RRC+DCI. While for DCI triggered PRS measurement report, the configuration parameters of PRS can be indicated by the same DCI.

Proposal 3: Support PRS measurement report by PUSCH including configured grant PUSCH and dynamic grant PUSCH.

Positioning for UE in Inactive state

In Rel-16 positioning, most discussions focus on connected UE. But there are some cases in which positioning for idle/inactive UE is necessary too. For example the IoT device, such as children’s watch with tracking function. In this scenario, periodic positioning for idle UE is needed. If only positioning for connected UE is supported, the UE needs to switch between idle and connected mode frequently. That will introduce high signaling overhead and UE power consumption. Thus positioning for inactive UE should be supported.

For inactive UE, there is no LPP message between UE and LMF. If the UE assisted DL positioning is initiated, inactive UE at least need to provide the PRS measurement report to gNB or LMF. According to the Rel-17 small data WI, the small data can be transmitted during the RACH procedure. So a straightforward way for inactive UE reporting the PRS measurement report is using PUSCH in Msg 3 or Msg A during RACH procedure.

Proposal 4: Measurement report can be sent to gNB by PUSCH in Msg 3 or Msg A during random access procedure for inactive UE.

Currently, the PRS configuration was provided to UE by a LPP message from LMF or by the positioning system information broadcasted by gNB for specific positioning methods. However, for LPP message, the inactive UE can’t directly receive the LPP message. Considering the PRS configuration does not change frequently, the PRS configuration could be pre-configured when the UE is in connected state. For example, the PRS configuration can be provided to UE by LPP message from LMF or by RRC/MAC CE/ DCI from gNB when UE is in connected state and then UE can apply the PRS configuration when UE transmits to inactive state.

Proposal 5: Consider to pre-configure the PRS for inactive UE when UE is in connected mode.

In order to support UL and DL&UL positioning methods for inactive UE, RACH procedure can be reused. And preamble can be used as UL positioning reference signal. Considering the PRACH resource and preamble for random access usage and positioning usage, it is possible to configure both common PRACH resource/preamble and dedicated PRACH resource/preamble for positioning usage. In order to support DL&UL positioning method, PUSCH in Msg.3 of 4-step RACH and PUSCH in Msg. A of 2-step RACH can be used to indicate the time difference between reference signal reception and transmission at UE side. However, there is one problem for inactive UE. Since multi-TRP is required to support positioning, it is necessary to send independent preamble to multi-TRPs for beam specific UL positioning reference signal. In order to reduce the positioning latency and improve the positioning accuracy, it is better to send more than one preamble to each TRP with different beam. And the beam is same as the detected SSB from each TRP. But if more than one preamble is sent to a TRP, it may introduce impacts on efficiency of random access and high UE power consumption. Thus we propose to limit the number of SSBs referring to which the preamble is sent in each TRP. However, it is easy to differentiate TRPs from different cell. But if there are more than one TRP in a cell, it is difficult to differentiate the SSBs from different TRPs.  From our point of view, it is better to indicate UE the TRP index of each SSB. It can be indicated by both explicit and implicit signaling.

Proposal 6: Random access procedure can be reused for UL and DL&UL positioning of Inactive UE.

Proposal 7: Random access preamble can be reused as UL reference signal for Inactive UE.

In addition, in order for the accuracy, it is better if SRS can be transmitted for UL and DL&UL positioning methods for inactive UE. Similar to DL PRS, the configuration of SRS can also be based on the pre-configuration when UE in connected mode. And the SRS transmission can be triggered by gNB through paging.

Proposal 8: SRS transmission for inactive UE can be triggered by gNB through paging.

Conclusion

In this contribution, we discussed about on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions. Based on above discussion, we provide the following proposals.

Proposal 1: on-demand PRS should support periodical transmission, semi-persistent transmission and aperiodic transmission.

Proposal 2: gNB initiated of on-demand PRS transmission can be supported by RRC, MAC CE and DCI.

Proposal 3: Support PRS measurement report by PUSCH including configured grant PUSCH and dynamic grant PUSCH.

Proposal 4: Measurement report can be sent to gNB by PUSCH in Msg 3 or Msg A during random access procedure for inactive UE.

Proposal 5: Consider to pre-configure the PRS for inactive UE when UE is in connected mode.

Proposal 6: Random access procedure can be reused for UL and DL&UL positioning of Inactive UE.

Proposal 7: Random access preamble can be reused as UL reference signal for Inactive UE.

Proposal 8: SRS transmission for inactive UE can be triggered by gNB through paging.

References

- RP-210903 Revised WID on NR Positioning Enhancements