3GPP TSG-RAN WG1#105-e			R1-2105589

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.1.2.1

Title:	Enhancements on Multi-TRP for PUCCH and PUSCH

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Rel-16 eMIMO introduced enhancements for multi-TRP transmission of PDSCH. Rel-17 FeMIMO WI will extend the multi-TRP support to PDCCH, PUCCH and PUSCH [1].

- Extend specification support in the following areas [RAN1] - Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline

# Discussion

## 2.1 PUCCH

In RAN1#104b-e, the following was agreed.

Agreement For the case of multi-TRP, to support per-TRP power control in FR1, the linking of PUCCH resource with [one or] two power control parameter sets, the following is supported - MAC-CE indicates RRC IE that configures power control parameter sets (p0, pathloss RS ID, and a closed-loop index). - The exact design of RRC IE is up to RAN2 but from RAN1 point of view, one possible example is to reuse PUCCH-SpatialRelationInfo except for the referenceSignal Note: It is common understanding in RAN1 that one PUCCH resource can be linked to one power control parameter set. Conclusion With reference to the normative work on NR-feMIMO: Related to the support of switching gap between UL transmissions towards two TRPs in RAN1 specifications, there is no consensus in RAN1 to specify symbol gap(s) for the following cases - PUSCH Type A - PUCCH scheme 1 - PUSCH Type B - PUCCH scheme 3 The above applies for the case included in the LS from RAN4 in R1-2102297. Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact). Agreement Confirm the following Working Assumption: For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions. - FFS: Applicability of mapping patterns for different beam switching gaps - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows, - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions. - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions. Agreement Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED): - For beam mapping /power control parameter set mapping for PUCCH repetitions, - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions). - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

Power control is important for PUCCH performance. If the dynamics of per-TRP closed loop power control is worse than for single-TRP, some of the multi-TRP gains may be lost. Therefore, we support a duplication of the Rel-15/16 closed loop power control for the second TRP, i.e. a second TPC field is added in DCI formats 1_1 / 1_2.

Proposal 1: Support Option 3: A second TPC field is added in DCI formats 1_1 / 1_2.

Regarding frequency hopping for scheme 1, the question seems to be if “TRP/beam hopping” prior to frequency hopping should be supported, i.e. frequency hopping is performed among the repetitions with the same beam for cyclical mapping pattern. In the case of blockage of one TRP, frequency hopping may be of little help, since the blockage affects a wide band. In this case, lower PUCCH detection latency could be achieved by first performing TRP/beam switching (i.e. cyclical mapping) and secondly frequency hopping.

Proposal 2: Support Option 1: frequency hopping is performed among the repetitions with the same beam.

PUCCH repetition scheme 1 has been agreed. Given that scheme 3 is a working assumption, there seems to be no need to also support scheme 2, which is quite similar.

Proposal 3: Do not support PUCCH repetition scheme 2 in Rel-17.

## 2.2 PUSCH

In RAN1#104b-e, the following was agreed:

Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting Working Assumption For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation - FFS: Whether the new field is 1 bit or 2 bits

The PUSCH transmissions to different TRPs may operate with separate power control. They may also be transmitted from different panels with potentially different power constraints. Hence, it’s meaningful to calculate two PHRs, each corresponding to a TRP, and to report those values to the gNB, in order to facilitate the UL power control and scheduling.

Proposal 4: Support Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs.

A new DCI field will be introduced to dynamically indicate at least S-TRP or M-TRP operation (working assumption). If S-TRP operation is indicated, some of the other fields may be unused, e.g. the second SRI field, and may be used for TRP indication. Hence, it’s sufficient with 1 bit.

Proposal 5: The new field in DCI to indicate at least the S-TRP or M-TRP operation is 1 bit.

# Conclusion

In this contribution, we considered aspects of multi-TRP PUCCH/PUSCH enhancement. We have the following proposals:

Proposal 1: Support Option 3: A second TPC field is added in DCI formats 1_1 / 1_2.

Proposal 2: Support Option 1: frequency hopping is performed among the repetitions with the same beam.

Proposal 3: Do not support PUCCH repetition scheme 2 in Rel-17.

Proposal 4: Support Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs.

Proposal 5: The new field in DCI to indicate at least the S-TRP or M-TRP operation is 1 bit.

# References

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, Samsung, RAN#89e, Sep. 2020