3GPP TSG RAN WG1 #105-e			R1-2105600

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.12.1 Mechanisms to support group scheduling for RRC_CONNECTED UEs

Title:	Discussion on group scheduling mechanism for RRC_CONNECTED UEs

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Agreement:

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

Agreement:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- l  Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- l  Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- l  Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Agreement:

The down-selection of Option 2A and Option 2B for CFR for multicast of RRC-CONNECTED UEs will be made before the end of RAN1#105-e.

Agreement:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

In the RAN1 #104b-e meeting, the following agreements were reached on MBS group scheduling mechanism [1]:

In this contribution, we provide our views on the potential group scheduling mechanism to support the NR multicast and broadcast.

# Discussion

## 2.1 Dynamic scheduling for MBS

In the past RAN1 meetings, it has been agreed that PTM transmission scheme 1 is supported for the initial transmission. When PTM transmission scheme 1 is used for initial transmission, both PTM transmission scheme 1 and PTP transmission are supported for retransmission. Using group-common PDCCH, i.e., PTM transmission scheme 1, has the advantage of low signaling overhead when the number of the UEs to be scheduled is large. However, as another option, using UE-specific PDCCH to schedule the PDSCH for MBS, e.g., using PTP transmission or PTM transmission scheme 2, may be beneficial in some cases, e.g., when the number of the UEs to be scheduled is small, for the purpose of improving the robustness of the DCI transmission. Therefore, we propose PTP transmission and PTM transmission scheme 2 should be supported for initial transmission for MBS. Also, PTM transmission scheme 2 should be supported for retransmission for MBS.

Proposal 1: PTP transmission and PTM transmission scheme 2 should be supported for initial transmission for MBS.

Proposal 2: PTM transmission scheme 2 should be supported for retransmission for MBS.

As UE-specific PDCCH with CRC scrambled by C-RNTI is also used for scheduling the PDSCH for unicast service, when a UE receives a UE-specific PDCCH, the UE needs to distinguish which type of PDSCH, i.e., MBS PDSCH or unicast PDSCH, that the DCI is scheduling. Therefore, 1-bit field needs to be introduced in the DCI for the UE to distinguish between these two types, namely the UE-specific PDCCH scheduling the MBS PDSCH and UE-specific PDCCH scheduling the unicast PDSCH.

Proposal 3: 1-bit field is introduced in the DCI format for the UE to distinguish between the UE-specific PDCCH scheduling the MBS PDSCH and scheduling the unicast PDSCH.

In the MBS WID [2], one of the scopes is that “Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception.”

In current RAN1 discussion, common frequency resource (CFR) is define for both the UEs in RRC_IDLE/ RRC_INACTIVE states and the UE in RRC_CONNECTED state. With the guidance of the WID and to reduce the standardization efforts, we prefer to avoid the solutions that could cause separate designs for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs. For example, adopting option 2A for RRC_CONNECTED UEs while adopting option 2B for RRC_IDLE/INACTIVE UEs is not desirable. Unified CFR design should be used for MBS for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs.

Proposal 4: Unified CFR design should be used for MBS for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs.

Another issue that was heavily discussed in past RAN1 meetings is whether the dedicated MBS BWP (option 2A) or the common frequency resource confined within UEs’ active BWPs (option 2B) should be used as the common frequency resource (CFR) for the MBS operation for the RRC_CONNECTED UEs. Similar problem is also under discussion for the MBS operation for the RRC_IDLE/INACTIVE UEs.

For the option 2B, the common frequency resource is confined within UEs’ active BWPs and the intersection of the UEs’ active BWPs will be used for NR MBS. It is obvious that this approach works only when such common frequency resource exists. However, in NR, different UEs can be configured and activated with different BWPs. Also, different numerologies may be used for different BWPs. In a practical system, such common frequency resource may not always exist unless the network forces it to happen which could result in restriction to the system. Therefore, it is not desirable to select an approach that cannot be fulfilled in all the MBS scenarios, especially when we are designing the very basic and fundamental function of the MBS. Also, option 2B doesn’t work for the RRC_IDLE/INACTIVE UEs when a CFR wider than the initial BPW is needed as it can be only equal to or narrower than the associated unicast BWP.

Option 2A uses the dedicated MBS BWP for the MBS operation. In this approach, the network has the full flexibility to configure and activate the MBS BWP used by the UEs to receive the MBS. For example, when the common frequency resource confined within UEs’ active BWPs exists, the network may configure it as the MBS BWP and utilize the advantages it offers. When such common frequency resource does not exist, the network can still configure and activate some other frequency band as the MBS BWP to make the system work. From our view, the previous approach is a subset of the dedicated MBS BWP approach, where the dedicated MBS BWP approach can address all the different use cases.

During the past meetings, some companies raised that BWP switching is needed between the MBS specific BWP and the dedicated unicast BWP even when the MBS specific BWP is confined within the dedicated unicast BWP, which will introduce delay to the system. In our view, BWP switching is not needed as the UE does not need to retune its RF chain to receive the MBS and such argument does not hold. Therefore, we propose that dedicated MBS BWP (option 2A) should be supported for RRC_CONNECTED UEs in NR MBS.

Proposal 5: Dedicated MBS BWP (option 2A) should be supported for RRC_CONNECTED UEs and RRC_IDLE/INACTIVE UEs in NR MBS.

In NR, CORESET is introduced to reduce the UE’s blind decoding efforts. For NR MBS, CORESET also needs to be defined. As the BWP for unicast and the CFR for MBS are likely to occupy different frequency range, the CORESET configured for unicast BWP may not be valid for MBS. Therefore, dedicated CORESET(s) should be configured for NR MBS in addition to the existing CORESETs in UE dedicated BWP.

Proposal 6: Dedicated CORESET(s) should be configured for NR MBS in addition to the existing CORESETs in UE dedicated BWP.

## 2.2 Semi-persistent scheduling for MBS

SPS scheduling has been agreed to be supported for NR MBS and group-common PDCCH has been supported for the activation and deactivation DCI, while UE-specific PDCCH is still under discussion. Similar to the dynamic scheduling, in our view, we think both options have its own pros and cons and can be beneficial for some certain scenarios. Therefore, UE-specific PDCCH should be also supported for activation/deactivation DCI for MBS SPS.

Proposal 7: UE-specific PDCCH should be supported for activation/deactivation DCI for MBS SPS.

For retransmission of an MBS SPS PDSCH transmission, PTM transmission scheme 1, and PTP transmission have been agreed to be supported. Similar to dynamic scheduling, in some use cases, PTM transmission scheme 2 may be beneficial. Therefore, PTM transmission scheme 2 should be considered for the MBS SPS PDSCH retransmission.

Proposal 8: PTM transmission scheme 2 should be considered for the MBS SPS PDSCH retransmission.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: PTP transmission and PTM transmission scheme 2 should be supported for initial transmission for MBS.

Proposal 2: PTM transmission scheme 2 should be supported for retransmission for MBS.

Proposal 3: 1-bit field is introduced in the DCI format for the UE to distinguish between the UE-specific PDCCH scheduling the MBS PDSCH and scheduling the unicast PDSCH.

Proposal 4: Unified CFR design should be used for MBS for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs.

Proposal 5: Dedicated MBS BWP (option 2A) should be supported for RRC_CONNECTED UEs and RRC_IDLE/INACTIVE UEs in NR MBS.

Proposal 6: Dedicate CORESET(s) should be configured for NR MBS in addition to the existing CORESETs in UE dedicated BWP.

Proposal 7: UE-specific PDCCH should be supported for activation/deactivation DCI for MBS SPS.

Proposal 8: PTM transmission scheme 2 should be considered for the MBS SPS PDSCH retransmission.

# References

- Chairman Notes, RAN1 #104b-e, April 12th – 20th, 2021

- RP-201038, Revised New Work Item on NR Multicast and Broadcast Services